3GPP TSG-RAN WG2 Meeting #116-e		R2-2109694

Online, 1st -12th November 2021

Source:	CATT

Title:	Consideration on NR and MR-DC MG enhancements

Agenda Item:	8.22

Document for:	Discussion and Decision

Introduction

In order to optimize the efficiency of RRM functionalities on both network and UE sides, the WID on NR and MR-DC measurement gap enhancements [1] are approved in Rel-17. RAN4 sent their agreements on concurrent MG [2] and pre-configured MG [3]. In this contribution, we discuss the possible impacts based on the two LSs.

Discussion

- Concurrent MG

With additional features and enhancements in Rel-16 NR, in addition to the traditional SSB based measurement, more measurements (such as CSI-RS, PRS based measurement) requires gaps. The concurrent MG can provide flexible multiple independent gaps for measurements which can reduce the total measurement delay.

Currently per-UE or per-FR measurement gaps can be configured, depending on UE capability to support independent FR measurement and network preference. Per-UE gap applies to both FR1 (E-UTRA, UTRA-FDD and NR) and FR2 (NR) frequencies. For per-FR gap, two independent gap patterns (i.e. FR1 gap and FR2 gap) are configured for FR1 and FR2 respectively. In Rel-15/16, one UE may be configured with a per-UE gap or with one or two per FR gap(s).

With concurrent MG it is straightforward that multiple measurement gaps with the same gap type can be configured to one UE. For example, two per-UE measurement gaps can be configured to one UE, or two FR1 measurement gaps can be configured to one UE. But it is unclear whether multiple measurement gaps with different gap types are allowed. For example, it is unclear if it is allowed to configure one per-UE measurement gap and one FR1 measurement gap as concurrent MGs to one UE, or it is unclear if it is allowed to configure one per-UE measurement gap and one FR2 measurement gap as concurrent MGs to one UE. RAN2 can discuss the issue first and sent an LS to RAN4 if needed.

Moreover, it is unclear on the maximum number of concurrent MGs to be configured to a UE. We need to ask RAN4 if sending a response LS.

Proposal 1: For concurrent MG, multiple measurement gaps with the same gap type can be configured to one UE concurrently.

Proposal 2: RAN2 to discuss if it is allowed to configure multiple concurrent MGs with different gap types. If needed, ask RAN4 to confirm the understanding.

Proposal 3: Ask RAN4 the maximum number of concurrent MGs to be configured to a UE.

    - NR SA

- Use cases for the association between concurrent MG and frequency layer

- The measurement gap can be associated to one or multiple use cases in the following, while the detail on how to implement the association is left to RAN2 - One or more MO(s) for same or different RATs - SSB and/or CSI-RS in each associated NR MO - PRS - It is feasible that one of the concurrent gap is purely used for measuring LTE and other gaps are used for other MOs, e.g., - One gap is associated with only LTE measurement - One gap is associated with other measurements including NR. Note: - It is RAN4 common understanding that frequency layer includes positioning frequency layer. - Each frequency layer can be associated with only one MG (leave it for RAN2 on how to implement the association) - SSB, CSI-RS and PRS are treated as different frequency layers One MG can be associated with multiple frequency layers, while one frequency layers can only be associated to a single MG.

According to the above agreements from RAN4 [2], we can observe:

- One concurrent MG can be associated with one or more MO(s) for same or different RATs.

- One concurrent MG can be associated with one or more MO(s) with further RS type indicator, if the corresponding MO includes both SSB and CSI-RS measurements.

- One concurrent MG can be associated with PRS frequency layer(s).

- It is allowed to configure one concurrent MG associated with different RS types. For example: One concurrent MG can be associated with one or more MO(s) with SSB, one or more MO(s) with CSI-RS, and PRS frequency layer(s).

Proposal 4: RAN2 support to configure:

- One concurrent MG can be associated with one or more MO(s) for same or different RATs, optional with further RS type indicator.

- One concurrent MG can be associated with PRS frequency layer(s).

- It is allowed to configure one concurrent MG associated with different RS types.

According to the above highlighted part from RAN4 LS, RAN4 think it is feasible that one of the concurrent gap is purely used for measuring LTE and other gaps are used for other MOs. There are two possible configuration options to achieve the use case.

Option 1: The network realizes the use case via implementation. For example, concurrent MGs are configured to be associated with multiple MOs explicitly. And it is network implementation to ensure all these MOs associated with one concurrent MG belong to LTE while other MOs associated with other gaps are not for LTE.

Option 2:  An indicator for only LTE measurement is introduced. No need to further configure explicitly which MO(s) for LTE measurement are associated with one concurrent MG. This can reduce signaling overhead for the association between concurrent MG and frequency layer.

In order to reduce signaling overhead, option 2 is preferred.

Proposal 5: RAN2 support to introduce an indicator to indicate that the concurrent MG is associated with only LTE measurement.

- Signalling for concurrent MG

First, we would like to discuss how to configure concurrent MG configurations.

- How to configure concurrent MG configurations

RAN4 agreed:

- Concurrent gaps are multiple measurement gaps configured by RRC message(s) - Either by same or separate RRC messages - Whether and how to introduce new IE(s) or duplicate the existing IE is left to RAN2. - Note: if existing IE is to be used, the configuration mechanism shall allow NW to use the same IE to either configure additional concurrent MGP or update the configured MGP.

Measurement gap configurations for per UE MG, per FR1 MG and per FR2 MG are optional configured in IE MeasGapConfig. It is straightforward to extend IE MeasGapConfig for concurrent MG.

When concurrent MG mechanism is applied, it means that multiple MGs at least with the same gap type are applied concurrently. In order to allow the network to configure additional concurrent MGP or update the configured MGP with delta signaling, ToAddModList and ToReleaseList are used for concurrent MG configurations.

An example is shown below:

-- ASN1START -- TAG-MEASGAPCONFIG-START MeasGapConfig ::= SEQUENCE { gapFR2 SetupRelease { GapConfig } OPTIONAL, -- Need M ..., [[ gapFR1 SetupRelease { GapConfig } OPTIONAL, -- Need M gapUE SetupRelease { GapConfig } OPTIONAL -- Need M ]] [[ congapFR2ToAddModList-r17 CongapToAddModList-r17 OPTIONAL, -- Need N congapFR2ToRemoveList-R17 Congaplist-r17 OPTIONAL, -- Need N congapFR1ToAddModList-r17 CongapToAddModList-r17 OPTIONAL, -- Need N congapFR1ToRemoveList-R17 Congaplist-r17 OPTIONAL, -- Need N congapUE ToAddModList-r17 CongapToAddModList-r17 OPTIONAL, -- Need N congapUEToRemoveList-R17 Congaplist-r17 OPTIONAL -- Need N } CongapToAddModList-r17::= SEQUENCE (SIZE (1..maxNrofCongap)) OF Congap-r17 Congap-r17 ::= SEQUENCE { congapId-r17 CongapId-r17, congapConfig-r17 GapConfig, ..., } Congaplist-r17 ::= SEQUENCE(SIZE (1..FFS)) OF CongapId-r17 CongapId-r17 ::= INTEGER (SIZE (1..FFS)) //skip irrelevant part

Proposal 6: Introduce ToAddModList and ToReleaseList in IE MeasGapConfig for concurrent MG configuration.

- How to introduce the association between concurrent MG and frequency layer

According to RAN4 LS, the association between concurrent MG and frequency layer needs also be introduced. Two possible options can be considered.

- Alt1: The association between concurrent MG and frequency layer are configured within measurement gap configuration.

For example, the association can be added in the configuration for concurrent MG.

Congap ::= SEQUENCE { congapId CongapId, congapConfig GapConfig, --Add the association with frequency layer, detail is FFS ..., }

- Alt2: The association between concurrent MG and frequency layer are configured within measurement object

For example, the association can be added in the IE MeasObjectNR.

MeasObjectNR ::= SEQUENCE { ssbFrequency ARFCN-ValueNR OPTIONAL, -- Cond SSBorAssociatedSSB ssbSubcarrierSpacing SubcarrierSpacing OPTIONAL, -- Cond SSBorAssociatedSSB smtc1 SSB-MTC OPTIONAL, -- Cond SSBorAssociatedSSB smtc2 SSB-MTC2 OPTIONAL, -- Cond IntraFreqConnected refFreqCSI-RS ARFCN-ValueNR OPTIONAL, -- Cond CSI-RS referenceSignalConfig ReferenceSignalConfig, absThreshSS-BlocksConsolidation ThresholdNR OPTIONAL, -- Need R absThreshCSI-RS-Consolidation ThresholdNR OPTIONAL, -- Need R nrofSS-BlocksToAverage INTEGER (2..maxNrofSS-BlocksToAverage) OPTIONAL, -- Need R nrofCSI-RS-ResourcesToAverage INTEGER (2..maxNrofCSI-RS-ResourcesToAverage) OPTIONAL, -- Need R quantityConfigIndex INTEGER (1..maxNrofQuantityConfig), offsetMO Q-OffsetRangeList, cellsToRemoveList PCI-List OPTIONAL, -- Need N cellsToAddModList CellsToAddModList OPTIONAL, -- Need N blackCellsToRemoveList PCI-RangeIndexList OPTIONAL, -- Need N blackCellsToAddModList SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement OPTIONAL, -- Need N whiteCellsToRemoveList PCI-RangeIndexList OPTIONAL, -- Need N whiteCellsToAddModList SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement OPTIONAL, -- Need N ..., [[ freqBandIndicatorNR FreqBandIndicatorNR OPTIONAL, -- Need R measCycleSCell ENUMERATED {sf160, sf256, sf320, sf512, sf640, sf1024, sf1280} OPTIONAL -- Need R ]], [[ smtc3list-r16 SSB-MTC3List-r16 OPTIONAL, -- Need R rmtc-Config-r16 SetupRelease {RMTC-Config-r16} OPTIONAL, -- Need M t312-r16 SetupRelease { T312-r16 } OPTIONAL -- Need M ]] [[ --The association with concurrent MG, detail is FFS ]] }

With alt2, we need to introduce configurations of PRS frequency layer in measurement object configuration. Furthermore, if the association between concurrent MG and frequency layer changes, the network needs to update measurement object configuration which will lead the UE to remove the measurement reporting entry for the measid associated with this measurement object.

Proposal 7: The association between concurrent MG and frequency layer are configured within measurement gap configuration.

    - MR-DC

According to the following note in [1], cocurrent MG also applied to MR-DC scenarios.

Notes: - The joint requirements for objectives (1), (2) and (3) can be discussed in the WI stage after stabilizing individual requirements. - The work on all objectives should consider UEs in RRC CONNECTED mode only. - The work includes EN-DC, NE-DC, NR-NR DC and standalone operations and includes both per-FR MG capable UE and per-UE MG capable UE.

Proposal 8: Cocurrent MG also applied to MR-DC scenarios.

- Which node decides cocurrent MG configuration

- Alt1: Follow the same framework for MG configuration in MR-DC scenarios

In NE-DC and NR-DC, MN decides per UE concurrent MG, FR1 concurrent MG and FR2 concurrent MG.

In (NG)EN-DC, MN decides per UE concurrent MG, FR1 concurrent MG while SN decides FR2 concurrent MG.

- Alt2: If one current MG is applicable to only measurement objects in SN, SN decides the concurrent MG. Otherwise, MN decides concurrent MG.

Alt2 introduces additional complexity. For example, we need to discuss how the handle the concurrent MG configurations if both MN and SN decide to configure concurrent MG with the same type simultaneously.

Alt1 is preferred.

Proposal 9: For concurrent MG in MR-DC scenarios, follow the same framework for MG configuration in MR-DC scenarios in Rel-15, i.e.:

- In NE-DC and NR-DC, MN decides per UE concurrent MG, FR1 concurrent MG and FR2 concurrent MG.

- In (NG)EN-DC, MN decides per UE concurrent MG, FR1 concurrent MG while SN decides FR2 concurrent MG.

- Pre-configured MG

According to the LS, RRC-based activation/deactivation method is agreed.

- NW can control activation/deactivation of pre-configured MG - RRC-based activation/deactivation method is supported. - Network can indicate active/deactive status per BWP

However, it is not clear how pre-configured MG mechanism works with RRC-based activation/deactivation method in CA and MR-DC scenarios. RAN4 has already deprioritized pre-configured MG with MR-DC scenario, and would discuss pre-configured MG with CA scenario in the second stage. As pre-configured MG mechanism with RRC-based activation/deactivation method in CA may impact RRC signalling and pre-configured MG mechanism with other possible methods (e.g. MAC CE method for pre-configured MG) are still discussed in RAN4, we can wait for RAN4’s further conclusions on the topic for RAN2 discussion.

Proposal 10: Wait for RAN4’s further conclusions on pre-configured MG mechanism.

Conclusion

In this contribution, we discuss scenarios and possible impacts based on the RAN4 LS on concurrent MG, and briefly discuss pre-configured MG.

Concurrent MG:

Proposal 1: For concurrent MG, multiple measurement gaps with the same gap type can be configured to one UE concurrently.

Proposal 2: RAN2 to discuss if it is allowed to configure multiple concurrent MGs with different gap types. If needed, ask RAN4 to confirm the understanding.

Proposal 3: Ask RAN4 the maximum number of concurrent MGs to be configured to a UE.

Proposal 4: RAN2 support to configure:

- One concurrent MG can be associated with one or more MO(s) for same or different RATs, optional with further RS type indicator.

- One concurrent MG can be associated with PRS frequency layer(s).

- It is allowed to configure one concurrent MG associated with different RS types.

Proposal 5: RAN2 support to introduce an indicator to indicate that the concurrent MG is associated with only LTE measurement.

Proposal 6: Introduce ToAddModList and ToReleaseList in IE MeasGapConfig for concurrent MG configuration.

Proposal 7: The association between concurrent MG and frequency layer are configured within measurement gap configuration.

Proposal 8: Cocurrent MG also applied to MR-DC scenarios.

Proposal 9: For concurrent MG in MR-DC scenarios, follow the same framework for MG configuration in MR-DC scenarios in Rel-15, i.e.:

- In NE-DC and NR-DC, MN decides per UE concurrent MG, FR1 concurrent MG and FR2 concurrent MG.

- In (NG)EN-DC, MN decides per UE concurrent MG, FR1 concurrent MG while SN decides FR2 concurrent MG.

Pre-configured MG:

Proposal 10: Wait for RAN4’s further conclusions on pre-configured MG mechanism.

Reference

- RP-211591, Revised WID on NR and MR-DC measurement gap enhancements, RAN#92e

- R2-2109361, LS on R17 NR MG enhancements – Concurrent MG, CATT, Mediatek, RAN2#116e

- R2-2109367, LS on R17 NR MG enhancements – Pre-configured MG, Huawei,Vivo, RAN2#116e