3GPP TSG-RAN WG2 Meeting #116-e	R2-2109714

Online, Nov 1st - Nov 12th, 2021

Title:	(Draft) LS on the alternative IMSI

Release:	Release 17

Work Item:	NG_RAN_PRN_enh-Core

Source:	RAN2

To:	CT1, SA2

Cc:

Contact Person:

Name:	WentingLi

E-mail Address:	Li.wenting@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1. Overall Description:

About the IMSI offset for the EPC paging collision, RAN2 notice that SA2 has defined an alternative IMSI calculated as below:

alternative IMSI value = [MCC] [MNC] [(MSIN value + Accepted IMSI Offset) mod (MSIN address space)]

While the CT1 has specified that “the MUSIM capable UE shall forward the IMSI offset value to lower layers”

During RAN2#116_e meeting, RAN2 has discussed 2 options for the alternative IMSI calculation and Indication.

- Option 1: The UE upper layer calculates the alternative IMSI value and indicate it to the AS layer.

- Option 2: The UE upper layer indicates the IMSI offset, then UE AS layer calculates the alternative IMSI based on the IMSI offset and the MSIN address space.

RAN2’s understanding is that the option 1 is aligned with the SA2’s understanding while the option 2 is aligned with CT1’s. From RAN2 perspective, option 2 would also require the UE upper layer to indicate the MSIN address space for the alternative IMSI calculation. Thus the option 1 is more preferred. RAN2 respectfully asks CT1/SA2 to confirm RAN2’s understanding and also confirm whether the option 1 is acceptable from SA2/CT1 aspect. If unacceptable and thus the option 2 was selected, will the upper layer also indicate the MSIN address space to the UE AS layer.

2. Actions:

RAN2 respectfully asks CT1/SA2 to confirm RAN2’s understanding and also confirm whether the option 1 is acceptable from SA2/CT1 aspect. If unacceptable and thus the option 2 was selected, will the upper layer also indicate the MSIN address space to the UE AS layer.

# 3. Date of Next RAN WG2 Meetings:

RAN2#116bis-e                        17th Jan - 25th Jan 2022	Online

RAN2#117-e                             21st Feb- 3rd Mar 2022	Online