3GPP TSG-RAN WG2 #115-e	R2-2109719

Electronic meeting, 1st – 12th November, 2021

Agenda Item:	8.15.3

Source:	NEC

Title:	Discussion on RAN2 impacts for supporting inter-UE coordination Scheme 1 with preferred resource set

Document for:	Discussion, Decision

Introduction

The WID [1] defines inter-UE coordination as one of the possible resource allocation enhancement techniques to overcome the lack of inadequate sensing results. RAN1 has discussed inter-UE coordination for several meetings and achieved a good progress in RAN1#106-e and RAN1#106bis-e meetings. Agreements/work assumptions of RAN1#106-e and RAN1#106bis-e meetings are summarized in APPENDIX A and B as references.

This contribution provides our initial analyses on possible RAN2 impacts for supporting inter-UE coordination scheme 1 with preferred resource set, based on the agreements/work assumptions achieved in RAN1#106-e and RAN1#106bis-e meetings.

Discussion

For scheme 1, RAN1#106-e meeting agreed to support following inter-UE coordination information signalling from UE-A (coordination UE) to UE-B (coordinated UE).

- Set of resources preferred for UE-B’s transmission

- Set of resources non-preferred for UE-B’s transmission

RAN1#106-e meeting also agreed that

- In scheme 1, at least following UE-B’s behavior in its resource (re-)selection is supported when it receives inter-UE coordination information from UE-A:

  - For preferred resource set, the following two options are supported:

    - Option A): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information

    - Option B): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information

Observation 1 UE-B’s behavior Option A) could have more preferred resource set candidates than Option B) by merging UE-B’s sensing result (if available) and the preferred resource set received from UE-A.

The next issue we want to discuss here is how to merge UE-B’s sensing result and the received coordination information. Firstly, let us check the format of them.

In Rel-16 sidelink resource allocation mode 2, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission. Then the physical layer reports a subset of candidate single-slot resource for transmission Rx,y with a set of LsubCH contiguous sub-channels of resources to the higher layer. Here, x is the lowest sub-channel ID in frequency domain whereas y is a slot ID within the time interval of selection window. In other words, the physical layer sends “the set of resources preferred for transmission” to the higher layer in Rel-16. Here, “the set of resources preferred for transmission” sending from physical layer to higher layer is based on UE-B’s sensing result.

Observation 2 A form of candidate single-slot resource for transmission, sending from physical layer to higher layer in Rel-16, can be used to present UE-B’s sensing result.

Regarding the format of “the received coordination information”, RAN1#106bis-e meeting agreed followings:

- for inter-UE coordination scheme 1, the set of resources preferred for UE-B’s transmission is a form of candidate single-slot resource as specified in Rel-16 TS 38.214 Section 8.1.4.

- When the inter-UE coordination information transmission is triggered by UE-B’s explicit request, the candidate single-slot resource(s) are determined in the same way according to Rel-16 TS 38.214 Section 8.1.4 with at least following parameters provided by signaling from UE-B. FFS whether or not to apply RSRP threshold increase in Step 7) of Rel-16 TS 38.214 Section 8.1.4.

  - Priority value to be used for PSCCH/PSSCH transmission

    - It replaces prio_TX

  - Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot

    - It replaces L_subCH

  - Resource reservation interval

    - It replaces P_rsvp_TX

    - FFS: Starting/ending time location of resource selection window

Observation 3 A form of candidate single-slot resource as specified in Rel-16 TS 38.214 Section 8.1.4 can be used to present “the received coordination information of the set of resources preferred for UE-B’s transmission.

When merging UE-B’s physical layer “preferred resource set” and the inter-UE coordination information received from UE-A, a possible issue is how to draw out resource set report when there is no sensing results due to half-duplex constraint and DRX inactive. In Rel-16, the relevant UE behavior for those slots in which its own transmission occur was based on a hypothetical SCI format 1-A with 'Resource reservation period' field set to periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList and indicating all sub-channels of the resource pool in this slot. It is reasonable to assume the same mechanism is reused for drawing out resource set report when there is no sensing results due to DRX inactive.

Proposal 1 RAN2 to confirm that the hypothetical SCI format 1-A mechanism which defined in Rel-16 TS38.214 Section 8.1.4 for handling half-duplex constraint is reused for DRX inactive slots.

Then similar to Rel-16, the single-slot resource candidate resource Rx,y included in the inter-UE coordination information is derived from either a decoded SCI format 1-A or a hypothetical SCI format 1-A. In order to correctly handle inter-UE coordination information received from UE A, RAN2 to discuss whether an additional information (i.e., the single-slot resource candidate resource Rx,y included in the inter-UE coordination information is derived from either a decoded or a hypothetical SCI format 1-A) is beneficial or not.

Proposal 2 RAN2 to discuss whether an additional information (i.e., the single-slot resource candidate resource Rx,y included in the inter-UE coordination information is derived from either a decoded or a hypothetical SCI format 1-A) is beneficial or not.

Summary

This contribution provides our initial analyses on possible RAN2 impacts for supporting inter-UE coordination scheme 1 with preferred resource set, based on the agreements/work assumptions achieved in RAN1#106-e and RAN1#106bis-e meetings.

Observation 1 UE behavior Option A) could have more preferred resource set candidates than Option B) by  merging UE-B’s sensing result and the preferred resource set received from UE-A.

Observation 2 A form of candidate single-slot resource for transmission, sending from physical layer to higher layer in Rel-16, can be used to present UE-B’s sensing result.

Observation 3 A form of candidate single-slot resource as specified in Rel-16 TS 38.214 Section 8.1.4 can be used to present “the received coordination information of the set of resources preferred for UE-B’s transmission”.

Proposal 1 RAN2 to confirm that the hypothetical SCI format 1-A mechanism which defined in Rel-16 TS38.214 Section 8.1.4 for handling half-duplex constraint is reused for DRX inactive slots.

Proposal 2 RAN2 to discuss whether an additional information (i.e., the single-slot resource candidate resource Rx,y included in the inter-UE coordination information is derived from either a decoded or a hypothetical SCI format 1-A) is beneficial or not.

- References

[1] RP-202846, “WID revision: NR sidelink enhancement”, LG Electronics, 3GPP RAN#90e, December 2020.

- APPENDIX A RAN1#106-e agreements on “Inter-UE coordination for Mode 2 enhancements”

Agreement

For scheme 1, the following inter-UE coordination information signalling from UE-A is supported. FFS details including condition(s)/scenario(s) under which each information is enabled to be sent by UE-A and used by UE-B.

- Set of resources preferred for UE-B’s transmission

- Set of resources non-preferred for UE-B’s transmission

Agreement

For scheme 2, the following inter-UE coordination information signalling from UE-A is supported. FFS details including condition(s)/scenario(s) under which each information is enabled to be sent by UE-A and used by UE-B

- Presence of expected/potential resource conflict on the resources indicated by UE-B’s SCI

  - FFS: UE behaviour when the presence of expected/potential resource conflict is detected by the transmitter

- FFS: Whether to additionally support the presence of detected resource conflict on the resources indicated by UE-B’s SCI

Agreement

- In scheme 1, the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination information transmission triggered by an explicit request in Mode 2:

  - A UE that sends an explicit request for inter-UE coordination information can be UE-B

  - A UE that received an explicit request from UE-B and sends inter-UE coordination information to the UE-B can be UE-A

  - Working assumption At least a destination UE of a TB transmitted by UE-B can be UE A

  - The above feature can be enabled or disabled or controlled by (pre-)configuration

  - FFS: Details on how to support this, including (pre-)configuration signaling granularity

  - FFS: Additional details and conditions on UE-A and UE-B

- Working Assumption In scheme 1, the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination information transmission triggered by a condition other than explicit request reception in Mode 2:

  - A UE that satisfies the condition mentioned in the main bullet and sends inter-UE coordination information is UE-A

  - A UE that received inter-UE coordination information from UE-A and uses it for resource (re-)selection is UE-B

  - The above feature can be enabled or disabled or controlled by (pre-)configuration

    - FFS: Details on how to support this, including (pre-)configuration signaling granularity

  - FFS: Additional details and conditions on UE-A and UE-B

Agreement

In scheme 2, at least the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination transmission triggered by a detection of expected/potential resource conflict(s) in Mode 2:

- A UE that transmitted PSCCH/PSSCH with SCI indicating reserved resource(s) to be used for its transmission, received inter-UE coordination information from UE-A indicating expected/potential resource conflict(s) for the reserved resource(s), and uses it to determine resource re-selection is UE-B

- A UE that detects expected/potential resource conflict(s) on resource(s) indicated by UE-B’s SCI sends inter-UE coordination information to UE-B, subject to satisfy one of the following conditions, is UE-A

  - Working assumption At least a destination UE of one of the conflicting TBs, i.e., TBs to be transmitted in the expected/potential conflicting resource(s)

    - Whether a non-destination UE of a TB transmitted by UE-B can be UE-A is (pre-)configured

  - FFS: Additional details and condition(s) on UE-A and UE-B

- The above feature can be enabled or disabled or controlled by (pre-)configuration

  - FFS: Details on how to support this, including (pre-)configuration signaling granularity

- FFS: Definition of expected/potential resource conflict(s) and other details (if any)

Agreement

In scheme 2, the following UE-B’s behavior in its resource (re)selection is supported when it receives inter-UE coordination information from UE-A:

- UE-B can determine resource(s) to be re-selected based on the received coordination information

  - UE-B can reselect resource(s) reserved for its transmission when expected/potential resource conflict on the resource(s) is indicated

    - FFS: Other details (if any)

Agreement

In scheme 1, at least following UE-B’s behavior in its resource (re-)selection is supported when it receives inter-UE coordination information from UE-A:

- For preferred resource set, the following two options are supported:

  - Option A): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information

    - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set in combination with its own sensing result

      - UE-B uses in its resource (re-)selection, resource(s) not belonging to the preferred resource set when condition(s) are met

        - FFS: Details of condition(s)

      - This option is supported when UE-B performs sensing/resource exclusion

      - FFS: Other details (if any)

  - Option B): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information

    - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set

      - This option is supported at least when UE-B does not support sensing/resource exclusion

        - FFS: Whether the support is conditional or UE capability

      - FFS: Other details (if any)

  - FFS: Other option(s), and other details (if any)

- For non-preferred resource set,

  - UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information

    - UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set

      - FFS: Details including

        - Whether/how UE-B can use in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set, definition of the overlap, and other details (if any)

        - When UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set

    - FFS: UE-B reselects in its resource (re-)selection, resource(s) to be used for its transmission when the resource(s) are fully/partially overlapping with the non-preferred resource set

  - FFS: Other option(s), and other details (if any)

Agreement

In scheme 2, at least the following is supported to determine inter-UE coordination information:

  - Among resource(s) indicated by UE-B’s SCI, UE-A considers that expected/potential resource conflict occurs on the resource(s) satisfying at least one of the following condition(s):

  - Condition 2-A-1:

    - Other UE’s reserved resource(s) identified by UE-A are fully/partially overlapping with resource(s) indicated by UE-B’s SCI in time-and-frequency

    - FFS: Other details (if any)

    - FFS: Whether/how to specify additional criteria and other details (if any) including signaling details of conflict indication

  - (Working Assumption) Condition 2-A-2:

    - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

      - FFS: Other details (if any)

  - FFS: Other condition(s)

  - FFS: Other details (if any)

Agreement

In scheme 1, at least the following is supported to determine inter-UE coordination information of preferred resource set:

  - UE-A considers any resource(s) satisfying all the following condition(s) as set of resource(s) preferred for UE-B’s transmission

    - Condition 1-A-1:

      - Resource(s) excluding those overlapping with reserved resource(s) of other UE identified by UE-A whose RSRP measurement is larger than a RSRP threshold

        - FFS: Other details (if any)

    - FFS: Condition 1-A-2:

      - Resource(s) excluding slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B

        - FFS: Other details (if any)

    - FFS: Condition 1-A-3:

      - Resource(s) satisfying UE-B’s traffic requirement (if available)

        - FFS: Other details (if any)

    - FFS: Other condition(s)

  - FFS: Other details (if any)

Agreement

In scheme 1, at least the following is supported to determine inter-UE coordination information of non-preferred resource set:

- UE-A considers any resource(s) satisfying at least one of the following condition(s) as set of resource(s) non-preferred for UE-B’s transmission

  - Condition 1-B-1:

    - Reserved resource(s) of other UE identified by UE-A from other UEs’ SCI (including priority field) and RSRP measurement

      - FFS: Other details (if any)

  - FFS: Condition 1-B-2:

    - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B

      - FFS: Other details (if any)

  - FFS: Other condition(s)

- FFS: Other details (if any)

- APPENDIX B RAN1#106b-e agreements on “Inter-UE coordination for Mode 2 enhancements”

Agreement

For Scheme 2, PSFCH format 0 is used to convey the presence of expected/potential resource conflict on reserved resource(s) indicated by UE-B’s SCI

Agreement

For Condition 2-A-1 of Scheme 2, down-select one or more of following additional criteria to determine resource(s) where expected/potential resource conflict occurs

- Option 1: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a RSRP threshold according to the priorities included in the SCI:

  - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations

  - Strive to reuse Rel-16 specification wherever possible

- Option 2: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is within a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

  - FFS: Whether the threshold depends on priority

- Option 3: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) and the other UE is within a distance threshold of UE-B as determined by both UEs’ SCIs.

- Option 4: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

  - FFS: Whether the threshold depends on priority

- FFS: In case of collisions of resources for two UEs having TBs with UE A as destination UE, if needed

Working Assumption

For Condition 1-B-1 of Scheme 1, the following two options are supported

- Option 1: Reserved resource(s) of other UE(s) identified by UE-A whose RSRP measurement is larger than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s)

- Option 2: Reserved resource(s) of other UE identified by UE-A whose RSRP measurement is smaller than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s) when UE-A is a destination of a TB transmitted by the UE(s)

Working Assumption

For Scheme 1 with non-preferred resource set, support following condition:

- Condition 1-B-2:

  - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

Agreement

For Condition 1-A-1 of Scheme 1, the set of resources preferred for UE-B’s transmission is a form of candidate single-slot resource as specified in Rel-16 TS 38.214 Section 8.1.4

- When the inter-UE coordination information transmission is triggered by UE-B’s explicit request, the candidate single-slot resource(s) are determined in the same way according to Rel-16 TS 38.214 Section 8.1.4 with at least following parameters provided by signaling from UE-B. FFS whether or not to apply RSRP threshold increase in Step 7) of Rel-16 TS 38.214 Section 8.1.4.

  - Priority value to be used for PSCCH/PSSCH transmission

    - It replaces prio_TX

  - Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot

    - It replaces L_subCH

  - Resource reservation interval

    - It replaces P_rsvp_TX

  - FFS: Starting/ending time location of resource selection window

- FFS : In addition to Rel-16 procedure, use inter-UE coordination information from other UEs

  - If there is no consensus in RAN1#106bis-e, no further discussions for Rel-17

Conclusion

No consensus that UE-A uses inter-UE coordination information from other UEs when it determines the preferred resource set for Condition 1-A-1 of Scheme 1.

Working Assumption

For Scheme 1 with preferred resource set, support following condition:

- Condition 1-A-2:

  - Resource(s) excluding slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

  - This can be disabled by RRC (pre-)configuration

Agreement

For allocating PSFCH resources in Scheme 2, at least following can be (pre)configured separately from those for SL HARQ-ACK feedback.

- Set of PRBs for PSFCH transmission/reception (sl-PSFCH-RB-Set)

Agreement

For Scheme 2,

- Index of a PSFCH resource for inter-UE coordination information transmission is determined in the same way according to Rel-16 TS 38.213 Section 16.3 with at least following modification

  - P_ID is L1-Source ID indicated by UE-B’s SCI

  - M_ID is 0

- FFS: How to set m_CS

- FFS: How to set m_0

- FFS: Whether M_ID can be (pre)configured