3GPP TSG-RAN WG2 #116-e	R2-2109720

Electronic meeting, 1st –12th November, 2021

Agenda Item:	8.15.2

Source:	NEC

Title:	Further discussion on identified FFS/ open issues of unicast sidelink DRX overall flow

Document for:	Discussion, Decision

Introduction

In the last meeting (RAN2#115-e), regarding SL DRX, RAN2 achieved many agreements [1] and assigned a post meeting e-mail discussion 716 to further discuss following identified FFS/open issues:

[POST115-e][716][V2X/SL] Identified FFS/open issues (CATT)

Scope: Discuss identified FFS/open issues including: 1) FFS whether a TX profile identifies a release, or one or more sidelink feature groups, 2) FFS whether a TX profile needs to be provided with service type information or L2 id when upper layer indicates to AS layer, 3) FFS on slot or symbol where the start of SL-specific drx-HARQ-RTT-Timer and SL-specific drx-RetransmissionTimer, 4) FFS on the specific values of HARQ RTT that can be used for HARQ disabled case, 5) How to handle cases when a transmission may cause these timers to be running at the RX UE is FFS. FFS on groupcast. FFS on whether any spec impact (in agreement 14 and 15 in SL DRX timer maintenance, 6) what information is included in the assistance information from RX UE to TX UE? 7) Need of SL DRX assistance information REQ from TX UE to RX UE, 8) If SL DRX assistance information REQ is needed, what information is included? 9) FFS on the interpretation if assistance information is not provided, 10) FFS on the following TX/RX UE behaviours when reject happens, 11) FFS on whether the new rejection cause for SL DRX needs to be defined, 12) FFS on whether RRCReconfigurationFailureSidelink or RRCReconfigurationCompleteSidelink is used in Step 2, 13) Need of down-selection for SL DRX configuration when multiple QoS profiles are associated for same DST L2 ID, 14) Common or separate default SL DRX configuration for GC and BC? 15) FFS on whether default SL BC DRX configuration or which SL BC DRX configuration for DCR message should be used, 16) Whether SL DRX is applied after DCR message and before SL unicast DRX configuration is applied? 17) Whether we can confirm the WA that DRX configuration for V2X group management signaling is out of RAN2 scope.

This contribution discussed identified FFS/open issues 6)-12) (highlighted in yellow) which are related to unicast sidelink DRX overall flow.

Discussion

Identified FFS/open issues 6): What information is included in the assistance information from RX UE to TX UE?

Regarding the assistance information from RX UE to TX UE, RAN2#115-e meeting agreed that

- For SL unicast, RX UE may include its desired SL DRX configuration in the assistance information which is transmitted to TX UE.

Therefore the discussion point here is details of the desired SL DRX configuration. Considering the RX UE could have individual preference on any parameters of the SL DRX configuration, it is reasonable to define the desired SL DRX configuration by copying the conventional SL DRX parameters (e.g., DRX cycle, onduration timers, RTT timers and etc). In order to reduce signaling overhead, only those parameters which RX UE has preference/requirement should be included (or presented) in the desired SL DRX configuration. How to determine its desired SL DRX configuration (including selection of interested SL DRX configuration parameters and determination of the corresponding value) is up to RX UE implementation.

Proposal 1 What information is included in the assistance information from RX UE to TX UE is up to RX UE implementation

Identified FFS/open issues 7): Need of SL DRX assistance information REQ from TX UE to RX UE

Basically, both RX UE and TX UE can trigger unicast TX-UE centric DRX configuration negotiation procedure. For the TX UE initiated case, RX UE can consider SidelinkReconfigurationRequest including SL DRX configuration from the TX UE as the REQ, then an explicit SL DRX assistance information REQ is not necessary. For the RX UE initiated case, RX UE will trigger the procedure by sending SL DRX assistance information directly. It doesn’t make a sense to design an unnecessary REQ message.

Proposal 2 SL DRX assistance information REQ from TX UE to RX UE is not supported in Rel-17.

Identified FFS/open issues 9) FFS on the interpretation if assistance information is not provided

In order to reduce signaling overhead in the TX UE initiated SL DRX configuration negotiation, it is beneficial to let the RX UE skip to send the assistance information when RX UE has no preference on SL DRX configuration. Instead of that, the RX shall provide an explicit acceptance/rejection information to let TX UE know when the SL DRX configuration can be applied.

Proposal 3 The assistance information absent case may occur when the RX UE has no preference on SL DRX configuration.

Identified FFS/open issues 10) FFS on the following TX/RX UE behaviours when reject happens

In RAN2#115-e meeting, it was agreed that [1]:

4:	For unicast, a two-step process (i.e., RX UE accepts or rejects TX UE’s suggestion) is adopted as a baseline, i.e., FFS on the following TX/RX UE behaviours when reject happens.

• Step 1: TX UE sends RRCReconfigurationSidelink containing a SL DRX configuration to be applied by RX UE to RX UE

• Step 2: RX UE replies with a PC5-RRC signalling indicating acceptance or rejection for the SL DRX configuration. FFS on whether the new rejection cause for SL DRX needs to be defined. FFS on whether RRCReconfigurationFailureSidelink or RRCReconfigurationCompleteSidelink is used in Step 2.

When the RX UE indicates its rejection of the SL DRX configuration proposed by the TX UE, the TX UE shall not apply the SL DRX configuration included in the previous RRCReconfigurationSidelink message. If there is a DRX configuration applied between the TX and RX UEs pair, then the TX UE can continue using it. If the SL DRX operation is not applied yet, the selection of the following behaviour is up to TX UE implementation.

- Fully reuse the legacy Tx UE behaviors (i.e., don’t apply DRX operation between the TX and RX UEs pair)

- restarts the T400 and resends the RRC reconfiguration including a new DRX configuration.

- informs DRX rejection to gNB (when the TX UE is a connected UE)

Proposal 4 If the SL DRX operation is not applied yet, TX UE behaviours when reject happens is up to TX UE implementation.

Identified FFS/open issues 11) FFS on whether the new rejection cause for SL DRX needs to be defined

Identified FFS/open issues 12) FFS on whether RRCReconfigurationFailureSidelink or RRCReconfigurationCompleteSidelink is used in Step 2

Identified FFS/open issues 11) and 12) are interdependency, i.e., only when the answer of issue 12) is RRCReconfigurationFailureSidelink, issue 11) is valid. Therefore, we provide our view on 12) firstly.

Besides of carrying a SL DRX configuration to be applied, the RRCReconfigurationSidelink message could be used to modify a PC5-RRC connection, e.g. to establish/modify/release sidelink DRBs, to (re-)configure NR sidelink measurement and reporting, to (re-)configure sidelink CSI reference signal resources and CSI reporting latency bound. When RX UE replies with a PC5-RRC signalling indicating rejection for the SL DRX configuration, it is beneficial to let TX UE be able to modify a RRC-RRC connection by falling back to legacy UE behaviour when the RX UE has no preference/requirement on DRX operation itself. Therefore, we think it is necessary to distinguish the DRX rejection case from legacy failure cases, then prefer to indicate RX UE DRX rejection via RRCReconfigurationCompleteSidelink message.

Proposal 5 Add a new DRX acceptance/rejection indication in RRCReconfirationCompleteSidelink message.

When RX-UE accepts the delivered DRX configuration from TX-UE, it will feedback ACK indication via RRCReconfirationCompleteSidelink, then DRX operation start point at TX-UE/RX-UE can be defined as upon receiving/transmitting the corresponding RRCReconfirationCompleteSidelink message with ACK indication of DRX configuration as shown in Fig.1.

Fig.1 Start point of DRX operation (RX-UE trigger case)

Proposal 6 RAN2 to confirm that DRX operation start point at TX-UE/RX-UE is upon receiving/transmitting RRCReconfirationCompleteSidelink message with ACK indication of TX-UE delivered DRX configuration.

Summary

This contribution discussed identified FFS/open issues 6)-12) in the post-115e-716 email discussion which are related to unicast sidelink DRX overall flow.

Proposal 1 What information is included in the assistance information from RX UE to TX UE is up to RX UE implementation

Proposal 2 SL DRX assistance information REQ from TX UE to RX UE is not supported in Rel-17.

Proposal 3 The assistance information absent case may occur when the RX UE has no preference on SL DRX configuration.

Proposal 4 If the SL DRX operation is not applied yet, TX UE behaviours when rejects happens is up to TX UE implementation.

Proposal 5 Add a new DRX acceptance/rejection indication in RRCReconfirationCompleteSidelink message.

Proposal 6 RAN2 to confirm that DRX operation start point at TX-UE/RX-UE is upon receiving/transmitting RRCReconfirationCompleteSidelink message with ACK indication of TX-UE delivered DRX configuration.

- References

[1] R2-2108838	Report from session on LTE V2X and NR SL	Report