3GPP TSG-RAN WG2 Meeting #116e                                                             R2-2109774

# Electronic, November 1st – 12th, 2021

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | 0212 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RRM relaxation of higher priority frequencies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-Core |  |  |  |  |  | Date: |  |  | 2021-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to RAN4 LS in R4-2108230, there is inconsistency between RAN2 and RAN4 specs on relaxation of higher priority carriers when the lowMobilityEvalutation criterion is fulfilled and when highPriorityMeasRelax is configured for inter-frequency measurements or inter-RAT measurements. According to clause 4.2.2.10.2 in TS 38.133, for each higher layer priority, the UE shall search for inter-frequency layers/E-UTRA inter-RAT frequency layers of higher priority at least every Nlayers hours. However, in the current TS 38.304, 1 hour was captured there. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Refer to clause 4.2.2.10.2 in TS 38.133 for the correct relaxation requirements. Impact analysis Impacted 5G architecture options: NR SA, NE-DC,NR-DC Impacted functionality: RRM relaxation of higher priority carriers Inter-operability: - If UE implements this and network does not, there is no inter-operability issue. - If network implements this and UE does not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE will not perform the correct relaxation of higher priority carriers when the lowMobilityEvalutation criterion is fulfilled and when highPriorityMeasRelax is configured for inter-frequency measurements or inter-RAT measurements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4.9.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.2.4.9.0	Relaxed measurement rules

When the UE is required to perform measurements of intra-frequency cells or NR inter-frequency cells or inter-RAT frequency cells according to the measurement rules in clause 5.2.4.2:

-	if lowMobilityEvaluation is configured and cellEdgeEvaluation is not configured; and

-	if the UE has performed normal intra-frequency, NR inter-frequency, or inter-RAT frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell; and

-	if the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP:

-	the UE may choose to perform relaxed measurements for intra-frequency cells according to relaxation methods in clauses 4.2.2.9 in TS 38.133 [8];

-	if the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ:

-	if highPriorityMeasRelax is configured with value true:

-	the UE may choose to perform relaxed measurements for NR inter-frequency cells or inter-RAT frequency cells on frequencies of higher priority according to relaxation methods in clause 4.2.2.10.2 in TS 38.133 [8];

-	else (i.e. the serving cell fulfils Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ):

-	the UE may choose to perform relaxed measurements for NR inter-frequency cells or inter-RAT frequency cells according to relaxation methods in clauses 4.2.2.10, and 4.2.2.11 in TS 38.133 [8];

-	if cellEdgeEvaluation is configured and lowMobilityEvaluation is not configured; and

-	if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled:

-	the UE may choose to perform relaxed measurements for intra-frequency cells according to relaxation methods in clauses 4.2.2.9 in TS 38.133 [8];

-	if the serving cell fulfils Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ:

-	the UE may choose to perform relaxed measurements for NR inter-frequency cells or inter-RAT frequency cells according to relaxation methods in clauses 4.2.2.10, and 4.2.2.11 in TS 38.133 [8];

-	if both lowMobilityEvaluation and cellEdgeEvaluation are configured:

-	if the UE has performed normal intra-frequency, NR inter-frequency, or inter-RAT frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell; and

-	if the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP; and

-	if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled:

-	for any intra-frequency, NR inter-frequency, or inter-RAT frequency, if less than 1 hour has passed since measurements of corresponding frequency cell(s) for cell reselection were last performed:

-	the UE may choose not to perform measurement for measurements on this frequency cell(s);

-	else:

-	if the UE has performed normal intra-frequency, NR inter-frequency, or inter-RAT frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell, and the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP; or,

-	if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled:

-	if combineRelaxedMeasCondition is not configured:

-	the UE may choose to perform relaxed measurements for intra-frequency cells, NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to relaxation methods in clauses 4.2.2.9, 4.2.2.10, and 4.2.2.11 in TS 38.133 [8];

-	if the serving cell fulfils Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ:

-	the UE may choose to perform relaxed measurement for NR inter-frequency cells of higher priority, or inter-RAT frequency cells of higher priority according to relaxation methods in clauses 4.2.2.10, and 4.2.2.11 in TS 38.133 [8];

The above relaxed measurements and no measurement are not applicable for frequencies that are included in VarMeasIdleConfig, if configured and for which the UE supports dual connectivity or carrier aggregation between those frequencies and the frequency of the current serving cell.