3GPP TSG-RAN WG2 Meeting #116-e	R2-2109828

# Electronic, 1st – 12th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1825 | rev |  | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of missing TEI15 features |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility (Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2021-10-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The following two features were introduced as TEI15 with cat “B” but the descriptions on their support are missing: - Support for logging of “Any cell selection” state For this feature the following RAN2 CRs were agreed: - TS 37.320: CR0072 in R2-1808615, agreed in RAN2#102 (May 2018) - TS 36.304: CR0420r1 in R2-1808803, agreed in RAN2#102 (May 2018) - TS 36.331: CR3803r5 in R2-1819200, agreed in RAN2#104 (November 2018) Acc. to the agreed CRs a UE that is configured to perform logged measurements in RRC_IDLE, shall continue logging in any cell selection state in RRC_IDLE. Therefore, the support of this feature is conditionally mandatory for a UE that supports logged measurements in RRC_IDLE upon request from the network. - MBMS reception using Receive Only Mode For this feature the following RAN2 CRs were agreed: - TS 36.300: CR1207r1 in R2-1818596, agreed in RAN2#104 (November 2018) - TS 36.331: CR3776r1 in R2-1818597, agreed in RAN2#104 (November 2018) Acc. to the agreed CRs an MBMS capable UE may receive or be interested to receive broadcast MBMS service(s) in receive only mode from a different eNB while receiving unicast or MBMS service(s) from serving eNB. In this case the UE uses the MBMSInterestIndication message to inform the unicast serving eNB about the receive only mode MBMS service(s) which the UE is receiving or interested to receive. To our understanding, the support of this feature is optional for an MBMS capable UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Support for logging of “Any cell selection” state The support of this feature has been defined as conditionally mandatory for UEs which support logged measurements in RRC_IDLE upon request from the network. - MBMS reception using Receive Only Mode The support of this feature has been defined as an optional feature w/o capability signaling for an MBMS capable UE. Impact analysis Impacted functionality: UE capability Inter-operability: There are no inter-operability issues since the support of the missing TEI15 features does not require any capability signaling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of the two TEI15 features remains missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.x (new), 7.10.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

## 6.3	MBMS features

It is optional for UE to support MBMS procedures as specified in TS 36.331 [5].

### 6.3.1	MBMS Service Continuity

It is optional for UE to support MBMS Service Continuity for UEs supporting MBMS as specified in TS 36.331 [5].

### 6.3.2	MBMS reception with 256QAM

It is optional to support MBMS reception with 256QAM for UEs supporting MBMS. A UE which supports MBMS reception with 256QAM shall also support dl-256QAM-r12 as specified in TS 36.331 [5], except UEs configured to operate in Receive Only Mode as defined in TS 23.246 [31].

### 6.3.3	Void

### 6.3.4	Void

### 6.3.4	Void

### 6.3.x	MBMS reception using Receive Only Mode

It is optional to support indication of MBMS reception using Receive Only Mode in an MBMSInterestIndication message for UEs supporting MBMS as specified in TS 36.331 [5].

Next change

## 7.10	Other features

### 7.10.1	Logged MDT measurement suspension due to IDC interference

It is mandatory to support Logged MDT measurement suspension due to IDC interference for UEs which are supporting logged measurements in RRC_IDLE upon request from the network and in-device coexistence indication as well as autonomous denial functionality as specified in TS 36.331 [5].

### 7.10.2	Support of extended reporting of WLAN measurements

It is mandatory to support reporting of extended number of measurements of WLAN IDs for UEs which are supporting WLAN measurements as specified in TS 36.331 [5].

### 7.10.3	wlan-ReportAnyWLAN-r14

Indicates whether UE supports reporting of measurements of unknown WLAN as specified in TS 36.331 [5]. It is mandatory to support reporting of measurements of unknown WLAN ID for UEs which are supporting WLAN measurements as specified in TS 36.331 [5].

### 7.10.4	wlan-PeriodicMeas-r14

This parameter indicates whether the UE supports periodic reporting of WLAN measurements. It is mandatory to support periodic reporting of WLAN measurements for UEs which are supporting WLAN measurements as specified in TS 36.331 [5].

### 7.10.x	MDT measurement logging in any cell selection state

It is mandatory to support MDT measurement logging in any cell selection state in RRC_IDLE for UEs which are supporting logged measurements in RRC_IDLE upon request from the network as specified in TS 36.331 [5] and TS 36.304 [14].

End of changes