3GPP TSG RAN WG2 Meeting #116-e                                                        	   R2-2109880

E-Meeting, 1st – 12th November, 2021

Agenda Item:	8.9.2.1

Source: 	Intel Corporation

Title:	Further considerations for subgrouping

Document for: 	Discussion/Decision

# Introduction

An email discussion [3] further discussed in general the network based subgrouping and UE based UE-ID subgrouping.

In this contribution, we provide further our view on the following topics

  - Need of Assistance information (UE assistance to network, gNB assistance to CN and vice versa)

  - Coexistence between UE-ID based subgrouping and CN based subgrouping

  - Support of mobility for PEI/subgrouping

# Discussion

## Need of Assistance information

### UE assistance information

In the last meeting, it is agreed that CN directly allocates the subgrouping ID. On the other hand, it is not concluded on whether CN needs further assistance to allocate the subgrouping ID.  In our view, which UE characteristics should be considered for allocating the subgroups can be left to network implementation. CN has the required information needed for assigning the subgroup for a UE. In the email discussion [1], there was more support for the following UE characteristics for subgrouping: UE ID, paging probability and power consumption sensitivity level. These UE characteristics are already or can be made to be known to the CN:

- UE ID is known to CN as it is allocated by CN for mobility management and legacy paging operation.

- In the case of paging probability, if it is just for differentiating the paging probability between Redcap UE and eMBB UEs. This can be known to the CN via UE subscription.

- If static power consumption sensitivity level is needed (e.g. eMBB UEs, IoT UE), this can be again known to the CN via the UE subscription.

Observation#1: The combinations of UE characteristics to perform subgroup categorization is already available to CN:

- UE ID is known to CN as it is allocated by CN for mobility management and legacy paging operation.

- In the case of paging probability, if it is just for differentiating the paging probability between Redcap UE and eMBB UEs. This can be based on UE subscription.

- If static power consumption sensitivity level is needed (e.g. eMBB UEs, IoT UE), this can be again known to the CN via the UE subscription

Even though UE subscription information may not provide more “dynamic” changing information (e.g. UE power consumption sensitive level may change depending on the battery life etc.), we think this aspect can be studied further in future release in view of the limited time left in Rel-17.

Proposal#1: New UE assistance is not provided by UE in this release to help in the allocation of the UEs in the different UE paging subgroups.

### gNB/CN assistance information

In the last meeting, RAN2 discussed via an offline discussion on how the CN assigned subgroup is mapped to the physical subgroup and there were some proposals that gNB provides the AMF with the configured number of physical subgroups.  With the agreement (working assumption) below:

- R2 assumes that All the cells within the registration area supports the same number of CN assigned subgroups, i.e. no remapping of CN assigned group ID to RAN subgroup ID (will revisit only if serious issues are found).

The question then is on how AMF and RAN nodes can ensure that all the cells within a registration area supports the same number of CN assigned subgroups. There are basically 2 approaches:

Approach A: Coordination via the interface between the AMF and gNB (i.e. CN informs gNBs within the registration area of the number of CN subgroups needed) or

Approach B: Ensure that the number of CN assigned subgroups on the gNBs over a registration area are uniform via OAM.

Approach A will mean that some work is needed in SA2 and RAN3 while Approach B means no further work is needed across other WGs. In view of the time left for Rel-17, Approach B will be sufficient in our view.

Proposal#2: How to ensure all the cells within the registration area supports the same number of CN assigned subgroups can be left to OAM and there is no need for CN assistance information in this respect. Inform SA2 and RAN3 on this decision if agreed.

There are also some supports to indicate the paging probability for UE is in Inactive mode [1].  The main purpose of Inactive mode is mainly to reduce the signalling overhead during call establishment. Even though the paging probability may be different to Idle mode while UE is in Inactive mode, this may not be that essential and in our view can be left out from Rel-17.

Proposal#3: No need of any gNB assistance in this release.

## Coexistence between UE-ID based subgrouping and CN based subgrouping

RAN2 had also discussed whether a cell should have a mix of both UE-ID based subgrouping and CN based subgrouping.Based on the offline discussion [2], there are the following options:

- Option b1: Hard split between CN assigned subgroups and UE-ID based subgroups with each broadcasted (I.e. subgrouping bits in the DCI of the PDCCH based PEI are hard split between CN assigned subgroups and UE-ID based subgroups)

- Option b2: CN assignment and UE-ID based can share the same subgroups (I.e. subgrouping bits in the DCI of the PDCCH based PEI are the same for CN assigned subgroups and UE-ID based subgroups)

- Option b3: either NW controlled subgrouping or UE ID based subgrouping is used in a cell without mixing them (I.e. subgrouping bits in the DCI of the PDCCH based PEI are either for CN assigned subgroups or UE-ID based subgroups)

- Option b4: gNB can decide by itself on the number of subgroups it wants to allocate to UE-ID based UEs. It is up to gNB implementation whether there can be any overlap between CN-assigned subgroups and UE-ID based subgroups in its cell. Although hard partition between two types of subgroups is desirable (for avoiding false alarm), we think we can leave that decision to gNB to “keep things simple”.

Options b1, b2 or b4 are acceptable to us.  For Option b2, there is a possibility of false alarm.  Option b1 is the simplest but hard partition may not lead to uniform distribution depending on the number of UEs that are assigned by CN and gNB. Option b4 provides the most flexibility in that it can also be used to implement Options b1 or b2 or b3. We do not see a motivation for the network to do such mixing of UEID based subgroup and CN assigned subgroup since the CN assigned subgrouping can also take into consideration of the UE ID while allocating the CN assigned subgroup ID. At the same time, we do not see any additional effort to allow the mix of UEs in a cell with option b4 and hence we do not see a need to explicitly prohibit it.

Observation#2: Option b4 is a superset of Options b1 to b3

Proposal#4: It can be left to gNB implementation whether to allow overlapping or hard split among the subgrouping bits in the DCI of the PDCCH based PEI or whether all the DCI grouping bits are for UEID based subgrouping or for CN based subgrouping.

## Support of PEI/subgrouping for mobility

In the last RAN2 meeting, the support of mobility for PEI/subgrouping for the RRC_Inactive case is left as a FFS

It is FFS when a UE in RRC_INACTIVE has been assigned by CN a Paging subgroup, whether some signaling should be introduced between gNBs to inform each other about the UE’s subgroup for RAN paging.

Even though majority of the companies think that this can be supported for RRC_Inactive mode in the last email disc, one company holds the view that RAN2 has not discussed whether PEI/subgrouping should be supported for mobility. The argument is that by not supporting mobility for PEI/subgrouping, it prevents waking up other UEs in the same subgroup in the whole of UE paging area/TA when the UE moves out of its last used cell (i.e. the network escalate the paging to the whole the UE paging area/TA, instead of paging it on the last used cell).  However, this is done at the expense of the UE being paged as it will have to follow the legacy paging when it moves out of the last used cell. Also this is also assuming a certain form of network paging strategy (i.e. paged on last used cell before paging over UE paging area). Hence it is difficult to base on this to judge whether mobility for PEI/subgrouping is supported in idle mode and RRC Inactive.

It is also brought up that the (G)WUS in current LTE eMTC and NBIoT which is equivalent to PEI/subgrouping is only performed on the last used cell. We think the scenario is a bit different with eMTC/NBIoT where the UEs are mainly stationary. In this case where UE power saving also applies to eMBB, mobility may need to be considered for PEI.

Observation#3: Not supporting mobility for PEI/subgrouping comes at the expense of the UE being paged as it will have to follow the legacy paging when it moves out of the last used cell while not be paged for a very long time.

Observation#4: Since the power saving is applicable to eMBB, it may seem like support of mobility for PEI/subgrouping seems needed to improve power saving for the eMBB UE.

Before we can decide on whether to support mobility for PEI/subgrouping in idle mode and RRC_Inactive, there is also a need to understand whether there is any issue if mobility is supported. In Rel-15, the Inter-node message for the UE capability for paging (i.e. UERadioPagingInformation) is populated by the gNB and hence if the gNB does not support PEI and/or subgrouping, it could not include any information of the UE supporting PEI and/or subgrouping to the UERadioPagingInformation. Hence subsequently during CN or RAN paging, the paging gNB does not know the exact support of PEI and/or subgrouping by the UE.

As a result of this, the following 2 issues occur in NR during CN/RAN paging:

Issue 1: If the first or any of the previous gNB (i.e. the gNB where the UE was previously connected) does not support PEI and/or subgrouping, all subsequent CN paging will not be able to use PEI and/or subgrouping even if the current gNB supports paging PEI and/or subgrouping. The UE does not hence know if PEI will be used.

Issue 2: For RAN paging, due to issue 1, the UERadioPagingInformation received by the source gNB from CN does not include the UE support of PEI and/or subgrouping, the UE can’t be paged with PEI and/or subgrouping even if the UE and the current gNB supports it.

Because of the above issues, gNB that supports PEI and/or subgrouping may not know that the UE supports PEI and/or subgrouping while UE configures to monitor the PEI and/or subgrouping by the serving cell. For example, if the gNB supports UEID based subgrouping (i.e. it broadcast PEI/subgrouping support for UEID based subgrouping) and the UE supports UEID based subgrouping, the UE will assume that UEID based subgrouping is performed. However, the gNB does not have the support of PEI/subgrouping in the UERadioPagingInformation either from CN (i.e. CN paging) or from source gNB (i.e. RAN paging), it will not page the UE with PEI/subgrouping

This will result in paging being missed by UE as gNB thinks that the UE is not monitoring PEI/subgrouping while the UE is monitoring PEI/subgrouping. The issue will occur for UEID based subgrouping since UERadioPagingInformation is used during CN paging and also provided by source gNB to target gNB during RAN paging.

For CN based subgrouping, this issue does not occur for CN paging if the UE support of CN based subgrouping is provided to CN via NAS signalling. If it is based on gNB forwarding UE support of PEI/subgrouping, it will have the same issue. For RAN paging, the similar issue may also occur if the source gNB does not forward the CN allocated subgrouping ID to the target gNB.

Observation#5: If PEI/subgrouping is supported for mobility, it is not sufficient for the UE to perform PEI/subgrouping monitoring based only on the serving cell and UE support of PEI/subgrouping since it depends on whether gNB in which the UE performs initial registration supports PEI/subgrouping.

One straightforward way to solve the above issue is that the support of subgrouping/PEI by gNB is uniform over the UE’s paging area. This seems logical since RAN2 has made similar assumption on the number of CN assigned subgroup (I.e. All the cells within the registration area supports the same number of CN assigned subgroups).   If this can’t be guaranteed or is too restrictive for network deployments, it is also possible to inform the UE whether it can monitor PEI/Subgrouping whenever it enters idle mode or RRC_Inactive. However, this will require further discussion which RAN2 may not have time for in Rel-17.

Observation#6: The issues observed in Observation#5 can be resolved if the support of subgrouping/PEI by gNB is uniform over the UE’s paging area.

Based on Observations#3, 4 and 6, it is proposed:

Proposal#5: PEI/subgrouping is supported for UE mobility (i.e. the PEI/subgrouping is not limited to the last used cell by the UE).

# Conclusion

It is requested that RAN2 discussed the following observations and proposals:

Proposal#1: New UE assistance is not provided by UE in this release to help in the allocation of the UEs in the different UE paging subgroups.

Proposal#2: How to ensure all the cells within the registration area supports the same number of CN assigned subgroups can be left to OAM and there is no need for CN assistance information in this respect. Inform SA2 and RAN3 on this decision if agreed.

Proposal#3: No need of any gNB assistance in this release

Observation#2: Option b4 is a superset of Options b1 to b3

Proposal#4: It can be left to gNB implementation whether to allow overlapping or hard split among the subgrouping bits in the DCI of the PDCCH based PEI or whether all the DCI grouping bits are for UEID based subgrouping or for CN based subgrouping.

Observation#3: Not supporting mobility for PEI/subgrouping comes at the expense of the UE being paged as it will have to follow the legacy paging when it moves out of the last used cell while not be paged for a very long time.

Observation#4: Since the power saving is applicable to eMBB, it may seem like support of mobility for PEI/subgrouping seems needed to improve power saving for the eMBB UE.

Observation#5: If PEI/subgrouping is supported for mobility, it is not sufficient for the UE to perform PEI/subgrouping monitoring based only on the serving cell and UE support of PEI/subgrouping since it depends on whether gNB in which the UE performs initial registration supports PEI/subgrouping.

Observation#6: The issues observed in Observation#5 can be resolved if the support of subgrouping/PEI by gNB is uniform over the UE’s paging area.

Proposal#5: PEI/subgrouping is supported for UE mobility (i.e. the PEI/subgrouping is not limited to the last used cell by the UE).

# References

- R2-2108685, Summary of [Post114-e][076][ePowSav] Paging SubGrouping, CATT

- R2-2109094, [AT115-e][043][ePowSav] Paging Subgrouping (Nokia), Nokia

- R2-21xxxx, Summary of [Post115-e][089][ePowSav] Paging Subgrouping