3GPP TSG-RAN WG2 Meeting #116e                             R2-2109894

e-meeting, 1st – 12th November 2021

Source: 			ZTE Corporation, Sanechips,

Title: 	Consideration on Msg3 repetition in CE

Agenda item:		8.19.2

Document for: 	Discussion and Decision

- Introduction

For Msg3 repetition in Rel-17 coverage enhancement, RAN2 discussed the potential RAN2 impact during RAN2#116e meeting, and made the following agreements:

Agreements:

- RAN2 should focus on Msg3 repetition for 4-step RACH, unless RAN1 makes solid conclusion to support Msg3 repetition for fallbackRAR

- Msg3 repetition is applicable to all cases that trigger 4-step CBRA procedure (can come back if we identify that some specific case should not be covered)

- A separate RSRP threshold is introduced for requesting Msg3 repetition

Agreements via email - from offline 111:

- Extension of ra-ResponseWindow and ra-ContentionResolutionTimer are not needed for Msg3 repetition.

- RAN2 confirms enhancing MAC RAR for indicating MSG3 repetition is not supported.

- Postpone the discussion on UE capability (i.e. whether explicit UE capability is needed for indicating the support of Msg3 repetition).

In addition, for RACH indication and partitioning, RAN2 has made the following agreements in RAN2#116e:

Agreements: 1. Preamble partitioning is defined on a feature and/or feature combination basis. FFS on signalling. 2step RA and CE is excluded, if RAN1 decided to exclude 2. Preambles associated with a Rel-17 feature should never be chosen by legacy UEs in the case of RO sharing. 3. New feature and/or feature combination specific preambles can be defined in a) Separate time-frequency resources, not defined through legacy RRC signalling, b) Within the Contention free preamble resources (i.e. within the preambles not used for contention based) defined through legacy RRC signalling. FFS on c) Within the “not available” preambles defined at the end of a RO through the legacy totalNumberOfRA-Preambles 4. A common RRC CR capturing the signalling framework for RACH resource configuration across all the WIs should be used and this CR should be maintained as part of the common RACH agenda item. Each WI is expected to provide the necessary parameters to include in the signalling. 5. A common MAC CR capturing the changes to sections 5.1.1 and section 5.1.1a of the MAC spec can also be considered and if agreeable, this CR should also be maintained as part of the common RACH agenda item. 6. As a baseline, the RA procedure design for Rel-17 should adhere to the following general principles: a: Carrier selection (between NUL/SUL) should happen ahead of the initial RACH resource selection (i.e. feature combination is not considered in carrier selection). b: Initial RACH resource should be selected based on the selected carrier for the selected feature combination (i.e., selected slice, SDT or not, REDCAP or not etc). Only the RACH resource matching the feature and/or feature combination of current RACH procedure will be considered as available in the RACH resource selection. c: As a general rule, all RACH retransmissions (if any are needed, until RACH failure happens) shall be performed over the same RACH resources (and same carrier – NUL/SUL) as the one selected for initial RACH resource. However, we can discuss fallback on a case by case basis if there is a strong motivation and discuss them together in this AI.

Specifically, the general principle for RA procedure design is related to our discussion of Msg3 repetition. In addition, after RAN2#116e meeting, an LS with several questions was sent to RAN1, and we just received the reply from RAN1 in [1]. In this contribution, we further discuss the remaining issues and provide our views.

- Discussion

  - NUL and SUL

Based on RAN1’s reply LS in [1], RAN1 confirms it is feasible to support Msg3 repetition on both NUL and SUL, and separate RSRP thresholds will be needed for requesting Msg3 repetition. From RAN2 signalling point of view, the Msg3 repetition related parameters (e.g. RSRP threshold and RACH resource) will be defined in RACH-ConfigCommon, which is per UL carrier configured. So there is no additional signalling effort for supporting Msg3 repetition on both NUL and SUL.

Proposal 1: Confirm Msg3 repetition is supported on both NUL and SUL, and network can configure different RSRP thresholds for requesting Msg3 repetition on NUL and SUL.

  - Group B preambles

Support of Group B with Msg3 repetition has been discussed in RAN1 and RAN2, based on RAN1’s reply LS in [1], majority companies see the necessity of this feature, but no consensus was reached in RAN1.  According to the discussion in RAN1, the proponent mainly concerns on the large payload size for Msg3 repetition. From RAN2 perspective, one use case of Group B is RRCResume with full resumeID, and it is controlled by network (based on RA range). If UE is requested to use large RRCResume message, we think the UE should also be allowed to trigger Msg3 repetition when it is near the cell edge. In addition, Msg3 repetition can also be helpful for data transmission (e.g. SDT). We see no reason to preclude Group B with Msg3 repetition, and it is up to network to decide whether to configure Group B together with Msg3 repetition.

Proposal 2: Group B preambles with Msg3 repetition is supported, it is up to network to decide whether to configure Group B together with Msg3 repetition.

And follow RAN1 reply LS, we can introduce separate parameters for this scenario:

Proposal 3: If Group B preambles with Msg3 repetition is configured, network can configure separate parameters for requesting Msg3 repetition, including ra-Msg3SizeGroupA, messagePowerOffsetGroupB and numberOfRA-PreamblesGroupA.

  - RA-type selection

In Rel-16, the RACH procedure involves the following steps in that order:

- RACH triggering;

- Carrier selection (NUL or SUL);

- RA-Type selection and RA specific variables are initialized;

- SSB selection and RACH resource selection based on selected SSB;

- Msg1/MsgA transmission/retransmission.

Based on RAN1/2 agreements, Msg3 repetition is applicable to all cases that trigger 4-step CBRA procedure, but it is not applicable to 2-step RA. In addition, RAN2 agrees to introduce a separate RSRP threshold for requesting Msg3 repetition, only if the RSRP of serving cell/SSB is below the threshold, UE may request Msg3 repetition.

As mentioned above, Msg3 repetition is only applicable to 4-step RA, so it is reasonable to check CE necessity only if 4-step RA is selected (i.e. after RA-Type selection). More detailed, following principles should be considered when network enables 2-step RA or CE.

- Principle 1: If only 2-step RA configuration is signaled, network is not allowed to enable Msg3 repetition (does not configure RACH resource for requesting Msg3 repetition)

- Principle 2: If both 2-step RA configuration and 4-step RA configuration are signaled, the RSRP threshold used for requesting Msg3 repetition should be configured lower than msgA-RSRP-Threshold-r16.

Principle 1 is straight forward, for Principle 2, in theory, 2-step RA is used when UE is at cell center (with very good quality), while msg3 repetition is used to the solve the PUSCH transmission problem for cell edge UEs, So when UE fulfills 2-step threshold, it does not make sense to fulfill the threshold defined for requesting Msg3 repetition.

Proposal 4: Agree the following principles, and capture them in stage 3 specification.

- Principle 1: If only 2-step RA configuration is signaled, network is not allowed to enable Msg3 repetition (does not configure RACH resource for requesting Msg3 repetition)

- Principle 2: If both 2-step RA configuration and 4-step RA configuration are signaled, the RSRP threshold used for requesting Msg3 repetition should be configured lower than msgA-RSRP-Threshold-r16.

  - SSB selection and CE determination

Considering RAN2 agrees to introduce a separate RSRP threshold for requesting Msg3 repetition. So on top of steps of legacy RACH procedure (shown in section 2.3), we need to discuss when (in which step) the UE should estimate this new RSRP threshold, in order to determine the necessity of Msg3 repetition.

Before going into the detail, we notice the RAN1 LS [1] also includes the following recommendation:

- From RAN1 perspective, it can be beneficial to separately configure rsrp-ThresholdSSB for requesting Msg3 PUSCH repetition with shared RO on a given UL carrier.

Assuming RAN2 agree to introduce a separate rsrp-ThresholdSSB for requesting Msg3 repetition, then for a given carrier, there will be 3 RSRP thresholds:

- legacy rsrp-ThresholdSSB

  - used to select SSB and corresponding RACH resource;

- new rsrp-ThresholdSSB-Msg3Rep

  - used to select SSB for requesting Msg3 repetition;

- new rsrp-Threshold-Msg3Rep

  - used to determine whether Msg3 repetition is needed.

In our view, the motivation of introducing separate rsrp-ThresholdSSB for Msg3 repetition, is that UE can have more opportunity to select good beams for requesting Msg3 repetition. An example is given in below figure: (The green line represents the legacy threshold i.e. rsrp-ThresholdSSB, the red line represents the newly added rsrp-ThresholdSSB for requesting Msg3 repetition, and the dotted blue line represents the RSRP threshold used to determine the necessity of Msg3 repetition. )

Figure 1 An example of SSB selection under multiple RSRP thresholds

If only legacy rsrp-ThresholdSSB can be configured, based on current spec, if no SSB above rsrp-ThresholdSSB, UE will select any SSB. Then it is possible to select an SSB (i.e. SSB#1 or SSB#4) that is not good enough. But if separate rsrp-ThresholdSSB can be configured (i.e. red line), then the UE is able to first select SSB#2 or SSB#3, and then requesting Msg3 repetition. The mechanism is similar to BFR, that network can configure separate rsrp-ThresholdSSB for BFR RACH procedure.

So for flexibility, we suggest RAN2 to follow RAN1 guidance and introduce separate rsrp-ThresholdSSB for requesting Msg3 repetition, and the new threshold is used when UE is capable of Msg3 repetition, and when the cell has enabled Msg3 repetition.

Proposal 5: Introduce separate rsrp-ThresholdSSB for requesting Msg3 repetition.

Proposal 6: The new rsrp-ThresholdSSB threshold is used (instead of legacy rsrp-ThresholdSSB) when the UE supports Msg3 repetition and the cell enables Msg3 repetition.

Regarding the order between SSB selection and CE determination, we suggest to first select SSB, and evaluate Msg3 repetition criterion then. The UE behaviour can be:

1>	if at least one of the SSBs with SS-RSRP above Msg3 repetition specific rsrp-ThresholdSSB is available:

2>	select an SSB with SS-RSRP above the rsrp-ThresholdSSB.

1>	else:

2>	select any SSB.

1>	if RSRP of selected SSB above rsrp-Threshold-Msg3Rep (i.e. Msg3 repetition criterion is met):

2>	select the RACH pool configured for Msg3 repetition.

1>	else:

2>	select the RACH pool not configured for Msg3 repetition.

Proposal 7: UE first selects SSB (based on Msg3 repetition specific rsrp-ThresholdSSB), and then determines whether Msg3 repetition is needed or not.

  - Msg1 retransmission

One thing we need to discuss, is whether the UE can switch from CE to non-CE (or non-CE to CE) during Msg1 retransmission?

According to the agreement made in RACH partitioning session (see below). Which means UE is not expected to change its RACH resource pool in each Msg1 retransmission in one RACH procedure. And the decision can be revisited if problem is found.

c: As a general rule, all RACH retransmissions (if any are needed, until RACH failure happens) shall be performed over the same RACH resources (and same carrier – NUL/SUL) as the one selected for initial RACH resource. However, we can discuss fallback on a case by case basis if there is a strong motivation and discuss them together in this AI.

Different from RedCap identification, RAN slicing and SDT, for Msg3 repetition in CE, the UE determines the necessity of Msg3 repetition based on the RSRP measurement results. In legacy RACH procedure, UE is required to perform SSB measurement during RACH procedure, and evaluate the RSRP thresholds before each Msg1 retransmission. So if the UE moves, it is possible the decision on Msg3 repetition may change. If we disallow the change, there may be performance impact. For instance, upon RACH trigger, UE first decides not to use Msg3 repetition (because its RSRP above rsrp-Threshold-Msg3Rep), and the RSRP gets worse, then during Msg1 retransmission, it makes sense to allow UE to request Msg3 repetition to improve the RACH successful rate.

However, RAN2 discussed the similar issue for carrier selection (NUL or SUL) and RA-type selection (2-step RA or 4-step RA), like whether UE can switch from NUL to SUL, or from 2-step RA to 4-step RA upon Msg1 retransmission, and the conclusion is “No”.

So far, both options have Pros and Cons, so we suggest to RAN2 to discuss this issue and make a decision, the outcome of this issue will impact the discussion on RACH identification and partitioning.

Proposal 8: RAN2 to discuss whether UE can switch from CE (i.e. requesting Msg3 Repetition) to non-CE (i.e. not requesting Msg3 repetition), or vice versa upon Msg1 retransmission.

In addition, we need to further discuss Msg1 retransmission in case of 2-step RA. In legacy procedure, once 2-step RA is selected, the UE is now allowed to change the RA-type unless reaches msgA-TransMax. Then for Msg3 repetition, can UE change the RA-type in order to use Msg3 repetition upon Msg1 repetition?

For instance, assuming both 2-step RA and 4-step RA configurations are signaled by network, and the UE first selects 2-step RA. Later, during MsgA retransmission, if the RSRP drops below Msg3 repetition requesting threshold (e.g. rsrp-Threshold-Msg3Rep), what should UE do? So far, two options are on the table:

- Option 1: Ignore Msg3 repetition criterion and continue with 2-step RA retransmissions until max retransmission is reached; (same as legacy behaviour)

- Option 2: Switch to 4-step RA immediately, and requesting Msg3 repetition by using the RACH resources configured for Msg3 repetition.

Among the two options, we slightly prefer option 1, because it is simpler and has less specification work. For Option 2, it looks flexibility, but it is more complex, and we need to further discuss whether UE can switch back to 2-step RA once RSRP becomes better.

Proposal 9: Once 2-step RA is triggered, the UE ignores the Msg3 repetition configuration. UE cannot change the RA type unless max MsgA retransmission is reached.

- Conclusion and proposals

RAN2 is kindly asked to discuss and adopt the following proposal:

Proposal 1: Confirm Msg3 repetition is supported on both NUL and SUL, and network can configure different RSRP thresholds for requesting Msg3 repetition on NUL and SUL.

Proposal 2: Group B preambles with Msg3 repetition is supported, it is up to network to decide whether to configure Group B together with Msg3 repetition.

Proposal 3: If Group B preambles with Msg3 repetition is configured, network can configure separate parameters for requesting Msg3 repetition, including ra-Msg3SizeGroupA, messagePowerOffsetGroupB and numberOfRA-PreamblesGroupA.

Proposal 4: Agree the following principles, and capture them in stage 3 specification.

- Principle 1: If only 2-step RA configuration is signaled, network is not allowed to enable Msg3 repetition (does not configure RACH resource for requesting Msg3 repetition)

- Principle 2: If both 2-step RA configuration and 4-step RA configuration are signaled, the RSRP threshold used for requesting Msg3 repetition should be configured lower than msgA-RSRP-Threshold-r16.

Proposal 5: Introduce separate rsrp-ThresholdSSB for requesting Msg3 repetition.

Proposal 6: The new rsrp-ThresholdSSB threshold is used (instead of legacy rsrp-ThresholdSSB) when the UE supports Msg3 repetition and the cell enables Msg3 repetition.

Proposal 7: UE first selects SSB (based on Msg3 repetition specific rsrp-ThresholdSSB), and then determines whether Msg3 repetition is needed or not.

Proposal 8: RAN2 to discuss whether UE can switch from CE (i.e. requesting Msg3 Repetition) to non-CE (i.e. not requesting Msg3 repetition), or vice versa upon Msg1 retransmission.

Proposal 9: Once 2-step RA is triggered, the UE ignores the Msg3 repetition configuration. UE cannot change the RA type unless max MsgA retransmission is reached.

- Reference

[1] R1-2110585 Reply LS on Msg3 repetition in coverage enhancement    From RAN1, To RAN2