3GPP TSG-RAN WG2 #116e	R2-2109905

Electronic meeting, 2021-11-01 - 2021-11-12

Agenda Item:	8.7.2.4

Source:	Ericsson

Title:	Aspects for QoS management with SL relay

Document for:	Discussion, Decision

# 1	Introduction

One objective of the SL relay WI is to specify mechanisms for E2E, i.e. PC5 and Uu, QoS management for L2 relay [1]. In this paper, we discuss QoS aspects for L2 relay.

# 2	Discussions

The general QoS handling framework for L2 UE-to-Network Relay has been studied in SI phase [2]. The conclusion is that the gNB implementation can handle the QoS breakdown over Uu and PC5 for end-to-end QoS enforcement, and this breakdown can be flexibly tailored to the AS conditions on SL and Uu. Remote UE operates using the configuration provided in SIB or dedicated RRC signaling with overall better QoS performance than using pre-configuration. QoS can be enforced for each E2E bearer as the gNB can decide whether an E2E bearer is admitted or not depending on the current radio situation.

The QoS framework for L2 UE-to-Network Relay has been studied in SI phase.

In our view, we also think gNB should have full control on QoS configuration such as QoS breakdown over Uu and PC5 and bearer to Uu/PC5 RLC channel mapping in L2 relay scenario, how to do the QoS configuration should be up to gNB implementation. Therefore we propose to confirm the SI conclusion.

Confirm that how to do the QoS configuration in L2 relay scenario is up to gNB implementation.

In RAN2#115, it has been agreed that gNB should configure the relay UE and the remote UE with the PC5 PDB for PC5 hop of relay traffic. One issue is in what granularity the PC5 PDB should be configured. PDB is a QoS characteristics associated with a QoS flow, however, Uu QoS flow and Uu QoS bearer of relay traffic is blind to relay UE thus it does not know how to apply the PC5 PDB if it is configured on QoS flow or QoS bearer level. One feasible option is to configure the relay UE with the PC5 PDB on PC5 RLC bearer level. Similarly, for remote UE, the PC5 PDB is also configured on PC5 RLC bearer level, as Uu QoS flow and Uu QoS bearer of relay traffic is end to end thus we cannot split PDB for each hop on QoS flow or QoS bearer level.

The PC5 PDB for PC5 hop of relay traffic is configured on PC5 RLC bearer level.

Besides, some companies also proposed that gNB should configure the relay UE and the remote UE with the PC5 PER and PC5 priority for PC5 hop of relay traffic. In our understanding, there is no benefit to configure PC5 PER for relay traffic as PER is not used at AS layer (neither in resource allocation nor in grant generation), i.e., there is no way to enforce PER over PC5, thus there is no need to configure PC5 PER for PC5 hop of relay traffic. For the PC5 priority, we agree this is needed, however this information can be conveyed as part of SL RLC bearer configuration as in the legacy, i.e., no need to explicitly convey it.

There is no need to configure PC5 PER for PC5 hop of relay traffic.

PC5 priority for PC5 hop of relay traffic can be conveyed as part of SL RLC bearer configuration as in the legacy.

One open issue is the handling in case PC5 RLC channels with different E2E QoS are mapped to the same Uu RLC channel. The concern here is that this may result in loss of QoS differentiation for the PC5 RLC channels on the Uu link especially for UL where the UE determines which Uu RLC channel to transmit w/o caring the PC5 RLC channels mapped to that Uu RLC channel. In our understanding, this can be resolved by proper gNB implementation, e.g. gNB may only map PC5 RLC channels with different E2E QoS to the same Uu RLC channel if the QoS breakdown over Uu is the same for these PC5 RLC channels so that they can be treated equally over Uu.

Mapping PC5 RLC channels with different E2E QoS to the same Uu RLC channel may result in loss of QoS differentiation on the Uu link.

Proper gNB implementation can avoid the impact from loss of QoS differentiation on the Uu link.

On the other hand, remote UE traffic and relay UE’s own traffic should not be multiplexed in the same Uu RLC bearer of the relay UE, as remote UE’s traffic has PC5/Uu adaptation layer header while relay UE’s own traffic does not have Uu adaptation layer, multiplexing SDUs with the adaptation layer and SDUs without adaptation layer in the same Uu RLC bearer is infeasible since the gNB will not be able to parse the SDUs correctly.

Remote UE traffic and relay UE’s own traffic should not be multiplexed in the same Uu RLC bearer.

Another remained open issue is whether remote UE can be configured to use configured grant type 1. In our view, using configured grant type 1 will lead to that the remote UE cannot be scheduled with (additional) retransmission(s) as it cannot receive DCI from gNB, therefore we should not allow remote UE to use configured grant type 1.

Remote UE is not allowed to use configured grant type 1 in Rel-17.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	The QoS framework for L2 UE-to-Network Relay has been studied in SI phase.

Observation 2	Mapping PC5 RLC channels with different E2E QoS to the same Uu RLC channel may result in loss of QoS differentiation on the Uu link.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Confirm that how to do the QoS configuration in L2 relay scenario is up to gNB implementation.

Proposal 2	The PC5 PDB for PC5 hop of relay traffic is configured on PC5 RLC bearer level.

Proposal 3	There is no need to configure PC5 PER for PC5 hop of relay traffic.

Proposal 4	PC5 priority for PC5 hop of relay traffic can be conveyed as part of SL RLC bearer configuration as in the legacy.

Proposal 5	Proper gNB implementation can avoid the impact from loss of QoS differentiation on the Uu link.

Proposal 6	Remote UE traffic and relay UE’s own traffic should not be multiplexed in the same Uu RLC bearer.

Proposal 7	Remote UE is not allowed to use configured grant type 1 in Rel-17.

# References

[1] RP-210893, “New WID on NR Sidelink Relay”

[2] TR 38.836, v17.0.0, “Study on NR sidelink relay; (Release 17)”