+3GPP TSG-RAN WG2 #116-e	R2-2109912

Electronic meeting, November 1st – 12th 2021

Agenda Item:	9.1.3

Source: 	Ericsson

Title:	Analysis of Rmax based solution and carrier-based solution

Document for: 	Discussion, Decision

# Introduction

In Rel-17 one of the objectives in the WID is:

- Introduce support for NB-IoT carrier selection based on the coverage level, and associated carrier specific configuration (e.g. maximum repetitions UL/DL, DRX configurations, etc.). [NB-IoT] [RAN2, RAN3]

In RAN2#111-e the below agreements were made:

Agreements - Paging carrier selection Improvements based on CE level is considered - Paging carrier selection Improvements based on DRX cycle may be considered - whether DRX cycle is considered as part of CE level (Rmax) or can be also considered separately - Enhancements for NPRACH Carrier selection carrier may be considered - Paging carrier selection Improvements solely based on WUS or GWUS is not considered - FFS service based

In RAN2#113-e

- Paging carrier selection Improvements based on CE level is considered - Paging carrier selection Improvements based on DRX cycle may be considered - whether DRX cycle is considered as part of CE level (Rmax) or can be also considered separately - Paging carrier selection Improvements solely based on WUS or GWUS is not considered - FFS service based - Select between one of the options: - Option 1: UE selects a paging carrier based on a rule configured by the network - Option 2: NW configures a specific paging carrier Working assumption: For both options, when coverage changes, mechanism that requires UE to report the update of coverage is not introduced.

In RAN2 #114-e

- Agreements - Rel-17 paging carriers and the legacy paging carriers should be exclusive. - RAN2 assumes S1AP/NGAP update is not needed. - Carrier selection criteria does not include power boosting or service - FFS: For option 1, whether DRX can be part of the carrier selection criteria - Rel-17 paging carrier configuration is provided in broadcast signalling. - Select between the following sub-options: - Option 1c: Network enables UE to select a Rel-17 paging carrier by providing the coverage information (CEL/Rmax) for the carrier selection to the UE in dedicated signalling - Option 2a: NW indicates the carrier to use explicitly via dedicated signalling based on information determined within the NW. - FFS for both options whether there is a report from the UE to suggest a carrier or provide a metric report - Working assumption: UE metric for determining carrier suitability and selection is based on measured NRSRP. FFS whether to use a hysteresis/longer averaging/timer - For option 1, upon cell change, FFS: - Alt 1: based on previously determined CEL and broadcasted paging carrier configuration in the new cell. - Alt 2: UE needs to perform fallback mechanism. - For option 2, upon cell change, UE needs to perform fallback mechanism. - Whenever the R17 coverage based carrier criteria is met, UE uses the R17 coverage based carrier, otherwise UE should use the fallback mechanism - For both options, fall back carrier is legacy paging carrier based on UE_ID

In RAN2 #115-e

- Agreements - Support coverage or carrier specific DRX configurations, FFS details. - UE capability for Rel-17 paging carrier selection should be introduced - UE metric for determining carrier suitability and selection is based on NRSRP. - Use a hysteresis/longer averaging/timer for UE metric based on NRSRP. - FFS whether to introduce new UE report and/or whether to mandate support of existing Msg5 reporting.

In this contribution we further clarify some key aspects for carrier selection and give a summary of comparison between option 1 and option 2.

# Discussion

## UE report

In RAN2#115-e, UE report remained as an FFS:

- FFS whether to introduce new UE report and/or whether to mandate support of existing Msg5 reporting.

In our view, we can reuse msg5; however we can make it conditional such that UE supporting carrier selection based upon coverage mandatorily provides the report whereas for other cases it is Optional.

Proposal 1	Msg5 based reporting can be reused by appending conditional tag.

## Signaling

RRC Release (CEL/Rmax)

Paging (CEL/Rmax)

Provide CEL/Rmax

From signaling aspect, there is not much difference between option 1 and option 2.

In RAN2#114-e, a consensus on S1AP/NGAP is reached:

- RAN2 assumes S1AP/NGAP update is not needed.

For both option 1 and option 2, only RRC signaling update is needed.

Proposal 2	For both options, RRCConnectionRelease-NB update is needed to include information of CEL/Rmax or dedicated paging carriers.

Proposal 3	For both options, UEPagingCoverageInformation-NB update is needed to include information of CEL/Rmax or dedicated paging carriers between MME and eNB.

Proposal 4	For the detailed signaling aspect for paging carrier selection, it should wait until paging carrier selection mechanism based on either option 1c or option 2a is determined.

## Load balance

If paging carrier selection is purely based on coverage information, i.e., not considering other factors, e.g., DRX configuration. With option 1, for load balance between Rel-17 paging carriers and legacy paging carriers, it can be achieved by SIB configuration. For load balance among Rel-17 paging carriers with different CE levels, it can be achieved together by SIB configuration and NW dynamic control. As NW will decide the coverage information to the UE, NW has the possibility to assign a higher CEL/Rmax to the UE for load balance reason. For load balance among the Rel-17 paging carriers within same CE level, UE_ID based method can be used as in legacy paging carrier load balance scheme.

However, if paging carrier selection is to consider other factors, e.g., DRX configuration, then 2-level paging carrier selection may be adopted. If so, for load balance among Rel-17 paging carriers within same CE level, in real implementation, the number of paging carrier is limited, then the DRX configuration will be a major factor to choose paging carriers within this CE level, and there will be less room left for UE_ID based load balance. On the other hand, UE need to implement the paging carrier selection jointly considering both CEL/Rmax and DRX configuration, which will complex UE implementation.

For option 2, NW can dynamically and combinedly control load balance among Rel-17 paging carriers with different CE levels and also among Rel-17 paging carriers within same CE level. Moreover, except coverage information and DRX configuration, NW has other observability such as UE paging performance, paging success rate KPIs per carrier, UL RSSI, quality report of different carriers etc, which can be utilized additionally for load balancing. From UE’s perspective, there is no UE implementation complexity as it will directly use the paging carrier indicated by NW.

Therefore, from load balancing perspective, option 2 is better for achieving load balancing, and it’s extendable for future improvement by NW implementation.

Observation 1	From load balancing perspective, option 2 is better for achieving load balancing, and it’s extendable by NW implementation.

In previous meeting, there are views that option 2 cannot handle the case when NW removes a carrier from the list for a certain Rmax. This is not true, first of all, the possibility of system information change is small, and if it happens, UE can fall back to legacy paging carrier, which is easy and straightforward. On the other hand, when there is paging or direct indication on SIB change, option 1 cannot use the former paging carrier to receive this paging or direct indication message, it should also need to fall back to legacy paging carrier. The only difference is that, after this SI change paging or direct indication message, option 1 can choose another paging carrier from the list for the former certain Rmax if UE’s coverage level is not changed, while for option 2, UE should stick to the legacy paging carrier. After all, as the SI change possibility is small, this should not be a problem.

Another solution for this problem for option 2 is that it can simply be addressed by assigning an index number to each carrier so that the NW can assign the index number to a UE and when a carrier has to be replaced with another one only the mapping between the index value and the paging carrier needs to be changed. The mapping can be broadcasted in the serving cell.

Observation 2	Removing a paging carrier will impact both solutions. Fallback mechanism can be applied in such case. However, change in system information is rare.

## Cell change

In RAN2#114-e, regarding cell change, the following agreements are achieved. A consensus is reached for option 2, while for option 1, there is an FFS left:

- For option 1, upon cell change, FFS:

- Alt 1: based on previously determined CEL and broadcasted paging carrier configuration in the new cell.

- Alt 2: UE needs to perform fallback mechanism.

In previous meeting, some companies assume that there is high possibility that UE’s coverage is not changed even cell is changed, and the previously determined CEL can be used in the new cell. However, we think that possibility can not be predictable, there is no guarantee that the UE will have the same CEL/Rmax in another cell. If the required Rmax in the new cell is larger than the previously determined Rmax from last cell, then UE cannot be paged in the new cell using the previously determined Rmax, and it need to fall back to legacy paging carrier later. In this case, there will be paging latency and resource waste. Moreover, if DRX is to be considered, as the number of paging carriers is limited, and different cells may have different paging configurations, even if these two cells both have paging carriers with similar Rmax configuration, they may not have similar DRX configuration. Further, the communication between the legacy cell and the new cell maybe needed before making the assessment regarding if the previously determined CEL can be used in the new cell, which will make things even more complex

Thus, we think that for option 1, upon cell change, Alt 1 will make things complex considering the limited benefit UE may get. So, Alt 2 is suggested to be used upon cell change.

Proposal 5	For option 1, upon cell change, Alt 2 should be adopted: UE needs to perform fallback mechanism.

# Conclusion

In the previous sections we made the following observations:

Observation 1	From load balancing perspective, option 2 is better for achieving load balancing, and it’s extendable by NW implementation.

Observation 2	Removing a paging carrier will impact both solutions. Fallback mechanism can be applied in such case. However, change in system information is rare.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Msg5 based reporting can be reused by appending conditional tag.

Proposal 2	For both options, RRCConnectionRelease-NB update is needed to include information of CEL/Rmax or dedicated paging carriers.

Proposal 3	For both options, UEPagingCoverageInformation-NB update is needed to include information of CEL/Rmax or dedicated paging carriers between MME and eNB.

Proposal 4	For the detailed signaling aspect for paging carrier selection, it should wait until paging carrier selection mechanism based on either option 1c or option 2a is determined.

Proposal 5	For option 1, upon cell change, Alt 2 should be adopted: UE needs to perform fallback mechanism.

# References

RP-201306, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, RAN #88e, Electronic Meeting, June 29th – July 3rd, 2020.