3GPP TSG RAN WG2#116-e                                                                   R2-2110010

Electronic, Nov 1 – Nov 12, 2021

Source:	CATT

Title:	Further Analysis on PSCell MHI Enhancement

Agenda Item:	8.13.2.3

Document for:	Discussion and Decision

Introduction

In previous meeting we discussed many issues about PSCell MHI Enhancement, and has reach some agreements for baseline [1],

SN Related MHI Information:

2	RAN2 to confirm that the PSCell transition is part of MHI.

3	PSCell MHI is reported only to PCell.

4	UEInformationResponse message is used to convey the PSCell MHI to the MN.

5	Take Option 1 ‎(PSCell MHI nested within the PCell MHI) as baseline.

In this contribution, we intend to discuss the rest detailed issues about PSCell MHI enhancement.

Discussion

The recording of MHI of PSCell

Since the solution of “PSCell MHI should be nested within the PCell MHI” is agreed as the baseline in last meeting, the recording action of UE and the format of record entries for PSCells and PCells could be further defined based on the baseline.

Recording action of UE

For recording the PSCell, it is already agreed that “If PSCell MHI is introduced, at least include PSCell ID (may include CGI or frequency+PCI) and the time UE stayed in each PSCell into PSCell MHI.” [1]. But only recording the time UE stayed in each PSCell cannot exactly reflect in which time UE stayed in the PSCell when UE stayed in the specific PCell. Some cases such as PSCell release, PSCell addition cannot be reflected. Since the node needs to use the continuous information to estimate the UE mobility state otherwise the estimation is not correct because the network may estimate the UE mobility based on two PSCells which are not continuous in the mobility history [2], it is proposed that the cases of PSCell release, PSCell addition should be reflected.

Proposal 1: The PSCell addition and PSCell release should be reflected in the PSCell history information.

Another case is UE handover to the target PCell without PSCell change. Even the PSCell is not changed, when the UE handover to another PCell, the recording of PSCell needs to be cut off and set to the entries of different PCells with the time UE stayed in each PCell.

Proposal 2: The information about the same PSCell should be respectively nested within different PCells for the case of UE handover to the target PCell without PSCell change.

Format of Record Entries for PSCells and PCells

To reflect the proposed cases above, a better way is to design the PSCell history information as below:

Within each entry of PCell, a list of entries of PSCells and “no PSCell” markers are extended in chronological order:

- For PSCell, the PSCell ID and the time UE stayed in the cell should be recorded;

- For “no PSCell” marker, the time UE elapsed without PSCell should be recorded.

Note that if UE connected to the PCell without connecting PSCell in the beginning or in the end of the PCell, the “no PSCell” marker should also be recorded. In this recording format, the total time spent in a PSCell entry list should be equal to the time UE recorded in the corresponding PCell entry.

To use this format, the network could clearly know in which time the UE connected to a PSCell within the PCell, and know whether there is a UE handover to the target PCell without PSCell change.

Proposal 3: The time UE stayed in each PSCell and UE stayed without PSCell should all be recorded in each PCell entry in chronological order.

Variable of PSCells MHI

Since the solution of “PSCell MHI should be nested within the PCell MHI” is agreed as the baseline, the  PSCell MHI is a sub-level of PCell MHI and cannot be cut apart, therefore the original variable of PCell MHI can be used to include both the PCell and PSCell MHI information.

Proposal 4: The original variable of PCell MHI is used to include both the PCell and PSCell MHI information.

The Appropriate DC Scenarios for PSCell MHI

Since the appropriate DC scenarios for PSCell MHI can be discussed separately from the technical issues, it is proposed to postpone scenario discussions, so we did not discuss it in lasting meetings. And for detail design of stage-3, it is needed to re-discuss this issue.

It is RAN3 agreement that “Enhancement of UE History Information for Secondary Node applies to all MR-DC scenario”. But the scope of UE history enhancement is only about EN-DC in the WID [3]. For the description in the WID, we think it is mainly focus on the solution about how to perform the UE history enhancement for DC scenario, i.e. the recording of PSCell history information. Therefore together with the RAN3 agreement which reflects the requirement of RAN3 SON purpose, it is proposed to extend the WID scenario scope of UE history information to “UE history information in EN-DC scenarios”.

Proposal 5: RAN2 re-discusses the DC scenarios for PSCell MHI and extends the WID scenario scope of UE history information to include more DC scenarios other than EN-DC.

Then we could discuss the appropriate DC scenario includes (NG)EN-DC, NR-DC and NE-DC. Since NE-DC is also not a commonly used scenario, it is suggest only enhance the scenarios of (NG)EN-DC and NR-DC. To perform the enhancement, if UE changes/enters/leaves a SgNB, the PSCell related information should be recorded. The modification involves both the TS36.331 and TS 38.331.

Proposal 6: The extended DC scenarios for UE history information include (NG)EN-DC and NR-DC.

Capability of PSCell MHI

In LTE, the UHI is one of the optional features without UE radio access capability parameters [2]:

It is optional for UE to support the storage of mobility history information and the reporting in UEInformationResponse message as specified in TS 36.331 [5].

In NR, it is similar as in LTE [3]:

Mobility history information storage It is optional for UE to support the storage of mobility history information and the reporting in UEInformationResponse message as specified in TS 38.331 [9]. UE is not required to report this capability.

This optional feature is only applied for MN, and to enhanced the support of PSCell record and report, an parallel optional feature could be defined, for (NG)EN-DC in LTE and for NR-NR DC in NR respectively.

Proposal 7: Define optional feature for PSCell mobility history information storage and reporting.

Conclusion

According to the discussion in section 2, we propose:

Proposal 1: The PSCell addition and PSCell release should be reflected in the PSCell history information.

Proposal 2: The information about the same PSCell should be respectively nested within different PCells for the case of UE handover to the target PCell without PSCell change.

Proposal 3: The time UE stayed in each PSCell and UE stayed without PSCell should all be recorded in each PCell entry in chronological order.

Proposal 4: The original variable of PCell MHI is used to include both the PCell and PSCell MHI information.

Proposal 5: RAN2 re-discusses the DC scenarios for PSCell MHI and extends the WID scenario scope of UE history information to include more DC scenarios other than EN-DC.

Proposal 6: The extended DC scenarios for UE history information include (NG)EN-DC and NR-DC.

Proposal 7: Define optional feature for PSCell mobility history information storage and reporting.

Reference

- Draft_R2-115e_Meeting_Report_v1.docx

- R2-2101253 Discussion on other SON aspects Huawei, HiSilicon

- RP-201281 Revised WID on enhancement of data collection for SON/MDT in NR and EN-DC