3GPP TSG-RAN WG2 Meeting #115-e		R2-2110041

Electronic, 1 November – 12 November, 2021

Agenda item:	8.13.2.1

Source:	Apple

Title:	UP measurements of HO interruption time

Document for:	Discussion and Decision

# 1	Introduction

The issue of UP measurements for Successful Handover Report, triggered by a RAN3 LS [2], was discussed in the last RAN2 meetings and agreed [1] to be addressed. In the present paper we provide our views on how to address the RAN3 requirements.

# 2   	Discussion

In their LS [2], RAN3 asked RAN2 to further study the introduction of User Plane measurements (e.g. user plane interruption time at HO) in for Successful Handover Report, including DAPS HO and CHO. The following agreement [1] was reached in RAN2#115:

Agreement:

1	UP measurements for Successful Handover Report will be introduced as RAN3 required. FFS the details.

We propose to structure the discussion as follows:

- First, discuss whether to support UP measurements in DL, UL or both.

- Then, agree at which protocol stack layer handover interruption time should be measured.

- Finally, decide on the metrics.

In this exercise we should of course ensure applicability of the solution to all the use cases (HO, DAPS HO, CHO).

Since we are discussing UE measurements, it of course makes the most sense to measure DL. UL measurements, if needed, can be performed by the network, which (if required) would be discussed in RAN3. Furthermore, typical traffic is usually DL dominated, so it is more likely that a UE would have DL traffic ongoing during a HO rather than UL. Therefore, we propose to focus on DL measurements only.

Proposal 1: to focus on DL handover interruption time measurements.

With regards to the question of which protocol stack layer to perform the measurements at, the candidates would be MAC and PDCP. Performing handover interruption time UP measurements in the MAC layer has the advantage of being more accurate. However, that option might be hard to realize for one of the important use cases – DAPS. That is because in DAPS there are two MAC entities, and it may not be trivial for a UE to get synchronized measurements between the two.

We therefore suggest adopting the option of measurements at the PDCP layer, as it would be easier for a UE implementation and would naturally cover all the use cases: regular HO, DAPS and CHO. Furthermore, measurements at the PDCP layer are closer to what a user experiences, which is what we should strive to optimize.

Proposal 2: to adopt measurements at the PDCP layer.

Regarding the measurement itself, one option, as suggested in [3], is to adopt as the measurement the time between the last packet received in the source cell and the first packet received in the target cell. This might be a useful metric, but it may be affected by the traffic patterns – that is, there is no guarantee that there are always DL packets queued in the buffer.

If the DL buffer in the source cell had no packets queued when the HO was triggered, that may artificially increase the measured HO interruption time and thus provide incorrect information. In other words, the last DL packet received in the source cell may have been received some time before the handover occurs and that time when there was no DL traffic would be counted towards the reported handover interruption time.

Observation 1: the time between the last packet received in the source cell and the first packet received in the target cell can be a useful metric, but it is affected by traffic patterns and in some cases may provide incorrect results.

We are generally supportive of the concept and think the idea should be explored further, while addressing the issue of dependency on traffic patterns. One option to do that is to define the measurement in such a way that it is only triggered when the time between the reception of the last packet in the source cell and the HO trigger is less than a certain predefined threshold. This will ensure that all the HO interruption time measurements collected are consistent and do not depend on traffic variations.

Proposal 3: to define the HO interruption time measurement as the time between the last packet received in the source cell and the first packet received in the target cell, measured at the PDCP layer and triggered only if the time between the reception of the last packet in the source cell and the HO trigger is less than a certain predefined threshold.

The exact value for the threshold can be further discussed.

# 3	Conclusions

Proposal 1: to focus on DL handover interruption time measurements.

Proposal 2: to adopt measurements at the PDCP layer.

Observation 1: the time between the last packet received in the source cell and the first packet received in the target cell can be a useful metric, but it is affected by traffic patterns and in some cases may provide incorrect results.

Proposal 3: to define the HO interruption time measurement as the time between the last packet received in the source cell and the first packet received in the target cell, measured at the PDCP layer and triggered only if the time between the reception of the last packet in the source cell and the HO trigger is less than a certain predefined threshold.

# 4	References

- R2-2108965	Report of [Offline-872][SONMDT] Logged MDT enhancements (Ericsson)

- R2-2106942	LS on UP measurements for Successful Handover Report (R3-212935; contact: Ericsson)

- R2-2108419	LS Reply On user plane masurements for successful handover report	Ericsson	discussion