3GPP TSG-RAN WG2 Meeting #116-e                                                        R2-2110092

Electronic, November 1 - November 12, 2021

Agenda item:	8.2.2.2

Source:	Apple

Title:	Simple MCG recovery procedure using deactivated SCG for Rel-17

Document for:	Discussion

# 1	Introduction

Post RAN2-115e email discussion [1] has in it a discussion on the UE initiated SCG re-activation procedure and the usage of this option for MCG link recovery when the SCG is deactivated. In this short paper we intend to show that SCG re-activation can be avoided for using the MCG link recovery even when the SCG is deactivated and intend to show that the existing spec (with minimal additions if needed) can handle this effectively for Rel-17 without any additional optimizations.

# 2   Discussion

The following are the broad steps the UE takes when in NR-DC (we limit to NR-DC for ease of discussion) when there is a MCG link failure. The preconditions for using SCG for recovery are that split SRB1 or SRB3 should be configured, and the NW should configure the UE for this recovery procedure (T316).

We assume the same pre-conditions apply for the SCG deactivated case of MCG link recovery.

- The UE detect the conditions that fulfill the link failure in MCG (T310/T312 based for eg)

- Suspends all SRB/DRB except SRB0 for MCG (no change to SCG RBs)

- Reset MCG MAC (no change to SCG)

- If the SCG is not suspended, the UE prepares the MCG failure information RRC message, with the relevant cause.

- Depending on whether split-SRB1 or SRB3 is configured, the UE submits the RRC message to lower layers on SCG for transmission.

## 2.1    Impact from deactivated SCG on RRC

It was discussed in RAN2-115e about the DRB handling for deactivated SCG (pasted below) and it can be noted that all the discussion is about DRB handling and data, while the SRB3 (if configured) or split-SRB1 can be assumed to be not changed. It is the view of RAN2 that without any UL grant, the UE cannot anyway use the SRB for sending anything even when the SRBs are not suspended.

- PDCP entity is not suspended at SCG deactivation for at least AM DRB. FFS for Stage-3 details - UL data processing is not prohibited during SCG deactivation for at least AM DRB. FFS for Stage-3 details - UL data transmission to SCG is prohibited during SCG deactivation. FFS for Stage-3 details - UE-initiated activation is still FFS.

Observation 1:  SRBs on SGC (split-SRB1 or SRB3) does not need to be suspended in SCG deactivation, as the UE will anyway not be able to use these without UL grants on SCG.

## 2.2    Impact from deactivated SCG on SCG MAC

Post RAN2-115e email discussion 213 on the MAC running CR [2] (shown below) shows that the MAC of the SCG is not reset during SCG deactivation. As can be seen in 2.1, the PDCP is not suspended, and no further action in MAC is needed as along the below actions are done in SCG MAC

- else if an SCG is configured to deactivated via RRC signaling: 2> deactivate all the SCells associated with the PSCell according to clause 5.9; 2> clear any configured downlink assignment and any configured uplink grant Type 2 associated with the PSCell respectively; 2> clear any PUSCH resource for semi-persistent CSI reporting associated with the PSCell; 2> suspend any configured uplink grant Type 1 associated with the PSCell; 2> flush all HARQ buffers associated with the PSCell; 2> cancel, if any, triggered consistent LBT failure for the PSCell. 2> PSCell is deactivated according to the timing defined in TS 38.xxx [xx], including: 3> not transmit SRS on the PSCell: 3> not transmit on UL-SCH on the PSCell: 3> not monitor the PDCCH on the PSCell.

Observation 2: SCG MAC is not  reset at SCG deactivation, and other than the requirement that the UE cannot monitor PDCCH on PSCell, there is nothing preventing the UE in SCG to initiate a RACH

Observation 3: If we allow UE to RACH on SCG during SCG deactivated state, the current procedure would allow the UE to trigger an SR or RACH for transmitting the MCG failure RRC message, and the UE would use the SCG C-RNTI for the RACH procedure.

Observation 4: Since the MN-SN configuration for the SCG would not be changed, the SN can handle the MSG3 containing the Ul DCCH message: MCG failure information from the UE without any change to the specification, i.e, pass it to the MN for either of split-SRB1 or SRB3 ) after decoding that this is a container from MN).

Observation 5: MN can then decide the next steps, again without additional spec changes, as SCG re-activation or NR_DC reconfig is all part of the current Rel-17 spec which handles SCG deactivation.

Based on the above, we propose the following as a response:

Proposal 1: UE can trigger a MCG failure information on split-SRB1 or SRB3 on the deactivated SCG. UE is allowed to RACH on the SCG for this purpose.

## 2.3	Anticipated Text proposal

- From the running CR in 38.331 on SCG deactivation #### 5.3.5.3 Reception of an RRCReconfiguration by the UE The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO or CPC): 1> if the RRCReconfiguration includes the scg-State: 2> consider the SCG to be deactivated; 1> else: 2> consider the SCG (if any) to be activated; NOTE: in SCG deactivated state, only the MCGFailureInformation is allowed on SRB3 (if configured) 1> if the RRCReconfiguration is applied due to a conditional reconfiguration execution upon cell selection performed while timer T311 was running, as defined in 5.3.7.3: ### 5.7.3b MCG failure information #### 5.7.3b.1 General Figure 5.7.3b.1-1: MCG failure information The purpose of this procedure is to inform the network about an MCG failure the UE has experienced i.e. MCG radio link failure. A UE in RRC_CONNECTED, for which AS security has been activated with SRB2 and at least one DRB setup or, for IAB, SRB2, may initiate the fast MCG link recovery procedure in order to continue the RRC connection without re-establishment. #### 5.7.3b.2 Initiation A UE configured with split SRB1 or SRB3 initiates the procedure to report MCG failures when neither MCG nor SCG transmission is suspended for the SRB, t316 is configured, and when the following condition is met: 1> upon detecting radio link failure of the MCG, in accordance with 5.3.10.3, while T316 is not running. Upon initiating the procedure, the UE shall: 1> stop timer T310 for the PCell, if running; 1> stop timer T312 for the PCell, if running; 1> suspend MCG transmission for all SRBs, DRBs, except SRB0, and, if any, BH RLC channels; 1> reset MCG MAC; 1> stop conditional reconfiguration evaluation for CHO, if configured; 1> stop conditional reconfiguration evaluation for CPC, if configured; 1> initiate transmission of the MCGFailureInformation message in accordance with 5.7.3b.4. NOTE: The handling of any outstanding UL RRC messages during the initiation of the fast MCG link recovery is left to UE implementation. #### 5.7.3b.3 Failure type determination The UE shall set the MCG failure type as follows: 1> if the UE initiates transmission of the MCGFailureInformation message due to T310 expiry: 2> set the failureType as t310-Expiry; 1> else if the UE initiates transmission of the MCGFailureInformation message due to T312 expiry: 2> set the failureType as t312-Expiry; 1> else if the UE initiates transmission of the MCGFailureInformation message to provide random access problem indication from MCG MAC: 2> if the random access procedure was initiated for beam failure recovery: 3> set the failureType as beamFailureRecoveryFailure; 2> else: 3> set the failureType as randomAccessProblem; 1> else if the UE initiates transmission of the MCGFailureInformation message to provide indication from MCG RLC that the maximum number of retransmissions has been reached: 2> set the failureType as rlc-MaxNumRetx: 1> else if the UE initiates transmission of the MCGFailureInformation message due to consistent uplink LBT failures on the MCG: 2> set the failureType as lbt-Failure; 1> else if connected as an IAB-node and the MCGFailureInformation message is initiated due to the reception of a BH RLF indication on BAP entity from the MCG: 2> set the failureType as bh-RLF. #### 5.7.3b.4 Actions related to transmission of MCGFailureInformation message The UE shall set the contents of the MCGFailureInformation message as follows: 1> include and set failureType in accordance with 5.7.3b.3; 1> for each MeasObjectNR configured by a measConfig associated with the MCG, and for which measurement results are available: 2> include an entry in measResultFreqList; 2> if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to ssb: 3> set ssbFrequency in measResultFreqList to the value indicated by ssbFrequency as included in the MeasObjectNR; 2> if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to csi-rs: 3> set refFreqCSI-RS in measResultFreqList to the value indicated by refFreqCSI-RS as included in the associated measurement object; 2> if a serving cell is associated with the MeasObjectNR: 3> set measResultServingCell in measResultFreqList to include the available quantities of the concerned cell and in accordance with the performance requirements in TS 38.133 [14]; 2> set the measResultNeighCellList in measResultFreqList to include the best measured cells, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows; 3> ordering the cells with sorting as follows: 4> based on SS/PBCH block if SS/PBCH block measurement results are available and otherwise based on CSI-RS; 4> using RSRP if RSRP measurement results are available, otherwise using RSRQ if RSRQ measurement results are available, otherwise using SINR; 3> for each neighbour cell included: 4> include the optional fields that are available. 1> for each EUTRA frequency the UE is configured to measure by measConfig for which measurement results are available: 2> set the measResultFreqListEUTRA to include the best measured cells, ordered such that the best cell is listed first using RSRP to order the cells if RSRP measurement results are available for cells on this frequency, otherwise using RSRQ to order the cells if RSRQ measurement results are available for cells on this frequency, otherwise using SINR to order the cells, based on measurements collected up to the moment the UE detected the failure, and for each cell that is included, include the optional fields that are available; 1> for each UTRA-FDD frequency the UE is configured to measure by measConfig for which measurement results are available: 2> set the measResultFreqListUTRA-FDD to include the best measured cells, ordered such that the best cell is listed first using RSCP to order the cells if RSCP measurement results are available for cells on this frequency, otherwise using EcN0 to order the cells, based on measurements collected up to the moment the UE detected the failure, and for each cell that is included, include the optional fields that are available; 1> if the UE is in NR-DC: 2> include and set measResultSCG in accordance with 5.7.3.4; 1> if the UE is in NE-DC: 2> include and set measResultSCG-EUTRA in accordance with TS 36.331 [10] clause 5.6.13.5; NOTE 1: The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported. NOTE 2: Field measResultSCG-Failure is used to report available results for NR frequencies the UE is configured to measure by SCG RRC signalling. NOTE 3: Field measResultSCG-EUTRA is used to report available results for E-UTRAN frequencies the UE is configured to measure by E-UTRA RRC signalling. 1> if SRB1 is configured as split SRB and pdcp-Duplication is not configured: 2> if the primaryPath for the PDCP entity of SRB1 refers to the MCG: 3> set the primaryPath to refer to the SCG. The UE shall: 1> start timer T316; 1> if SRB1 is configured as split SRB: 2> submit the MCGFailureInformation message to lower layers for transmission via SRB1, upon which the procedure ends; 1> else (i.e. SRB3 configured): 2> submit the MCGFailureInformation message to lower layers for transmission embedded in NR RRC message ULInformationTransferMRDC via SRB3 as specified in 5.7.2a.3. #### 5.3.10.3 Detection of radio link failure The UE shall: 1> if any DAPS bearer is configured and T304 is running: 2> upon T310 expiry in source SpCell; or 2> upon random access problem indication from source MCG MAC; or 2> upon indication from source MCG RLC that the maximum number of retransmissions has been reached; or 2> upon consistent uplink LBT failure indication from source MCG MAC: 3> consider radio link failure to be detected for the source MCG i.e. source RLF; 3> suspend the transmission and reception of all DRBs in the source MCG; 3> reset MAC for the source MCG; 3> release the source connection. 1> else: 2> during a DAPS handover: the following only applies for the target PCell; 2> upon T310 expiry in PCell; or 2> upon T312 expiry in PCell; or 2> upon random access problem indication from MCG MAC while neither T300, T301, T304, T311 nor T319 are running; or 2> upon indication from MCG RLC that the maximum number of retransmissions has been reached; or 2> if connected as an IAB-node, upon BH RLF indication received on BAP entity from the MCG; or 2> upon consistent uplink LBT failure indication from MCG MAC while T304 is not running: 3> if the indication is from MCG RLC and CA duplication is configured and activated for MCG, and for the corresponding logical channel allowedServingCells only includes SCell(s): 4> initiate the failure information procedure as specified in 5.7.5 to report RLC failure. 3> else: 4> consider radio link failure to be detected for the MCG, i.e. MCG RLF; 4> discard any segments of segmented RRC messages stored according to 5.7.6.3; NOTE: Void. 4> if AS security has not been activated: 5> perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';- 4> else if AS security has been activated but SRB2 and at least one DRB or, for IAB, SRB2, have not been setup: 5> store the radio link failure information in the VarRLF-Report as described in subclause 5.3.10.5; 5> perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'; 4> else: 5> store the radio link failure information in the VarRLF-Report as described in subclause 5.3.10.5; 5> if T316 is configured; and 5> if SCG transmission is not suspended; and 5> if neither PSCell change nor PSCell addition is ongoing (i.e. timer T304 for the NR PSCell is not running in case of NR-DC or timer T307 of the E-UTRA PSCell is not running as specified in TS 36.331 [10], clause 5.3.10.10, in NE-DC): 6> initiate the MCG failure information procedure as specified in 5.7.3b to report MCG radio link failure. 5> else: 6> initiate the connection re-establishment procedure as specified in 5.3.7. NOTE: Deactivation of the SCG does not result in SRB transmission getting suspended in the SCG

# 3	Conclusions

Observation 1:  SRBs on SGC (split-SRB1 or SRB3) does not need to be suspended in SCG deactivation, as the UE will anyway not be able to use these without UL grants on SCG.

Observation 2: SCG MAC is not  reset at SCG deactivation, and other than the requirement that the UE cannot monitor PDCCH on PSCell, there is nothing preventing the UE in SCG to initiate a RACH

Observation 3: If we allow UE to RACH on SCG during SCG deactivated state, the current procedure would allow the UE to trigger an SR or RACH for transmitting the MCG failure RRC message, and the UE would use the SCG C-RNTI for the RACH procedure.

Observation 4: Since the MN-SN configuration for the SCG would not be changed, the SN can handle the MSG3 containing the Ul DCCH message: MCG failure information from the UE without any change to the specification, i.e, pass it to the MN for either of split-SRB1 or SRB3 ) after decoding that this is a container from MN).

Observation 5: MN can then decide the next steps, again without additional spec changes, as SCG re-activation or NR_DC reconfig is all part of the current Rel-17 spec which handles SCG deactivation.

Proposal 1: UE can trigger a MCG failure information on split-SRB1 or SRB3 on the deactivated SCG. UE is allowed to RACH on the SCG for this purpose.

# 4	References

- R2-21xxxxx Email discussion [Post115-e][219][R17 DCCA] UE-initiated SCG activation (Huawei)