3GPP TSG-RAN WG2 Meeting #116 electronic            	                   R2-2110106

E-Meeting, 1 – 12 November 2021

Source:	vivo, Ericsson, InterDigital Inc., Lenovo, Motorola Mobility, CATT, ASUSTek

Title:	Discussion on SL-DRX for ProSe

Agenda Item:	8.15.2

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss whether/how to support SL-DRX for ProSe discovery and ProSe communication, including both the non-relaying case and the relaying case.

- Discussion

In RAN#93e, there was a discussion in [1] on whether SL-DRX should also be supported for ProSe communication and ProSe discovery. As seen from [2], there was no explicit conclusion or RAN guideline reached in RAN plenary, and this issue was finally left to RAN2 for the final decision.

Observation 1: Per discussion in RAN2 #93e, it is up to RAN2 to decide whether/how to support SL-DRX for ProSe communication and ProSe discovery.

In addition, during the RAN plenary discussion, companies commented that it has already been clear in the WID description that all of V2X, public safety and commercial use cases are within the scope of SL enh. WI, meaning that the support of ProSe communication/discovery is not excluded from SL-DRX design.

Observation 2: All V2X, public safety and commercial uses cases are within the scope of SL enh. WI, as clearly stated in the current WID.

In above Observation 2, the public safety and commercial uses cases are referring to the ProSe services specified in TS 23.304 [3], which can be further categorized into non-relay related ProSe discovery/communication and relay related ProSe discovery/communication as summarized in the following table (subclause numbers in the table referring to those in [3]):

Table 1. ProSe Services specified in [3]

| Non-Relaying related ProSe |  | Relaying related ProSe |  |
| --- | --- | --- | --- |
| ProSe Direct communication - Bcast (e.g. in 6.4.1) - Gcast (e.g. in 6.4.2) - Ucast (e.g. in 6.4.3) | Non-relay ProSe discovery - ProSe Direct discovery (e.g. for commercial use in 6.3.2.1) - Group Member Discovery (e.g. in 6.3.2.2) | ProSe relay communication (e.g. in 6.5) | ProSe relay discovery (e.g. in 6.3.2.3) |

Below, we discuss how to support SL-DRX for ProSe communication and ProSe discovery based on the ProSe services defined by [3], for the non-relaying case and the relaying case respectively.

2.1	SL-DRX for ProSe direct communication

Let’s start from ProSe direct communication, where data transfer is terminated among UEs in SL thus having nothing to do with SL relay. As specified in [3, 5.6.1], the QoS handling of ProSe direct communication reuses the PC5 QoS framework in TS 23.287 which was specified for NR PC5 reference point in R16 for NR V2X. Therefore, the PC5 QoS based SL-DRX framework concluded in SL enh. WI is able to be directly reused for the user plane data transfer in ProSe direct communication.

In addition, the PC5-S signalling and procedures specified in TS 23.287 for PC5 unicast link management in Rel-16 is also reused for the Unicast mode of ProSe direct communication, as also seen from [3]. Therefore, SL-DRX mechanism for PC5-S signalling (i.e. SL-SRB0/1/2) under the discussion in SL enh. WI can be directly reused for the Unicast mode of ProSe direct communication as well.

To this end, it is proposed that RAN2 confirms the SL-DRX design in SL enh. WI can be directly reused for ProSe direct communication w/o extra Spec impact needed, including both PC5 QoS based SL-DRX framework and SL-DRX mechanism for PC5-S signalling.

Proposal 1: RAN2 confirms that current SL-DRX design in SL enh. WI can be directly reused for ProSe direct communication (including both PC5 QoS based SL-DRX framework and SL-DRX mechanism for PC5-S signalling). No extra Spec impact is needed.

2.2	SL-DRX for non-relay ProSe discovery

According to [3], two models are supported for 5G ProSe discovery transfer, Model-A and Mode-B. There is also a new protocol stack introduced specifically for ProSe discovery (i.e. ProSe protocol), where different types of discovery messages are defined for different use cases, e.g. commercial cases, group discovery, relay discovery, etc.

From AS perspective, however, all discovery messages are actually transmitted in the following manners with only a slight difference located in what DST L2 ID is used:

- Broadcast/Groupcast-like discovery message transmission: this is mainly for the transfer of discovery announcement message in Model-A and transfer of discovery solicitation message in Model-B, where a Broadcast or Groupcast DST L2 ID is used for the discovery message transmission in the AS (as specified in [3, 5.1.2.1]).

- Unicast-like discovery message transmission: this is mainly for the transfer of discovery response message in Model-B, where the SRC L2 ID of the peer UE is used as the DST L2 ID for the discovery message transmission in the AS (as specified in [3, 6.3.2.1]).

Therefore, from the AS perspective, the transmission mechanism of the discovery message is conceptually quite similar to that of the DCR message which can also use either broadcast DST L2 ID or unicast DST L2 ID. Further considering that RAN2 concluded to have a similar SL-DRX design for SL broadcast and groupcast, it seems feasible to reuse the SL-DRX design for DCR message in SL enh. WI (e.g. default SL-DRX configuration) also to ProSe discovery message with no further Spec impact needed. This is proposed as follows.

Proposal 2: RAN2 confirms that the SL-DRX design for DCR message is reused for non-relay ProSe discovery. No extra Spec impact is needed.

2.3	SL-DRX for ProSe relay discovery/communication

It was agreed in SL relay WI to introduce a new SL-SRB (e.g. SL-SRB4) to carry the ProSe relay discovery message. Considering that the relay discovery messages and non-relay related discovery messages only differ in the message content with no big difference expected in the AS transmission schemes, both of them are expected to be mapped into the same SL-SRB (e.g. SL-SRB4) for AS transmission. Therefore, the conclusion on SL-DRX for non-relay ProSe discovery can be reused for ProSe relay discovery.

As to ProSe relay communication, it is only related to SL unicast between the Remote UE and Relay UE. In the SL enh. session, it was agreed in RAN2 #114e that for SL unicast the SL DRX configuration is configured to an RRC_CONNECTED TX UE by the network via dedicated signalling and then forwarded to the peer UE, as follows (below agreement 4) [4]. This means that the SL DRX configuration in this case is decided by the NW implementation based on the QoS of NR SL unicast communication.

Agreements on TX-UE centric or RX-UE centric DRX configuration determination

3:	In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other UE as Rx-UE, when Tx-UE is in-coverage and in RRC_CONNECTED state, Tx-UE may report the information received in signaling-1 (Rx->Tx) to the serving network.

4:	In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other as Rx-UE, when Tx-UE is in-coverage and in RRC_CONNECTED state, Tx-UE may obtain DRX configuration from dedicated RRC to generate signalling-2 (Tx->Rx).

5:	In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other as Rx-UE, when Rx-UE is in-coverage and in RRC_CONNECTED state, Rx-UE report the DRX configuration received in signalling-2 (Tx->Rx) to the serving network.

When both the Relay UE and the Remote UE are in RRC_CONNECTED, as the QoS for their communications in sidelink is also under the control of the gNB, it is feasible to reuse the above agreement 4 from SL enh. session and leave how to configure the SL DRX configuration in both directions (i.e. Relay  Remote and Remote  Relay) to NW implementation.

On the other hand, however, there might also be some SL relay specific aspects/scenarios that have not been considered in the SL enh. WI. For example, it is also allowed for an RRC_IDLE/INACTIVE Remote UE to connect with a Relay UE which needs to forward Uu RRC signalling specific to RRC_IDLE/INACTIVE for the Remote UE (e.g. paging message, SI, RRC setup/resume related messages, etc), and the Relay UE can be either in RRC_CONNECTED or in RRC_IDLE/INACTIVE. This is clearly a case that does not exist in the non-relaying related SL communication, and since it is related to the transfer of Uu RRC signalling in PC5, it is currently unclear how the Remote UE and/or the Relay UE (if in RRC_IDLE/INACTIVE) sets SL DRX configurations to meet the transmission requirements of these Uu RRC signalling which has no QoS requirements as UP data.  As another example, agreements 3 and 5 in the box above were made based on the assumption that the TX UE and RX UE may be served by different serving gNBs (thus reporting SL DRX assistance message and SL DRX configuration acquired from its peer UE/NW to its own gNB, respectively); however, this case is not supported in SL relay in this release where the remote UE and relay UE need to be served by the same gNB which knows their paring relationship and provides necessary SL configurations for both of them, so that it is not clear whether such agreements made in SL enh. session need to apply to SL relay also.

There might be other SL relay specific issues to consider, if one intends to apply SL-DRX to SL relay as well, not limited to the ones listed above. As a result, in order to make a reliable conclusion on whether such inter-WI feature of SL-DRX for SL relay can really be supported in this release, extensive discussions are inevitably needed to check such SL relay specific aspects/issues one by one. Such discussions themselves are not easy and can be quite time consuming, considering also potentially different views among companies on e.g., whether directly reusing current SL-DRX already covers each relay specific issue/aspect.

Considering that we are at the late stage of Rel-17 and there are still some essential designs not yet completed, there is not enough time left for such an inter-WI discussion in this release. To sum up, it is proposed that RAN2 to conclude not pursuing any SL relay specific SL-DRX design and thus not supporting SL-DRX for SL relay in this release.

Proposal 3: RAN2 concludes that SL-DRX for ProSe relay discovery and communication is not supported in this release.

Such an inter-WI feature of SL-DRX for SL relay may be further revisited in the next release (pending final decision in RAN plenary).

- Conclusion

In this contribution, SL-DRX for ProSe is discussed with the following observations and proposals:

Observation 1: Per discussion in RAN2 #93e, it is up to RAN2 to decide whether/how to support SL-DRX for ProSe communication and ProSe discovery.

Observation 2: All V2X, public safety and commercial uses cases are within the scope of SL enh. WI, as clearly stated in the current WID.

Proposal 1: RAN2 confirms that current SL-DRX design in SL enh. WI can be directly reused for ProSe direct communication (including both PC5 QoS based SL-DRX framework and SL-DRX mechanism for PC5-S signalling). No extra Spec impact is needed.

Proposal 2: RAN2 confirms that the SL-DRX design for DCR message is reused for non-relay ProSe discovery. No extra Spec impact is needed.

Proposal 3: RAN2 concludes that SL-DRX for ProSe relay discovery and communication is not supported in this release.

- Reference

- RP-212628	Moderator's summary for discussion [93e-14-Sidelink-Progress]	LG Electronics

- draft_MeetingReport_RAN_93e_eom

- TS 23.304

- R2-2106901	Report of 3GPP TSG RAN WG2 meeting #114-e	Online	19 - 27 May, 2021

Even so, the DCR message and discovery message are transmitted without PC5 unicast link/RRC connection established as normal SL unicast communication.

When a UE in SL unicast is in RRC_IDLE/INACTIVE, it was agreed in RAN2 #115e that the SL inactivity timer is set by the UE itself based on implementation which conclusion is likely to apply to other SL DRX parameters as well.