3GPP TSG-RAN WG2 Meeting #116 e-meeting                         R2-2110108

Online, November 1st – 12th, 2021

Source:          ZTE, Sanechips, China Southern Power Grid Co., Ltd, TCL

Communication Ltd., vivo

Title:             N and combined Tx-side timer for IIoT QoS

Agenda item:     8.5.4

Document for:    Discussion and Decision

- Introduction

The new WID of NR Industrial Internet of Things (IoT) and URLLC support was approved in RAN#86 and revised in RAN#88e [1]. In which, the following objective is included:

... - RAN enhancements based on new QoS related parameters if any, e.g. survival time, decided from SA2. [RAN2, RAN3]

In Rel-16, there were some discussion on the survival time parameter in RAN2#105bis. The following agreements have been made:

- RAN2 think that knowledge of survival time is beneficial to gNB. FFS whether there would be any impact to AS specifications to make use of this, and such discussions would have lower priority, as it is not explicitly a WI objective. There are also concerns that QoS framework may be impacted due to survival time being provided explicitly.

In RAN2#113 e-meeting, based on the contributions, the following agreements have been achieved:

Assumptions: - Communication service availability (CSA) is not needed on top of survival time. Send a reply LS to SA2 to notify such confirmation - RAN2 confirms that specification enhancement for survival time support may only needed for uplink. Downlink is addressed by implementation and no specification impacts. - Support for survival time in UCE is up to network configuration. - Continue discussing whether burst spread and burst ending time is beneficial from RAN2 perspective, but trigger the discussion after SA2 progress in February - Communication service reliability (CSR) is not needed on top of survival time - Only periodic traffic is considered for survival time work in Rel-17 - AN2 assumes one application message is conveyed by one PDCP SDU, and may further consider the cases where one application message is conveyed by varying number of PDCP SDUs depending on the progress

In RAN2#114 e-meeting, the following agreements on survival time have been achieved:

- RAN2 does not consider the Burst Spread parameter in RAN - The Burst End Time parameter in RAN is out of scope for Rel-17 IIoT WI. - No specific enhancements in support of Survival Time in UCE will be studied in R17, but we should aim for solutions for Survival time that also work in UCE - When Survival Time information is provided in TSC AI, RAN action (gNB and/or UE) can utilize it to improve the associated link reliability so that the survival time requirement is met - Study fast mechanisms for survival time handling and the need - RAN2 takes the performance requirements of the top 3 rows of Table 5.2-1 from TS 22.104 (transfer interval = survival time = 0.5/1/2ms) - Survival Time triggered proactively based on Sequence Number is deprioritized - UE-based reactive solution based on RLC-NACK is not pursued - RAN2 will work/study UE-based reactive solutions to address survival time on top of gNB implementation. RAN2 assumes that gNB implementation solutions on their own are not sufficient.

In RAN2#115 e-meeting, the following agreements have been achieved:

- RAN2 does not assume that physical HARQ-NACK messages are always available, i.e. RAN2 will not mandate explicit HARQ-NACK feedback - Given the application message size range under study, RAN2 will not optimize the ST design based on case of segmentation of message into multiple TBs. (This does not preclude the use of RLC segmentation; instead, it rules out optimizations for the case with RLC segmentation) - Following entry into the Survival Time state, PDCP duplication for ST configuration is activated. The gNB pre-configures which RLC entities can be activated for duplication when entering ST state. FFS the number of supported RLC entities. - RAN2 will at least continue working and discussing the HARQ NACK solution. Details are FFS

In the on-going post-meeting email discussion “[Post115-e][513][IIoT] QoS survival time”, the details of HARQ NACK solution has been discussed. However, due to diverse views, there are still some pending issues. In this contribution, we will focus on whether and how Tx-side timer can be combined to the basic HARQ-NACK-based solution and give our proposals.

- Discussion

  - #Issue 1: The value of N

In the email discussion “[Post115-e][513]”, many companies think N values larger than 1 can avoid the issue of “too early entering ST state” and “triggering PDCP duplication too often” which shall lead to unnecessary resource waste. At the same time, a bit more companies prefer fixing N=1 with consideration on simple UE/network implementation and specification changes. Moreover, considering that more companies prefer MAC entity to handle the counting of N and there will be no impact of different N values outside of MAC entity, rapporteur finally propose to regard fixing N=1 as baseline and whether N can be larger than 1 can be discussed later.

As we know, for the HARQ-NACK based solution, some companies have high level concern that the retransmission of the previously failed message (with higher reliability) is not essential to meet the survival time requirement and what really matters is the subsequent message. Moreover, companies think the flexibility for network implementation, e.g., the network may decide not to retransmit a previously failed message but to allocate more radio resources for the subsequence message, should be taken into account. We have sympathy with the former part concern and think N larger than 1 can be used to alleviate the issue of over-satisfying survival time requirement which is naturally in a solution based on only one HARQ-NACK. But for the later part of the concern, we don’t think it’s so reasonable. We assume for the service with Survival Time requirement and configured with CG resources, the network would try to schedule retransmission for each time failed transmission.

Here we want to reiterate and summary the positive views on N larger than 1 mentioned by the supported companies:

- To allow that N is configurable and can be larger than 1 is more aligned with survival time requirement provided by SA.

- A configurable option can cover a range of scenarios as well as prevent the UE from entering ST state unnecessarily which is also not an efficient use of resources.

- For some use cases with a bit long survival time and PDB (e.g. transfer interval = survival time = 2ms), force UE to enter ST state after only 1 retransmission grant is received will cause unnecessary PDCP duplication activation and resource waste.

- There is a view from chipset/UE vendors that managing a counter of HARQ-NACK N at the MAC would not add much complexity, or the complexity introduced is acceptable.

- Here we add one more point, we think the setting of N can correspond to the network’s retransmission scheduling strategy. For IIoT service with high reliability requirement, if only one retransmission is scheduled, more resources would be needed. So scheduling multiple retransmissions might be needed so that it can allow less resources for each retransmission that in turn would bring benefit of reducing block to other services/UEs. Therefore, N larger than 1 can be seen as a trade-off that try to use the least resources to meet the survival time requirements of SA2 under a highly possible scheduling strategy.

Take into account all the previous discussion and the above mentioned benefits or feasibility, and also in order to facilitate the stage-3 discussion, we suggest RAN2 to agree the following proposal 1 as early as possible:

Proposal 1: The N for counting HARQ-NACK can be configurable and larger than 1.

  - #Issue 2: Combination of HARQ-NACK counting and Tx-side timer

In previous meeting and also during the email discussion “[Post115-e][513]”, some critical issues of HARQ-NACK-based option have been mentioned:

- In N HARQ NACK solution, a counter is required to keep track of N-1 received NACKs and survival time state is then triggered upon receiving the Nth NACK. For such solution, it may be possible to miss or lose intermediate NACKs from the gNB, which could result in survival time state not being triggered when required.

- Even the loss of (all) HARQ-NACKs may be very rare, due to the following reasons, it’s still easy to occur the delayed HARQ-NACKs which may also result in survival time state not being triggered as expectation. Then the activation of PDCP duplication is also delayed, even later than the next packet:

  - The availability of network resources (PDCCH, PUSCH resources) is uncertain. Even the IIoT service may have higher QoS, when there are several UEs with higher QoS service, the prioritized scheduling for each UE or each services cannot be always guaranteed.

  - In the case of N greater than 1, when the previous one or few retransmission grant(s) are delayed or even lost, the start of counting retransmission grant might be delayed or the total counting time period might be prolonged. All these issues may also result in survival time state not being triggered timely.

- In unlicensed spectrum case, the gNB may prefer letting CG retransmission timer to expire to triggering retransmission rather than sending an explicit retransmission grant. That means, due to deliberate missing of retransmission grant, the above mentioned problem may be more serious in unlicensed case.

The proponent companies think combination of HARQ-NACK counting and Tx-side timer can resolve the above issues. But it seems the proponent companies have not reached a consensus understanding on the details of the combined Tx-side timer. At least there is ambiguity in the following aspect:

- What’s the UE behaviors when the Tx-sider timer expires?

  - In previous meeting, in order to handle the case that all the N HARQ NACKs are lost, at least two companies has suggested that on expiration of Tx-side timer, UE can directly send indication to PDCP layer to trigger ST state and activate PDCP duplication. But later we realize that no introduction of explicit HARQ-ACK and in the case without implicit HARQ-ACK (without initial transmission grant), UE cannot distinguish this abnormal case from normal case of successful transmission. And such process may cause unnecessary activation of PDCP duplication. Therefore, we think such process may be not suitable in license spectrum case:

    - As unlicensed spectrum case is different and explicit HARQ-NACK/ACK has been supported in unlicensed spectrum case, such process, e.g., to let UE directly send indication to PDCP layer to trigger ST state when the Tx-sider timer expires would be suitable for unlicensed spectrum case. We can easy understand that not only the explicit HARQ-ACK can be used to timely stop the Tx-sider timer to avoid unnecessary triggering PDCP duplication, but also the Tx-sider timer can be used as a complementary to N HARQ-NACKs counting to perfectly resolve the issue of HARQ-NACK missing in unlicensed spectrum case.

  - Another proponent company suggest that when the Tx-sider timer expires, the PDCP PDU can be considered as transmitted successfully, and the counter should be reset. We can agree with some other company’ concern that such process cannot resolve the issue of loss of N HARQ NACKs. The opposing company think such timer would be useless as it looks like a somewhat redundant implicit ACK, similar to CG timer (we assume in the company’s thinking, it’s the configuredGrantTimer). We agree with the view of opposing company and also think such usage of Tx-side timer is incorrect and cannot resolve the issue of loss of N HARQ NACKs. The intention of Tx-side timer would be very different from that of configuredGrantTimer. As autonomous retransmission and explicit HARQ-NACK/ACK have already been supported in unlicensed spectrum case, we assume there is no issue for triggering retransmission there. The main purpose of configuredGrantTimer might be to avoid the UE continuously occupying the HARQ process resources. But for our survival time feature, now we need a “protect timer” to handle the case of without retransmission grant and without trigger for PDCP duplication. So at least, we need a “protect” timer to avoid being unable to enter ST state or entering ST state too late when transmission failure occurs.

  - As mentioned above, in order to avoid unnecessary activation of PDCP duplication, UE cannot directly send indication to PDCP layer when the Tx-sider timer expires. That means, we have to live with the very rare case that all the N HARQ NACKs are lost.

  - As we assume such case would be more occurred, e.g., one or a few retransmission grant(s) are lost but finally at least one retransmission grant can be received, we propose another scheme to alleviate the above issues, e.g., to change to use a minimum N as soon as a configured Tx-side timer expires.

Based on all the analysis, the detailed combined Tx-side timer can be summarized as following:

- The Tx-side timer could be configurable by the network if a scenario requires it.

- The Tx-sider timer is configured with length equal or less than AN PDB.

- The Tx-side timer is started when a PDCP PDU is delivered to lower layer.

- When the Tx-side timer is running:

  - If a new transmission grant is received, the Tx-side timer should be stopped. (See Figure 1)

  - If N retransmission grants or explicit HARQ-NACKs have been received, UE should send indication to the PDCP layer to trigger ST state. And the Tx-side timer should be stopped. (See Figure 1)

  - In unlicensed spectrum case, if HARQ-ACK is received, the Tx-side timer should also be stopped.

Figure 1

- When the Tx-side timer expires:

  - Even if N doesn’t reaches the threshold (or even no any retransmission grant has been received during timer is running), MAC entity in UE would also send indication to the PDCP layer to trigger ST state on reception of the first retransmission grant after expiration of timer.(See Figure 2)

Figure 2

  - In unlicensed spectrum case, UE should send indication to the PDCP layer to trigger ST state.

Based on the above analysis, we give the following proposal:

Proposal 2: To introduce a combined Tx-side timer for the HARQ-NACK-based option. The following details can be further discussed:

- The Tx-side timer could be configurable by the network if a scenario requires it.

- The Tx-sider timer is configured with length equal or less than AN PDB.

- The Tx-side timer is started when a PDCP PDU is delivered to lower layer.

- When the Tx-side timer is running:

  - If a new transmission grant is received, the Tx-side timer should be stopped.

  - If N retransmission grants or explicit HARQ-NACKs have been received, UE should send indication to the PDCP layer to trigger ST state. And the Tx-side timer should be stopped.

  - In unlicensed spectrum case, if HARQ-ACK is received, the Tx-side timer should also be stopped.

- When the Tx-side timer expires:

  - Even if N doesn’t reaches the threshold (or even no any retransmission grant has been received during timer is running), MAC entity in UE would also send indication to the PDCP layer to trigger ST state on reception of the first retransmission grant after expiration of timer.

  - In unlicensed spectrum case, UE should send indication to the PDCP layer to trigger ST state.

Furthermore, an example of text proposal (based on the rapporteur’s text proposal) to reflect the main functions of the combined Tx-side timer can be found in Annex (please note some details, e.g., start or restart of the timer, usage in unlicensed spectrum case, are still under consideration and not reflected now).

- Conclusions

In this contribution, we make the following proposals:

Proposal 1: The N for counting HARQ-NACK can be configurable and larger than 1.

Proposal 2: To introduce a combined Tx-side timer for the HARQ-NACK-based option. The following details can be further discussed:

- The Tx-side timer could be configurable by the network if a scenario requires it.

- The Tx-sider timer is configured with length equal or less than AN PDB.

- The Tx-side timer is started when a PDCP PDU is delivered to lower layer.

- When the Tx-side timer is running:

  - If a new transmission grant is received, the Tx-side timer should be stopped.

  - If N retransmission grants or explicit HARQ-NACKs have been received, UE should send indication to the PDCP layer to trigger ST state. And the Tx-side timer should be stopped.

  - In unlicensed spectrum case, if HARQ-ACK is received, the Tx-side timer should also be stopped.

- When the Tx-side timer expires:

  - Even if N doesn’t reaches the threshold (or even no any retransmission grant has been received during timer is running), MAC entity in UE would also send indication to the PDCP layer to trigger ST state on reception of the first retransmission grant after expiration of timer.

  - In unlicensed spectrum case, UE should send indication to the PDCP layer to trigger ST state.

References

- 3GPP, RP-201310, Revised WID: Core part: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR, RAN #88e, June, 2020.

Annex

- Text Proposal for TS 38.321

FIRST CHANGE

5.4.2	HARQ operation

5.4.2.1	HARQ Entity

The MAC entity includes a HARQ entity for each Serving Cell with configured uplink (including the case when it is configured with supplementaryUplink), which maintains a number of parallel HARQ processes.

The number of parallel UL HARQ processes per HARQ entity is specified in TS 38.214 [7].

Each HARQ process supports one TB.

Each HARQ process is associated with a HARQ process identifier. For UL transmission with UL grant in RA Response or for UL transmission for MSGA payload, HARQ process identifier 0 is used.

NOTE:	When a single DCI is used to schedule multiple PUSCH, the UE is allowed to map generated TB(s) internally to different HARQ processes in case of LBT failure(s), i.e. UE may transmit a new TB on any HARQ process in the grants that have the same TBS, the same RV and the NDIs indicate new transmission.

The maximum number of transmissions of a TB within a bundle of the dynamic grant or configured grant is given by REPETITION_NUMBER as follows:

-	For a dynamic grant, REPETITION_NUMBER is set to a value provided by lower layers, as specified in clause 6.1.2.1 of TS 38.214 [7];

-	For a configured grant, REPETITION_NUMBER is set to a value provided by lower layers, as specified in clause 6.1.2.3 of TS 38.214 [7].

If REPETITION_NUMBER > 1, after the first transmission within a bundle, at most REPETITION_NUMBER – 1 HARQ retransmissions follow within the bundle. For both dynamic grant and configured uplink grant, bundling operation relies on the HARQ entity for invoking the same HARQ process for each transmission that is part of the same bundle. Within a bundle, HARQ retransmissions are triggered without waiting for feedback from previous transmission according to REPETITION_NUMBER for a dynamic grant or configured uplink grant unless they are terminated as specified in clause 6.1 of TS 38.214 [7]. Each transmission within a bundle is a separate uplink grant delivered to the HARQ entity.

For each transmission within a bundle of the dynamic grant, the sequence of redundancy versions is determined according to clause 6.1.2.1 of TS 38.214 [7]. For each transmission within a bundle of the configured uplink grant, the sequence of redundancy versions is determined according to clause 6.1.2.3 of TS 38.214 [7].

For each uplink grant, the HARQ entity shall:

1>	identify the HARQ process associated with this grant, and for each identified HARQ process:

2>	if the received grant was not addressed to a Temporary C-RNTI on PDCCH, and the NDI provided in the associated HARQ information has been toggled compared to the value in the previous transmission of this TB of this HARQ process; or

2>	if the uplink grant was received on PDCCH for the C-RNTI and the HARQ buffer of the identified process is empty; or

2>	if the uplink grant was received in a Random Access Response (i.e. in a MAC RAR or a fallback RAR); or

2>	if the uplink grant was determined as specified in clause 5.1.2a for the transmission of the MSGA payload; or

2>	if the uplink grant was received on PDCCH for the C-RNTI in ra-ResponseWindow and this PDCCH successfully completed the Random Access procedure initiated for beam failure recovery; or

2>	if the uplink grant is part of a bundle of the configured uplink grant, and may be used for initial transmission according to clause 6.1.2.3 of TS 38.214 [7], and if no MAC PDU has been obtained for this bundle:

3>	if there is a MAC PDU in the MSGA buffer and the uplink grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload was selected; or

3>	if there is a MAC PDU in the MSGA buffer and the uplink grant was received in a fallbackRAR and this fallbackRAR successfully completed the Random Access procedure:

4>	obtain the MAC PDU to transmit from the MSGA buffer.

3>	else if there is a MAC PDU in the Msg3 buffer and the uplink grant was received in a fallbackRAR:

4>	obtain the MAC PDU to transmit from the Msg3 buffer.

3>	else if there is a MAC PDU in the Msg3 buffer and the uplink grant was received in a MAC RAR; or:

3>	if there is a MAC PDU in the Msg3 buffer and the uplink grant was received on PDCCH for the C-RNTI in ra-ResponseWindow and this PDCCH successfully completed the Random Access procedure initiated for beam failure recovery:

4>	obtain the MAC PDU to transmit from the Msg3 buffer.

4>	if the uplink grant size does not match with size of the obtained MAC PDU; and

4>	if the Random Access procedure was successfully completed upon receiving the uplink grant:

5>	indicate to the Multiplexing and assembly entity to include MAC subPDU(s) carrying MAC SDU from the obtained MAC PDU in the subsequent uplink transmission;

5>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity.

3>	else if this uplink grant is a configured grant configured with autonomousTx; and

3>	if the previous configured uplink grant, in the BWP, for this HARQ process was not prioritized; and

3>	if a MAC PDU had already been obtained for this HARQ process; and

3>	if the uplink grant size matches with size of the obtained MAC PDU; and

3>	if none of PUSCH transmission(s) of the obtained MAC PDU has been completely performed:

4>	consider the MAC PDU has been obtained.

3>	else if the MAC entity is not configured with lch-basedPrioritization; or

3>	if this uplink grant is a prioritized uplink grant:

4>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity, if any;

3>	if a MAC PDU to transmit has been obtained:

4>	if the uplink grant is not a configured grant configured with autonomousTx; or

4>	if the uplink grant is a prioritized uplink grant:

5>	deliver the MAC PDU and the uplink grant and the HARQ information of the TB to the identified HARQ process;

5>	instruct the identified HARQ process to trigger a new transmission;

5>	if the uplink grant is a configured uplink grant:

6>	start or restart the configuredGrantTimer, if configured, for the corresponding HARQ process when the transmission is performed if LBT failure indication is not received from lower layers;

6>	start or restart the cg-RetransmissionTimer, if configured, for the corresponding HARQ process when the transmission is performed if LBT failure indication is not received from lower layers.

5>	if the uplink grant is addressed to C-RNTI, and the identified HARQ process is configured for a configured uplink grant:

6>	start or restart the configuredGrantTimer, if configured, for the corresponding HARQ process when the transmission is performed if LBT failure indication is not received from lower layers.

5>	if cg-RetransmissionTimer is configured for the identified HARQ process; and

5>	if the transmission is performed and LBT failure indication is received from lower layers:

6>	consider the identified HARQ process as pending.

3>	else:

4>	flush the HARQ buffer of the identified HARQ process.

2>	else (i.e. retransmission):

3>	if the uplink grant received on PDCCH was addressed to CS-RNTI and if the HARQ buffer of the identified process is not empty:

4>	if the MAC PDU stored in the HARQ buffer contains data from DRB(s) configured with SurvivalTimeSupport:

5> if NACK_COUNTER = nackCountMax and SurvivalTimeTimer is running:

6>	indicate Survival Time state information to upper layers for each DRB.

6> reset NACK_COUNTER.

6> stop SurvivalTimeTimer, if running.

5> else if NACK_COUNTER < nackCountMax and SurvivalTimeTimer is configured and not running:

6>	indicate Survival Time state information to upper layers for each DRB.

6> reset NACK_COUNTER.

5> else:

6>	increment NACK_COUNTER by 1.

Editor’s Note 1: FFS whether DG addressed to C-RNTI is considered as Survival Time state trigger.

Editor’s Note 2: Details of “SurvivalTimeSupport” to be specified in RRC CR.

Editor’s Note 3: FFS whether N (>1) HARQ NACKs is considered as Survival Time state trigger.

3>	if the uplink grant received on PDCCH was addressed to CS-RNTI and if the HARQ buffer of the identified process is empty; or

3>	if the uplink grant is part of a bundle and if no MAC PDU has been obtained for this bundle; or

3>	if the uplink grant is part of a bundle of the configured uplink grant, and the PUSCH duration of the uplink grant overlaps with an uplink grant received in a Random Access Response (i.e. MAC RAR or fallbackRAR) or an uplink grant determined as specified in clause 5.1.2a for MSGA payload for this Serving Cell; or:

3>	if the MAC entity is not configured with lch-basedPrioritization and this uplink grant is part of a bundle of the configured uplink grant, and the PUSCH duration of the uplink grant overlaps with a PUSCH duration of another uplink grant received on the PDCCH; or:

3>	if the MAC entity is configured with lch-basedPrioritization and this uplink grant is not a prioritized uplink grant:

4>	ignore the uplink grant.

3>	else:

4>	deliver the uplink grant and the HARQ information (redundancy version) of the TB to the identified HARQ process;

4>	instruct the identified HARQ process to trigger a retransmission;

4>	if the uplink grant is addressed to CS-RNTI; or

4>	if the uplink grant is addressed to C-RNTI, and the identified HARQ process is configured for a configured uplink grant:

5>	start or restart the configuredGrantTimer, if configured, for the corresponding HARQ process when the transmission is performed if LBT failure indication is not received from lower layers.

4>	if the uplink grant is a configured uplink grant:

5>	if the identified HARQ process is pending:

6>	start or restart the configuredGrantTimer, if configured, for the corresponding HARQ process when the transmission is performed if LBT failure indication is not received from lower layers;

5>	start or restart the cg-RetransmissionTimer, if configured, for the corresponding HARQ process when the transmission is performed if LBT failure indication is not received from lower layers.

4>	if the identified HARQ process is pending and the transmission is performed and LBT failure indication is not received from lower layers:

5>	consider the identified HARQ process as not pending.

When determining if NDI has been toggled compared to the value in the previous transmission the MAC entity shall ignore NDI received in all uplink grants on PDCCH for its Temporary C-RNTI.

When configuredGrantTimer or cg-RetransmissionTimer is started or restarted by a PUSCH transmission, it shall be started at the beginning of the first symbol of the PUSCH transmission.