3GPP TSG-RAN WG2 Meeting #116e	R2- 2110164

# Online Meeting, 2021 November 2021

Agenda item:	8.7.2.2

Source:	LG Electronics Inc.

Title: 	Service continuity – depending on relay state

Document for:	Discussion and Decision

1.	Introduction

WID of Sidelink relay (RP-210904) was agreed in RAN#91e [1]. The related WID objectives on service continuity are:

- Specify mechanisms for service continuity - Limited to intra-gNB cases [RAN2] NOTE 1: RAN requests RAN2 to strive for completion of the common parts (objective 1) by RAN#92 (June). RAN understands that RAN2 will also initially work on other aspects that have cross-group dependencies. NOTE 2: For L2 UE-to-Network Relay, it is assumed that the Remote UE has a single active connection towards gNB via only a single Relay UE at a given time in this release. NOTE 3: Only NR Uu interface, i.e. gNB, and 5GC is considered, and it is limited to NR SA scenario in this release. NOTE 4: Work specific to the mobility scenario of “between indirect (via a first Relay UE) and indirect (via a second Relay UE)”, and the group mobility is not supported in this release.

In this contribution, we investigate some considerations on the states of relay UE when Remote UE performs direct-to-indirect path switching.

2. Discussion

2.1 Direct-to-indirect path switching

In the 38.300 CR[2], as the below, service continuity is considered only the relay UE in RRC CONNECTED during direct-to-indirect path switching (e.g., step2). We need to discuss whether service continuity is supported when relay UE is in RRC IDLE/INACTIVE.

For service continuity of L2 U2N Relay, the following procedure is used, in case of a UE switching to U2N Relay UE: Figure 16.x.6.2-1: Procedure for U2N Remote UE switching to indirect Relay UE 1. The U2N Remote UE reports one or multiple candidate U2N Relay UE(s) and legacy Uu measurements, after it measures/discovers the candidate U2N Relay UE(s). - The UE may filter the appropriate U2N Relay UE(s) according to Relay selection criteria before reporting. The UE shall report only the U2N Relay UE candidate(s) that fulfil the higher layer criteria. - The reporting can include at least U2N Relay UE ID, U2N Relay UE’ s serving cell ID, and SD-RSRP information. 2. The gNB decides to switch the U2N Remote UE to a target U2N Relay UE. Then the gNB sends an RRCReconfiguration message to the target U2N Relay UE, which can include at least Uu and PC5 RLC configuration for relaying, and bearer mapping configuration. Editor's Note: At step 2, the gNB may decide to perform a normal handover rather than a path switch to an indirect path. 3. The gNB sends the RRCReconfiguration message to the U2N Remote UE. The contents in the RRCReconfiguration message can include at least U2N Relay UE ID, PC5 RLC configuration for relay traffic and the associated end-to-end radio bearer(s). The U2N Remote UE stops UP and CP transmission over Uu after reception of RRCReconfiguration message from the gNB. 4. The U2N Remote UE establishes PC5 connection with target U2N Relay UE 5. The U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the gNB via the Relay UE. 6. The data path is switched from direct path to indirect path between the U2N Remote UE and the gNB. Editor's Note: FFS in case the target relay UE is in IDLE/INACTIVE, if supported.

According to the path switching procedure from direct to indirect described above, gNB configures PC5 RLC and Uu RLC of remote UE SRB1 for RRCReconfigurationComplete message to relay UE. To do this, gNB has to know the relay UE is in RRC CONNECTED. Remote UE reports layer2 ID of candidate relay UEs and the layer2 ID may be used as SRC ID in discovery message from candidate relay UEs. If the candidate relay UE in RRC CONNECTED reports its layer2 ID used as SRC ID of discovery message to gNB, the gNB can know which candidate relay UE is in RRC CONNECTED as comparing it to the reported candidate relay UE ID from remote UE.

Proposal 1: Relay UE reports its layer2 SRC ID using in discovery message to the gNB for its identification.

Proposal 2: gNB can identify which relay UE is in RRC CONNECTED among candidate relay UEs by comparing candidate relay UE IDs reported from remote UE to the relay UE IDs reported from relay UE in RRC CONNECTED.

The number of candidate relay UEs that can be reported from remote UE to gNB can be restricted. Because if a large number of candidate relay UE is reported, the message size will be increased. So, if there is a restriction to the number of candidate relay UEs that can be reported to gNB, remote UE should report RRC CONNECTED relay UE first. And remote UE can report high SD-RSRP relay UE in order among RRC CONNECTED candidate relay UEs.

Proposal 3: if there is a restriction to the number of candidate relay UEs that can be reported to gNB, it is efficient that remote UE reports RRC CONNECTED relay UEs first.

However, Remote UE doesn’t know which candidate relay UE is in RRC CONNECTED. So, in order for remote UE to report RRC CONNECTED relay UE first, the RRC state of relay UE should be included in a discovery message from relay UE.

Proposal 4: To report candidate relay UE in RRC CONNECTED first, remote UE has to know which candidate relay UE is in RRC CONNECTED.

Proposal 5: It can be helpful that the discovery message includes the RRC state of relay UE when the number of candidate relay UE to be reported is limited.

In the above direct-to-indirect path switch procedure, if the step5 (transmission RRC Reconfiguration Complete message from Remote UE) or step 4 (PC5 connection establishment) procedure is failed, Remote UE can report the failure and the cause value of the failure using the current direct path to gNB. The gNB that receives this information selects a new Relay UE based on the previously reported information from Remote UE and informs it to Remote UE. If the candidate Relay UE reported was only one, Remote UE keeps the current direct path.

Proposal 6: If remote UE fails RRC re-configuration via indirect path or fails PC5 connection establishment with relay UE selected by gNB, the remote UE informs the failure with a cause value to the gNB.

Proposal 7: If gNB receives a path switching failure message from remote UE, the gNB can select another relay UE among the previously reported candidate relay UE and inform that to the remote UE. If the reported candidate relay UE is only one, the remote UE can keep the current direct connection.

Meanwhile, we’ve discussed whether the legacy T304 can be reused for path switch procedure on RAN2 114e-meeting e-mail discussion [3]. But we did make any agreement about this issue. We need to consider that a T304-like timer will reuse in remote UE during direct-to-indirect path switching. The legacy T304 timer is for handover operation. When a UE receives the RRCReconfiguration message related to handover, the T304 timer starts. And when the UE completes random access, the T304 timer will stop. In the direct-to-indirect path switching procedure, remote UE doesn’t perform random access. So, remote UE may need to have the T304-like timer instead of the legacy T304 timer. Or if remote UE uses the same as legacy T304 timer, the stop condition may be described differently.

Proposal 8: We need to discuss whether remote UE can use the legacy T304 timer during path switching from direct to indirect.

Proposal 8a: During direct-to-indirect path switching, a T304-like timer may be required for remote UE.

Proposal 8b: If remote UE uses the same as legacy T304 timer, the stop condition should be described differently.

If the T300 timer stop condition describes like that the T304 timer stops when the RRCReconfigurationComplete message arrived gNB, how is the remote UE convinced that the message arrived gNB well? Maybe, relay UE should send a confirmation message to remote UE when the relay UE is convinced that the message arrived at gNB well

Proposal 9: If the T300 timer stop condition describes like that the T304 timer stops when the RRCReconfigurationComplete message arrived at gNB, we need to discuss how remote UE is convinced the message arrived at gNB well.

2.2 Indirect-to-direct path switching

For service continuity of L2 U2N relay, the following procedure is used, in case of U2N Remote UE switching to direct Uu cell: Figure 16.x.6.1-1: Procedure for U2N Remote UE switching to direct Uu cell 1. The Uu measurement configuration and measurement report signalling procedures are performed to evaluate both relay link measurement and Uu link measurement. The measurement results from U2N Remote UE are reported when configured reporting criteria is met. The SL relay measurement report shall include at least U2N Relay UE ID, serving cell ID, and SL-RSRP information. 2. The gNB decides to switch the U2N Remote UE onto direct Uu path. 3. The gNB sends RRCReconfiguration message to the U2N Remote UE. The U2N Remote UE stops UP and CP transmission via U2N Relay UE after reception of RRCReconfiguration message from the gNB. 4. The U2N Remote UE synchronizes with the gNB and performs Random Access. 5. The UE (i.e. previous U2N Remote UE) sends the RRCReconfigurationComplete message to the gNB via target path, using the configuration provided in the RRCReconfiguration message. From this step, the U2N Remote UE moves the RRC connection to the target gNB 6. The gNB sends RRCReconfiguration message to the U2N Relay UE to reconfigure the connection between the U2N Relay UE and the gNB. The RRCReconfiguration message to the U2N Relay UE can be sent any time after step 3 based on gNB implementation (e.g. to release Uu and PC5 RLC configuration for relaying, and bearer mapping configuration between PC5 RLC and Uu RLC). 7. Either U2N Relay UE or U2N Remote UE can initiate the PC5 unicast link release (PC5-S). The timing to execute link release is up to UE implementation. The U2N Relay UE can execute PC5 connection reconfiguration to release PC5 RLC for relaying upon reception of RRC Reconfiguration by gNB in Step 6, or the UE (i.e. previous U2N Remote UE) can execute PC5 connection reconfiguration to release PC5 RLC for relaying upon reception of RRC Reconfiguration by gNB in Step 3. 8. The data path is switched from indirect path to direct path between the UE (i.e. previous U2N Remote UE) and the gNB. Step 8 can be executed in parallel or after step 5, which is independent of step 6 and step 7. The DL/UL lossless delivery during the path switch is done according to PDCP data recovery procedure.

If Remote UE fails step4 (RA to gNB) or step5 (RRC Reconfiguration Complete message) procedure, the Remote UE can maintain the current indirect link. In this case, the Remote UE sends a resume message, which means the Relay UE is allowed to send messages to the Remote UE.

Proposal 10: In indirect-to-direct path switching, if remote UE fails path switching, the remote UE may send a message to the gNB via relay UE for resuming. The relay UE receiving this message can send messages to the remote UE.

3.  Conclusion

Proposal 1: Relay UE reports its layer2 SRC ID using in discovery message to the gNB for its identification.

Proposal 2: gNB can identify which relay UE is in RRC CONNECTED among candidate relay UEs by comparing candidate relay UE IDs reported from remote UE to the relay UE IDs reported from relay UE in RRC CONNECTED.

Proposal 3: if there is a restriction to the number of candidate relay UEs that can be reported to gNB, it is efficient that remote UE repots RRC CONNECTED relay UEs first.

Proposal 4: To report candidate relay UE in RRC CONNECTED first, remote UE has to know which candidate relay UE is in RRC CONNECTED.

Proposal 5: It can be helpful that the discovery message includes the RRC state of relay UE when the number of candidate relay UE to be reported is limited.

Proposal 6: If remote UE fails RRC re-configuration via indirect path or fails PC5 connection establishment with relay UE selected by gNB, the remote UE informs the failure with a cause value to the gNB.

Proposal 7: If gNB receives a path switching failure message from remote UE, the gNB can select another relay UE among the previously reported candidate relay UE and inform that to the remote UE. If the reported candidate relay UE is only one, the remote UE can keep the current direct connection.

Proposal 8: We need to discuss whether remote UE can use the legacy T304 timer during path switching from direct to indirect.

Proposal 8a: During direct-to-indirect path switching, a T304-like timer may be required for remote UE.

Proposal 8b: If remote UE uses the same as legacy T304 timer, the stop condition should be described differently.

Proposal 9: If the T300 timer stop condition describes like that the T304 timer stops when the RRCReconfigurationComplete message arrived at gNB, we need to discuss how remote UE is convinced the message arrived at gNB well.

Proposal 10: In indirect-to-direct path switching, if remote UE fails path switching, the remote UE may send a message to the gNB via relay UE for resuming. The relay UE receiving this message can send messages to the remote UE.

4.  References

- RP-193253, “New SID: Study on NR sidelink relay”

- [Post115-e][601][Relay] Relaying CR to 38.300 (MediaTek)

- R2-2108196, “Feature summary of AI 8.7.2.2.”