3GPP TSG-RAN WG2 Meeting #116-e 	R2-2110178

Electronic, 1st - 12th Nov, 2021

Agenda item:	8.11.2

Source:	Huawei, HiSilicon

Title:	Discussion on latency reduction techniques from other groups

Document for:	Discussion and Decision

- Introduction

In RAN2 meeting #115-e, there were some discussions about latency reduction [1]. In this paper, we will further discuss this issue.

- Discussion

  - Scheduled location time

In the last RAN2 meeting, it was agreed that the scheduled location time can reduce the latency for the location preparation phase in a certain use case and it does not allow latency reduction for the location execution phase [1].

Figure, scheduled location time

The scheduled location time is mainly used for the use case where a LCS Client that is requesting the location of a target UE knows a time T at which the location should be obtained. From RAN2’s perspective, the current spec is already able to support the functionalities for Scheduled Location Time. The LMF can request the location measurements with a proper value for the field of response time, to obtain the location measurements at time T.

In the RAN3#113 meeting, it was agreed to include the response time IE in the MEASUREMENT REQUEST message [2]. Therefore, both LPP and NRPPa have already support the LMF to obtain measurements with the response time. There is no further RAN stage 3 specification impacts to support the scheduled location time described by the SA2.

Proposal 1: RAN2 to conclude that the scheduled location time has no impacts on RAN stage 3 specifications.

  - Storing positioning capability in AMF

During the last meeting, RAN2 has discussed whether positioning related UE capabilities can be variable, and the following agreement and reply LS to SA2 (R2-2108945[2]) has been reached .

Agreement:

Proposal 6 (modified): In response to the question asked by SA2 regarding UE positioning capability, it is proposed to capture that the positioning related UE capabilities can be variable.

NOTE: P6 was edited after agreement for clarity (deletion marked with strikeout).  Checked in email discussion [AT115-e][600].

The approach of storing capabilities in the AMF for latency reduction has been adopted as part of the SA2 R17 WI with the LS in S2-2105153, with CR S2-2105123 [3] to TS 23.273 as attachment. In CR S2-2105123, the feature of UE positioning capability storage in AMF is introduced to reduce positioning latency, where the corresponding correction is extracted as below:

Functions which may be performed by an AMF to support location services include the following. - Initiate an NI-LR location request for a UE with an IMS emergency call. - Receive and manage location requests from a GMLC for a 5GC-MT-LR and deferred 5GC-MT-LR for periodic, triggered and UE available location events. - Receive and manage location requests from a UE for a 5GC-MO-LR. - Receive and manage Event Exposure request for location information from an NEF. - Select an LMF. - Receive updated privacy requirements from a UE and transfer to a UDR via UDM. - Support cancelation of periodic or triggered location reporting for a target UE. - Support change of a serving LMF for periodic or triggered location reporting for a target UE. - When assistance data is broadcast by 5GS in ciphered form, the AMF receives ciphering keys from the LMF and forwards to suitably subscribed UEs using mobility management procedures. - Store UE Positioning Capability received from an LMF and send the UE Positioning Capability along with the received location request to an LMF. NOTE X: Details of UE Positiong Capability is defined in TS 37.355[xx].

According to the current LPP/NRPPa procedure, AMF may not get a whole picture of UE positioning capability. The main reason is that LMF may only request its interested positioning capability instead of all the positioning capability from UE via Capability Transfer procedure. From the UE side, it only feedbacks the positioning capability corresponding to the request from LMF. Even with the request from the LMF, it is also up to the UE’s implementation to report the capabilities that it wants to report. The UE may not report all the request capabilities.  The details can be found in TS 37.355.

5.1.1 Capability Transfer procedure The Capability Transfer procedure is shown in Figure 5.1.1-1. Figure 5.1.1-1: LPP Capability Transfer procedure 1. The server sends a RequestCapabilities message to the target. The server may indicate the types of capability needed. 2. The target responds with a ProvideCapabilities message to the server. The capabilities shall correspond to any capability types specified in step 1. This message shall include the endTransaction IE set to TRUE.

According to the CR S2-2105123, AMF only stores the UE positioning capability received from an LMF, which may be only a fraction of positioning capability and would cause an ambiguity problem. Assume LMF request positioning capability A from the AMF and finds it is not available at the AMF’s storage, the LMF can’t tell positioning capability A is not supported by UE or it has never been requested from UE before. To be specific, there may be two interpretation the non-availability of positioning capability A at AMF side.

- Interpretation1: The UE actually supports positioning capability A, but the LMF never requested this capability previously.

- Interpretation 2: The UE doesn’t support positioning capability A. In this case, no matter the LMF has once requested this capability, it’s not available at AMF’s storage.

Therefore, the LMF needs to check the problem with UE, i.e., initiating a capability transfer procedure, which may introduce extra delay and makes the solution AMF storing UE positioning capability meaningless in terms of latency reduction. Accordingly, we think this issue should be solved and several potential solutions are provided as follows.

- Solution 1: Following the legacy solution for AMF storing UE radio capability. AMF should store additional information for the UE positioning capability, e.g., the request record of the UE positioning capability by LMF from UE, the positioning capability not supported by UE, etc.

- Solution 2: AMF should store the complete capability of the UE. One possible way is that LMF requests the whole UE positioning capability from UE.

Proposal 2: For AMF storing positioning capabilities for latency reduction, RAN2 should down-select from the following options:

- Option1: Legacy solution for AMF storing UE radio capability (e.g., the request record of the UE positioning capability);

- Option2: AMF should store the complete capability of the UE.

- Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1: RAN2 to conclude that the scheduled location time has no impacts on RAN stage 3 specifications.

Proposal 2: For AMF storing positioning capabilities for latency reduction, RAN2 should down-select from the following options:

- Option1: Legacy solution for AMF storing UE radio capability (e.g., the request record of the UE positioning capability);

- Option2: AMF should store the complete capability of the UE.

- Reference

- Report of 3GPP TSG RAN WG2 meeting #115-e, Online

- R2-2108945, Response LS on storage of UE Positioning Capabilities.

- S2-2105123, UE positioning capability storage.