3GPP TSG-RAN WG2 Meeting #116-e 	R2-2110183

Electronic, 1st - 12th Nov, 2021

Agenda item:	8.6.5

Source:	Huawei, HiSilicon

Title:	Small data transmission with CG-based scheme

Document for:	Discussion

- Introduction

Small Data Transmission in RRC INACTIVE topic was discussed during the previous meetings where numerous agreements were made for CG-SDT. In this paper, we discuss further aspects for the small data transmission with CG-based scheme.

- Discussion on CG-SDT

2.1	CG-SDT resource release

One point needs to be considered is when receiving RRCResume message in response to RRCResumeRequest for SDT, the UE then transitions to RRC_CONNECTED, and on the other hand the UE shall release part of the CG-SDT configuration. In RRC_CONNECTED, configurations like association between SSB and CG resource, RSRP threshold for TA validation, etc. are not needed at all. Hence, this part of configuration can be released.

Proposal 1: CG-SDT configuration should be released when transitioning to RRC_CONNECTED.

2.2	CG-SSB mapping

2.2.1	Triggering for SSB re-evaluation

At RAN2#115e meeting, RAN2 agreed for the purpose of CG resource selection, UE re-evaluates the SSB for subsequent CG transmission and can initiates legacy RACH procedure if UE cannot find qualified SSB for CG-SDT resource during the subsequent new CG transmission phase. However we think another cases are not yet considered, i.e. how to select SSB when multiple SSBs are qualified. If the first CG-SDT transmission for a HARQ process is successfully completed, the UE can select any qualified SSB for second CG-SDT transmission for another HARQ process similarly as for the first CG-SDT transmission. However, if the first CG-SDT transmission is not successful completed (i.e. retransmission can take place over DG) at SSB re-evaluation for second CG-SDT transmission for another HARQ process, it can be up to UE implementation to select any qualified SSB as well as the first CG-SDT transmission but we must keep in mind is the network cannot know it when UE reselects to a new SSB until receiving the second CG-SDT transmission associated to the new SSB successfully. Before that moment, the network may still schedule UE by DG over CS-RNTI PDCCH for first CG-SDT retransmission based on the previous selected SSB (not the new SSB). We thus think UE should keep selecting the one for first CG-SDT transmission (if it is qualified) to be aligned with network on SSB used for DG for retransmission.

Proposal 2: UE shall keep selecting the previous selected SSB if it is qualified during the subsequent transmission phase.

The next question is when to trigger the SSB selection. One option is to trigger the SSB selection at each CG occasion. However, this is unnecessary since for certain selected SSB, there is only one CG occasion that corresponds to the SSB within one SSB to CG-PUSCH association period. Hence, it is enough to select SSB at the beginning of each SSB to CG-PUSCH association period and consider the CG occasions corresponding to the selected SSB as valid.

Proposal 3:  Define a SSB to CG-PUSCH association period that SSB reselection is triggered at the beginning of each such association period.

2.2.2	Reconfiguration of CG-SSB mapping

The following has been included in the LS from RAN1 R1-2106335 for the association between SSBs and CG resources for CG-SDT

Association between SSBs and CG resources for CG-SDT RAN1 has further discussed the association between SSBs and CG resources for CG-SDT, and the following agreements have been achieved. RAN1 will continue working on the details of the association. In RAN1#104b-e - It is RAN1’s common understanding that the CG configuration mechanism in licensed band can be reused for CG-SDT in principle. - CG resources per CG configuration are associated with a set of SSB(s) configured by explicit signalling. - FFS how to define an SSB-to-PUSCH resource mapping within the CG configuration. - FFS specific changes to the CG configuration to support the additional SSB-to-PUSCH mapping, if any. In RAN1#105-e - The SSB-to-PUSCH resource mapping within the CG configuration is implicitly defined. - The ordering of the SSB and CG PUSCH resources are to be captured in RAN1 spec. - A PUSCH resource refers to a transmission occasion and a DMRS resource used for PUSCH transmission - The ordering of the SSB can reuse from the SSB-to-RO mapping - The ordering of CG PUSCH resources can reuse from that of MsgA PUSCH as much as possible - FFS determination of mapping ratio and association period, e.g., explicitly signaled or implicitly derived - FFS any limitation on the combination of the parameters for CG resources.

RAN1 agreed that “CG resources per CG configuration are associated with a set of SSB(s) configured by explicit signaling”. Since gNB may not map all the SSBs to CG configurations in order to reduce the number of reserved resources, it may happen that a UE may move out of the coverage of SSBs mapped with CG configurations. We think it would be beneficial to have a possibility of reconfiguring the CG-SDT resources and their mapping to SSBs during subsequent data phase of SDT to account for UE mobility within a cell.

Proposal 4: The network should be able to reconfigure CG-SDT resources and the mapping between CG configuration(s) and SSBs during subsequent transmission phase of the SDT procedure.

2.3	TA aspects

2.3.1	TA validation

A reply LS was sent from RAN1 where RAN1 requests RAN2 to down-select the following options for SSB subset.

- Option 1: Within a set of SSBs configured per CG configuration

- Option 2: Within a set of SSBs configured for all CG configurations

- Option 3: Within a set of all SSBs actually transmitted as indicated in SIB1

- Option 4: Highest N SSBs of all SSBs actually transmitted as indicated in SIB1

RAN1 agreed each CG configuration can be configured to be associated with a set of SSBs (it is implied some of SSBs can be not associated with CG configuration). However keeping in mind is TA mainly depends on the distance between network and UE. To correctly estimate the distance and further derive a correct TA, we think all SSBs actually transmitted including the SSB without CG configuration associated can be used for SSB subset determination. Another point we would like to point out is even though TA is considered as valid by taking the SSB without CG configuration associated into account, we still need to check RSRP of each SSB with CG configuration associated at initiation of CG-SDT procedure and CG-SDT is not initiated when no qualified SSB with CG configuration associated is found. Based on the above principle, option 1 and option 2 are excluded. We think option 3 can be agreed as a baseline but whether highest N SSBs of all actually transmitted SSBs are used or not can be discussed later.

Proposal5: All actually transmitted SSBs as indicated in SIB1 can be used for SSB subset determination for TA validation.

With the SSBs for the determination of TA validation, the next step is how to obtain the RSRP based on the subset of SSBs for TA validation. In TS 38.304, the following has been defined for RSRP derivation for a cell quality for cell reselection:

For cell reselection in multi-beam operations, including inter-RAT reselection from E-UTRA to NR, the measurement quantity of this cell is derived amongst the beams corresponding to the same cell based on SS/PBCH block as follows: - if nrofSS-BlocksToAverage (maxRS-IndexCellQual in E-UTRA) is not configured in SIB2/SIB4 (SIB24 in E-UTRA); or - if absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA) is not configured in SIB2/SIB4 (SIB24 in E-UTRA); or - if the highest beam measurement quantity value is below or equal to absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA): - derive a cell measurement quantity as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [11]. - else: - derive a cell measurement quantity as the linear average of the power values of up to nrofSS-BlocksToAverage (maxRS-IndexCellQual in E-UTRA) of highest beam measurement quantity values above absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA).

We think for the TA validation for CG, the RSRP for multiple beams can be obtained in the same way as in the IDLE/INACTIVE mode procedure.

Proposal6: The RSRP for TA validation is derived by the SSB consolidation procedure in IDLE/INACTIVE mode spec.

Another question is that, since we have subsequent uplink transmission in SDT and it can be performed on CG-SDT, whether TA validation should be performed during subsequent uplink transmission. While for initial transmission, TA validation is by comparing the RSRP of the UE when previous UL transmission accurs with the current RSRP, we think that during subsequent uplink when the TA can be controlled by the network, the TA mechanism should be similar to that of the CONNECTED mode that validation is by whether the TAT is running. Hence, we think TA validation for CG by comparing RSRP with that at the previously UL transmission should only be applicable for initial CG transmission.

Proposal7: TA validation by comparing RSRP with that at the previously UL transmission for CG is only applied for the initial CG-SDT.

2.3.2	TA maintenance

Based on the current NR MAC specification, UE receives a TA command included in either MSG2 or MSGB (i.e. fallbackRAR and successRAR) during the on-going random-access procedure and immediately applies the TA command to get an updated NTA and maintains the updating NTA for subsequent uplink transmissions (e.g. MSG3 or HARQ feedback for MSG4 or for successRAR) when the timeAlignmentTimer is running as shown below. Otherwise UE ignores the received TA command in MSG2 or MSGB since on one hand, it already has a valid TA and on the other hand, the received TA command may be incorrect and actually targeted for other UE using the same RO and preamble but the UE cannot know at that moment until it later loses from contention resolution at MSG4 stage.

1> when a Timing Advance Command is received in a Random Access Response message for a Serving Cell belonging to a TAG or in a MSGB for an SpCell: 2> if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble: 3> apply the Timing Advance Command for this TAG; 3> start or restart the timeAlignmentTimer associated with this TAG. 2> else if the timeAlignmentTimer associated with this TAG is not running: 3> apply the Timing Advance Command for this TAG; 3> start the timeAlignmentTimer associated with this TAG; 3> when the Contention Resolution is considered not successful as described in clause 5.1.5; or 3> when the Contention Resolution is considered successful for SI request as described in clause 5.1.5, after transmitting HARQ feedback for MAC PDU including UE Contention Resolution Identity MAC CE: 4> stop timeAlignmentTimer associated with this TAG. 2> else: 3> ignore the received Timing Advance Command.

The running state of a TA timer (i.e. timeAlignmentTimer) is used for considering whether or not the maintained TA is valid for UEs both in RRC_CONNECTED and RRC_IDLE/INACTIVE, i.e. expiry of the TA timer means the maintained TA is invalid. In addition, in previous meeting, RAN2 agreed a separate TA timer (i.e. CG-SDT-TAT) for CG-SDT which has the same functionality with timeAlignmentTimer on TA maintenance as mentioned below. This CG-SDT-TAT can be running over RRC_INACTIVE.

- A new TA timer for TA maintenance specified for configured grant based small data transfer in RRC_INACTIVE should be introduced. FFS on the procedure, the validity of TA, and how to handle expiration of TA timer. The TA timer is configured together with the CG configuration in the RRCRelease message. - CG-SDT-TAT is started upon receiving the CG-SDT-TAT configuration from gNB, i.e. RRCrelease message, and can be (re)started upon reception of TA command.

Based on the above agreement, UE starts CG-SDT-TAT after being directed to RRC_INACTIVE by RRCRelease message and the CG-SDT-TAT may be still running for following RA cases. One point is worth pointing out is how to handle the following cases for TA maintenance for CG-SDT after receiving TA command in MSG2 or MSGB while the CG-SDT-TAT is running:

Case 1: RA is triggered, e.g. due to non-SDT data earlier arrival, periodic RNAU or something else, before the initiation of CG-SDT session but not yet completed. For Case1, the scenario is the same as the case when the UE is released from CONNECTED to INACTIVE by RRCRelease with suspendConfig. In this case, the UE should maintain the NTA that the UE has maintained previously, either in RRC_INACTIVE or RRC_CONNECCTED. UE may have received TA command in MSG2 or MSGB before the initiation of CG-SDT session. The UE shall be known whether to apply such TA command for CG-SDT session. This case was also discussed in PUR and clarified in LTE MAC specification, i.e. UE considers the received TA in random access response to be a temporary TA as mentioned below. But note in LTE only 4-step RA is supported.

5.2 Maintenance of Uplink Time Alignment - when a Timing Advance Command is received in a Random Access Response message for a serving cell belonging to a TAG: - if the UE is configured with pur-Config (see TS 36.331 [8]) and if a NTA has been stored or maintained and no temporary NTA has been stored: - store current NTA as temporary NTA (see clause 5.4.7.2). 5.4.7.2 Maintenance of PUR Uplink Time Alignment - upon considering a Random Access procedure successfully completed: - start or restart the pur-TimeAlignmentTimer, if configured; - indicate to upper layers that the Timing Advance value has been adjusted; - if a temporary NTA has been stored, delete the stored temporary NTA. - upon considering a Random Access procedure unsuccessfully completed, if a temporary NTA has been stored: - set the NTA to the stored temporary NTA; - delete the stored temporary NTA.

- Case 2: RA is triggered in the middle of the CG-SDT session due to SR triggered by second SDT data arrival as agreed in previous meeting. Even if UE has valid TA during CG-SDT session, the UE may also receive TA command in MSG2 or MSGB for updating TA. The UE shall be known whether to apply such TA command for subsequent CG-SDT transmission as well.

For case of TA command received in fallbackRAR and 4-step RA, if UEs do not have a valid TA, they apply the TA command immediately for MSG3 transmission and wait for contention resolution in MSG4 phase. If UE fails for contention resolution, it discards the maintained NTA. However, if UEs have a valid TA, (i.e. the CG-SDT-TAT is running) they shall use the maintained NTA to avoid applying a potential incorrect one received in MSG2 or fallbackRAR for MSG3 transmission, similarly as in legacy.

Proposal8: When the CG-SDT-TAT is started, the UE shall maintain a CG-SDT-NTA different from the legacy NTA for the timing advance for CG-SDT.

Proposal9: UE shall apply TA command to NTA in random access response (4-step RA) and fallbackRAR when CG-SDT-TAT is running and use this NTA the uplink timing alignment for the rest of the RACH procedure.

In addition, for case of TA command received in successRAR, note in legacy, TA timer seems never running for UEs in RRC IDLE and RRC inactive at the reception of successRAR. However for SDT, case may be a little bit of different since we have CG-SDT-TAT to be running over RRC_inactive. As a consequence, network may find the maintained UE’s TA becomes invalid based on reception of MSGA preamble and payload for SDT. The TA command in successRAR was tailor-made by network for updating the maintained UE’s TA and as a consequence, UE identifies its successRAR from MSGB multiplexing with other UE’s succesRAR or fallbackRAR based on its contention resolution ID and immediately applies the identified TA command in successRAR for subsequent SDT transmission.

When coming to co-existence of two TA timers and two NTAs, in general we prefer to let two TA timers to control TA validation for different uplink transmissions in RRC_INACTIVE. For timeAlignmentTimer it can be used only for controlling uplink transmissions for RA (i.e. MSG3 and HARQ feedback for MSG4) while for CG-SDT-TAT it can be used for controlling uplink transmissions during CG-SDT session except transmissions during RA in the middle of the on-going CG-SDT session.

Upon RA is successful completed, the maintained TA will be further confirmed by network to be valid so CG-SDT-TAT can be started or restarted at that moment similarly as in LTE PUR (see the below box) but timeAlignmentTimer shall be stopped after either transmitting HARQ feedback for MSG4 or failure from contention resolution since there is no need to keep two TA timers are both running during subsequent SDT transmission phase which is a little bit of different from LTE PUR which only one-shot transmission is supported.

- upon considering a Random Access procedure successfully completed: - start or restart the pur-TimeAlignmentTimer, if configured; - indicate to upper layers that the Timing Advance value has been adjusted; - if a temporary NTA has been stored, delete the stored temporary NTA.

Proposal10: CG-SDT-TAT can be started or restarted and the NTA for RACH shall be applied for CG-SDT-NTA upon successful completion of Random Access procedure.

Proposal11: timeAlignmentTimer shall be stopped after RACH completion, similar to on-demand SI request.

In addition to TA command in MSG2 or MSGB, UE may receive TAC MAC CE during subsequent transmission phase to update TA which make UE to start or restart CG-SDT-TAT as agreed in previous meeting. As a consequence, we think there is no need to start or restart timeAlignmentTimer as well though timeAlignmentTimer is indeed started or restarted in connected mode for above cases.

Proposal12: timeAlignmentTimer is not started when timing advance command is received during SDT subsequent transmission phase.

2.3.3	UE actions upon CG-SDT-TAT expiry

Another issue we think that should be discussed is what the behavior is after expiry of CG-SDT-TAT. In previous meeting RAN2 agreed to release CG configuration upon expiry of CG-SDT-TAT but for other behavior like those for timeAlignmentTimer as mentioned below which is still unclear.

1> when a timeAlignmentTimer expires: 2> if the timeAlignmentTimer is associated with the PTAG: 3> flush all HARQ buffers for all Serving Cells; 3> notify RRC to release PUCCH for all Serving Cells, if configured; 3> notify RRC to release SRS for all Serving Cells, if configured; 3> clear any configured downlink assignments and configured uplink grants; 3> clear any PUSCH resource for semi-persistent CSI reporting; 3> consider all running timeAlignmentTimers as expired; 3> maintain NTA (defined in TS 38.211 [8]) of all TAGs.

We think upon expiry of CG-SDT-TAT, the MAC entity shall clear configured grant for SDT, flush HARQ buffers. Since at this time, the configuration for CG-SDT has already be cleared, including the CG-TAT configuration, there is no point anymore to still maintain the NTA for CG-SDT.

Proposal13: UE clears configured grants, flushes HARQ buffer and stop maintaining CG-SDT-NTA upon expiry of CG-SDT-TAT.

2.4	UL carrier selection for subsequent CG-SDT

The issue of re-checking for SSB selection during subsequent transmission phase with mobility was covered in email discussion. We think it is beneficial to let network knowing timely on good DL beam change. In addition to that, we think re-checking for UL carrier and RSRP change should be also considered for subsequent transmission with mobility.

In previous meeting, RAN2 agreed to have UL carrier selection based on RSRP based on RSRP change for the first SDT transmission. However RAN2 did not discuss whether or not it also holds for subsequent UL transmissions. If the UE is moving in middle of the on-going SDT session, in addition to UE may reselect to another SSB for subsequent CG transmission and it may have some possibilities to make RSRP lower than the configured threshold for carrier selection. In such case we think UE shall reselect to another UL carrier for subsequent CG transmission

Proposal14: Uplink carrier reselection shall be re-evaluated for each subsequent transmission for CG-SDT.

2.6	RAN1 aspects regarding CG

2.6.1	Power control aspects

Two types of power control schemes have been defined by RAN1: open loop and closed loop power control. For SDT, initial uplink transmission can only be based on open loop power control. Then, for subsequent uplink transmission, a closed loop power control is beneficial to improve its performance if the subsequent uplink transmission can be based on dynamic grant. It can also be supported by the network send the power control command to the UE with L1-ACK, which RAN2 has discussed and agreed during the last meeting.

However, which scheme is selected is rather a RAN1 issue and should be determined by RAN1.

2.6.2	UL beam maintenance for SDT

In NR, beam operation is a critical feature. In RRC_CONNECTED, UL beam management is supported to always maintain an optimal UL beam. The network may trigger the UE to transmit SRS with different UL beams and measures all the SRS to determine the best UL beam, which can be subsequently indicated by the network for CG transmission.

For SDT over RRC_INACTIVE, the question comes how to determine the UL beam for CG transmission in RRC_INACTIVE, e.g. whether UL beam management is introduced. From our point view there are three possible solutions to this issue:

- Option1: The network indicates the UL beam in the RRCRelease message according to the beam measurements from UE in RRC CONNECTED. In RRC INACTIVE, the UE uses the indicated UL beam for SDT. This is mainly applicable to the UEs which are rather stationary, e.g. for the smart meter scenario.

- Option2: SRS configuration and transmission in INACTIVE is introduced. On one hand, it causes higher power consumption, but on the other hand it may improve the performance of SDT.

- Option3: UE determines the UL beam based on UE implementation, similar to RACH.

2.6.3	TAC delivery for CG-SDT

Anther issue is the issue of TAC delivery for CG-SDT. In legacy PUR, the TAC can be delivered either by MAC CR or DCI. Although our understanding is that the TAC in DCI is mainly used for the control plane solution while control plane solution is not supported in SDT, we would like RAN1 to confirm on their understanding on this.

As the above issues are rather RAN1 issues, we think it should be discussed by RAN1 and an LS should be sent asking RAN1 to analyze it.

Proposal 15: Send LS to RAN1, regarding:

- To define the power control mechanism for CG-SDT and;

- To determine the UL beam aspects for CG-SDT;

- To define how to deliver the TA command to the UE in CG-SDT.

- Conclusion

Based on the above discussion, we make the following observations and recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: CG-SDT configuration should be released when transitioning to RRC_CONNECTED.

Proposal 2: UE shall keep selecting the previous selected SSB if it is qualified during the subsequent transmission phase.

Proposal 3:  Define a SSB to CG-PUSCH association period that SSB reselection is triggered at the beginning of each such association period.

Proposal 4: The network should be able to reconfigure CG-SDT resources and the mapping between CG configuration(s) and SSBs during subsequent transmission phase of the SDT procedure.

Proposal5: All actually transmitted SSBs as indicated in SIB1 can be used for SSB subset determination for TA validation.

Proposal6: The RSRP for TA validation is derived by the SSB consolidation procedure in IDLE/INACTIVE mode spec.

Proposal7: TA validation by comparing RSRP with that at the previously UL transmission for CG is only applied for the initial CG-SDT.

Proposal8: When the CG-SDT-TAT is started, the UE shall maintain a CG-SDT-NTA different from the legacy NTA for the timing advance for CG-SDT.

Proposal9: UE shall apply TA command to NTA in random access response (4-step RA) and fallbackRAR when CG-SDT-TAT is running and use this NTA the uplink timing alignment for the rest of the RACH procedure.

Proposal10: CG-SDT-TAT can be started or restarted and the NTA for RACH shall be applied for CG-SDT-NTA upon successful completion of Random Access procedure.

Proposal11: timeAlignmentTimer shall be stopped after RACH completion, similar to on-demand SI request.

Proposal12: timeAlignmentTimer is not started when timing advance command is received during SDT subsequent transmission phase.

Proposal13: UE clears configured grants, flushes HARQ buffer and stop maintaining CG-SDT-NTA upon expiry of CG-SDT-TAT.

Proposal14: Uplink carrier reselection shall be re-evaluated for each subsequent transmission for CG-SDT.

Proposal 15: Send LS to RAN1, regarding:

- To define the power control mechanism for CG-SDT and;

- To determine the UL beam aspects for CG-SDT;

- To define how to deliver the TA command to the UE in CG-SDT.

- Reference