3GPP TSG-RAN WG2 Meeting #116-e                                            　    　　　　　　 	　　　R2-2110200

Online, Nov 1st – Nov 12th 2021

Source: 	KDDI Corporation

Title:	Discussion on RLM for inter-cell Multi-TRP

Tdoc Type:	Discussion

Agenda Item:	8.17.3

Document for:	Discussion and decision

Release:	Rel-17

Introduction

In this contribution, we would like to share our view on the RLM mechanism and Random Access problem of serving cell and non-serving cell for inter-cell Multi-TRP operation

Discussion

In the current RAN2 specification, RLF is declared separately for the MCG and the SCG. If radio link failure is detected for MCG and if fast MCG link recovery is not configured, the UE initiates the RRC connection re-establishment procedure, if the radio link failure is detected for SCG, if MCG transmissions of radio bearers are not suspended, the UE suspends SCG transmissions for all radio bearers and report SCGfailureinformation to the MN, instead of triggering re-establishment.

However, in the Rel-17 inter-cell multi-TRP discussion, although in RAN1, they are using term of “serving cell” and “not serving cell”. As mentioned in[1], we are not sure whether the current radio link failure detection mechanism works well for Rel-17 inter-cell multi-TRP , where “serving cell” and “not serving cell” are used simultaneously.  So we think it is better that RAN2 analyse the scenario and discuss the feasibility of the current radio link failure detection mechanism.

In our view, if we discuss the above mechanism, there might be two options, one is to reuse/enhance the current method, which is to detect the serving cell and not serving cell at the same time e.g both Pcells, one is a serving cell, the other one is not serving cell. The reason is that both serving cell and non-serving cell can still be considered to provide UE network connections, although they are defined with different names. Therefore, it might be possible if we consider them as one radio link failure detection group. Another option is to separate the detection for those two cells. The reason is that the functionality of serving cell and non-serving cell is still different. When there is radio link failure detected in serving cell, the radio link in non-serving cell might still be fine. Instead of initiating RRC re-establishment, maybe a lower layer link switch to the non-serving cell would be better.

Proposal 1: RAN2 agree to discuss RLM mechanism for Inter-cell TRP, combined RLM or separate RLM

Similar as radio link failure detection, in the current RAN2 specification, the UE action while receiving the random access problem indication from MCG or SCG is different, which means the random access problem indication of Pcell and Pscell is different. Therefore, do we combine the serving cell and non-serving cell as a Pcell group and apply the current mechanism for them? Or do we need to consider serving cell and non-serving cell separately, like serving cell can take the action similar as Pcell, while non-serving cell can take the action similar as Psell? It is better that we discuss the merit and demerit of each mechanism in RAN2.

Proposal2:RAN2 agree to discuss Random Access problem detection for Inter-cell TRP, combined detection or separate detection.

Conclusion

We introduce the following proposals.

Proposal 1: RAN2 agree to discuss RLM mechanism for Inter-cell TRP, combined RLM or separate RLM

Proposal2:RAN2 agree to discuss Random Access problem detection for Inter-cell TRP, combined detection or separate detection.

Reference

[1] R2-2107948 Multi-cell support for multi-TRP   Nokia, Nokia Shanghai Bell