3GPP TSG-RAN WG2 #116-e	R2-2110201

Electronic meeting, 1th –12th November, 2021

Agenda Item:	8.5.4

Source:	  NTTDOCOMO, INC.

Title:	Discussion on survival time state

Document for:	Discussion and Decision

1	Introduction

In the last RAN2 115e meeting [1], the following agreement are reached.

Agreements

- RAN2 does not assume that physical HARQ-NACK messages are always available, i.e. RAN2 will not mandate explicit HARQ-NACK feedback

- Given the application message size range under study, RAN2 will not optimize the ST design based on case of segmentation of message into multiple TBs. (This does not preclude the use of RLC segmentation; instead, it rules out optimizations for the case with RLC segmentation)

- Following entry into the Survival Time state, PDCP duplication for ST configuration is activated.  The gNB pre-configures which RLC entities can be activated for duplication when entering ST state.  FFS the number of supported RLC entities.

- RAN2 will at least continue working and discussing the HARQ NACK solution.  Details are FFS.

After R2-115e post meeting email discussion, there are still following two issues that need for further discussion.

Issue#1: UE duplication behavior for DC split bearer

Issue#2: whether UE activate all pre-configured RLC legs when entering into ST state.

Issue#3: Misalignment of PDCP duplication activation state between UE and network

2	Discussion

Issue#1

In case of DC split bearer, it is possible that UE receives HARQ NACK from both MN and SN side, so it is not clearer regarding the condition that UE should enter ST state. Generally, there are two options as following:

Option1: UE enters ST state when receiving N times of HARQ NACK at either MCG or SCG leg.

Option2: UE enters ST state when receiving HARQ NACK at both MCG and SCG leg and total count of NACK is larger than N times.

Option1 is agile for UE to immediately enter ST state to increase redundancy to avoid data delay while it is not resource allocation efficient from system level point of view. Moreover, it is still possible network could reconfigure the split bearer to MCG/SCG bearer where the radio condition is still good at either side. Therefore, we propose as following:

Proposal1: For DC split bearer, UE enters ST state when receiving HARQ NACK at both MCG and SCG leg and the total count of NACK is larger than N times.

Figure1 UE receives HARQ NACK at both MCG and SCG legs in case of DC split bearer

Issue#2

In R2-115e post meeting email discussion, one issue is controversial i.e. following entry into ST state, whether all pre-configured RLC legs for a DRB needs to be activated. As activating all RLC legs regardless of each leg’s channel condition is not resource efficient considering all UEs’ resource allocation in a cell level, it is more reasonable that UE could decide to activate which pre-configured RLC leg based on the DL channel condition (i.e. radio quality of the associated serving cells)

Observation2: UE has knowledge of the radio quality of pre-configured RLC leg, it is reasonable up to UE to decide which pre-configured RLC legs to be activated based on the radio quality of RLC leg.

Propposal2: Following entry into ST state, UE decides to activate which pre-configured RLC leg based on the DL channel condition (i.e. radio quality of the associated serving cells).

Issue#3

The understanding of duplication activation state between UE and network may not be consistent. The following cases may cause misalignment of PDCP duplication activation state between UE and network.

Case 1. In case the radio quality of RLC legs is bad in SCG, UE may only activate part of pre-configured RLC legs in SCG. As MN is not aware which pre-configured RLC legs is activated by UE at SCG, misalignment of PDCP duplication activation state between UE and MN may exist

Case 2. In case there is no HARQ feedback received from network due to misdetection, UE may enter ST state autonomous. Since network has no knowledge that UE has entered ST state, misalignment of PDCP duplication activation state between UE and network may exist

To resolve the issue above, the following solutions can be considered

Solution1: UE always activated all pre-configured RLC legs after entering ST state

Solution2: Network pre-configure with UE which RLC legs should be activated after entering ST state

Solution3: UE reports to network about which RLC legs have been activated after entering ST state

For Solution1, it is not resource allocation efficient as discussed above. For solution2, it is also not preferable since network does not know the radio condition in advance, it may wrongly pre-configure UE to activate RLC legs that has good radio condition. It is more reasonable that UE decides to activate which pre-configured RLC legs based on the channel condition and report the activated RLC legs to network (e.g. using Duplication RLC Activation/Deactivation MAC CE).

Observation3: In certain cases, the understanding of duplication activation state between UE and network may not be consistent.

Proposal3: To avoid misalignment of duplication activation state between network and UE, UE reports to network about which pre-configured RLC haven been activated after entering ST state.

3. Conclusion

Based on the discussion in the previous sections, we made the following proposals:

Observation1: For DC split bearer, it is possible that UE receives HARQ NACK from both MN and SN side, it is still open issue regarding the condition that UE should enter ST state.

Observation2: UE has knowledge of the radio quality of pre-configured RLC leg, it is reasonable up to UE to decide which pre-configured RLC legs to be activated based on the radio quality of RLC leg.

Observation3: In certain cases, the understanding of duplication activation state between UE and network may not be consistent.

Proposal1: For DC split bearer, UE enters ST state when receiving HARQ NACK at both MCG and SCG leg and the total count of NACK is larger than N times.

Propposal2: Following entry into ST state, UE decides to activate which pre-configured RLC leg based on the DL channel condition (i.e. radio quality of the associated serving cells).

Proposal3: To avoid misalignment of duplication activation state between network and UE, UE reports to network about which pre-configured RLC haven been activated after entering ST state.

4. References

- RAN2 115-e meeting chairman notes.