3GPP TSG-RAN WG2 #116-e	R2-2110202

Electronic meeting, 1th –12th November, 2021

Agenda Item:	8.12.2.2

Source:	  NTTDOCOMO, INC.

Title:	Access restriction on RedCap UE

Document for:	Discussion and Decision

1	Introduction

There is one LS [1] regarding RedCap UE access control from RAN3 as following:

---------------------------------------------------------Excerpted from [1] -------------------------------------------------------------

RAN3 would appreciate clarification on legacy gNB’s behaviour not supporting RedCap UEs, specifically:

- Can RAN2 confirm that RedCap UEs should not attempt to camp/access in legacy cells or be handed over to such cells; if so, can RAN2 please explain how access control will work for legacy gNBs. This is related to one option considered in RAN3, where it is assumed that the broadcast in supporting cells would be designed to indicate support (or access allowed), and the presence (or contents) of such broadcast would be indicated at Xn level by the possible introduction of new information elements, rather than a barring indication as mentioned in the LS.

- Can RAN2 confirm whether a legacy gNB can detect via the (RedCap) UE Radio Capabilities (e.g. at Handover preparation) that it cannot configure or serve the RedCap UE? This is related to another option considered by RAN3 in which a Rel-17 gNB can perceive, e.g., the support or barring by a neighbour gNB cell of RedCap UE via the handover preparation failure with signalling a proper cause value at XnAP level.

The above clarifications can help RAN3 to assess appropriate solutions for mobility control, including cases where legacy gNBs are involved.

---------------------------------------------------------End of Excerpt -------------------------------------------------------------

2	Discussion

Regarding how to avoid idle RedCap UE to access legacy gNB that does not support RedCap UEs, as RAN2 made the following agreement in the last meeting that UE can only be allowed to camp on the cell if RedCap-specific barring indication and IFRI is set as allowed in SIB1. Since legacy gNB does not broadcast this indication in SI, there is no issue for idle RedCap UE to camp on legacy gNB cell.

      - Specify separate indications in SIB1 for barring RedCap UEs with 1 Rx chain and 2 Rx chains.

      - Specify a RedCap specific IFRI in SIB1.

      - IFRI for RedCap UEs in SIB1 is common for UEs with 1 Rx or 2 Rx branches.

      - If RedCap-specific IFRI is absent from broadcast SI, the UE considers the cell does not support RedCap.

Observation1: For idle RedCap UE, broadcasting RedCap specific barring indication and IFRI in SI are used to control UE access. As legacy gNB does not broadcast this indication in SI, there is no issue for idle RedCap UE to camp on legacy gNB cell.

As for connected UE, as described in RAN3 LS there are two solutions as shown below on the table to choose. Generally, both two option1 and 2 can work. While option1 is more straightforward and it is more preferable for source gNB to know whether its neighbor gNB support RedCap UE in advance to avoid unnecessary handover failure. Detecting UE type from its UE capability is also not always easy work for target gNB. Moreover, when RedCap UE emerges in the market in near future, it is possible that operators do not upgrade all the gNBs in their network to support RedCap UE. In such case, Option2 would bring a lot handover preparation failures which may increase work load for network optimization.

Figure1: partial legacy gNBs may be upgraded to support RedCap UE in the near future

Option1: A new indication on Xn-AP message can be created (an example is as shown below [2]) to coordinate between RAN nodes on whether its serving or neighbor cells support 1Rx or 2Rx RedCap UE.

1 Rx RedCap access allowed 	EUMERATED {true} 	Optional,

2 Rx RedCap access allowed 	EUMERATED {true} 	Optional,

Option2: Legacy gNB can detect via the UE Radio Capabilities in Handover preparation information message that it cannot configure or serve the RedCap UE [3]. E.g. Detecting UE radio capabilities can be checking the mandatory UE capabilities to identify its UE type. If target gNB does not support RedCap UE, it will send appropriate cause value to source gNB with the following two options.

- If target gNB does not support RedCap UE, it reject the Handover request and include an appropriate cause value (“Control Processing Overload” if it is temporary or “Handover Target not Allowed”).

- If target gNB does not support RedCap UE, it reject the Handover request by sending HANDOVER PREPARATION FAILURE and include an appropriate cause value (e.g. insufficient UE capabilities)

HandoverPreparationInformation-IEs ::=  SEQUENCE {handover re

ue-CapabilityRAT-List                   UE-CapabilityRAT-ContainerList,

sourceConfig                            AS-Config                                       OPTIONAL, -- Cond HO

rrm-Config                              RRM-Config                                      OPTIONAL,

as-Context                              AS-Context                                      OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                                     OPTIONAL

}

Observation2: Option1 is straightforward and it is more preferable for source gNB to know whether its neighbor gNB support RedCap UE in advance to avoid unnecessary handover failure.

Observation3: when RedCap UE emerges in the market in near future, it is possible that operators do not upgrade all the gNBs in their network to support RedCap UE. Option2 would bring a lot handover preparation failures which may increase work load for network optimization.

Proposal: RAN2 send an reply LS to RAN3 to inform them the following:

- For idle RedCap UE, broadcasting RedCap specific barring indication and IFRI in SI are used for UE access control.

- For connected RedCap UE, A new indication on Xn-AP message can be created to coordinate between RAN nodes on whether its serving or neighbor cells support 1Rx or 2Rx RedCap UE in advance of handover.

- For connected RedCap UE, during handover preparation, target gNB can detect UE radio capabilities by checking the mandatory UE capabilities to identify its UE type. So the 2nd candidate solution in LS that if target does support RedCap UE, it reject the Handover request and include an appropriate cause value is also workable from RAN2 point of view.

3. Conclusion

Based on the discussion in the previous sections, we made the following proposals:

Observation1: For idle RedCap UE, broadcasting RedCap specific barring indication and IFRI in SI are used to control UE access. As legacy gNB does not broadcast this indication in SI, there is no issue for idle RedCap UE to camp on legacy gNB cell.

Observation2: Option1 is straightforward and it is more preferable for source gNB to know whether its neighbor gNB support RedCap UE in advance to avoid unnecessary handover failure.

Observation3: when RedCap UE emerges in the market in near future, it is possible that operators do not upgrade all the gNBs in their network to support RedCap UE. Option2 would bring a lot handover preparation failures which may increase work load for network optimization.

Proposal: RAN2 send an reply LS to RAN3 to inform them the following:

- For idle RedCap UE, broadcasting RedCap specific barring indication and IFRI in SI are used for UE access control.

- For connected RedCap UE, A new indication on Xn-AP message can be created to coordinate between RAN nodes on whether its serving or neighbor cells support 1Rx or 2Rx RedCap UE in advance of handover.

- For connected RedCap UE, during handover preparation, target gNB can detect UE radio capabilities by checking the mandatory UE capabilities to identify its UE type. So the 2nd candidate solution in LS that if target does support RedCap UE, it reject the Handover request and include an appropriate cause value is also workable from RAN2 point of view.

- The 2nd candidate solution in LS would bring a lot handover preparation failures, which may increase work load for network optimization.

4. References

- R2-2109342 Reply LS on the coordination between gNBs on the supporting of RedCap UEs

- R3-213267  Access Restriction for RedCap UEs 	Qualcomm Incorporated

- R3-213836  Discussion on the gNB coordination aspect for restricting RedCap UE access during Handover 		Ericsson