3GPP TSG-RAN WG2 Meeting #116 electronic                                           R2-2110264

Online, November, 2021

Agenda Item:	8.16.3

Source: 	CMCC

Title:  	Remaining Issues on supporting UE on-boarding and remote provisioning

WID/SID:          NG_RAN_PRN_enh

Document for:	Discussion and Decision

Introduction

In previous RAN2 meetings, the most related issues on supporting UE on-boarding and provisioning for NPN have been addressed. The left issues are related to the follows:

- Whether a common list of GINs used for on-boarding and SNPN access using external credentials

- Maximum number of GINs

In this contribution, we would concentrate on the left issues discussed in TR 23700-07 [2] and summary document [3] to support UE on-boarding and provisioning in idle mode and connected mode for SNPN.

Discussion

2.1 Re-usage of list of GINs used for on-boarding and SNPN access using external credentials

The left issues are related to the follows:

- Wait for SA2 reply LS on the issue whether a common list of GINs used for on-boarding and SNPN access using external credentials.

Regarding the issue of Group IDs for Network selection (GINs), SA2 sent a reply LS to RAN2 as below [5]:

- SA2 thanks RAN2 for the LS on "Group IDs for Network selection (GINs)" - RAN2 asked the following: - Question 1a: Can a common list of GINs be used for both onboarding and SNPN access using external credentials? - SA2 answer: Yes - Question 1b: Can RAN2 assume that higher layers may use different GIN values for each feature? - SA2 answer: SA2 does not make any assumption i.e. either same or different GIN value for each feature is possible. - Question 2: Can SA2 clarify whether a GIN is encoded: - as an SNPN ID (i.e., a PLMN ID plus a NID); or - as a NID. - SA2 answer: SA2 agrees that GIN is encoded as a) an SNPN ID (i.e., a PLMN ID plus a NID).

As replied from SA2, a common list of GINs can be used for both on-boarding and SNPN access using external credentials. And SA2 does not make any assumption i.e. either same or different GIN value for each feature is possible.

If RAN2 decides to further restrict that the same GIN values can be used for both on-boarding and SNPN access using external credentials, this may cause the UE supporting different features to confuse the meaning of GINs. Therefore, it is reasonable that both same and different GIN values for each feature are needed to be specified.

Observation 1:  Per replied from SA2, a common list of GINs can be used for both on-boarding and SNPN access using external credentials, and SA2 does not make any assumption i.e. either same or different GIN value for each feature is possible. This means both same and different GIN values for each feature are needed to be specified.

Proposal 1: Both same and different GIN values for each feature are needed to be supported in NPN.

2.2 Maximum number of GINs

In the current Running CR TS38.331, the maximum number of GINs is still FFS:

- - ASN1START - -- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-START - maxNrofGIN-r17 INTEGER ::= FFS -- Maximum number of GINs in gin-List-r17 - -- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-STOP - -- ASN1STOP

In Rel-16, the maximum number of networks is set to 12 in the RRC signalling. And in RAN2 #114e meeting, RAN2 had agreed that the supported Group IDs are broadcasted in a new SIB, which will provide more room for Group ID broadcasting.  Moreover, in RAN2 #115e meeting, it is agreed that the maximum number of GINs is specified per cell and from the LS sent by SA2 [5] indicates that GIN is encoded as an SNPN ID (i.e., a PLMN ID plus a NID).  Therefore a typical GIN size is 68 bits generally, in case of the GIN constructed by 24 bits-PLMN ID and 44 bits-NID. As specified that the maximum size of a SIB message is 2976 bits, then it is possible that up to 43 GINs can fit in a new SIB. However, since, in common cases, there is no so much GID requirement for different commercial purpose in one cell, it is preferred that the maximum number of GINs can be 12 separately.

Observation 2: Generally, a typical GIN size is 68 bits, in case of the GIN constructed by 24 bits-PLMN ID and 44 bits-NID. As specified that the maximum size of a SIB message is 2976 bits, then it is possible that up to 43 GINs can fit in a new SIB. However, considering in common cases there is no so much GID requirement, it is preferred that the maximum number of GINs can be 12 separately.

Proposal 2: The maximum number of GINs listed in the new SIB can be 12.

Conclusion

Observation 1:  Per replied from SA2, a common list of GINs can be used for both on-boarding and SNPN access using external credentials, and SA2 does not make any assumption i.e. either same or different GIN value for each feature is possible. This means both same and different GIN values for each feature are needed to be specified.

Proposal 1: Both same and different GIN values for each feature are needed to be supported in NPN.

Observation 2: Generally, a typical GIN size is 68 bits, in case of the GIN constructed by 24 bits-PLMN ID and 44 bits-NID. As specified that the maximum size of a SIB message is 2976 bits, then it is possible that up to 43 GINs can fit in a new SIB. However, considering in common cases there is no so much GID requirement, it is preferred that the maximum number of GINs can be 12 separately.

Proposal 2: The maximum number of GINs listed in the new SIB can be 12.

Reference

- 3GPP TS 23.501 v16.1.0 System Architecture for the 5G System; stage2.

- 3GPP TS 23.700-07	 v17.0.0(2021-03) Study on enhanced support of non-public networks (NPN); (Release 17)

- R2-2109033 [Pre115-e][009][eNPN] Summary 8.16.2 ext credentials + 8.16.3 onboarding (Nokia); Nokia	discussion	Rel-17	NG_RAN_PRN_enh-Core

- 3GPP TS 36.304 v16.2.0 User Equipment (UE) procedures in idle mode

- S2-2106708 Reply to LS on Group IDs for Network Selection(GINs),SA2, Ericsson