3GPP TSG-RAN WG2 Meeting #116-e	R2-2110286

Electronic, 1 - 12 November 2021

Agenda Item:	8.1.3.2

Source: 	TCL

Title: 	Indication for Multicast Activation notification

Document for:	Discussion and decision

1	Introduction

In RAN2#114 meeting, PCCH based group notification for multicast for MBS supporting nodes is agreed.

Use PCCH for Multicast activation notification (also for MBS supporting nodes).

Confirm that we convey the MBS session ID in the notification.

Use of paging in all (legacy) PO with PRNTI is the baseline assumption (can still discuss other variants)

In RAN2#115 meeting，the following agreements have been made.

Provided RAN3 confirms, paging for multicast activation notification is used in the relevant legacy POs for the UEs with deactivated multicast session(s).

RAN2 sends an LS to RAN3 and SA2 to indicate its preference for paging for multicast activation notification to be used in the relevant legacy POs for the UEs with non activated multicast session(s). Further, RAN2 requests RAN3 for confirmation and if so, also specifying required network signalling.

Confirm extending the unicast paging message to include a new paging record list ( pagingGroupList) for group activation notification of multicast sessions.

In this contribution, we will further discuss the indication for multicast activation notification in corresponding paging message.

- Discussion

In order to avoid legacy UE or UE not interested in MBS monitor R-RNTI unnecessarily, in case that paging message is transmitted only for multicast activation indication, during the email discussion [AT115-e][048][MBS], it has been discussed whether the indication for multicast activation notification should be introduced or not, and what kind of mechanism should be used. The discussion is summarized in the report R2-2109078. Some companies support the proposal that short message indication is used while some others consider WUS/PEI based indication.

The proponents of short message indication mechanism consider this is a straight way forward, and can be easily achieved by making use of the existing short message or short messaged indicator field in DCI scrambled by P-RNTI, e.g., the previously reserved state‘00’can be defined in R17 to indicate that the current paging message in the scheduled PDSCH contains multicast activation notification only.

However, there are very limited reserved values for short message indicator or short message, especially for short message indicator which has only one value reserved, therefore we should use these values very carefully.

Power saving issue have been discussed during the past several releases, and some mechanisms have been introduced. WUS initially introduced for NB-IOT devices can be a viable solution to notify that paging contains MBS paging only.

Additionally, the ongoing PowSav WI in R17 is discussing PEI which is used to inform an idle/inactive UE before a paging occasion whether to monitor Paging PDCCH or not. UEs are divided into subgroups and each group is assigned a subgroup ID. The PEI approach can be used for group notification. The possible way forward used for indicating weather the paging is for multicast activation indication only could be PEI-based indication to indicate UE to monitor a PO if UE’s subgroup ID is paged, otherwise UE is not required to monitor a PO if UE does not detect PEI-based indication at the target PO.

Proposal 1: RAN2 to agree that WUS/PEI based indication for multicast activation in corresponding paging message is used.

In RAN1, it has been discussed if a new RNTI for PEI should be introduced. Some companies assume legacy P-RNTI would be enough for this kind of indication. However, we think a new RNTI for PEI can be considered in order to differentiate it from Paging and other DCI formats.

Proposal 2: If proposal 1 is agreed, a new RNTI for WUS/PEI should be specified.

3	Conclusions

Based on our discussion, we have the following proposals:

Proposal 1: RAN2 to agree that WUS/PEI based indication for multicast activation in corresponding paging message is used.

Proposal 2: If proposal 1 is agreed, a new RNTI for WUS/PEI should be specified.

4	References

- RAN2-114 Meeting Report

- RAN2-115 Meeting Report

- R2-2109078 [AT115-e][048][MBS] Notifications