3GPP TSG-RAN WG2 Meeting RAN2#116 electronic                        R2- 2110331

Online, November 1st – November 12th, 2021

Agenda item:		8.12.3.1

Source:		Lenovo, Motorola Mobility

Title:		       Consideration on eDRX for RedCap UE

Document for:		Discussion and Decision

- Introduction

Based on RAN2#115-e, the following agreements on eDRX for Redcap UE were made as in [1]:

Agreements:

- When IDLE eDRX cycle is longer than 10.24s, PH calculation formula defined in LTE is re-used, i.e.

PH_CN:  H-SFN mod TeDRX,_CN,H= (UE_ID_H mod TeDRX_CN,H)

-  where TeDRX_CN,H is equal to IDLE eDRX cycle.

2.	When IDLE eDRX cycle is longer than 10.24s, CN PTW_end calculation formula defined in LTE is re-used, i.e.

PTW_end is radio frame satisfying SFN = (PTW_start + L*100 - 1) mod 1024,

- where L is PTW length configured by upper layers.

3.	For RRC_IDLE UE, when eDRX cycle is no longer than 10.24s, T is determined by IDLE eDRX cycle. When eDRX cycle is longer than 10.24s, during the CN PTW, T is determined by the shortest of UE specific DRX cycle, if configured by upper layer, and default paging cycle.

4.	For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and Inactive eDRX cycle is not configured, during CN PTW, T is determined by the shortest of UE specific DRX cycle, if configured by upper layer, RAN paging cycle and default paging cycle.

5.	For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and Inactive eDRX cycle is no longer than 10.24s, outside CN PTW, T is determined by INACTIVE eDRX cycle.

Agreements via email - from offline 105 second round:

- RAN2 considers the configuration as an invalid case, where INACTIVE eDRX cycle is configured but IDLE eDRX cycle is not configured. FFS whether to capture this restriction in RAN2 spec.

- RAN2 considers the configuration as invalid case, where INACTIVE eDRX cycle is longer than IDLE eDRX cycle. FFS whether to capture this restriction in RAN2 spec.

- The maximum PTW length is 40.96s when IDLE eDRX cycle is longer than 10.24s.

- The minimum PTW length is 1.28s and the step length/granularity of PTW length is 1.28 when IDLE eDRX cycle is longer than 10.24s.

- Introduce an additional new IE for INACTIVE eDRX to contain all values of INACTIVE eDRX cycles (also include values >10.24, if agreed in future).

- For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is no longer than 10.24s, T is determined by the shortest of IDLE eDRX cycle and INACTIVE eDRX cycle.

- For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is no longer than 10.24s, during CN PTW, T is determined by the shortest of UE specific DRX cycle, if configured by upper layer, INACTIVE eDRX cycle and default paging cycle.

- eDRX feature is optional for any UE (including RedCap and non-RedCap UEs).

Agreements via email - from offline 105 third round

- eDRX is optional for any gNB (either supporting RedCap or not), which means it is up to gNB implementation whether to support eDRX

Working Assumption:

- When IDLE eDRX cycle is longer than 10.24s, CN PTW_start calculation formula defined in LTE is re-used as the baseline, as below. FFS whether CN PTW_start position could be configurable by network and in case which node decides the N value. Note: this formula would be revisited if INACTIVE eDRX cycle can be above 10.24s

PTW_start denotes the first radio frame of the PH that is part of the PTW and has SFN satisfying the following equation:

SFN = 1024/N* ieDRX, where

ieDRX = floor(UE_ID_H /TeDRX,H) mod N

FFS N = 4 or 8, FFS if N can take other values

Agreements online:

- For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is not configured, FFS which option below is adopted for paging monitoring:

Option 1: T is determined by the shortest of RAN paging cycle, IDLE eDRX cycle, and default paging cycle.

Option 2: T is determined by the shortest of RAN paging cycle and IDLE eDRX cycle.

- For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is not configured, outside CN PTW, FFS which option below is adopted for paging monitoring:

Option 1: T is determined by the shortest of RAN paging cycle and default paging cycle.

Option 2: T is determined by RAN paging cycle.

In this contribution, it mainly focuses on the design of a same PTW starting location for RAN paging and CN paging, and the issue on UE behaviour if RAN PTW and CN PTW are both configured or only CN PTW is configured for UE.

- Discussion

In LTE, the UE will monitor the PTW starting based on the parameter ieDRX (SFN = 256* ieDRX , ieDRX = floor(UE_ID_H /TeDRX,H) mod 4) in the starting PH. Based on the agreement in RAN2#115-e meeting, if IDLE eDRX cycle is longer than 10.24s, CN PTW_start calculation formula defined in LTE is re-used as the baseline. eDRX is configured for both RAN and CN paging, the PTW based on eDRX configuration for RAN paging and the PTW based on the eDRX configuration for CN paging should have the same PTW starting location in the coincided PH (H-SFN mod TeDRX,H= (UE_ID_H mod TeDRX,H) ). Two options are proposed as follows.

- option.1, ieDRX will be a value from the set {ieDRX based on eDRX for RAN paging, ieDRX based on eDRX for CN paging}

- option.2, a new formulation is used for ieDRX, for example, SFN = (UE_ID mode 1024)

For option.1, some condition could be considered to choose the value as ieDRX, for example, the early value in the set for ieDRX to avoid the missed paging message, or the PTW starting location for CN mode will follow the ieDRX for inactive mode if ieDRX for CN paging is smaller than ieDRX for inactive, the reason is CN paging is usually later than the RAN paging for the same paging procedure. For option.2, a simple formulation is applied to select one random SFN in HF as a common PTW starting.

Proposal 1: If eDRX is configured to both RAN paging and CN paging, PTW starting location could be a value from the set of { ieDRX based on eDRX for RAN paging, ieDRX based on eDRX for CN paging } based on legacy LTE mechanism or based on a new formulation(for example, SFN = (UE_ID mode 1024)).

In NR, based on current agreement, eDRX could be configured to CN paging and RAN paging independently. It is up to RAN to configure the length for PTW for RAN paging, which could be different from the one for CN paging configured by upper layers. Besides, the eDRX cycle for RAN paging could be also different from the one for CN paging.

If eDRX is configured to CN paging and RAN paging and both of the eDRX cycle are longer than 10.24s, for idle mode UE（such as inactive mode to idle mode）, since no RAN paging will be received by UE, it is reasonable to monitor the PO as the idle mode UE in LTW when eDRX is only configured for the CN paging. As in LTE, it will apply the eDRX configuration for CN paging and only monitor the PO associated to the PTW for CN paging. As in LTE, the DRX cycle T associated to PTW is reasonable to be the shortest of the UE specific DRX value, if allocated by upper layers, and a default DRX value broadcast in system information.

Proposal 2: If eDRX is configured for CN paging and RAN paging and both of the eDRX cycles are longer than 10.24s, idle mode UE will only monitor the PO associated to the PTW for CN paging, the DRX cycle T associated to PTW is suggested to be the shortest of the UE specific DRX value, if allocated by upper layers, and a default DRX value broadcast in system information.

If eDRX is configured to CN paging and RAN paging, for inactive mode UE, since both eDRX for RAN paging and eDRX for CN paging could be configured to UE and both of the eDRX cycle are longer than 10.24s, UE is better to monitor the PO only associated to each type of PTW from the view of UE power saving. If the PTW is overlapped with each other, we think both type of PTW needs to be monitored to avoid missing the paging. In details, in order to align the PTW and PO in each type of PTW as much as possible, the DRX cycle T associated to CN PTW and RAN PTW could be both defined as the shortest of the RAN paging cycle, the UE specific paging cycle, if allocated by upper layers and the default paging cycle.

Proposal 3: If eDRX is configured for CN paging and RAN paging and both of the eDRX cycles are longer than 10.24s, inactive mode UE is suggested to monitor the PO associated to PTW for CN paging and PTW for RAN paging.

Proposal 4: If eDRX is configured for CN paging and RAN paging, for inactive mode UE, the DRX cycle T associated to CN PTW and RAN PTW is suggested both to be the shortest of the RAN paging cycle, the UE specific paging cycle if allocated by upper layers, and the default paging cycle.

- Conclusion

In this contribution, the issues on the design of a same PTW starting location for RAN paging and CN paging, and the issue on UE behaviour based on eDRX configuration for CN paging and RAN paging are discussed, we have the following proposals:

Proposal 1: If eDRX is configured to both RAN paging and CN paging, PTW starting location could be a value from the set of { ieDRX based on eDRX for RAN paging, ieDRX based on eDRX for CN paging } based on legacy LTE mechanism or based on a new formulation(for example, SFN = (UE_ID mode 1024)).

Proposal 2: If eDRX is configured for CN paging and RAN paging and both of the eDRX cycles are longer than 10.24s, idle mode UE will only monitor the PO associated to the PTW for CN paging, the DRX cycle T associated to PTW is suggested to be the shortest of the UE specific DRX value, if allocated by upper layers, and a default DRX value broadcast in system information.

Proposal 3: If eDRX is configured for CN paging and RAN paging and both of the eDRX cycles are longer than 10.24s, inactive mode UE is suggested to monitor the PO associated to PTW for CN paging and PTW for RAN paging.

Proposal 4: If eDRX is configured for CN paging and RAN paging, for inactive mode UE, the DRX cycle T associated to CN PTW and RAN PTW is suggested both to be the shortest of the RAN paging cycle, the UE specific paging cycle if allocated by upper layers, and the default paging cycle.

- References

RAN2#115-e chairman note.

36.304, “User Equipment (UE) procedures in idle mode”.