3GPP TSG-RAN WG2 Meeting #116 electronic							R2-2110349

Online, November 1 – 12, 2021

Agenda Item:	8.6.4

Source: 	SONY

Title:	Remaining issues of RACH-based SDT in NR

Document for:	Discussion and Decision

# Introduction

RAN2 made good progress in the last few meetings (RAN2#112, RAN2#113, RAN2#113bis, RAN2#114 and RAN2#115) for SDT WI. However, there are still some open issues to be finalised in this meeting.

In this contribution, we discuss some remaining open issues for RA-SDT such as supporting SDT for RedCap UEs, RA-RNTI collision for SDT and non-SDT UEs, UE switching from SDT to non-SDT and Beam Change Reporting.

## RACH-based SDT

SDT for RedCap UEs: RAN1 discussed whether a RedCap UEs can be configured separate initial DL and UL BWPs and made the following agreement/working assumption:

Agreement

- For a cell that allows a RedCap UE to access, network can configure a separate initial UL BWP for RedCap UEs in SIB

  - It can be used both during and after initial access.

  - It is no wider than the maximum RedCap UE bandwidth.

  - It is always configured if the initial UL BWP for non-RedCap UEs is wider than the maximum RedCap UE bandwidth

  - This applies to both TDD and FDD (including FD FDD and HD FDD) cases

  - FFS whether part of the configuration is implicitly signaled

Working Assumption

- For a cell that allows a RedCap UE to access, network can configure a separate initial DL BWP for RedCap UEs in SIB.

  - Working assumption: It can be used during initial access

  - It can be used after initial access.

  - It is no wider than the maximum RedCap UE bandwidth.

  - FFS: It is always configured if the initial DL BWP for non-RedCap UEs is wider than the maximum RedCap UE bandwidth.

  - This applies to both TDD and FDD (including FD FDD and HD FDD) cases.

  - Working assumption: It applies at least after initial access for FR1 when MIB configured CORESET#0 is included

So, based on the above RAN1 agreement/working assumption, RAN2 should allow SDT to be available and configured for RedCap UEs in INACTIVE state because these are the type of devices that benefits the most from small data transmissions for power saving reasons, for example wearable devices and industrial wireless sensors. This means that RAN2 should take into account RAN1 agreement on RedCap feature for configuring RedCap initial BWP for SDT.

Proposal 1: RAN2 should take into account RAN1 agreement on RedCap feature, for configuring RedCap initial BWP for SDT.

RA-RNTI collision for SDT and non-SDT: As shown on Figure 1, SDT and non-SDT PRACH occasions (RO) can be configured independently in an FDM manner with the same starting OFDM symbol on the same slot of the same bandwidth part (BWP) of a cell.

From this figure, UE1 and UE2 have the same starting OFDM symbol and the same frequency offset for the ROs locating on the same slot of the same active BWP. Hence, after UE1 and UE2 have transmitted PRACH on their corresponding uplink ROs, they will monitor a DCI in the downlink with the same RNTI on the same PDCCH search space and this results in an RNTI collision or ambiguity unless some remedy is adopted.

In our view, the above issue can be solved simply by introducing a configurable offset (s-offset) for the symbol index of the SDT RNTI equation.

Figure 1. FDM multiplexing of SDT and non-SDT PRACH occasions with same starting symbol.

Proposal 2: RAN2 to discuss how to address the RA-RNTI collision for SDT and non-SDT when their ROs have the same starting OFDM symbol and the same frequency offset locating on the same slot of the same active BWP.

UE switching from SDT to non-SDT: RAN2 discussed in the past whether a UE can switch from SDT to non-SDT if the initial UL transmission (in msgA/Msg3) fails for a configured number of times, but no agreement was reached. From our point of view, in order to handle the case where there is no response from the network, there should be a fallback mechanism for the UE to initiate to move to connected mode. Hence, a UE should be able to switch from the failed SDT to non-SDT procedure.

Proposal 3: UE switches from SDT to non-SDT when initial SDT UL transmission (in msgA/Msg3) fails configured number of times.

Beam Change Reporting: RAN1 has just agreed that BFD/BFR procedure is not required for SDT in Rel-17. But leaves an open issue whether to report the beam change to gNB. In our view, if BFD/BFR procedure is not required for SDT due to small period of time for data transmission, then reporting beam change in UL message is not required, unless the legacy message of initiating RACH procedure is reused for SDT as well. The reason why initiating RACH procedure is acceptable because there are other cases for triggering RACH procedure such as when TAT expires and when no valid UL resource is available for RA-SDT.

Proposal 4: After beam failure for RA-SDT, UE triggers RACH procedure for reporting the beam change to gNB.

## Conclusions

In this contribution, we have discussed some remaining open issues for RA-SDT such as supporting SDT for RedCap UEs, RA-RNTI collision for SDT and non-SDT UEs, UE switching from SDT to non-SDT and Beam Change Reporting. We have the following proposals:

Proposal 1: RAN2 should take into account RAN1 agreement on RedCap feature, for configuring RedCap initial BWP for SDT.

Proposal 2: RAN2 to discuss how to address the RA-RNTI collision for SDT and non-SDT when their ROs have the same starting OFDM symbol and the same frequency offset locating on the same slot of the same active BWP.

Proposal 3: UE switches from SDT to non-SDT when initial UL transmission (in msgA/Msg3) fails configured number of times.

Proposal 4: After beam failure for RA-SDT, UE triggers RACH procedure for reporting the beam change to gNB.

## References

[1] RP-201305, “Updated Work Item on NR small data transmissions in INACTIVE state”, RAN#88e.

[2] TS 38.321 V16.6.0