3GPP TSG-RAN WG2 Meeting #116 Electronic	R2-2110368

1 – 12 November 2021

Agenda item:	8.16.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Cell selection for onboarding

WID/SID:	NG_RAN_PRN_enh - Release 17

Document for:	Discussion and Decision

# 1	Introduction

This paper discusses the initial cell selection for onboarding.

# 2	Discussion

## 2.1	Cell selection

In the reply LS R2-2102658 SA2 provided the following answers to RAN2's questions related to support of onboarding:

Question 3: Can RAN2 assume uniform support of onboarding in all cells in an O-SNPN? (I.e. can RAN2 assume that all cells of an O-SNPN broadcasts the support for onboarding or can some cells not set the ”onboardingEnabled” bit to e.g. control RAN congestion

[SA2 answer] The ”onboardingEnabled” bit can be set/enabled per cell  The onboarding functionality can be enabled only in a part of the SNPN network thereby avoiding the load from onboarding UEs in cells that does not indicate the support for onboarding. The parameter is used to assist the UE in network selection.

Even if there is no uniform support and a UE moves to a cell in an O-SNPN not supporting onboarding, SA2 foresees no impact to mobility procedures as remote provisioning can continue in the target cell. Once the PDU session for remote provisioning has been activated existing 5GS functionality applies for mobility.

According to the answer from SA2 the “onboardingEnabled” flag can be specific to each cell in the O-SNPN. When there are multiple cells of the selected O-SNPN that meet the cell selection criteria then it is up-to UE implementation which of those cells is selected for initial access. If the “onboardingEnabled” flag is not set in all cells of the O-SNPN and it is not checked during initial cell selection then the UE may select a cell that does not support onboarding in the selected O-SNPN, and UE access request will be rejected. This may lead to permanent failure for a UE, e.g., if onboarding is only supported in a specific frequency band in an O-SNPN and a UE implementation always selects a different band for initial access.

Therefore, our view is that the “onboardingEnabled” flag needs to be checked by the UE when it intends to access the O-SNPN to start an onboarding session. The onboarding indication should come from NAS. The requirement for the NAS indication is not new, as it was agreed that the RRCSetupComplete message includes the onboarding request indication.

Proposal 1: The "onboardingEnabled" flag shall be considered by the UE for initial cell selection without impacting cell reselection when NAS indicates that the session is triggered due to onboarding.

Proposal 1 can be implemented with a minor change in clause 4.5 (Cell Categories) in 38.304:

**** Start of changes ****

suitable cell:

For UE not operating in SNPN Access Mode, a cell is considered as suitable if the following conditions are fulfilled:

-	The cell is part of either the selected PLMN or the registered PLMN or PLMN of the Equivalent PLMN list, and for that PLMN either:

-	The PLMN-ID of that PLMN is broadcast by the cell with no associated CAG-IDs and CAG-only indication in the UE for that PLMN (TS 23.501 [10]) is absent or false;

-	Allowed CAG list in the UE for that PLMN (TS 23.501 [10]) includes a CAG-ID broadcast by the cell for that PLMN;

-	The cell selection criteria are fulfilled, see clause 5.2.3.2.

According to the latest information provided by NAS:

-	The cell is not barred, see clause 5.3.1;

-	The cell is part of at least one TA that is not part of the list of "Forbidden Tracking Areas for Roaming" (TS 22.011 [18]), which belongs to a PLMN that fulfils the first bullet above.

For UE operating in SNPN Access Mode, a cell is considered as suitable if the following conditions are fulfilled:

-	The cell is part of either the selected SNPN or the registered SNPN of the UE;

-	The onbordingEnabled is true if the cell is selected as a part cell selection process (see clause 5.2.3) and NAS indicated that session is triggered due to onboarding;

-	The cell selection criteria are fulfilled, see clause 5.2.3.2;

According to the latest information provided by NAS:

-	The cell is not barred, see clause 5.3.1;

-	The cell is part of at least one TA that is not part of the list of "Forbidden Tracking Areas for Roaming" which belongs to either the selected SNPN or the registered SNPN of the UE.

**** End of changes ****

Proposal 2: Include the above changes into clause 4.5 of 38.3044 to enable checking of "onboardingEnabled" flag during initial cell selection.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: The "onboardingEnabled" flag shall be considered by the UE for initial cell selection without impacting cell reselection when NAS indicates that the session is triggered due to onboarding.

Proposal 2: Include the above changes into clause 4.5 of 38.3044 to enable checking of "onboardingEnabled" flag during initial cell selection.