3GPP TSG-RAN WG2 Meeting #116 electronic	R2-2110399

Online, 1st -12th November 2021

Source:	CATT

Title:	Consideration on CP issues

Agenda Item:	8.6.3

Document for:	Discussion and Decision

Introduction

In this contribution, we would like to discuss some control plane issues, including considerations on control plane SDT, periodic RNA update during SDT procedure, SI acquisition, paging message reception, SDT failure detection timer and measurements.

Discussion

- Control plane SDT

Control plane SDT is agreed to consider in R17 SDT WID [1], as shown below:

Specify configuring of SRB1 and SRB2 for small data transmission in RRC_INACTIVE state by reusing the framework for DRBs.

According to current running 38.331 CR, some descriptions on control plane SDT has already been captured as listed below:

- Small data transmission for CP data is configured by the network on a per SRB basis via RRCRelease message.

- Data volume threshold for SDT selection is common for both CP SDT and UP SDT. Stored configuration in the UE context is also used for the RLC bearer configuration for any SRB which is configured for SDT.

- If SRB2 is configured for SDT by the network, the UE shall re-establish SRB2 PDCP entity and resume SRB2 upon initiating RRC resume procedure for SDT.

In RAN2#111e meeting, we agreed:

3 The first UL message (i.e. MSG3 for 4-step RACH, MSGA payload for 2-step RACH and the CG transmission for CG) may contain at least the following contents (depending on the size of the message): - CCCH message (needs to be included) LCP can be used to determine to priority of the content below that may be included - DRB data from one or more DRBs which are configured by the network for small data transmission - MAC CEs – (e.g. BSR). FFS other MAC CEs - Padding bits

CP data for SDT, which is different from RRCResumeRequest message, can also be sent in the first UL message. But it is not clear which RRC message would be used in the first UL message for control plane SDT. According to the current procedures, only NAS related messages as CP small data have the requirement to be transmitted with RRC_INACTIVE state. NAS related messages can be transmitted via ULInformationTransfer message if SRB1/SRB2 has already been resumed. Hence, we propose to confirm that ULInformationTransfer message which contains CP small data may be included in the first UL message for SDT. If there is some other CP small data need to be transmitted when the UE is in RRC_INACTIVE state, we can discuss and introduce additional RRC message in the first UL message for SDT case by case.

Proposal 1: ULInformationTransfer message which contains CP small data can be included in the first UL message for CP SDT if SRB2 is configured for SDT by the network.

- Periodic RNA update during SDT procedure

A UE in the RRC_INACTIVE state is required to initiate RNA update procedure when it moves out of the configured RNA and when periodic RNA Update timer (T380) expires. If event triggered RNA update is performed, cell reselection must happen. Thus, how to handle event triggered RNA update during SDT can be discussed further after the conclusion for cell reselection during SDT procedure.

According to the current TS 38.331, T380 for periodic RNA update is stopped upon reception of RRCResume message. It is possible that T380 expires and periodic RNA update is triggered during SDT procedure within RRC_INACTIVE. There are some possible options to handle the case:

Option 1: UE triggers another RRC Resume procedure.

Option 2: Periodic RNA update is cancelled.

Option 3: UE sends RNA update indicator via DCCH message.

Periodic RNA update procedure can ensure reachability between the UE and NG-RAN node. However during SDT procedure within RRC_INACTIVE, the connection between the UE and NG-RAN node is available. Therefore, periodic RNA update procedure is not necessary in this case. Hence, option 2 is preferred. As SDT procedure may fall back to normal RRC Resume procedure, it is reasonable to cancel periodic RNA update procedure if T380 expires during SDT procedure.

Proposal 2: Periodic RNA update is cancelled if T380 expires during SDT procedure.

- SI acquisition during SDT

In last RAN2#114 e-meeting, we agreed:

4. The UE needs to monitor paging after UE initiates SDT for system information change, PWS. FFS for other cases

And the UE continues performing cell reselection during SDT. That means the UE may need to acquire valid SI for cell reselection/PWS during SDT.

Proposal 3: The UE needs to receive SI for cell reselection/PWS via broadcast during SDT.

On-demand SI mechanism in RRC_IDLE/INACTIVE was introduced in R15 NR, then on-demand SI mechanism in RRC_CONNECTED was introduced in R16 NR. During SDT, required SI(s) for cell reselection/PWS may not be broadcasting. The UE needs to trigger on-demand SI procedure to acquire required SI(s). There are two possible ways for the UE to trigger on-demand SI procedure during SDT:

Option 1: Use on-demand SI procedure in RRC_IDLE/INACTIVE.

Option 2: Use on-demand SI procedure in RRC_CONNECTED.

On-demand SI procedure in RRC_IDLE/INACTIVE supports to request SI(s) for cell reselection/PWS. But on-demand SI procedure in RRC_CONNECTED in R16 only supports to request SIB(s) for positioning, NR sidelink communication and V2X sidelink communication. If on-demand SI procedure in RRC_CONNECTED is used during SDT within RRC_INACTIVE, some enhancements need to be introduced. Hence, we propose:

Proposal 4: During SDT, the UE applies on-demand SI procedure in RRC_IDLE/INACTIVE if needed.

- Paging message during SDT

In normal scenario, a RAN paging trigger event occurs due to incoming DL user plane, DL signalling from 5GC, etc.

According to the current TS, UEs in RRC_INACTIVE is required to monitor paging message. If the UE in RRC_INACTIVE receives a paging message which includes an identity that matches the UE's stored fullI-RNTI, the UE triggers the RRC connection resumption procedure. If the UE in RRC_INACTIVE receives a paging message which includes an identity that matches the UE identity allocated by upper layers, the AS layer of the UE enters RRC_IDLE and the NAS layer of the UE triggers RRC connection establishment procedure for MT. If a UE in RRC_INACTIVE misses paging message, it cannot trigger RRC connection resumption or RRC connection establishment procedure for MT timely. But if the UE has already triggered RRC connection resumption procedure for MO data or signaling with SDT, the UE doesn’t need to trigger RRC connection resumption or RRC connection establishment procedure for MT again. Hence, it is not an issue if the UE doesn’t receive paging message during SDT.

Proposal 5: During SDT, the UE doesn’t require to receive paging message.

- SDT failure detection timer

In email discussion [Post113-e][503][SDT][2], SDT failure detection timer was discussed. And it is still FFS whether SDT failure detection timer:

Option 1) is an extended T319 like timer to accommodate subsequent SDT, or

Option 2) is restarted upon (re)transmission or reception of small data.

It was agreed that SDT failure detection timer is started upon initiation of SDT procedure. Thus SDT failure detection timer includes the duration of RA procedure for the first UL message transmission. The UE may try RA procedure for the first UL message transmission with several times if the RA procedure is unsuccessful. Hence, the SDT failure detection timer with option 2 also needs to be long enough to cover the worst case for RA procedure with SDT. After successful retransmission with several times during SDT, the duration of SDT (T1) may be long. And the value (SDT failure detection timer with option 1– T1) may be smaller than SDT failure detection timer with option 2. Then after successful retransmission with several times, the UE with option 1 may detect SDT failure earlier than that with option 2. Therefore, it is not true that the UE would detect the failure of SDT transmission earlier with option 2.

In addition, it was agreed in RAN2#113bis e-meeting that T319 legacy stop conditions also apply to SDT failure detection timer, e.g. cell reselection, reception of RRCSetup, RRCResume and so on. Even if the timer with option 2 is controlled in MAC layer, inter-layer interactions need to be introduced to cover these stop conditions in RRC layer.

Thus, we prefer option 1 as it is simple. And it is also flexible as the value is configurable by the network.

Proposal 6: SDT failure detection timer is an extended T319 like timer to accommodate subsequent SDT.

- Measurement

According to WID [1], if needed, RAN4 will specify RRM core requirements for small data transmission in RRC_INACTIVE. But beside RRM measurements for cell (re)selection, a UE in RRC_INACTIVE, may perform logging of measurement results and perform idle/inactive measurement for fast DC/CA configuration when switching to RRC_CONNECTED. RAN2 can discuss how to handle these behaviors first.

If inter-frequency measurements are performed during SDT, measurement gap may be required. As there is no way to configure dedicated measurement gap for a UE during SDT procedure, it is preferred to avoid inter-frequency measurements as much as possible. Hence, even if idle/inactive measurement configuration for fast DC/CA is valid, it is preferred that the UE stops idle/inactive measurement during SDT.

When a UE performs logging of measurement results, the UE doesn’t need to perform additional new measurement but just log measurement results. Thus, the UE can continue logging of measurement results during SDT.

Proposal 7: During SDT, the UE stops idle/inactive measurement if running but continues logging of measurement results if configured.

Conclusion

In this contribution, we discuss some control plane issues, including considerations on control plane SDT, periodic RNA update during SDT procedure, periodic RNA update, SI acquisition, paging message reception, SDT failure detection timer and measurements.

On control plane SDT, some descriptions on control plane SDT has already been captured in 38.331 running CR. we want to further confirm:

Proposal 1: ULInformationTransfer message which contains CP small data can be included in the first UL message for CP SDT if SRB2 is configured for SDT by the network.

On periodic RNA update issue, we propose:

Proposal 2: Periodic RNA update is cancelled if T380 expires during SDT procedure.

On SI acquisition issue, we propose:

Proposal 3: The UE needs to receive SI for cell reselection/PWS via broadcast during SDT.

Proposal 4: During SDT, the UE applies on-demand SI procedure in RRC_IDLE/INACTIVE if needed.

On paging message reception, we propose:

Proposal 5: During SDT, the UE doesn’t require to receive paging message.

On SDT failure detection timer, we propose:

Proposal 6: SDT failure detection timer is an extended T319 like timer to accommodate subsequent SDT.

On measurement, we propose:

Proposal 7: During SDT, the UE stops idle/inactive measurement if running but continues logging of measurement results if configured.

Reference

- RP-212594, Work Item on NR small data transmissions in INACTIVE state, RAN#93e

- R2-2103971, Report of [Post113-e][503][SDT] T319, cell reselection and re-establishment, InterDigital, RAN2#113bis-e