3GPP TSG-RAN WG2 Meeting#116-e		       					       R2-2110464

Electronic, 1st - 12th November 2021

Source: 			ZTE corporation, Sanechips, vivo

Title: 	PO determination in RRC_INACTIVE

Agenda item:		8.21.2.1

Document for: 	Discussion and Decision

- Introduction

There has been discussion on the PO misalignment for INACTVIE and IDLE state in the past RAN2#114e and RAN2#115e with the following conclusion [1]:

RAN2#114e

- [013] 1: RAN2 understand the index of the PO (i.e. the i_s) calculated based on the same UE ID may be different in inactive state and idle state when the DRX cycle for inactive and idle state are different. If a UE in inactive state only monitors the PO derived for inactive state, CN paging failure would happen in both NR and eLTE.

- [013] 2: For Rel-15, it is up to NW implementation to ensure RAN and CN paging occasions overlap in both NR and eLTE

- [013] Whether a standard solution should be supported in later releases (Rel-16 or Rel-17) for NR and eLTE, and if so, the choice of solution, is Postponed

RAN2#115e

- For R16, we assume similar as R15, that the network implementation need to ensure that there are no issues.

- We introduce a solution, from R17, where the following is the baseline:

    - R2-2109077 Solution 2 (i.e. UE in RRC _INACTIVE should use the same i_s to determine PO as for RRC _IDLE) is supported to address the RAN and CN paging PO non-overlap problem.

    - UE capability should be introduced to indicate support for using the same i_s in PO determination in RRC _INACTIVE state as in RRC _IDLE state.

CR discussion is postponed to next meeting. If needed can also further discuss variants of Solutions based on raised issues.

The problem has been acknowledged in RAN2#114e and it has been agreed to be left to NW implementation in Rel-15 and Rel-16 for NR and eLTE. While for the later release (Rel-17 and so on), the baseline solution, i.e. UE in RRC _INACTIVE should use the same i_s to determine PO as for RRC _IDLE) is supported to address the RAN and CN paging PO non-overlap problem.

In this paper, we share some further analysis on details of the baseline solution.

- Discussion

With the baseline solution, it is observed that only the RAN paging PO would be shifted if the i_s in RRC_INACTIVE is not the same as the i_s in RRC_IDLE. To ensure that UE and the network are aligned on how the RAN paging would be sent and received, we understand the baseline solution works when the following requirements are met:

- UE supports the baseline solution and is aware that this solution is enabled by the NW.

- NW supports the baseline solution and is aware that UE would monitor the RAN paging in the PO determined by the i_s as in RRC_IDLE.

Thus, we understand NW can enable the baseline solution via a indication, e.g. useIdlePO, in the RRCRelease message when releasing UE to RRC_INACTIVE after acquisition of the UE capability showing support of the baseline solution. Upon reception of such indication, UE would then monitor both CN paging and RAN paging in the PO determined with by the same i_s as for RRC_IDLE while staying in the same cell.

Proposal 1: An indication, e.g. useIdlePO, should be introduced in RRCRelease-> suspendConfig.

- NW should set useIdlePO to “true” if NW supports the baseline solution and UE indicates support for this solution.

- UE should monitor both CN paging and RAN paging in the POs determined by i_s of RRC_IDLE if the useIdlePO is included and set to “true” in RRCRelease-> suspendConfig.

After UE moves to another cell within the configured RNA, UE has to monitor both CN paging and RAN paging but has no other way to interact with the NW if no RNAU or UL/DL transmission is triggered, makes it difficult for UE and NW to have common understanding on how the RAN paging would be sent and received as the baseline solution should be enabled in a per UE manner.

The following options can be considered to enable the baseline solution for mobility in RRC_INACTIVE mode.

- Option 1: Indicate useIdlePO also in RAN PAGING message and it is up to NW to ensure that the baseline solution is supported by all the cells configured in the RNA and the gNB/ng-eNB would send RAN paging in the PO determined by the i_s for RRC_IDLE. UE would monitor both CN paging and RAN paging in the PO determined by the i_s in RRC_IDLE if it supports the baseline solution and does not move out of the configured RNA.

- Option 2: Indicate UE capability of supporting the baseline solution also in RAN PAGING message as part of UE Radio Capability for Paging and the target node would send the RAN paging and CN paging in the PO determined by the i_s of RRC_IDLE and broadcast ranPagingInIdlePO via system information. If cell reselection happens after UE get released to RRC_INACTIVE, UE would monitor both of the CN paging and the RAN paging in the PO determined by the i_s in RRC_IDLE if it supports the baseline solution and the ranPagingInIdlePO is broadcast by this newly re-selected cell.

Case 1: The newly re-selected cell also supports the baseline solution

Case 2: The newly re-selected cell does not the baseline solution

Comparison between option 1 and option 2 is also given in the following table:

Table 1: Comparison between option 1 and option 2

|  | Option 1 | Option 2 |
| --- | --- | --- |
| RAN2 impact | - Per UE indication (e.g. useIdlePO) RRCRelease message to enable the baseline solution. - It is up to NW to ensure such indication to enable the baseline solution is valid within the configured RNA, i.e. all the cells within the RNA support the baseline solution | - Per UE indication (e.g. useIdlePO) RRCRelease message to enable the baseline solution. - Broadcast a ranPagingInIdlePO - No requirement on the NW implementation and it is allowed for NW to include a cell who does not support the baseline solution when the RNA is configured. |
| RAN3 impact | Include per UE indication (e.g. useIdlePO ) in RAN PAGING message | Include UE capability in RAN PAGING message |

As analyzed in the above table, the major difference between option 1 and option 2 is related to the NW implementation. Option 1 has the requirement that only cells supporting the baseline solution will be included in the configured RNA so that the useIdlePO indication to enable the baseline solution would be valid in the whole RNA and NW and UE are aligned that the RAN paging and CN paging will be sent and received in the POs determined by the i_s of RRC_IDLE mode. Option 2 does not have any limitation on the NW configuration of RNA but requires broadcasting a ranPagingInIdlePO. Option 1 is slightly preferred from our perspective.

Proposal 2: RAN2 to down-select from the following options of PO determination after mobility in RRC_INACTVE:

- Option 1: useIdlePO valid in the whole RNA

Indicate useIdlePO also in RAN PAGING message and it is up to NW to ensure that the baseline solution is supported by all the cells configured in the RNA and the gNB/ng-eNB would send RAN paging in the PO determined by the i_s of RRC_IDLE. U

E would monitor both CN paging and RAN paging in the PO determined by the i_s in RRC_IDLE if it supports the baseline solution and it does not move out of the configured RNA.

- Option 2: useIdlePO valid only in the cell from which it is configured

Indicate UE capability of supporting the baseline solution also in RAN PAGING message and the target node would send the RAN paging and CN paging in the PO determined by the i_s of RRC_IDLE and broadcast ranPagingInIdlePO via system information.

If cell reselection happens after UE get released to RRC_INACTIVE, UE would monitor both of the CN paging and the RAN paging in the PO determined by the i_s in RRC_IDLE if it supports the baseline solution and the ranPagingInIdlePO is broadcast by this newly re-selected cell.

Proposal 3: Agree the corresponding text proposals for 38.304/38.331/38.306/36.304/36.331/36.306[2] after conclusions are made.

- Conclusion and proposals

With the above analysis, we have the following conclusions and proposals:

Proposal 1: An indication, e.g. useIdlePO, should be introduced in RRCRelease-> suspendConfig.

- NW should set useIdlePO to “true” if NW supports the baseline solution and UE indicates support for this solution.

- UE should monitor both CN paging and RAN paging in the POs determined by i_s of RRC_IDLE if the useIdlePO is included and set to “true” in RRCRelease-> suspendConfig.

Proposal 2: RAN2 to down-select from the following options of PO determination after mobility in RRC_INACTVE:

- Option 1: useIdlePO valid in the whole RNA

Indicate useIdlePO also in RAN PAGING message and it is up to NW to ensure that the baseline solution is supported by all the cells configured in the RNA and the gNB/ng-eNB would send RAN paging in the PO determined by the i_s of RRC_IDLE. U

E would monitor both CN paging and RAN paging in the PO determined by the i_s in RRC_IDLE if it supports the baseline solution and it does not move out of the configured RNA.

- Option 2: useIdlePO valid only in the cell from which it is configured

Indicate UE capability of supporting the baseline solution also in RAN PAGING message as part of UE Radio Capability for Paging and the target node would send the RAN paging and CN paging in the PO determined by the i_s of RRC_IDLE and broadcast ranPagingInIdlePO via system information.

If cell reselection happens after UE get released to RRC_INACTIVE, UE would monitor both of the CN paging and the RAN paging in the PO determined by the i_s in RRC_IDLE if it supports the baseline solution and the ranPagingInIdlePO is broadcast by this newly re-selected cell.

Proposal 3: Agree the corresponding text proposals for 38.304/38.331/38.306/36.304/36.331/36.306[2] after conclusions are made.

- References

- R2-2109077_Report of [AT115-e] [026] [NR16] System Information and Paging (ZTE) – Phase 1	ZTE Corporation, Sanschips

- R2-2110465_Text proposals on PO determination for UE in inactive state