3GPP TSG-RAN WG2 Meeting #116 electronic          	          R2-2110541

Online, November 1 – 12, 2021

Agenda Item:	8.9.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on criteria for the RLM/BFD relaxation

Document for:	Discussion and decision

Introduction

RAN4 has agreed two relaxation criteria for the RLM/BFD relaxation, and sent LS to RAN2 on criteria for RLM/BFD relaxation to trigger RAN2’s work.

According to the WID RP-193239, RAN2 work will be triggered by RAN4 LS. - Study the feasibility and performance impact of relaxing UE measurements for RLM and/or BFD, particularly for low mobility UE with short DRX periodicity/cycle, and specify, if agreed, relaxation in the corresponding requirements [RAN4] - NOTE: Supplementary RAN2 work, if needed, can be triggered by RAN4 LS RAN4 respectfully ask RAN2 take above conclusions into considerations and start work in RAN2. RAN2 may work on at least following aspects: - Specify corresponding signalling for the relaxation criteria defined by RAN4 If there is further RAN4 conclusions that need RAN2 work in the future meetings, it will be informed to RAN2.

In this contribution, we discuss criteria for the RLM/BFD relaxation from RAN2 point of view.

Discussion

Criterion definition

Low mobility criterion

For the low mobility criterion, RAN4 have achieved agreements so far as follows - Low mobility criteria (in RAN4 #100e) - Reuse Rel-16 low mobility criterion based on L3 RSRP measurement variation. - FFS the RSs for L3 RSRP measurement

In Rel-16, low mobility criterion is defined as below:

relaxedMeasurement-r16              SEQUENCE {

lowMobilityEvaluation-r16           SEQUENCE {

s-SearchDeltaP-r16                  ENUMERATED {

dB3, dB6, dB9, dB12, dB15,

spare3, spare2, spare1},

t-SearchDeltaP-r16                  ENUMERATED {

s5, s10, s20, s30, s60, s120, s180,

s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

}                                                                                   OPTIONAL,       -- Need R

According to RAN4 LS, two thresholds (s-SearchDeltaP and t-SearchDeltaP) can reused for providing low mobility criterion for RLM/BFD relaxation. However, low mobility criterion is only used in RRC_IDLE/INACTIVE state, the measurement is only based on SSB. But for RRC_CONNECTED state, SSB and CSI-RS (if configured) can be used for measurement. Thus, in RAN4 conclusion, the RSs for L3 RSRP measurement is FFS and RAN2 should wait for RAN4 discussion to further design the signalling and discuss the value range for the low mobility criterion.

Proposal 1: Two thresholds (s-SearchDeltaP and t-SearchDeltaP) are provided by the network for low mobility criterion for RLM/BFD relaxation. RAN2 waits for RAN4 progress on RS(s) type.

Cell quality criterion

For the cell quality criterion, RAN4 have achieved agreements so far as follows - Good serving cell quality criteria of RLM/BFD relaxation is defined as the radio link quality is better than a threshold. The radio link quality in good serving cell quality criteria for R17 RLM/BFD relaxation is based on SINR. (in RAN4 #98e-bis) - UE reuse the SINR for RLM/BFD evaluation when determine whether the serving cell quality criteria is fulfilled or not (in RAN4 #99e) - FFS: (in RAN4 #100e) - SINR definition for good serving cell quality criteria - predefined or configured threshold

Regarding the cell quality criterion, the cell quality criterion is defined as the radio link quality is better than a threshold. However, details are still unclear and under discussion in RAN4, for example, how the SINR defined, how the threshold defined (an offset to the Qin/Qout or absolute value), whether the threshold needs to distinguish the RS type or FR1/FR2, etc. Thus, RAN2 needs to wait for more inputs from RAN4.

Proposal 2: RAN2 waits for more RAN4 progress on the cell quality criterion for RLM/BFD relaxation.

General signalling design

For the relaxation criteria, RAN4 have also achieved agreements as follows. - Network to enable and disable this feature. (in RAN4 #98e) - Whether relaxed RLM/BFD requirements can be applied depends on both the serving cell quality and UE mobility state. (in RAN4 #98e-bis) - If the UE fulfills any of serving cell quality exit condition or low mobility exit condition, or DRX cycle length is NOT allowed for relaxation, UE will exit relaxation mode. (in RAN4 #99e) - If the UE applies a DRX cycle longer than 80ms, the UE is assumed not to perform relaxed RLM/BFD measurements and the existing RLM/BFD requirements would apply. (in RAN4 #100e) - When neither serving cell quality criteria nor low mobility criteria is configured, the existing RLM/BFD requirements shall apply. (in RAN4 #100e) - Note: It can be revisited if - dedicated or broadcast signalling to indicate the UE when it is allowed to relax the RLM/BFD measurements is agreed, or - good serving cell criteria is agreed to be predefined. - FFS: Whether low mobility criteria are necessary to be configured

Network can enable and disable the RLM/BFD relaxation. That is, if the RLM/BFD relaxation is enabled, the UE is allowed to further check the low mobility and/or cell quality criterion to decide if relaxed RLM/BFD measurements can be performed; if the RLM/BFD relaxation is disabled, the UE cannot perform relaxed RLM/BFD measurements and the existing RLM/BFD requirements apply. Since the RLM/BFD relaxation is only used for RRC_CONNECTED UE, dedicated signalling is more suitable to enable and disable the RLM/BFD relaxation. Broadcast signalling is not a good idea as this information is useless for RRC_IDLE/INACTIVE UE. Dedicated signalling can provide UE-specific control, e.g. based on UE capability and maybe UE service characteristic. From UE capability perspective, RLM/BFD relaxation should be an optional feature and UE needs to report whether this can be supported or not to the network.

UE

gNB

1. UE capability of supporting RLM/BFD relaxation

Check the criterion to decide if relaxed RLM/BFD measurements can be performed

2. Enable/disable the RLM/BFD relaxation

Figure 1. General signalling procedure for RLM/BFD relaxation

Proposal 3: Network can enable and disable the RLM/BFD relaxation by dedicated signalling.

Regarding the signalling of enabling and disabling the RLM/BFD relaxation, based on the RAN4 LS, low mobility criterion can reuse Rel-16 low mobility mechanism so the thresholds for low mobility criterion should be configured by the network; and for cell quality criterion, it is FFS whether it is configurable or predefined. Moreover, based on the RAN4 conclusion “Whether relaxed RLM/BFD requirements can be applied depends on both the serving cell quality and UE mobility state”, it seems both criteria need to be configured and relaxed RLM/BFD measurements can be performed only when both criteria is fulfilled. Otherwise, RAN4 has to define multiple RLM/BFD relaxation methods for multiple cases, i.e. the case of only low mobility criterion fulfilled, only cell quality criterion fulfilled, both fulfilled. Hence, to be simple, it is preferred to only have the case of both criteria configured. But this depends on the RAN4 discussion.

The following cases can be considered (low mobility criterion is configurable):

- Cell quality criterion is configurable

In this case, low mobility criterion and cell quality criterion need to be configured, the configuration of criterion provided by the network can implicitly indicate that the RLM/BFD relaxation is enabled.

- Cell quality criterion is predefined

In this case, 1) If both criteria need to be configured simultaneously, the configuration of low mobility criterion implicitly indicate that the RLM/BFD relaxation is enabled, and both criteria need to be checked (if configured) when decide if relaxed RLM/BFD measurements can be performed or not; 2) If either or both criteria are allowed to be configured, the configuration of low mobility criterion implicitly indicate that low mobility criterion based RLM/BFD relaxation is enabled, the configuration of cell quality criterion is not needed but the network still needs to explicitly indicate whether the cell quality criterion based RLM/BFD relaxation is enabled or not, e.g. by simple bit. Both criteria need to be checked (if both enabled) when decide if relaxed RLM/BFD measurements can be performed or not.

Proposal 4: To discuss if the RLM/BFD relaxation is enabled/disabled implicitly or explicitly.

After the RLM/BFD relaxation is enabled, UE checks the criterion to decide if relaxed RLM/BFD measurements can be performed or not. In Rel-17 RedCap, the discussion on RRM measurement relaxation for RRC_CONNECTED state, if the UE fulfill the criterion for RRM measurement relaxation, the UE needs to inform the network that the criterion is fulfilled, and the network indicate whether the UE can perform RRM measurement relaxation or not. The motivation is that the network needs to clearly know whether the UE is performing RRM measurement relaxation, so the network can, e.g. adjust the configured parameters to ensure the performance of handover. However, for RLM/BFD relaxation also for RRC_CONNECTED state, does the similar mechanism should be applied, i.e. the UE performs/exits RLM/BFD relaxation based on indication from the network, or the UE needs to inform the network if it performs/exits RLM/BFD relaxation.

Proposal 5: To discuss if the network needs to know or control UE performed RLM/BFD relaxation.

Conclusion

In this paper we discuss criteria for the RLM/BFD relaxation. Proposals are summarized as below.

Proposal 1: Two thresholds (s-SearchDeltaP and t-SearchDeltaP) are provided for low mobility criterion for RLM/BFD relaxation. RAN2 waits for RAN4 progress on RS(s) type.

Proposal 2: RAN2 waits for more RAN4 progress on the cell quality criterion for RLM/BFD relaxation.

Proposal 3: Network can enable and disable the RLM/BFD relaxation by dedicated signalling.

Proposal 4: To discuss if the RLM/BFD relaxation is enabled/disabled implicitly or explicitly.

Proposal 5: To discuss if the network needs to know or control UE performed RLM/BFD relaxation.

References

- R4-2115349, LS on criteria for RLM/BFD relaxation, RAN4.