3GPP TSG-RAN WG2 #116e	Tdoc R2-2110563

Electronic meeting, 2021-11-01 - 2021-11-12

Agenda Item:	6.1.4.3

Source:	Ericsson

Title:	Keeping or removing diffSCS-DAPS

Document for:	Discussion, Decision

# 1	Introduction

RAN2 discussed last time whether to remove the diffSCS-DAPS capability bits. The decision was postponed and here we propose to keep this capability bit.

# 2	Discussion

## 2.1	diffSCS-DAPS - What is it used for?

RAN4 requested in R4-2016850 to add the diffSCS-DAPS capability bits and are defined as follows:

interFreqDiffSCS-DAPS-r16 indicates whether the UE supports different SCSs in source PCell and inter-frequency target PCell in DAPS handover. The UE only includes this field if different SCSs can be supported in both UL and DL. If absent, the UE does not support either UL or DL SCS being different in DAPS handover.

intraFreqDiffSCS-DAPS-r16 indicates whether the UE supports different SCSs in source PCell and intra-frequency target PCell in DAPS handover. The UE only includes this field if different SCSs can be supported in both UL and DL. If absent, the UE does not support either UL or DL SCS being different in DAPS handover.

To illustrate what the capability bits do, let's consider four different UEs that all support both intra- and inter-frequency DAPS and say that all of them support operation of 15 kHz and 60 kHz SCS. The difference between these four UEs is which of diffSCS-DAPS bits they set to "supported", here is what they support:

|  | interFreqDiffSCS-DAPS | intraFreqDiffSCS-DAPS |
| --- | --- | --- |
| UE 1 | supported | supported |
| UE 2 | - | supported |
| UE 3 | supported | - |
| UE 4 | - | - |

In the network there are four cells:

- Cell 1: on frequency X with SCS 15 kHz

- Cell 2: on frequency X with SCS 60 kHz

- Cell 3: on frequency Y with SCS 15 kHz

- Cell 4: on frequency Y with SCS 60 kHz

The figure below shows which DAPS handovers the network can perform for the different UEs.

All UEs can support DAPS between Cell 1 and Cell 3 since the SCSs are the same. Similarly, all UEs can support DAPS between Cell 2 and Cell 4 since the SCSs on these cells are the same.

Only UE 1 and UE 3 can support DAPS handover between Cell 1 and Cell 2 (or between 3 and 4). UE 2 and UE 4 cannot do such DAPS handovers since (even if they happen to support SCS 15 and SCS 60) they cannot switch SCS during a DAPS handover when the cells are on the same frequency.

Only UE 1 and UE 2 can support DAPS handover between Cell 1 and Cell 4 (or between 2 and 3). UE 3 and UE 4 cannot do such DAPS handovers since (even if they happen to support SCS 15 and SCS 60) they cannot switch SCS during a DAPS handover when the cells are on different frequency.

We see that the diffSCS-DAPS capability bits do serve a purpose since they offer do allow for different UE implementations with regards to support of change SCS during DAPS handover.

The diffSCS-DAPS capability bits allow different UEs to support/not support SCS-change during a DAPS handover.

## 2.2	How to proceed

It was suggested during the last RAN2 meeting to remove the diffSCS-DAPS capabilities. We believe that RAN2 cannot remove these capability bits by themselves. RAN4 requested RAN2 to add these capabilities and RAN2 can assume that there was a good motivation for having these capability bits.

Companies can bring this topic up in RAN4. No LS would be needed for this.

We propose:

RAN2 does not remove the diffSCS-DAPS capability bits unless requested by RAN4.

# 3	Conclusion

Based on the discussion above we propose:

Proposal 1	RAN2 does not remove the diffSCS-DAPS capability bits unless requested by RAN4.