3GPP TSG-RAN WG2 #116-e	R2-2110594

E-meeting, 1st – 12th November, 2021

Agenda item:	8.6.4

Source:	Huawei, HiSilicon

Title:	Small data transmissions with RA-based schemes

Document for:	Discussion

# Introduction

In this paper, we further discuss the issues related to small data transmission (SDT) with random access (RA) based scheme based on the agreements made by RAN2 so far.

# RA-based schemes

## 2.1 Common RACH resources for SDT

In RAN2#113-e meeting, RAN2 agreed the following:

RAN2#113-e Agreement: - For RA-SDT, up to two preamble groups (corresponding to two different payload sizes for MSGA/MSG3) may be configured by the network - RAN2 continues to progress the work based the separate RACH resources for SDT (i.e. explicit mechanisms to support common resources won’t be pursued unless there is sufficient support for this. However, use of common RACH resources will not be precluded if possible via implementation

According to the above agreement, RAN2 agreed that different preamble groups can be utilized to indicate different MSGA/MSG3 payload sizes, which is similar to the preamble partitioning scheme in legacy RA procedure. For 4-step SDT, if the payload sizes can be indicated by different preambles, UE can assemble packets on the basis of UL grant in RAR. Hence, there is no need for the gNB to blindly detect MSG3 packet as had to be done when flexible TBS was applied, e.g. in LTE EDT. Without blind detection, the gNB behavior for reception of RA-SDT MSG3 and legacy MSG3 are exactly the same, meaning that it is not necessary for UEs to indicate the SDT intention through MSG1 from the perspective of gNB. If common RACH resources for SDT are applied, UE would use current preamble Group B when initiating RA-SDT procedure. UE could transmit the SDT data together with MSG3 if the UL grant is sufficient, or UE could include BSR if the UL grant is not sufficient and small data could be further sent over subsequent SDT transmissions. There may be cases that gNB sends an RRCResume message to move the UE into RRC_ CONNECTED upon receiving MSG3 not including any data nor BSR. However, we believe such scenario can be avoided by network implementation, e.g. by providing the UE with a grant sufficient for at least some UP data.

Proposal 1: For the case of common RACH resources for SDT, the network can distinguish SDT and non-SDT access attempts via LCID of UL data included in MSG3/MSGA, i.e. no additional indication is required.

## 2.2 Preamble Group Selection for RA-SDT

In legacy RA procedure, after the initialization of random access procedure, UE needs to perform the preamble group selection, where the preamble group A and preamble group B correspond to different payload sizes as well as different pathloss of the UE. Similarly, RAN2 has agreed that up to two preamble groups are supported for RA-SDT, which means that UE also needs to perform the preamble group selection for both types of RA-SDT, i.e. 4-step RA-SDT and 2-step RA-SDT if RA-SDT is initiated.

In RAN2#115-e meeting, RAN2 agreed the following:

RAN2#115-e Agreement: 5. For the RA-SDT preamble group selection, the UE should consider SDT data size plus MAC subheader in addition to CCCH SDU size plus MAC subheader and pathloss, same in legacy. FFS whether any additional things on top of legacy criteria is needed.

According to the agreements, UE takes the SDT data size into account for preamble group selection. Different from the legacy RA procedure, the RA SDT procedure triggered by SDT related RRC resume procedure also includes the transmission of SDT data, therefore the legacy data size threshold is not suitable for SDT and a new one can be introduced.

However, in the agreement for the RRC parameters for preamble group selection, the msgA and msg3 size are not mentioned.

RAN2#115-e Agreement: - At least the following parameters can be RA-SDT specific. - SSB selection related parameters, i.e., rsrp-ThresholdSSB, msgA-RSRP-ThresholdSSB. - Power control related parameters, i.e., preambleReceivedTargetPower/msgA-PreambleReceivedTargetPower, powerRampingStep/msgA-PreamblePowerRampingStep, msg3-DeltaPreamble/msgA-DeltaPreamble. - Preamble group related parameters, i.e., msg3-DeltaPreamble/msgA-DeltaPreamble, messagePowerOffsetGroupB for 2-step RA-SDT and 4-step RA-SDT.

Hence, we propose the following:

Proposal 2: When two preamble groups are configured for RA-SDT, the following data volume threshold should be SDT specific: ra-sdt-Msg3SizeGroupA for 4-step RA-SDT and ra-sdt-MsgA-SizeGroupA for 2-step RA-SDT.

## 2.3 SDT data in Msg4/MsgB

In legacy 4-step RA, the DL-SCH MAC PDU can include a MAC subPDU consisting of a MAC subheader and a MAC SDU of a single UE, since Msg4 is solely for the UE whose contention resolution succeeds.

In legacy 2-step RA, MSGB MAC PDU also can include a MAC subheader and MAC SDU for CCCH or DCCH. MAC subPDU(s) for MAC SDU are placed immediately after the 'MAC subPDU for successRAR' indicating presence of 'MAC subPDU(s) for MAC SDU', and at most one 'MAC subPDU for successRAR' indicating presence of 'MAC subPDU(s) for MAC SDU' is included in a MAC PDU. This means that only one UE’s SDU can be included in MSGB.

In SDT, after Msg3/MsgA with UL data is transmitted, there may be downlink data that needs to be transmitted. We think it is beneficial to allow this data to be included in Msg4/MsgB already, because the contention resolution is sent specifically for one UE to indicate a successful access, and UE already has the keys to receive the DL SDT data. Therefore, it is not necessary to send the DL SDT data to UE after successful contention resolution. When the DL data arrives, it can be sent together with the contention resolution within MsgB, which can reduce the delay of data transmission. In order to minimize the impact on the specification, we think it is sufficient to agree that Msg4/MsgB includes MAC SDU of only one UE, similarly as in the legacy specifications.

Proposal 3: SDT data can be included in Msg4/MsgB. Data of only a single UE can be included in MsgB.

## 2.4 RA-SDT parameters

In RAN2#115-e meeting, RAN2 agreed that the following parameters are SDT specific:

RAN2#115-e Agreement: - At least the following parameters can be RA-SDT specific. - SSB selection related parameters, i.e., rsrp-ThresholdSSB, msgA-RSRP-ThresholdSSB. - Power control related parameters, i.e., preambleReceivedTargetPower/msgA-PreambleReceivedTargetPower, powerRampingStep/msgA-PreamblePowerRampingStep, msg3-DeltaPreamble/msgA-DeltaPreamble. - Preamble group related parameters, i.e., msg3-DeltaPreamble/msgA-DeltaPreamble, messagePowerOffsetGroupB for 2-step RA-SDT and 4-step RA-SDT.

Besides the above parameters, there are also other RACH configuration related parameters which need to be discussed.

For 4-step RA-SDT related parameters, NW can configure the 4-step RA-SDT related configuration through RACH-ConfigCommonRA-SDT IE for separated RO case. For shared RO case, the similar way in legacy 2-step RA can be reused, for example, NW configures the preambles and the shared RO mask for UE.

For 2-step RA-SDT related parameters, NW can also configure the 2-step RA-SDT related configuration through RACH-ConfigCommonTwoStepRA-SDT IE for separated RO case. For shared RO case, in legacy 2-step RA, NW configures msgA-CB-PreamblesPerSSB-PerSharedRO if separated preamble with shared RO is applied. Similarly, for the shared RO case in 2-step RA-SDT, msgA-CB-PreamblesPerSSB-PerSharedRO can also indicate the preambles for UE, meaning that it’s better to introduce an SDT specific parameter. Moreover, a separated msgA-SSB-SharedRO-MaskIndex can indicate the ROs for UE to use achieving a different configuration from legacy 2-step RA. Finally, another two SDT specific parameters for 2-step RA-SDT are msgA-PUSCH-ResourceGroupA and msgA-PUSCH-ResourceGroupB, the probability of successful transmission can be achieved with a separated PUSCH resource different from legacy 2-step.

Proposal 4: The following parameters should be SDT specific

- msgA-CB-PreamblesPerSSB-PerSharedRO

- msgA-SSB-SharedRO-MaskIndex

- msgA-PUSCH-ResourceGroupA

- msgA-PUSCH-ResourceGroupB

In legacy 2-step RA, when 2-step RA is configured, a separate parameter preambleTransMax is only needed when only 2-step RA is configured. While when both 2-step and 4-step RA are configured, the preambleTransMax for 4-step RA can be reused.

While for SDT, when RA-SDT is configured, we think legacy RA, either 2-step RA or 4-step RA has to be configured in order to support cases without SDT or UE not support SDT accessing the same cell. Hence, we think when the UE performs RA-SDT, either by 2-step RA-SDT or 4-step RA-SDT, the UE can reuse the preambleTransMax configured for 4-step RA or for 2-step RA when only 2-step RA is configured.

Proposal 5: For RA-SDT, UE can reuse the preambleTransMax configured for 4-step RA or for 2-step RA when only 2-step RA is configured.

In legacy EDT, the contention resolution timer is extended compared with the contention resolution timer used in the previous releases under RACH-ConfigCommon, as follows:

edt-Parameters-r15			SEQUENCE {

edt-LastPreamble-r15		INTEGER(0..63),

edt-SmallTBS-Enabled-r15	BOOLEAN,

edt-TBS-r15				ENUMERATED {b328, b408, b504, b600, b712,

b808, b936, b1000or456},

mac-ContentionResolutionTimer-r15	ENUMERATED {sf240, sf480, sf960,

sf1920, sf3840, sf5760, sf7680, sf10240}		OPTIONAL -- Need OP

}	OPTIONAL		-- Cond EDT

The main reason is that compared with the processing within the period when the contention resolution timer is running in legacy releases, in EDT, the network side also needs to wait for the DL response from the network in the user plane.

For Small Data Transmission, we think the same rationale also holds that the network needs to wait for the downlink response from the UPF. Hence, similar to the change to contention resolution timer in EDT in LTE, we propose the following:

Proposal 6: The configurable values for ra-ContentionResolutionTimer and msgB-ResponseWindow should be extended for small data transmission.

## 2.5 RA-SDT failure

In legacy RACH, when the number of RACH transmissions exceed the threshold preambleTransMax, the MAC layer would consider the RACH transmission to be unsuccessful and indicate to the RRC layer the random access problem. Similarly, we think that when RA-SDT consistently fails and the number of RACH transmissions exceed the threshold, the MAC layer should also declare failure for SDT transmission and indicate to the upper layer the SDT failure. Hence, we propose the following.

Proposal 7: When the number of RACH transmissions for SDT exceeds the threshold preambleTransMax, SDT failure is detected and the MAC layer indicates SDT failure to upper layers.

## 2.6 Handling of downlink non-SDT Data arrival for non-anchor relocation case

During the email discussion [507][SData] Non-SDT data arrival handling for post 114-e, the issue of how to provide the security keys separation between the receiving and the last serving gNB and handling the switch from SDT to RRC_CONNECTED during an ongoing SDT session was discussed. For this issue, RAN2 has agreed the following after RAN2#115-e meeting:

RAN2#115-e Agreement: - No new solution is defined to prevent data loss or duplication for the scenario where the anchor relocation is required in the middle of an SDT session, i.e. network can release UE back into RRC_INACTIVE

This agreement allows to avoid complexities related to having to specify new procedures for anchor relocation during an ongoing SDT session. However, one thing worth noting is that when the DL non-SDT data arrives at the anchoring gNB in this scenario, in case RRCRelease is used to terminate the SDT procedure, the gNB would need to subsequently page the UE to bring it back to RRC Connected state. This would lead to unnecessary overhead and latency. To avoid this, it would be worth allowing the gNB to include an indication in the RRCRelease message informing the UE it should initiate a new resume procedure right-away.

Proposal 8: When releasing the connection due to DL non-SDT data arrival during an ongoing SDT procedure without anchor relocation, the anchor gNB can include an indication for the UE to initiate a new resume procedure right-away, in order to avoid having to page the UE.

When the UE initiates a new legacy resume procedure, the UE context can be relocated from the last serving gNB to the receiving gNB and the UE can be transitioned to the RRC_CONNECTED state by reusing the legacy procedure. Overall procedure for handling this scenario is shown in Figure 1.

Figure 1: Overall handling of Non-SDT Data arrival while anchoring

# Conclusion

Based on the above discussion, we make the following proposals and recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: For the case of common RACH resources for SDT, the network can distinguish SDT and non-SDT access attempts via LCID of UL data included in MSG3/MSGA, i.e. no additional indication is required.

Proposal 2: When two preamble groups are configured for RA-SDT, the following data volume threshold should be SDT specific: ra-sdt-Msg3SizeGroupA for 4-step RA-SDT and ra-sdt-MsgA-SizeGroupA for 2-step RA-SDT.

Proposal 3: SDT data can be included in Msg4/MsgB. Data of only a single UE can be included in MsgB.

Proposal 4: The following parameters should be SDT specific

- msgA-CB-PreamblesPerSSB-PerSharedRO

- msgA-SSB-SharedRO-MaskIndex

- msgA-PUSCH-ResourceGroupA

- msgA-PUSCH-ResourceGroupB

Proposal 5: For RA-SDT, UE can reuse the preambleTransMax configured for 4-step RA or for 2-step RA when only 2-step RA is configured.

Proposal 6: The configurable values for ra-ContentionResolutionTimer and msgB-ResponseWindow should be extended for small data transmission.

Proposal 7: When the number of RACH transmissions for SDT exceeds the threshold preambleTransMax, SDT failure is detected and the MAC layer indicates SDT failure to upper layers.

Proposal 8: When releasing the connection due to DL non-SDT data arrival during an ongoing SDT procedure without anchor relocation, the anchor gNB can include an indication for the UE to initiate a new resume procedure right-away, in order to avoid having to page the UE.