3GPP TSG-RAN WG2 Meeting #116-e	R2-2110745

# Electronic meeting, 1 – 12 November 2021

Agenda item:		8.1.3.2

Source:	Intel Corporation

Title:	Multicast Activation Notification and MCCH Change Notification

Document for:		Discussion and Decision

# Introduction

In RAN2 #115-e meeting, following was agreed:

Agreements:

- RAN2 waits for RAN1’s final decision on which RNTI/DCI (i.e. Alt1 and/or Alt 2 as identified by RAN1) for MCCH change notification to be adopted.

- Do not specify any mechanism to address the possibility of UE missing an MCCH change notification and it is left to UE implementation.

- Provided RAN3 confirms, paging for multicast activation notification is used in the relevant legacy POs for the UEs with deactivated multicast session(s).

- RAN2 sends an LS to RAN3 and SA2 to indicate its preference for paging for multicast activation notification to be used in the relevant legacy POs for the UEs with non activated multicast session(s). Further, RAN2 requests RAN3 for confirmation and if so, also specifying required network signalling.

- Confirm extending the unicast paging message to include a new paging record list ( pagingGroupList) for group activation notification of multicast sessions.

- NAS is expected to inform UE about multicast session release (e.g. to stop monitoring for multicast session activation).

- It is up to network implementation (e.g. paging repetitions) for addressing scenario of potential notification loss for UEs.

- RAN2 not to prioritize addressing of PRACH capacity issue due to group notification.

- It is FFS that short message or WUS based indication for multicast activation notification in corresponding paging message can be used.

- It is FFS to introduce MBS specific UAC.

- It is FFS on the establishment cause and resume cause for MBS.

- It is FFS if there is a need to prioritize a frequency with multicast support for idle/inactive UEs that monitor multicast activation notification.

In this contribution, we discuss multicast activation notification as well as MCCH change notification.

# Discussion

## Multicast activation notification

In RAN2 #115e meeting, following FFS are captured in Chairman’s Notes:

- It is FFS that short message or WUS based indication for multicast activation notification in corresponding paging message can be used.

Short message and WUS/PEI are proposed to reduce power consumption at the UE side. For short message, it is transmitted on PDCCH using P-RNTI with or without associated Paging message. Short message can use one or two bits to indicate the UE not to decode the associated paging message. However the UE most likely will take the whole slot to receive the paging message.

Recalling that PEI for paging is agreed to be supported in Rel-17 based on RAN1 agreement for power saving:

For PEI, a new DCI format is supported to include at least paging indications to UE group(s)/subgroups of the associated PO(s).

Compared with short message, PEI can be sent a few slots before the paging occasion. Since PEI can achieve larger power saving gain compared with Short message, it is proposed to not consider Short message for multicast activation notification.

Proposal 1: Short message is not supported for multicast activation notification in corresponding paging message.

With PEI, UE can save power and avoid receiving and decoding the paging message unless necessary, when the paging message is sent after some slots. As agreed in RAN1 #105-e meeting, there are only 8 subgroups supported for PEI:

Agreement: For UE subgroups indication in physical layer, maximum of 8 subgroups per PO is supported.

However, the agreement only assumes all UEs are receiving unicast traffic. It is unclear whether the performance will be impacted by using 1 subgroup for multicast or not.

As RAN2 Rel-17 power saving session is discussing paging subgrouping, the relevant impact on unicast paging performance needs to be evaluated there.

Proposal 2: It is up to RAN2 Rel-17 power saving session to decide whether one subgroup in PEI can be used to indicate multicast activation notification for power saving.

## MCCH change notification

In RAN1#105-e meeting, following was agreed:

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, study the following alternatives for MCCH change notification indication due to session start:

- Alt 1: Define a dedicated RNTI to scramble the CRC of a DCI indicating a MCCH change notification;

- Alt 2: Use of a field in a DCI format scheduling a MCCH without a dedicated RNTI for MCCH change notification;

Other solutions are not precluded and it is also not precluded whether to support both Alt1 and Alt2.

In RAN1#106bis-e meeting, RAN1 agreed on following working assumption:

Working assumption:

Alt 2 (from previous agreement) is supported for broadcast reception with RRC_IDLE/RRC_INACTIVE UEs for the notification of MCCH configuration changes.

- Send an LS to RAN2 with the mechanism agreed in RAN1

According to RAN1 agreement, MCCH change notification indication is based on a field in a DCI format scheduling MCCH, without a dedicated RNTI for MCCH change notification. It is proposed that RAN2 confirms RAN1 working assumption from RAN2’s perspective.

Proposal 3: RAN2 to confirm RAN1 working assumption (Alt 2) that MCCH change notification indication is based on a field in a DCI format scheduling MCCH, without a dedicated RNTI for MCCH change notification.

From RAN2 perspective, one open issue is the name and value of the RNTI scrambled in the DCI to schedule MCCH. In email discussion “[Post115-e][091][MBS] Remaining control plane issues” [1], RNTI name was discussed and there are two alternatives: MCCH-RNTI and M-RNTI. Naming is basically a choice of taste. We slightly prefer M-RNTI as it is succinct and more consistent with other fixed RNTI names like SI-RNTI and P-RNTI. Although M-RNTI is used in LTE for MCCH Information change notification, we don’t think there is confusion in NR. The value of M-RNTI can be fixed, taking one reserved value of FFFD, based on TS 38.321 Table 7.1-1 as copied below:

Table 7.1-1: RNTI values.

| Value (hexa-decimal) | RNTI |
| --- | --- |
| 0000 | N/A |
| 0001–FFF2 | RA-RNTI, MSGB-RNTI, Temporary C-RNTI, C-RNTI, CI-RNTI, MCS-C-RNTI, CS-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, PS-RNTI, SL-RNTI, SLCS-RNTI SL Semi-Persistent Scheduling V-RNTI, and AI-RNTI |
| FFF3–FFFD | Reserved |
| FFFE | P-RNTI |
| FFFF | SI-RNTI |

Proposal 4: The RNTI scheduling MCCH is named as M-RNTI, and the value is fixed to FFFD.

# Conclusion

In this contribution, we discuss multicast activation notification as well as MCCH change notification.

We propose the following:

Proposal 1: Short message is not supported for multicast activation notification in corresponding paging message.

Proposal 2: It is up to RAN2 Rel-17 power saving session to decide whether one subgroup in PEI can be used to indicate multicast activation notification for power saving.

Proposal 3: RAN2 to confirm RAN1 working assumption (Alt 2) that MCCH change notification indication is based on a field in a DCI format scheduling MCCH, without a dedicated RNTI for MCCH change notification.

Proposal 4: The RNTI scheduling MCCH is named as M-RNTI, and the value is fixed to FFFD.

# References

[1] R2-2110604, Huawei et al, "[Report of] e-mail discussion: [Post115-e][091][MBS] Remaining control plane issues (Huawei)"