3GPP TSG RAN WG2 Meeting #116-e                                                        	R2-2110815

e-Meeting, November 1 – November 12, 2021

Agenda item:	8.10.3.3

Source:	Samsung

Title:	Measurements and handover

Document for:	Discussion and Decision

# 1. Introduction

RAN2 made a good progress for RRC connected measurements and HO in NTN at the latest e-meetings. In this contribution, we would like RAN2 to discuss some further considerations.

# 2. Discussion

SMTC:

In the RAN2#115-e meeting, RAN2 made following agreements on SMTC configuration [1].

- The specific maximum number of SMTC configuration in one measurement object with the same ssbFrequency can be 4. And a LS will be sent to RAN4 to confirm the conclusion.

- In NTN, NW-based solution is supported, i.e. the final SMTC/measurement gap configuration is generated and provided by NW in NTN to a given UE (based on the propagation delay difference between at least one target cell and the serving cell of a given UE). FFS whether UE-based solution is supported or not.

- In NTN, it is necessary of the UE to report assistant information to the NW (which can be configured by NW or upon NW’s request) to assist NW calculating the offset for SMTC/GAP configurations. FFS the detailed information.

- The UE can be configured with multiple SMTCs per carrier. FFS if the UE can use only a partial set or all of them in parallel, and in case FFS whether based on network configuration or UE implementation

RAN2-115e agreed that, NW-based solution is supported, i.e. the final SMTC/measurement gap configuration is generated and provided by NW in NTN to a given UE and it is necessary for the UE to report assistant information to the NW (which can be configured by NW or upon NW’s request) to assist NW calculating the offset for SMTC/GAP configurations. The detailed assistant information is FFS.

To identify the offset, i.e. propagation delay difference for serving cell and neighbour cell, service link delay and feeder link delay have to be considered for both serving cell and neighbour cell. NW is aware of service link delay and feeder link delay for the serving cell, as well as the feeder link delay for the neighbour cell which depends on the network deployment. Thus, NW only needs to calculate service link delay for the neighbour cell based on UE assistant information report. The service link delay can be calculated based on ephemeris of the satellite associated to the neighbour cell and UE location. Thus, for NW-based solution, it is necessary for UE to report location information to assist NW determine offset for SMTC. UE location reporting has been agreed in RAN2-115e agenda item 8.10.3.1 as follows [1].

- If SA3 has no concern reporting coarse location during initial access, the coarse location information is reported in Msg5, i.e., via RRCSetupComplete/RRCResumeComplete message.

- For coarse UE location reporting during initial access, the location granularity is not indicated to UE via SIB

- Enhancements to validate the UE ’s coarse location information is not needed from RAN2 perspective. Whether this is needed by the network is up to other WGs.

- After AS security is established, gNB can obtain a GNSS-based location information from the UE using existing signalling method, i.e., by configuring includeCommonLocationInfo in the corresponding reportConfig. It is up to SA3 to decide whether User Consent is required before NW acquires location information from the UE in NTN. RAN2 discuss whether to send LS to SA3

- Aperiodic location reporting (e.g., via DCI) is not supported.

- If accepted by SA3, if the gNB has user consent to obtain UE location in NTN, reporting of finer location information/full GNSS coordinates in RRC_CONNECTED can be supported after AS security is enabled

- Periodic location reporting can also be configured by gNB to obtain UE location update of mobile UEs in RRC_CONNECTED. RAN2 discuss whether it is part of existing periodic measurement report configuration or a new configuration for periodic reporting of UE location.

Therefore, it is natural that the reported UE location is used as assistant information for NW calculating the offset for SMTC/GAP configuration.

[Proposal 1]: For NW calculating the offset for SMTC/GAP configuration, no additional information is required besides UE location information. Note UE location information can be reported by the already agreed UE location reporting mechanism.

RAN2-115e agreed that the specific maximum number of SMTC configuration in one measurement object with the same ssbFrequency can be 4. However, not all SMTC configurations in one measurement object may be used in parallel at the same time. Based on UE assistant information, NW can activate a set of SMTC configurations to be used by UE dynamically by MAC CE.

[Proposal 2]: NW dynamically activates a set of SMTC configurations to be used by UE by MAC CE based on reported assistant information.

Measurement reporting event:

In the RAN2#115-e meeting, RAN2 made following agreements on new measurement reporting event.

Working assumption

- Combination of serving and target cell reference location is supported for location report trigger event and for CHO location trigger

- Specify that measurement reports can be configured to be piggybacked with location report when location based event triggers it

Agreement via email –from offline 103:

- The following event is supported: condEvent L4: Distance between UE and the PCell’s reference location becomes larger than absolute threshold1 AND the distance between UE and the Conditional reconfiguration candidate becomes shorter than absolute threshold2. FFS other options.

- Specify hysteresis and time to trigger for the location event for RRM and CHO

- Timing information from RRCReconfiguration message in RRC running CR is removed

- UE is allowed to perform HO only during T1 to T2

- Agree to limit to A or B and continue discussion between options A and B

Option A: UTC time + duration/timer, e.g. 00:00:01 + 40s

Option B: Two UTC time to indicate the start (T1) and end time (T2) of the candidate cell, e.g. 00:00:01 + 00:00:41

With the newly introduced event L4, the UE can let the network know when it moves out of the serving cell and comes into a neighbouring cell. Although event L4 can be useful for the network to decide HO, we think the measured RSRP/RSRQ should be the baseline and event L4 should be taken into account on top of measured RSRP/RSRQ, e.g. even though the UE is closer to the target cell, if the target cell doesn’t provide good radio condition (good measured RSRP/RSRQ), HO to the target cell may not happen. To consider both UE location and measured RSRP/RSRQ, we may rely on multiple events configured by the network in separate or radio measurements piggybacked with the location report. However, this kind of means would bring the signalling overheads, which is against the intention of event-based measurement report/location report. To provide more configuration flexibilities to the network and to avoid the signalling overhead problem, it is proposed the network can configure the combined reporting event, e.g. UE sends measurement and location report when both event A3/A4 and event L4 are met.

[Proposal 3]: RAN2 is asked to consider combined reporting event, e.g. UE sends measurement and location report when both event A3/A4 and event L4 are met.

Handover signalling:

Hundreds or thousands of UEs may need to be handed over from one cell to another every second based on the cell size, number of UEs per cell, and the NTN type.

Transmissions of unicast and dedicated UE-specific handover signalling messages would consume a significant amount of radio resources in a short time period. This may delay handover signalling messages depending upon the available channel bandwidth, PDCCH capacity, and RRC message sizes. Furthermore, since such dedicated signalling messages are needed for many users within a short time period, the combined signalling load for the handover users will be very high, and, fewer radio resources would be left for user traffic. The achievable user throughput and cell throughput will be adversely affected.

It is possible to classify typical RRC signalling Information elements (IEs) into different classes, e.g. cell common configuration and UE dedicated configuration. Then, the most efficient signalling mode can be chosen for a given class of IEs, e.g. broadcast for cell common configuration and unicast for UE dedicated configuration. Such classification of signalling mechanisms will significantly reduce the amount of overhead and enhance user throughput.

[Proposal 4]: RAN2 is asked to consider a means to reduce handover signalling in NTN.

# 3. Conclusion

In this contribution, we have discussed further considerations on measurements, handover, and made the following proposals.

[Proposal 1]: For NW calculating the offset for SMTC/GAP configuration, no additional information is required besides UE location information. Note UE location information can be reported by the already agreed UE location reporting mechanism.

[Proposal 2]: NW dynamically activates a set of SMTC configurations to be used by UE by MAC CE based on reported assistant information.

[Proposal 3]: RAN2 is asked to consider combined reporting event, e.g. UE sends measurement and location report when both event A3/A4 and event L4 are met.

[Proposal 4]: RAN2 is asked to consider a means to reduce handover signalling in NTN.

# 4. Reference

[1] R2-2108832	Report from Break-out session on R17 NTN, REDCAP and CE	ZTE