3GPP TSG-RAN WG2 Meeting #116 Electronic	R2-2110818

Online, 1-12 November 2021

Agenda item:	8.6.3

Source:	Nokia, Nokia Shanghai Bell

Title:	SDT control plane aspects

WID/SID:	NR_SmallData_INACTIVE - Release 17

Document for:	Discussion and Decision

# 1	Introduction

The WID of NR small data transmissions in INACTIVE state [RP-193252] defines the following objectives, which target mechanisms enabling UL small data transmissions (SDT) using either RACH procedures or pre-configured PUSCH resources.

This work item enables small data transmission in RRC_INACTIVE state as follows: - For the RRC_INACTIVE state: - UL small data transmissions for RACH-based schemes (i.e. 2-step and 4-step RACH): - General procedure to enable UP data transmission for small data packets from INACTIVE state (e.g. using MSGA or MSG3) [RAN2] - Enable flexible payload sizes larger than the Rel-16 CCCH message size that is possible currently for INACTIVE state for MSGA and MSG3 to support UP data transmission in UL (actual payload size can be up to network configuration) [RAN2] - Context fetch and data forwarding (with and without anchor relocation) in INACTIVE state for RACH-based solutions [RAN2, RAN3] Note 1: The security aspects of the above solutions should be checked with SA3 - Transmission of UL data on pre-configured PUSCH resources (i.e. reusing the configured grant type 1) – when TA is valid - General procedure for small data transmission over configured grant type 1 resources from INACTIVE state [RAN2] - Configuration of the configured grant type1 resources for small data transmission in UL for INACTIVE state [RAN2] No new RRC state should be introduced in this WID. Transmission of small data in UL, subsequent transmission of small data in UL and DL and the state transition decisions should be under network control.

The following was agreed in RAN2#111e:

Agreements

1 	Small data transmission with RRC message is supported as baseline for RA-based and CG based schemes

2	RRC-less can be studied for limited use cases (e.g. same serving cell and/or for CG) with lower priority

3	Context fetch and data forwarding with anchor re-location and without anchor re-location will be considered.   FFS if there are problems with the scenario “without anchor relocation”.

4	From RAN2 perspective, stored “configuration” in the UE Context is used for the RLC bearer configuration for any SDT mechanism (RACH and CG).

5	The 2-step RACH or 4-step RACH should be applied to RACH based uplink small data transmission in RRC_INACTIVE

6	The uplink small data can be sent in MSGA of 2-step RACH or msg3 of 4-step RACH.

7	Small data transmission is configured by the network on a per DRB basis

8	Data volume threshold is used for the UE to decide whether to do SDT or not.   FFS how we calculate data volume.

FFS if an “additional SDT specific” RSRP threshold is further used to determine whether the UE should do SDT

9	UL/DL transmission following UL SDT without transitioning to RRC_CONNECTED is supported

10	When UE is in RRC_INACTIVE, it should be possible to send multiple UL and DL packets as part of the same SDT mechanism and without transitioning to RRC_CONNECTED on dedicated grant.  FFS on details and whether any indication to network is needed.

The following was agreed in RAN2#112e:

Agreements: 1 For small data, for RACH and CG based solutions when the UE receives RRC release with Suspend config, the UE at least performs the following actions (i.e. same action as in legacy): - MAC is reset and default MAC cell group configuration is released - RLC entities for SRB1 are re-established - SRBs and DRBs are suspended except SRB0 NOTE: SDT termination will be discussed with later papers 2 For both RACH and CG based solutions, upon initiating RESUME procedure for SDT initiation (i.e. for first SDT transmission), the UE shall re-establish at least the SDT PDCP entities and resume the SDT DRBs that are configured for small data transmission (along with the SRB1). FFS for non-SDT DRBs. FFS on implicit vs. explicit. FFS on whether we a new Resume cause. FFS on whether we need to deal with suppressing PDCP status report 3 The first UL message (i.e. MSG3 for 4-step RACH, MSGA payload for 2-step RACH and the CG transmission for CG) may contain at least the following contents (depending on the size of the message): - CCCH message (needs to be included) LCP can be used to determine to priority of the content below that may be included - DRB data from one or more DRBs which are configured by the network for small data transmission - MAC CEs – (e.g. BSR). FFS other MAC CEs - Padding bits FFS if we need to ensure that SDT data only is included. Depends on whether the UE initiates legacy/normal resume 4 For RACH and CG, the existing UAC procedure to determine whether access attempt is allowed, will be reused for SDT. 5 SDT is transparent to NAS layer (i.e. NAS generates one of the existing resume causes and AS decides SDT vs non-SDT access) 6 In case of RRC-based solution, for both RACH and CG based solutions, the CCCH message contains ResumeMAC-I generated using the stored security key for RRC integrity protection – i.e same as Rel-16. 7 For both RACH and CG based solutions, new keys are generated using the stored security context and the NCC value received in the previous RRCRelease message (i.e. same as legacy procedure) and these new keys are used for generating the data of DRBs that are configured for SDT. 8 For RACH based solutions, upon successful completion of contention resolution, the UE shall monitor the C-RNTI. 9 Determine if RAN1 LS is needed later – current list of possible questions input on the coreset/search space for the C-RNTI (i.e. is it common or dedicated) 10: As a baseline, the RACH resource i.e. (RO+preamble combination) is different between SDT and non-SDT - If ROs for SDT and non SDT are different, preamble partitioning between SDT and non SDT is not needed. - If ROs for SDT and non SDT are same, preamble partitioning is needed FFS if common configuration should be allowed 11: If the RACH resource i.e. (RO+preamble combination) is different between SDT and non-SDT then there is no further need for any differentiation between MSG2/MSGB for SDT vs non-SDT

Agreements

Define a new timer.  FFS whether it has the same definition as T319 or it is restarted every UL/DL

Agreements:

- The configuration of configured grant resource for UE uplink small data transfer is contained in the RRCRelease message.  FFS if other dedicated messages can configure CG in INACTIVE CG. Configuration is only type 1 CG with no contention resolution procedure for CG.

- The configuration of configured grant resource can include one type 1 CG configuration.  FFS if multiple configured CGs are allowed

- A new TA timer for TA maintenance specified for configured grant based small data transfer in RRC_INACTIVE should be introduced.  FFS on the procedure, the validity of TA, and how to handle expiration of TA timer.  The TA timer is configured together with the CG configuration in the RRCRelease message.

- The configuration of configured grant resource for UE small data transmission is valid only in the same serving cell.  FFS for other CG validity criteria (e.g. timer, UL/SUL aspect, etc)

- The UE can use configured grant based small data transfer if at least the following criteria is fulfilled (1) user data is smaller than the data volume threshold; (2) configured grant resource is configured and valid; (3) UE has valid TA.  FFS for the candidate beam criteria.

- From RAN2 point of view:  An association between CG resources and SSBs is required for CG-based SDT.  FFS up to RAN1 how the association is configured or provided to the UE.  Send an LS to RAN1 to start the discussion on how the association can be made.   Mention that one option RAN2 considered was explicit configuration with RRC Release message

- A SS-RSRP threshold is configured for SSB selection. UE selects one of the SSB with SS-RSRP above the threshold and selects the associated CG resource for UL data transmission.

The following was agreed in RAN2#113ebis:

Agreements

1	gNB can only configure MN terminated MCG bearer type for SDT

2	Non-SDT radio bearers are only resumed upon receiving RRCResume (same as today)

3	Down-scope to two solutions (CCCH or DCCH) and ask SA3 about security issues (explain that CCCH message will be repeated in same cell and ask if there is a question)

Agreements:

- SDT failure detection timer is started upon initiation of SDT procedure

- T319 legacy is not started if RRCResumeRequest or RRCResumeRequest1 is transmitted …

- T319 legacy stop conditions also apply to SDT failure detection timer

- RRC re-establishment procedure is not supported for SDT

5	An LS is sent to SA3 to verify feasibility/impacts of re-using same NCC/I-RNTI value temporarily for RRC Resume procedure in new cell during SDT procedure (include same cell question from 502]

6	FFS - RAN2 to select between the following options for cell re-selection during ongoing SDT procedure next meeting: 1) UE transitions to IDLE, possibly performing high-layer retransmission (8/25); or 2) UE remains in INACTIVE and sends RRC Resume to new cell

7	FFS Upon SDT failure detection timer expiry, the same procedure as T319 expiry is used (e.g. transition to IDLE as in the case of expiry of the T319 timer and attempts RRC connection setup)  (18/8)

The following was agreed in RAN2#114:

Agreements:

- Release of CG-SDT configuration by system information indication is not supported

- RAN2 thinks that some feedback may be beneficial in case CG is used for subsequent transmission.  RAN2 assumes that existing mechanism can be used.

- For initial CG transmission, UE does not select any SSB if none of the SSBs’ RSRP is above the RSRP threshold.  FFS if re-evaluation for every CG transmission is necessary

The following was agreed in RAN2#115:

Agreements

- No new solution is defined to prevent data loss or duplication for the scenario where the anchor relocation is required in the middle of an SDT session, i.e. network can release UE back into RRC_INACTIVE

- PDCP entities of only the non-SDT RBs are re-established (i.e. not for the SDT RBs) when the UE moves from RRC_INACTIVE with SDT session ongoing to RRC CONNECTED.

- Events that trigger a termination or failure of an ongoing SDT session 1) cell reselection, 2) expiry of the SDT failure detection timer, 3) when Max retx is reached in RLC.  RLC AM max retransmission functionality remains unchanged.

- When a UE detects a failure of an ongoing SDT session, UE transitions autonomously into RRC_IDLE (as baseline solution).   If time allows or have a ready solution we can consider further optimizations.

In this paper, we discuss further control plane aspects.

# 2	Discussion

In UP-EDT, the network broadcasts the maximum transport block size (TBS) that can be used for Msg3 [TS 36.321].

Observation 1: The maximum allowed packet size - and the uplink data size when using RACH-based EDT has to be less than or equal to a TB size indicated in the system information.

It has been agreed use data volume threshold for the UE to decide whether to do SDT or not:

Agreements

8	Data volume threshold is used for the UE to decide whether to do SDT or not.   FFS how we calculate data volume.

However with above agreement it is not clear whether UL data volume is for the 1st transmission (MSGA or MSG3) or whether it is for the whole SDT procedure including subsequent transmissions.

Proposal 1: RAN2 to decide whether UL data volume threshold is applicable to SDT initiation or for the whole SDT procedure.

It may not be efficient for the network if the UE would initiate an SDT procedure anytime small data becomes available for transmission. Every SDT procedure comes at a certain radio resource cost for the network, in terms of PRACH preambles (if RA-SDT is used) and PUSCH resources in order to carry the small payload itself plus its encapsulation overhead (e.g. PHY/MAC/RLC/PDCP headers) as well as the RRC Resume Request message (48 or 64 bits). Therefore, the network has clear advantage in avoiding (frequent) SDT procedures carrying very limited data amount to reduce such resource costs. In some cases, SDT type of data is not very delay sensitive and transmission may be delayed. In such cases the UE could collect more data before SDT procedure is initiated.

Proposal 2: NW can configure minimum and maximum UL data volume threshold which are used by UE to decide whether to do SDT or not

RRC Resume Request may include short or long version of I-RNTI. Long version of the I-RNTI may accommodate more bits for the UE and cell identifier. It has been agreed that in case CG-SDT the configuration is valid only for the same serving cell. Therefore longer version of the I-RNTI is not needed for RRC Resume Request for CG-SDT.

Proposal 3: Short version of the I-RNTI is used in RRC Resume Request for CG-SDT.

Agreements:

- The configuration of configured grant resource for UE small data transmission is valid only in the same serving cell.  FFS for other CG validity criteria (e.g. timer, UL/SUL aspect, etc)

According to the WID objective in RP-193252 flexible payload sizes should be supported for MSGA and MSG3:

	Enable flexible payload sizes larger than the Rel-16 CCCH message size that is possible currently for INACTIVE state for MSGA and MSG3 to support UP data transmission in UL (actual payload size can be up to network configuration) [RAN2]

We would like to confirm the above objective.

Proposal 4: The network can configure the payload size for CCCH message for both MSGA and MSG3.

We remark that for 2-step RACH-based SDT, the network has to provision also the PUSCH resources configurations for MsgA. Although broadcast information may be the typical means used by the network to provision configuration for RACH-based SDT, we see no reasons to prevent UE-specific configuration.

Proposal 5: Both dedicated and broadcast RACH configuration is supported for SDT.

Furthermore, the network may benefit from suspending the use of SDT temporarily e.g. when facing sudden congestion and requiring the resources for other services. In addition, some UE implementation may misuse the SDT procedure so that data transmission is always started with SDT procedure followed by the resume to RRC Connected which should be avoided from the network perspective. Currently it is not clear if the agreed data volume threshold valid for the initial SDT transmission of for the whole SDT procedure including subsequent transmissions.

Proposal 6: RAN2 to discuss how to limit or suspend temporarily the use of SDT.

It was agreed that network can configure DRBs for SDT:

Agreements

7	Small data transmission is configured by the network on a per DRB basis

It would be useful to allow transmission of signalling messages via SDT procedure to avoid RRC connection establishment which requires extensive signalling between the UE and network and it also may consume UE battery. For example, NAS signaling (e.g. tracking area update), UE assistance information, MDT reporting etc could be performed via SDT procedure. The following working assumption was made in RAN2#113 meeting:

Working assumption

- Support configuring of SRB1 and SRB2 for small data transmission for carrying RRC and NAS messages.

- Upon initiating RRC Resume procedure for SDT initiation (i.e. for first SDT transmission), the UE shall also resume SRB2 that is configured for SDT, in addition to SDT DRBs that are configured for SDT

- RAN2 recommends to include SRB2 in WID

RAN2 working assumption was discussed in RAN and the following update was agreed in in the work item:

Specify configuring of SRB1 and SRB2 for small data transmission in RRC_INACTIVE state by reusing the framework for DRBs.

We propose to confirm the RAN2 working assumption:

Proposal 7: Support configuring of SRB1 and SRB2 for small data transmission for carrying RRC and NAS messages

Proposal 8: Upon initiating RRC Resume procedure for SDT initiation (i.e. for first SDT transmission), the UE shall also resume SRB2, if configured for SDT, in addition to SDT DRBs that are configured for SDT

In NR, the mapping of QoS flows onto radio bearers in the UE is controlled by mapping rules configured by the gNB. The mapping rules are configured either explicitly via RRC signalling or implicitly via Reflective QoS (RQoS). With RQoS, the UE derives the uplink mapping rule from the information available in the downlink. All QoS flows for which no mapping rules are configured are mapped by default on a default DRB. As a result, the 5G QoS framework allows a mode of operation where: 1) a default DRB with radio protocols configured to provide a default QoS is used to carry the bulk of traffic; and 2) only QoS flows with specific requirements are dynamically (re)mapped onto dedicated DRBs as they appear, without involving 5GC. This greatly minimises the amount of signalling required and reduces control plane latency. Furthermore, because the number of QoS flows to deal with is typically very high, configuring mapping rules in advance is seldom possible. With RQoS, the allocation of radio bearers is still controlled by the network, but control plane signalling is limited to the setup of the bearers: explicit signalling of the mapping rules for the plethora of flows is not required.

Since SDT can suffer data losses (e.g. cell re-selection after SDT transmission), it is not desirable to use SDT for flows that cannot afford data losses. Even if one bearer was configured for SDT, the mapping of QoS flows onto that bearer would still need to be carefully controlled. Unfortunately, as explained above, the number of QoS flows to deal with is typically very high and pre-configuring the mapping is seldom possible. In other words, the agreed DRB level configuration for SDT is not sufficient to efficiently deal with the NR QoS framework which relies on QoS flows. Therefore, we propose:

Proposal 9: Small data transmission can be further configured by the network on a per QoS flow ID (QFI) basis.

It was left FFS whether to use additional SDT specific RSRP threshold:

Agreements

FFS if an “additional SDT specific” RSRP threshold is further used to determine whether the UE should do SDT

We think that it would be beneficial to avoid the transmission of the SDT data on the cell-edge, because handling all the error cases and data forwarding can be very complex. Therefore, we propose to use already existing “not-at-cell-edge threshold for SDT.

Proposal 10: “Not at cell edge” threshold specified for REL16 UE power saving is re-used for SDT purpose i.e. the UE is not allowed to use SDT at cell edge

# 4	Conclusion

The following proposals have been made:

Proposal 1: RAN2 to decide whether UL data volume threshold is applicable to SDT initiation or for the whole SDT procedure.

Proposal 2: NW can configure minimum and maximum UL data volume threshold which are used by UE to decide whether to do SDT or not

Proposal 3: Short version of the I-RNTI is used in RRC Resume Request for CG-SDT.

Proposal 4: The network can configure the payload size for CCCH message for both MSGA and MSG3.

Proposal 5: Both dedicated and broadcast RACH configuration is supported for SDT.

Proposal 6: RAN2 to discuss how to limit or suspend temporarily the use of SDT.

Proposal 7: Support configuring of SRB1 and SRB2 for small data transmission for carrying RRC and NAS messages

Proposal 8: Upon initiating RRC Resume procedure for SDT initiation (i.e. for first SDT transmission), the UE shall also resume SRB2, if configured for SDT, in addition to SDT DRBs that are configured for SDT

Proposal 9: Small data transmission can be further configured by the network on a per QoS flow ID (QFI) basis.

Proposal 10: “Not at cell edge” threshold specified for REL16 UE power saving is re-used for SDT purpose i.e. the UE is not allowed to use SDT at cell edge