3GPP TSG-RAN WG2 Meeting #116-e                          R2-2110901

Online, November 1st – 12th 2021

Agenda item:	8.8.2

Source:	Samsung

Title:	Remaining Issues on Slice Info and Option 4

Document for:	Discussion & Decision

# Introduction

In RAN2#115-e meeting, RAN2 agreed the following on the topic of slice priority for cell reselection:

Agreements RAN2 needs to check with SA2/ CT1 if it is alright for AS to expect to receive slice list as well as slice priority information from NAS for cell (re)selection. Ask about both slices and slice groups. 1 Solution Option 4 is selected for further work i.e., resolve the FFSs, send any required LSs and consequently start to draft specification CRs. 2 Following is taken as the baseline for Solution Option 4: […] Step 0: NAS layer at UE provides slice information to AS layer at UE, including slice priorities. Step 1: AS sorts slices in priority order starting with highest priority slice. Step 2: Select slices in priority order starting with the highest priority slice. Step 3: For the selected slice assign priority to frequencies received from network. Step 4: Starting with the highest priority frequency, perform measurements (same as legacy). Step 5: If the highest ranked cell is suitable (as defined in 38.304) and supports the selected slice in step 2 then camp on the cell and exit this sequence of operation; FFS: How the UE determines whether the highest ranked cell supports the selected slice. Step 6: If there are remaining frequencies then go back to step 4. Step 7: FFS: If the end of the slice list has not been reached go back to step 2. Step 8: Perform legacy cell reselection.

RAN2 handled the following two email discussions:

- [Post115-e][245][Slicing] Running NR RRC CR for RAN slicing [1], and

- [Post115-e][246][Slicing] Running 38.304 CR for RAN slicing [2].

In this contribution we address some of the main issues discussed in those emails and provide adjustments to the Option 4 procedure accordingly.

# Discussion

## Remaining issues on slice information:

In RAN2#113bis-e meeting, RAN2 agreed the following:

5 UE is only configured with either the existing dedicated priority configuration or the slice info in RRC Release. 3 In the case that slice info is also provided to the UE in the RRC Release message while SIB also provides the slice info, UE follows the dedicated slice info from RRC Release while T320-like timer is running and only if it expires that it follows the slice info in the SIB 6 For UE supporting slice based cell reselection, the UE should use slice info in the SIB for cell reselection if both slice info and existing cell reselection priority is broadcast in the SIB.

2.1.1 How to handle slice frequency priorities

RAN2 agreed to provide slice info via SIB or RRCRelease message. However, it is not clear whether a new cell reselection priority (CRP) is also provided as part of the slice info. It is possibly beneficial for the network to have the mean of making UEs supporting slice-based cell reselection and legacy UEs apply different CRP on the same frequency for the load purpose. That is, it seems beneficial to introduce a new CRP associated with a slice group per frequency.

Observation 1: It would be beneficial to introduce a new CRP associated with a slice or slice group frequency.

For the legacy reselection priorities handling via broadcast signalling:

- As the serving frequency is the reference frequency for reselection priorities handling, then in order to apply measurement rule and cell reselection criteria, the CRP for intra-frequency is mandatory in SIB2, i.e. the value of CellReselectionPriority as highlighted below.

SIB2 ::= SEQUENCE { cellReselectionInfoCommon SEQUENCE { nrofSS-BlocksToAverage INTEGER (2..maxNrofSS-BlocksToAverage) OPTIONAL, -- Need S absThreshSS-BlocksConsolidation ThresholdNR OPTIONAL, -- Need S rangeToBestCell RangeToBestCell OPTIONAL, -- Need R q-Hyst ENUMERATED { dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10, dB12, dB14, dB16, dB18, dB20, dB22, dB24}, speedStateReselectionPars SEQUENCE { mobilityStateParameters MobilityStateParameters, q-HystSF SEQUENCE { sf-Medium ENUMERATED {dB-6, dB-4, dB-2, dB0}, sf-High ENUMERATED {dB-6, dB-4, dB-2, dB0} } } OPTIONAL, -- Need R ... }, cellReselectionServingFreqInfo SEQUENCE { s-NonIntraSearchP ReselectionThreshold OPTIONAL, -- Need S s-NonIntraSearchQ ReselectionThresholdQ OPTIONAL, -- Need S threshServingLowP ReselectionThreshold, threshServingLowQ ReselectionThresholdQ OPTIONAL, -- Need R cellReselectionPriority CellReselectionPriority, cellReselectionSubPriority CellReselectionSubPriority OPTIONAL, -- Need R - ... - },

However, for slice-aware cell reselection, it is NOT guaranteed that the serving frequency always broadcasts slice-specific CRP, w.r.t. the UE’s selected slice or slice group. That is because the cell selection process does not take into account the slice info. Moreover, it is possible that the serving frequency itself does not support any slice feature in Rel-17. On the other hand, the serving cell may broadcast slice-specific CRP for a certain inter-frequency w.r.t the UE’s selected slice or slice group.

Observation 2: Serving cell may not broadcast slice-specific CRP on serving frequency with respect to the UE's selected slice or slice group. As there is no reference slice-specific CRP, it is unclear how the UE handles the slice-specific CRP per inter-frequency if signalled (i.e. whether set to higher, lower, or equal priority), in order to apply measurement rules or cell reselection criteria for slice-aware cell reselection.

Note there is a rule how to handle CRP on dedicated signalling in TS 38.304 i.e.

When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than any of the network configured values).

To address the above issue on SIB based slice-based reselection priorities handling, we think the following could be potential examples of solution options for the UE to set a higher reselection priority of NR inter-frequency:

- Option 1: the UE shall consider the current frequency to be the lowest priority frequency, i.e. lower than any frequency(ies) assigned alike dedicated signalling.

- Option 2: If the UE assigns priority(ies) to inter-frequency(ies) received from NW for the selected slice or slice group, the UE shall consider such frequencies as higher reselection priority.

Proposal 1: If serving cell broadcasts only slice-specific CRP(s) on inter-frequency(ies) for the selected slice or slice group, RAN2 to discuss which option to support to set a higher reselection priority for NR inter-frequency for SIB based slice-based reselection priorities handling:

- Option 1: The UE shall consider the current frequency to be the lowest priority frequency, i.e. lower than any frequency(ies) assigned alike dedicated signalling.

- Option 2: If the UE assigns CRP(s) to inter-frequency(ies) received from NW for the selected slice or slice group, the UE shall consider such frequencies as higher reselection priority.

2.1.2 NR intra-frequency measurements

The legacy measurement rule on NR intra-frequency is that UE decides whether to perform NR intra-frequency measurements by comparing Srxlev/Squal with SIntraSearchP/Q i.e.

-     If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

-     Otherwise, the UE shall perform intra-frequency measurements.

However, as indicated in the issue 2.1.1, the NR intra-frequency may not support the UE’s selected slice or slice group. So in this case it would be better for this UE NOT to perform NR intra-frequency measurement regardless of the cell quality/threshold.

Proposal 2: The UE does not perform NR-intra-frequency measurement if the NR-intra frequency does not support the UE’s selected slice or slice group during slice-aware cell reselection.

## Adjustments of Option 4 procedure:

Based on the discussion in [2], we propose the following adjustments to solution Option 4 procedure:

Step 2: Select slices in priority order starting with the highest priority slice.

In the case of a slice group, which could include one or more slices, it is not clear how a UE selects the slice group with highest priority slice. Assuming that a slice group would include slices of the same priority, then Step 2 should be updated as follows:

Step 2: The UE selects the highest priority slice group

Observation 3: A slice group should include slices of similar/same priority

Step 3: For the selected slice assign priority to frequencies received from network. Step 4: Starting with the highest priority frequency, perform measurements (same as legacy).

As clarified during discussion on [2], it is feasible for the UE to perform measurements on multiple frequencies, i.e. the UE may perform measurements ‘simultaneously’ on multiple frequencies of different priorities. In this case it would be sufficient to refer to clause 5.2.4.2.

Moreover, after performing measurements, the UE performs cell reselection criteria according to clauses 5.2.4.5 and 5.2.4.6.

We think it is possible to combine Step 3 and Step 4, into a new Step 3:

Step 3: Based on slice specific priority of frequencies for the selected slice, the UE performs measurements according to clause 5.2.4.2, and applies cell reselection criteria according to clause 5.2.4.5 and 5.2.4.6.

Observation 4: It is feasible for a UE to perform simultaneous measurements on multiple frequencies of different assigned priorities.

Observation 5: The UE ranks cells according to highest-priority frequency, if more than one cell meets the cell reselection criteria in 5.2.4.5 or intra-frequency or equal priority inter-frequency case. Otherwise, the 5.2.4.5 is not executed (i.e. single best cell on highest-priority frequency).

Step 5: If the highest ranked cell is suitable (as defined in 38.304) and supports the selected slice in step 2 then camp on the cell and exit this sequence of operation; FFS: How the UE determines whether the highest ranked cell supports the selected slice.

Step 5 is updated as follows, based on discussion in [2]:

Step 5: If the highest ranked cell is suitable and supports the selected slice, then the UE camps on the cell and exits this sequence of operation. If the highest ranked cell is not suitable, then skip the measured frequency(ies) and go to Step 4.

Step 6: If there are remaining frequencies then go back to step 4.

Step 6 can be removed considering Observation 4.

Step 7: FFS: If the end of the slice list has not been reached go back to step 2.

We prefer to keep Step 7, in order to allow slice based cell reselection for next priority slice(s) in the UE slice list. Moreover, the NW can support UE’s slice(s) as much as possible based on UE’s requested slice info (NSSAI) as in legacy. We propose the following updated Step 7:

Step 7: FFS: If the end of the slice list has not been reached, select a slice in the next priority order and go back to Step 2.

Observation 6: To avoid delays to agreement on the cell reselection procedure, RAN2 should decide on Step 7 (keep/remove) independently to the outcome of whether measurement results can be used over multiple iterations. The re-use of measurement results is merely an optimization, to reduce the delay due to Step 7, that we do not need to support in this release.

Step 8: Perform legacy cell reselection.

In our understanding, the UE may not be allowed to perform Step 8 in a certain scenario. For example, assuming that the UE is configured with a slice based dedicated priority configuration in dedicated signalling; if the UE fails to re-select a suitable cell that supports the selected slice group, then the UE should fallback from slice-based cell reselection to legacy cell reselection in Step 8.

However, the UE cannot perform Step 8 considering that it is NOT allowed to perform legacy cell reselection if it is configured with a slice based dedicated priority configuration in dedicated signaling, and T320-like timer is running (or a T320-like timer is not configured).

Hence, we think it is necessary to clarify that Step 8 is only applicable when the UE performs slice-based cell reselection according to SIB.

Observation 7: The UE may not be allowed to go to Step 8 (legacy cell reselection), if it is configured with slice based dedicated priority configuration in dedicated signalling (e.g. RRCRelease).

Proposal 3: RAN2 to discuss whether/how the UE can perform legacy cell reselection if the UE is configured with slice based priority via dedicated signalling.

Proposal 4: RAN2 to agree on the following updated steps for Option 4:

Step 1: The UE selects the highest priority slice group. Step 2: Based on slice specific priority of frequencies for the selected slice, the UE performs measurements according to clause 5.2.4.2, and applies cell reselection criteria according to clause 5.2.4.5 and 5.2.4.6. Step 3: If the highest ranked cell is suitable and supports the selected slice, then the UE camps on the cell and exits this sequence of operation. If the highest ranked cell is not suitable, then skip the measured frequency(ies) and go to Step 4. Step 4: FFS: If the end of the slice list has not been reached, select a slice in the next priority order and go back to Step 2. Step 5: If the UE is configured with slice based priority via SIB, perform cell reselection.

# Conclusion

In this contribution, we discussed remaining issues on slice information and proposed updates to steps of Option 4. The following are the observations and proposals in this document:

Observation 1: It would be beneficial to introduce a new CRP associated with a slice or slice group frequency.

Observation 2: Serving cell may not broadcast slice-specific CRP on serving frequency with respect to the UE's selected slice or slice group. As there is no reference slice-specific CRP, it is unclear how the UE handles the slice-specific CRP per inter-frequency if signalled (i.e. whether set to higher, lower, or equal priority), in order to apply measurement rules or cell reselection criteria for slice-aware cell reselection.

Observation 3: A slice group should include slices of similar/same priority

Observation 4: It is feasible for a UE to perform simultaneous measurements on multiple frequencies of different assigned priorities.

Observation 5: The UE ranks cells according to highest-priority frequency, if more than one cell meets the cell reselection criteria in 5.2.4.5 or intra-frequency or equal priority inter-frequency case. Otherwise, the 5.2.4.5 is not executed (i.e. single best cell on highest-priority frequency).

Observation 6: To avoid delays to agreement on the cell reselection procedure, RAN2 should decide on Step 7 (keep/remove) independently to the outcome of whether measurement results can be used over multiple iterations. The re-use of measurement results is merely an optimization, to reduce the delay due to Step 7, that we do not need to support in this release.

Observation 7: The UE may not be allowed to go to Step 8 (legacy cell reselection), if it is configured with slice based dedicated priority configuration in dedicated signalling (e.g. RRCRelease).

Proposal 1: If serving cell broadcasts only slice-specific CRP(s) on inter-frequency(ies) for the selected slice or slice group, RAN2 to discuss which option to support to set a higher reselection priority for NR inter-frequency for SIB based slice-based reselection priorities handling:

- Option 1: The UE shall consider the current frequency to be the lowest priority frequency, i.e. lower than any frequency(ies) assigned alike dedicated signalling.

- Option 2: If the UE assigns CRP(s) to inter-frequency(ies) received from NW for the selected slice or slice group, the UE shall consider such frequencies as higher reselection priority.

Proposal 2: The UE does not perform NR-intra-frequency measurement if the NR-intra frequency does not support the UE’s selected slice or slice group during slice-aware cell reselection.

Proposal 3: RAN2 to discuss whether/how the UE can perform legacy cell reselection if the UE is configured with slice based priority via dedicated signalling.

Proposal 4: RAN2 to agree on the following updated steps for Option 4:

Step 1: The UE selects the highest priority slice group. Step 2: Based on slice specific priority of frequencies for the selected slice, the UE performs measurements according to clause 5.2.4.2, and applies cell reselection criteria according to clause 5.2.4.5 and 5.2.4.6. Step 3: If the highest ranked cell is suitable and supports the selected slice, then the UE camps on the cell and exits this sequence of operation. If the highest ranked cell is not suitable, then skip the measured frequency(ies) and go to Step 4. Step 4: FFS: If the end of the slice list has not been reached, select a slice in the next priority order and go back to Step 2. Step 5: If the UE is configured with slice based priority via SIB, perform cell reselection.

# References

[1] [Post115-e][245][Slicing] Running NR RRC CR for RAN slicing.

[2] [Post115-e][246][Slicing] Running 38.304 CR for RAN slicing.

[3] TR 38.832, Study on enhancement of Radio Access Network (RAN) slicing.

[4] RP-210912, New WID on enhancement of RAN Slicing for NR.