3GPP TSG-RAN WG2 #116-e	R2-2110902

Electronic meeting, November 1st – 12th, 2021

Agenda Item:	8.16.2

Source:	Ericsson

Title:	SNPN access using external credentials

Document for:	Discussion

# 1	Introduction

At RAN2#115-e, the following agreements were reached:

Wait for SA2 reply LS on the issue whether a common list of GINs used for onboarding and SNPN access using external credentials. RAN2 has not identified a need for modification of / addition to broadcast of HRNNs. RAN2 confirms that there is no impact on connected mode mobility when accessing an SNPN through CHs (was already assumed). maximum number of GINs is specified per cell new SIB specified to broadcast GINs acc to Option B: Single list of GINs with explicit assignment to SNPNs. Details on the explicit assignment are FFS. RAN2 didn’t identify a need for modification to access control for SNPN access using external credential (could be discussed in other groups)

From RAN2#115-e’s Agenda Item 8.16.2 + 8.16.3 Summary, R2-2109033 [1], the only open issue is related to how the new indicators are added to SIB1:

Proposal 4: RAN2 select among the options below how the new indicators ("access using external credentials is supported", "SNPN allows registration from UEs that are not explicitly configured to select the SNPN", "onboarding support") are added to SIB1. Option A: introduce the new indicators parallel with npn-IdentityInfoList in CellAccessRelatedInfo Option B: introduce the new indicators in npn-IdentityInfo Option C: introduce a new list for the new indicators in npn-IdentityInfo

However, as highlighted in the chair’s notes:

P4 - Chair wonder if we can let the RRC TS rapporteur decide this. […] - Chair: P4 decided in the CR work, ask RRC rapporteur to have an opinion.

Thus, in this contribution we touch upon open issues from SA2’s LS reply to RAN2 (see S2-2106708 [2]) and address how GINs should be associated with the SNPNs.

# 2	Discussion

## 2.1	GIN broadcast for different purposes

During RAN2#114-e, RAN2 asked SA2 (see LS R2-2106545 [3]) whether a common list of GINs can be used for both onboarding and to access an SNPN using external credentials. Since SA2 replied ‘yes’ to RAN2 (see S2-2106708 [2]), in order to reduce the broadcast burden, the newly agreed SIB for GINs should contain a single/common list for both purposes.

- Use a single/common list of GINs for onboarding and to access an SNPN using external credentials.

## 2.2	GIN broadcasting details

In addition to the previously mentioned question, the LS sent during RAN2#114-e (see LS R2-2106545 [3]) also asked SA2 whether the GIN uses SNPN ID or NID encoding.

SA2 answered in the LS reply S2-2106708 [2] that:

SA2 agrees that GIN is encoded as a) an SNPN ID (i.e., a PLMN ID plus a NID).

Since an SNPN ID comprises a PLMN ID plus a NID, the broadcasting overhead will thus be higher, compared to the NID encoding option.

The corresponding ASN.1 code for GINs could thus look as follows:

GIN-r17 ::=                 SEQUENCE {

plmn-Identity-r17             PLMN-Identity,

gin-Value-r17                 BIT STRING (SIZE (44)) 		-- name is FFS

}

gin-Value A gin-Value reuses the NID encoding, which is specified in TS 23.003 [XX]. The PLMN ID and a gin-Value in the GIN represent a Group ID for Network selection. [Note: The name/description is pending CT4 definition.]

As shown in Figure 1 below, the PLMN ID is composed of the MCC (optional) and MNC, where the MCC consists of 3 digits (0-12 bits) and the MNC consists of 2-3 digits (8-12 bits). The optionality indication for the MCC also requires the signalling of 1 bit, i.e., 1 bit to indicate absence or presence of the MCC. A PLMN ID is thus represented by 9-25 bits.

PLMN-Identity ::=                   SEQUENCE {

mcc                                 MCC     OPTIONAL,   -- Cond MCC     -- 1 bit or 1+12 bits

mnc                                 MNC									-- 8-12 bits

}

MCC ::=                             SEQUENCE (SIZE (3)) OF MCC-MNC-Digit

MNC ::=                             SEQUENCE (SIZE (2..3)) OF MCC-MNC-Digit

MCC-MNC-Digit ::=                   INTEGER (0..9)

Figure 1. ASN.1 code defining the PLMN-Identity (PLMN ID).

Since a NID is simply defined as a 44 bits string, this means that one GIN requires the broadcast of 53-69 bits, i.e., including the PLMN ID may add up to 25 bits corresponding to 57% overhead as compared to the NID encoding option. However, we believe that it is still possible to reduce the burden on the broadcast.

As explained in TR 23.700-07 [4] the GIN can identify a multi-national operator or an interconnection provider. An interconnection provider could also have partner networks from different countries.

Moreover, TR 23.700-07 [4] and TS 23.501 [5] further consistently state that the NID encoding can be self-managed (assignment mode 1) or globally unique (assignment mode 0).

For a multi-national operator and an association of multi-national partners the PLMN ID would not point to a specific country or even network, as the GIN represents a group of networks. Even more so, given that the NID itself can be globally unique, it could thus be questioned whether the PLMN ID would be needed in those scenarios.

Hence, when using globally unique NIDs, the PLMN ID can eventually be removed from the GIN encoding. Even for the self-managed case, the PLMN ID may not have any meaning and could thus be optional. This would avoid unnecessary broadcasting overhead.

Therefore, RAN2 should ask SA2 whether the PLMN ID is mandatory or can be optionally set.

Ask SA2 whether the PLMN ID in the GIN encoding can optionally be set, i.e., if the PLMN ID could be omitted in the broadcast for some cases.

## 2.3	How GINs should be associated with the SNPNs

At RAN2#113bis-e, the following was agreed:

GIDs are broadcasted per SNPN in network sharing scenarios.

While during RAN2#115-e it was agreed that:

maximum number of GINs is specified per cell new SIB specified to broadcast GINs acc to Option B: Single list of GINs with explicit assignment to SNPNs. Details on the explicit assignment are FFS.

The explicit assignment avoids having to broadcast the same GIN multiple times in network sharing scenarios (i.e., once for each supporting SNPN). Such an assignment can be in either direction, i.e., a GIN-to-SNPN mapping or an SNPN-to-GIN mapping could be used. These approaches are illustrated in Figure 2 and described in the sections below. In this specific example, we assume the following numbers:

- NSNPN_A = 4 “advanced” SNPNs which indicate that external credentials are supported and/or that onboarding is enabled (in short, SNPNs providing CH/onboarding indications)

- NGIN = 6 different GINs included in the new SIB

- Nassoc = 8 associations between the SNPNs and the GINs (8 lines connecting the nodes)

In Figure 2, SNPN-1 is not shown as it is a legacy Rel-16 SNPN. In the examples below, Rel-16 SNPNs are not represented in the bitmaps or not indexed, respectively.

Figure 2. SNPN-to-GIN vs. GIN-to-SNPN mapping.

### 2.3.1	Bitmap approaches

Option 1a: one bitmap is provided for each SNPN, and each bit in the bitmap is associated with a certain GIN, i.e., for each SNPN, the bitmap indicates which of the GINs (in the separately provided GIN list) is supported by that SNPN.

The SNPNs for which a bitmap is provided could be a subset of the SNPNs broadcast in the npn-IdentityInfoList, i.e., only SNPNs which  provide CH/onboarding indications.

As captured in the running RRC CR, R2-2108874 [6], the following field has been introduced in SIB1:

snpn-AccessInfoList This list indicates the support of external Credentials Holder and onboarding for each SNPN in npn-IdentityInfoList, see TS 23.501 [32]. The n-th entry of the list contains the indicators of the n-th SNPN in npn-IdentityInfoList.

So, the bitmap could be provided for those SNPNs for which one of the above indicators is set to true. In short, SNPNs with CH/onboarding indication.

One such example was illustrated in R2-2109033 [1].

A similar example, referred to as “Example A”, is provided in Figure 3:

Figure 3. Example A, where the bits in the bitmap indicate the supported GINs for a certain SNPN.

In this option, bitmaps are only provided for SNPNs which provide CH/onboarding indications (i.e., SNPNs 2-5). It should however be noted that there could be an optional bitmap for each of the SNPNs that are listed in SIB1, i.e., also for SNPN-1.

The corresponding ASN.1 code for the GIN info within the new SIB could look as follows:

SIBnn-r17 ::=                    SEQUENCE {

gin-List-r17			         SEQUENCE (SIZE (1..maxGIN-r17)) OF GIN-r17

gin-SupportPerSNPN-List-r17	     SEQUENCE (SIZE (1..maxNPN-r16)) OF GIN-SupportPerSNPN-r17

}

GIN-SupportPerSNPN-r17 ::=       SEQUENCE {

supportedGINs-r17                BIT STRING (SIZE (1..maxGIN-r17)) 	   OPTIONAL		 -- Need R

}

SIBnn field descriptions

gin The Group Identifier for Network selection (GIN) as defined in TS 23.003 [XX]. [Note: The name/description is pending CT4 definition.]

gin-List The ginList contains a list of all GINs supported by the cell.

gin-SupportPerSNPN-List Indicates the GIN support for each SNPN in snpn-AccessInfoList for which ‘extCH-Supportedand/or ‘onboardingEnabledis set to ‘true’. The number of list elements corresponds to the number of SNPNs providing such indication(s), and the first list element corresponds to the first SNPN in snpn-AccessInfoList which provides such indication(s), and the second list element corresponds to the second SNPN in snpn-AccessInfoList which provides such indication(s), and so on.

GIN-SupportPerSNPN field descriptions

supportedGINs Indicates which GIN provided in gin-List is supported by a given SNPN. The first/leftmost bit corresponds to the first GIN provided in gin-List, the second bit corresponds to the second GIN provided in gin-List, and so on. A bit set to ‘1’ means that the GIN is supported by the SNPN.

Figure 4. ASN.1 example where bitmap indicates supported GINs for a certain SNPN.

Option 1b: Only one list is provided in the new SIB, where each list element maps a given GIN with one or more SNPN. Hence, each list element consists of:

- a GIN, and

- a bitmap indicating the SNPNs supporting access for the corresponding GIN.

The corresponding ASN.1 code could look as follows:

SIBnn-r17 ::=            SEQUENCE {

gin-InfoList-r17         SEQUENCE (SIZE (1..maxGIN-r17)) OF GIN-Info-r17

}

GIN-Info-r17 ::=         SEQUENCE {

gin-r17                  GIN-r17,

supportingSNPNs-r17      BIT STRING (SIZE (1..maxNPN-r16))

...

}

SIBnn field descriptions

gin-InfoList The gin-InfoList is used to configure a set of GIN-Info elements. Each of those elements contains a list of GINs and additional information associated with those GINs.

GIN-Info field descriptions

gin The Group Identifier for Network selection (GIN) as defined in TS 23.003 [XX]. [Note: The name/description is pending CT4 definition.]

supportingSNPNs Indicates which of the SNPNs supports this gin. The first/leftmost bit corresponds to the first SNPN in snpn-AccessInfoList for which ‘extCH-Supportedand’ or ‘onboardingEnabled’ is set to ‘true’, the second bit corresponds to the second SNPN with such indications. A bit set to ‘1’ means that the GIN is supported by the SNPN.

Figure 5. ASN.1 example where bitmap indicates the supporting SNPNs for a certain GIN.

In the above example, the length of the bitmap corresponds to the number of SNPNs supporting/allowing external credentials and/or onboarding.

When comparing the above ASN.1 examples, we make the following observations:

|  | Option 1a) SNPN-to-GIN mapping | Option 1b) GIN-to-SNPN mapping |
| --- | --- | --- |
| Short description | Two separate lists: 1. GIN-List + 2. extended list for SNPNs with SNPN-to-GIN mapping | Single GIN-InfoList with all GIN related info: GIN + GIN-to-SNPN mapping |
| Logic | The GINs are provided in one list, while the mapping is provided in separate list which is parallel to the (filtered) SNPN list | Each GIN in the GIN-Info is listed together with the mapping information to SNPNs -> appears simpler |
| UE behavior on NAS level | The UE tries to find the GINs provided in the GIN-List within its preconfigured GIN list; for prioritized matching GIN, the UE needs to check for each listed SNPN, whether the matching GIN(s) is supported, to find the supporting SNPNs; then the UE can start SNPN selection -- all entries of the second list (each SNPN info) needs to be parsed for a certain GIN | The UE tries to find the GIN provided in the GIN-Info within its preconfigured GIN list; for prioritized matching GIN, the UE can immediately start network selection: the UE checks the bitmap to evaluate which SNPNs support the GIN ++ only the bitmaps corresponding to a mapping GIN need to be parsed |
| Overhead | Bitmap may become long, depending on the number of listed GINs / maxGIN; for each bitmap, there could be a size indication as well which requires SGIN = log2(maxGIN) bits | Bitmap is at most maxNPN = 12; size indication for each bitmap would add SNPN = log2(12) = 4 bits; so if NSNPN_A < 8, the bitmap becomes shorter than 8 bits, otherwise even up to 16 bits including the size indication |
| Overhead reduction | There may be SNPNs that do not (explicitly) support any GINs -> the bitmap can be absent -> potential for reduction of broadcast overhead (save up to maxGIN-1 bits) per omitted bitmap; however: - the saving can be at most (maxNPN-1) * (maxGIN-1), while the overhead for the GIN-List can already be up to maxGIN * (53…69) bits - while maxNPN is 12, the number of SNPNs in a realistic network sharing scenario is expected to be far lower than the max value | Each GIN is supported by at least one of the SNPNs, i.e. the bitmap is mandatory for each GIN |

Grouping of all GIN related information in the GIN-to-SNPN mapping scheme appears simpler in the ASN.1 code than providing 2 separate lists in the SNPN-to-GIN mapping scheme.

### 2.3.1	Overhead comparison

We calculate the number of broadcasted bits for the mapping approaches described above. It should be noted that the numbers vary depending on how many GINs will be broadcast and on how many SNPNs indicate that external credentials/onboarding is supported/allowed.

For the GIN-to-SNPN mapping approach, it is assumed that each GIN is always associated with at least one SNPN, such that the supportingSNPNs field is mandatory rather than optional. Whereas, in the SNPN-to-GIN mapping approach, not all SNPNs support a specific GIN, and thus, the bitmap list would not be needed.

Therefore, as an optimization, only the SNPNs supporting at least one GIN provide the mapping info. Here, the number of such SNPNs is denoted as NSNPN_G for “SNPNs supporting at least one GIN”.

- NSNPN_A ... number of “advanced” SNPNs with CH/onboarding indication(s)

- NSNPN_G ... number of “advanced” SNPNs which support at least one GIN

- NGIN ... number of GINs to be broadcast

- maxGIN ... maximum number of GINs to be broadcast, value FFS

- SNPN  = 4 bits to indicate variable SIZE (1..maxNPN), maxNPN = 12

- SGIN  = log2(maxGIN) to indicate variable SIZE (1..maxGIN)

Table 1. Comparing the mapping schemes.

| Mapping direction | Bitmap approach [bits] | Bitmap approach without size indication [bits] |
| --- | --- | --- |
| GIN overhead | NGIN * 69 + SGIN |  |
| SNPN-to-GIN mapping | NSNPN_G * (NGIN + SGIN) + SNPN + NSNPN optionality bits | NSNPN_G * maxGIN + maxNPN optionality bits |
| GIN-to-SNPN mapping | NGIN * (NSNPN_A + SNPN) | NGIN * maxNPN |

For the comparison illustrated in Table 2 below, we assume the values corresponding to the example in Figure 2, i.e., NSNPN_G = 3, NSNPN_A = 4, and NGIN = 6 and various other values.

Furthermore, we assume that maxGIN = 32, such that SGIN  = log2(maxGIN) = 5.

We also show the bitmap using variable size indication, and a version where a fixed bitmap length is used, i.e., Lbitmap = maxGIN = 32 for the SNPN-to-GIN mapping scheme, and Lbitmap = maxNPN = 12 for the GIN-to-SNPN mapping scheme.

Table 2. Comparing the mapping options using Example A and other values.

| Mapping direction | Bitmap with variable size [bits] | No size indication [bits] | Bitmap with variable size [bits] | No size indication [bits] | Bitmap with variable size [bits] |
| --- | --- | --- | --- | --- | --- |
|  | NGIN = 6 |  | NGIN = 16 |  | NGIN = 32 |
| GIN overhead | 6 * 69 + SGIN = 419 bits |  | 16 * 69 + SGIN = 1109 bits |  | 2213 |
| SNPN-to-GIN mapping NSNPN_G = 3 | 3 * (6+5) + 8 = 41 (10 %) | 3 * 32 + 12 = 108 (26 %) | 3 * (16+5) + 8 = 71 (6 %) | 3 * 32 + 12 = 108 (10 %) | 3 * (32+5) + 8 = 119 (5 %) |
| SNPN-to-GIN mapping NSNPN_G = 1 | 1 * (6+5) + 8 = 20 (5 %) | 1 * 32 + 12 = 44 (11 %) | 1 * (16+5) + 8 = 29 (3 %) | 1 * 32 + 12 = 44 (4 %) | 1 * (32+5) + 8 = 45 (2 %) |
| SNPN-to-GIN mapping NSNPN_G = 4 | 4 * (6+5) + 8 = 52 (12 %) | 4 * 32 + 12 = 140 (33 %) | 4 * (16+5) + 8 = 92 (8 %) | 4 * 32 + 12 = 140 (13 %) | 4 * (32+5) + 8 = 156 (7 %) |
| SNPN-to-GIN mapping NSNPN_G = 6 | 6 * (6+5) + 8 = 74 (18 %) | 6 * 32 + 12 = 204 (49 %) | 6 * (16+5) + 8 = 134 (12 %) | 6 * 32 + 12 = 204 (18 %) | 6 * (32+5) + 8 = 230 (10 %) |
| SNPN-to-GIN mapping NSNPN_G = 8 | 8 * (6+5) + 8 = 96 (23 %) | 8 * 32 + 12 = 268 (64 %) | 8 * (16+5) + 8 = 176 (16 %) | 8 * 32 + 12 = 268 (24 %) | 8 * (32+5) + 8 = 304 (14 %) |
| GIN-to-SNPN mapping NSNPN_A = 4 | 6 * (4+4) bits = 48 (11 %) | 6 * 12 = 72 (17 %) | 16 * (4+4) bits = 128 (12 %) | 16 * 12 = 192 (17 %) | 32 * (4+4) bits = 256 (12 %) |
| GIN-to-SNPN mapping NSNPN_A = 1 | 6 * (1+4) bits = 30 (7 %) | 6 * 12 = 72 (17 %) | 16 * (1+4) bits = 80 (7 %) | 16 * 12 = 192 (17 %) | 32 * (1+4) bits = 160 (7 %) |
| GIN-to-SNPN mapping NSNPN_A = 8 | 6 * (8+4) bits = 72 (17 %) | 6 * 12 = 72 (17 %) | 16 * (8+4) bits = 192 (17 %) | 16 * 12 = 192 (17 %) | 32 * (8+4) bits = 384 (17 %) |

For Example A, the SNPN-to-GIN mapping with size indication requires the smallest amount to be broadcast for the mapping. Also, when the number of GINs is larger, the SNPN-to-GIN mapping is more efficient. The relative overhead as compared to the GIN overhead ranges from 2-12% for NSNPN_G ≤ 4 for the SNPN-to-GIN mapping, while it is 7-12% for the GIN-to-SNPN mapping. However, the relative overhead increases for the SNPN-to-GIN mapping with the number of SNPNs which supports at least one GIN (NSNPN_G can be up to 12). This is illustrated in Table 3.

“Worst case” assumptions: NSNPN_A = NSNPN_G = 12  log2(NSNPN_A) = 4

Table 3. Comparing the mapping options with worst case assumptions (NSNPN_A = NSNPN_G = 12).

| Mapping direction | Bitmap approach [bits] NSNPN_A = NSNPN_G = 12 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | NGIN = 32 | NGIN = 16 | NGIN = 12 | NGIN = 8 | NGIN = 4 | NGIN = 1 |
| GIN overhead + size indicator for GIN (info) list | 2213 | 1109 | 833 | 555 | 281 | 74 |
| SNPN-to-GIN mapping Variable bitmap size | NSNPN_G * (NGIN + SGIN) + SNPN + NSNPN_A optionality bits = 12 * NGIN + 12*5 + 4 + 12 = 12 * NGIN + 76 |  |  |  |  |  |
| Variable bitmap size | 460 (21%) | 268 (24%) | 220 (26%) | 172 (31%) | 124 (44%) | 88 (119%) |
| No size indication 12*maxGIN + maxNPN | 396 (18%) | 396 (36%) | 396 (48%) | 396 (71%) | 396 (14 %) | 396 (535%) |
| GIN-to-SNPN mapping Variable bitmap size | NGIN * (NSNPN_A + SNPN) = NGIN * (12 + 4) = 16 * NGIN |  |  |  |  |  |
| Variable bitmap size | 512 (23%) | 256 (23%) | 192 (23%) | 128 (23%) | 64 (23%) | 16 (22%) |
| No size indication 12 * NGIN | 384 (17%) | 192 (17%) | 144 (17%) | 96 (17%) | 48 (17%) | 12 (16%) |

For a large number of NSNPN_G, the overhead for sending the size indications for the SNPN-to-GIN bitmap becomes relatively large compared to the GIN overhead, also with decreasing number of GINs to be broadcast, while the increase is proportional for the GIN-to-SNPN bitmap. For the latter, the relative overhead is stable, i.e., 23 % for the variable bitmap size with length indication and 17% for a fixed bitmap length of 12.

However, it should be noted that it may not be very likely to have 12 SNPNs in a network scenario, with all associated with the same and only GIN (NGIN = 1).

In principle, the actual overhead depends on various parameters, such as maxGIN, the number of GINs to be broadcast, and the number of SNPNs in a network sharing scenario with CH/onboarding indications and also, whether these SNPNs support at least one GIN.

If there are only few SNPNs in a network sharing scenario supporting at least one GIN, the SNPN-to-GIN mapping requires smaller overhead, but the relative overhead becomes larger with more SNPNs supporting at least one GIN.

The GIN-to-SNPN mapping allows for less overhead reduction if there are only few SNPNs supporting at least on GIN, but the relative overhead is stable also when many SNPNs support at least one GIN.

For the GIN-to-SNPN mapping scheme, a CHOICE structure could be used to select between a bitmap with variable size or fixed size (for NSNPN_A > 8).

RAN2 to select a mapping approach. We slightly prefer the bitmap approach using GIN-to-SNPN mapping.

## 2.4	Maximum number of GINs

It is unclear whether the maximum number of GINs should be in RAN2 scope. The TR 23.700-07 [4] states the following in clause 8.1.4:

NOTE 2: The number of supported Group IDs that can be broadcast will be determined by RAN WG2.

According to our understanding, SA2 only asks about the capacity for broadcasting the GINs.

SA2 wants to know the number of GINs that can be broadcast in SIB given the SIB size limitations but may not want RAN2 to specify the RRC multiplicity.

Regarding the capacity, we have the following NOTE in the RRC specification, TS 38.331, clause 5.2.1:

NOTE: The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits.

The upper limit for the maximum GIN number depends on the approach used to associate GINs to supporting SNPNs (see Section above). The worst-case scenario should be considered for such a calculation.

GIN length LGIN = LSNPN ID = 69 bits

- SNPN-to-GIN mapping (Lbitmap = NGIN):

SIB size = NGIN * LGIN + SGIN + NSNPN_G * (Lbitmap + SGIN) + NSNPN_A

= NGIN * LGIN + SGIN + NSNPN_G * (NGIN + SGIN) + NSNPN_A

= NGIN * (LGIN + NSNPN_A) + SGIN + NSNPN_A * SGIN + NSNPN_A

= NGIN * (69 + 12) + 5 + 12 * 5 + 12

= 81 * NGIN + 77

-> NGIN_max = (SSIB,max – 77) / 81

Option b) GIN-to-SNPN mapping (Lbitmap = 12):

SIB size = SGIN + NGIN * (LGIN + Lbitmap + SNPN)

= 5 + NGIN * (69 + 12 + 4)

= 5 + 85 * NGIN

-> NGIN_max = (SSIB,max -5) / 85

Table 2. Capacity for GIN broadcasting.

| Mapping approach | Bitmap approach | Index approach |
| --- | --- | --- |
| SNPN to GIN mapping | (2976-77) / 81 = 2899 / 81 = 35 | depends on number of associations |
| GIN to SNPN mapping | (2976-5) / 85 = 2971 / 85 = 34 | depends on number of associations |

The number of GINs that can be broadcast depends on the broadcasting details. A realistic number in terms of 2x would be 25 = 32 GINs.

If RAN2 is supposed to discuss such requirements for the number of GINs to be broadcast, this can be addressed as part of stage-3 discussion.

If in RAN2 scope, specify the maximum number of GINs during Stage-3 discussion.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Grouping of all GIN related information in the GIN-to-SNPN mapping scheme appears simpler in the ASN.1 code than providing 2 separate lists in the SNPN-to-GIN mapping scheme.

Observation 2	If there are only few SNPNs in a network sharing scenario supporting at least one GIN, the SNPN-to-GIN mapping requires smaller overhead, but the relative overhead becomes larger with more SNPNs supporting at least one GIN.

Observation 3	The GIN-to-SNPN mapping allows for less overhead reduction if there are only few SNPNs supporting at least on GIN, but the relative overhead is stable also when many SNPNs support at least one GIN.

Observation 4	SA2 wants to know the number of GINs that can be broadcast in SIB given the SIB size limitations but may not want RAN2 to specify the RRC multiplicity.

Observation 5	The number of GINs that can be broadcast depends on the broadcasting details. A realistic number in terms of 2x would be 25 = 32 GINs.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Use a single/common list of GINs for onboarding and to access an SNPN using external credentials.

Proposal 2	Ask SA2 whether the PLMN ID in the GIN encoding can optionally be set, i.e., if the PLMN ID could be omitted in the broadcast for some cases.

Proposal 3	RAN2 to select a mapping approach. We slightly prefer the bitmap approach using GIN-to-SNPN mapping.

Proposal 4	If in RAN2 scope, specify the maximum number of GINs during Stage-3 discussion.

# 4	References

R2-2109033, “[Pre115-e][009][eNPN] Summary 8.16.2 ext credentials + 8.16.3 onboarding”, Nokia, RAN2#115-e, 2021-08

S2-2106708, “Reply to LS on Group IDs for Network selection (GINs)”, SA2, SA2#146E, 2021-08

R2-2106545, “LS on Group IDs for network selection (GINs)”, RAN2#114-e, 2021-05

TR 23.700-07, “Study on enhanced support of non-public networks”, TSG SA, V17.0.0, 2021-03

TS 23.501, “System architecture for the 5G System (5GS)”, TSG SA, V17.2.0, 2021-09

R2-2108874, “Draft RRC CR: Non-Public Network enhancements”, Nokia, Nokia Shanghai Bell, RAN2#115-e, 2021-08