3GPP TSG-RAN WG2 Meeting #116-e		R2-2110921

Electronic, 1st November – 12th November 2021

Source:	Qualcomm Incorporated

Title:	NR-U Related Enhancements

Agenda Item:	8.13.2.3 Other WID related SON features

Document for:	Discussion and Decision

# Introduction & Background

Rel-17 SON/MDT [1] includes the following RAN2-led objectives in the context of MDT.

- Support of data collection for SON features, including CCO, inter-system inter-RAT energy-saving, inter-system load balancing, 2-step RACH optimization, mobility enhancement optimization, and leftovers of Rel-16 SON/MDT WI (PCI selection, energy efficiency (OAM requirements), Successful Handovers Reports, UE history information in EN-DC, load balancing enhancement, MRO for SN change failure, RACH Optimization enhancements) [RAN3, RAN2]

- Specification of the UE reporting is necessary to enhance the network configuration [RAN2].

In this contribution draft, we explore the possible enhancements to the UE history Information for SCG mobility robustness and reporting of NR-U parameters considering failure and non-failure scenarios.

# Discussion

2.1 NR-U Related Enhancements

In the Rel-16 and prior releases, NR-Unlicensed (NR-U) was discussed to enable transmissions over the shared spectrums. However, before the transmission channel must be sensed (Listen Before Talk (LBT)) and reserved for NR-U transmissions. Note that during LBT, many a time the LBT may fail, i.e., the channel is occupied with ongoing transmission. In Post RAN2-emeeting email discussions 851 [2], it is confirmed that NR-U related optimization will be discussed in rel-17.

Observation 1: It is confirmed that NR-U related enhancement should be discussed in Rel-17.

Proposal 1: RAN2 is requested to confirm whether LBT Failures statistics are important for the network optimizations in the following scenarios

- Nonfailure scenarios: for selection of appropriate BWP, such as successful handover, RACH optimizations

- Near failure scenarios: for selection of appropriate BWP, when multiple BWP attempted for connection establishments but no RLF, SCGFailures, or CEF

- Failure Scenarios, such as RLF, SCGFailures, CEF

Note that the LBT statistics can be used in all of the three (failure, non-failure, and near-failure) scenarios in proposal 1, where LBT resources can be optimized by the network to select proper LBT modes or parameters, for the adjustment/selection of resource configurations to increase LBT success rate, and minimization of RACH, RLF, and SCGFailures.

Observation 2: LBT statistics can be used by the network for the following:

- For the selection of proper LBT modes or parameters for the UEs

- For adjusting/selecting resource configuration to increase LBT successful rate for the UEs

- For the minimizations of RACH failures, RLFs, and SCG failures.

For the optimization of transmission opportunity in the NR-U band and the success rate of LBT, LBT failure statistics on different BWPs may be required. Thus, we propose to introduce a new SON message as “LBTFailureInformation”.  The statistics can include the number of LBTFailures on different BWPs during RACH procedures, UL transmission on PUSCH, UL transmission on PCCH, and/or during semi-persistent scheduled (SPS) transmission. Furthermore, the number of LBTFailures during different channel access schemes (cat 1, cat 2A, cat 2B, cat 4) can be used by the network for adapting efficient channel schemes in different scenarios. UE can also add the measured RSSI values and CO per BWP and/or RB set to the network for NR-U related optimizations and decision making. Channel access priority class (CAPC) used for transmission on the shared spectrum can further be useful to the network for the implementation of NR-U related optimizations.

Observation 3: Nonfailure scenario: During the normal operations, the network can achieve NR-U related enhancements by using the statistics/number of LBT failures on different BWPs.

As discussed in Rel-16 NR-U [4], the consistent UL LBTFailures on all BWPs in PCell on configured resources results in RLF, the consistent UL LBTFailures on all BWPs in PSCell on configured resources results in SCGFailures, and consistent UL LBTFailures on BWP in SCell on configured resources results in RLC Failures. In these events, the proposed LBTFailureInformation message can be used together with the RLF report, SCGFailure Report, and FailureInformation to understand the underlying cause. Further, using the analysis of LBTFailureInformation, the network can adapt schemes to minimize these events. Furthermore, note that even though RLF and SCG Failures are declared when LBTFailures happen at all BWPs, LBTFailureInformation (when consistent LBTFailures happen at multiple BWPs in PCell and PSCell but does not result in RLF and SCGFailure) can be used by the network to alleviate the RLF and SCGFailures in the future.

Observation 4: Near-Failure scenario: When LBT Failures happens on multiple BWPs in PCell on configured resources, PSCell on configured resources, and/or SCell on configured resources but does not result in RLF, SCGFailures, and/or RLC Failures, the LBT failures statistics as discussed previously can be used by the network for alleviating the RLF, SCGFailures, and RLCFailures on respective Cells.

Observation 5: In the scenario where RA failures/RLFs/SCGFailures happen due to LBT Failures, the LBT Failure statistics discussed previously will help understand the underlying cause of RLF, SCGFailure, or RLC Failures in NR-U.

Proposal 2: Introduce a new SON message as LBTFailureInformation for NR-U related enhancements.

Proposal 3: The LBTFailureInformation may contain the following fields:

- Cell information on which LBTFailure happened

- Timestamp (the time when LBT failure is detected by the MAC)

- Indicator whether the LBTFailure resulted in RACH/RLF/SCGFailure (for differentiating failure and non-failure scenarios)

- BWP used and no. of LBT failures observed (on each of the RB sets within BWPs and/or aggregated LBT Failures on each BW), if needed

- RSSI and CO measurement when LBT failure is detected by the MAC, if available

Proposal 4: LBTFailureInformation can be used together with other SON reports to further achieve optimizations in different nonfailure, near-failure, and failure scenarios.

# Conclusion

Observation 1: It is confirmed that NR-U related enhancement should be discussed in Rel-17.

Proposal 1: RAN2 is requested to confirm whether LBT Failures statistics are important for the network optimizations in the following scenarios

- Nonfailure scenarios: for selection of appropriate BWP, such as successful handover, RACH optimizations

- Near failure scenarios: for selection of appropriate BWP, when multiple BWP attempted for connection establishments but no RLF, SCGFailures, or CEF

- Failure Scenarios, such as RLF, SCGFailures, CEF

Observation 2: LBT statistics can be used by the network for the following:

- For the selection of proper LBT modes or parameters for the UEs

- For adjusting/selecting resource configuration to increase LBT successful rate for the UEs

- For the minimizations of RACH failures, RLFs, and SCG failures.

Observation 3: Nonfailure scenario: During the normal operations, the network can achieve NR-U related enhancements by using the statistics/number of LBT failures on different BWPs.

Observation 4: Near-Failure scenario: When LBT Failures happens on multiple BWPs in PCell on configured resources, PSCell on configured resources, and/or SCell on configured resources but does not result in RLF, SCGFailures, and/or RLC Failures, the LBT failures statistics as discussed previously can be used by the network for alleviating the RLF, SCGFailures, and RLCFailures on respective Cells.

Observation 5: In the scenario where RA failures/RLFs/SCGFailures happen due to LBT Failures, the LBT Failure statistics discussed previously will help understand the underlying cause of RLF, SCGFailure, or RLC Failures in NR-U.

Proposal 2: Introduce a new SON message as LBTFailureInformation for NR-U related enhancements.

Proposal 3: The LBTFailureInformation may contain the following fields:

- Cell information on which LBTFailure happened

- Timestamp (the time when LBT failure is detected by the MAC)

- Indicator whether the LBTFailure resulted in RACH/RLF/SCGFailure (for differentiating failure and non-failure scenarios)

- BWP used and no. of LBT failures observed (on each of the RB sets within BWPs and/or aggregated LBT Failures on each BW), if needed

- RSSI and CO measurement when LBT failure is detected by the MAC, if available

Proposal 4: LBTFailureInformation can be used together with other SON reports to further achieve optimizations in different nonfailure, near-failure, and failure scenarios.

# Reference

[1] RP-201281, “Revised WID on the enhancement of data collection for SON/MDT in NR and EN-DC”

[2] Post RAN2#113-e-emeeting email discussion [851]