3GPP TSG-RAN WG2 Meeting #116-e		R2-2110923

Electronic, 1st November – 12th November 2021

Source:	Qualcomm Incorporated

Title:	Logged measurement Enhancements

Agenda Item:	8.13.3.2 Logged MDT enhancements

Document for:	Discussion and Decision

# Introduction & Background

Rel-17 SON/MDT [1] includes the following RAN2-led objectives in the context of MDT.

- Support of data collection for MDT features for identified use cases, including 2-step RACH optimization and leftovers of Rel-16 SON/MDT WI (MDT enhancements and MDT for MR-DC) [RAN2, RAN3, RAN4]

- Enhancement of logged and immediate MDT (including coexistence with IDC) [RAN2, RAN3]

- Enhancement of reporting e.g. RLF and accessibility measurements, Successful Handover reporting [RAN2, RAN3].

- Specification of MDT for MR-DC [RAN2, RAN3, RAN4]

In RAN2#115-emeeting [2], we made the following agreements:

3	Signaling based logged MDT override protection is applicable in the following scenarios:

1)	Signaling based Logged MDT is configured, but no results are available e.g. so far nothing stored, or all previously stored results retrieved

2)	Signaling based Logged MDT configuration is stopped (i.e. the expiry of T330), but UE still has un-retrieved results that would be discarded upon accepting a new configuration

4	Include an indicator to indicate the signaling based logged MDT configuration availability in RRCSetupComplete / RRCConnectionSetupComplete and RRCResumeComplete / RRCConnectionResumeComplete.

FFS: Implicit (flag indicating T330 is running or not) vs explicit indication

5	UE includes an indication regarding whether the T330 timer is running or not in RRCSetupComplete / RRCConnectionSetupComplete and RRCResumeComplete / RRCConnectionResumeComplete.

It is still FFS that we need a separate explicit flag for indicating “whether signaling based MDT is configured at the UE” and “whether the T330 is running or not”.In this contribution draft, we discuss that whether two different flags are needed.

# Discussion

In this paper, we have the following discussions:

- Avoiding overwriting of signaling-based logged MDT: We had discussions in the last few meetings on methods to avoid overwriting of signaling-based logged MDT by management-based MDT. We further made the below agreements to achieve this.

3	Signaling based logged MDT override protection is applicable in the following scenarios:

1)	Signaling based Logged MDT is configured, but no results are available e.g. so far nothing stored, or all previously stored results retrieved

2)	Signaling based Logged MDT configuration is stopped (i.e. the expiry of T330), but UE still has un-retrieved results that would be discarded upon accepting a new configuration

4	Include an indicator to indicate the signaling based logged MDT configuration availability in RRCSetupComplete / RRCConnectionSetupComplete and RRCResumeComplete / RRCConnectionResumeComplete.

FFS: Implicit (flag indicating T330 is running or not) vs explicit indication

5	UE includes an indication regarding whether the T330 timer is running or not in RRCSetupComplete / RRCConnectionSetupComplete and RRCResumeComplete / RRCConnectionResumeComplete.

Based on the agreement, it is still FFS whether we need an indicator to indicate if signalling-based MDT is configured at the UE or not.

- Logging measurements of non-cellreselection frequency: Here, we discuss the methods for configuring UE so that UE can log measurements on non-cellreselection frequencies in the logged MDT report.

  - Avoiding overwriting of signaling-based logged MDT

As discussed above, in RAN2#115-emeeting [2], RAN2 agreed to introduce an explicit flag to indicate whether T330 is running at the UE. Whether a flag to indicate signaling-based MDT is configured at the UE is FFS. Furthermore, we agreed to avoid overwriting of signaling-based MDT “when signaling based MDT is configured, but no results are available e.g. so far nothing stored, or all previously stored results retrieved” and “Signaling based Logged MDT configuration is stopped (i.e. the expiry of T330), but UE still has un-retrieved results that would be discarded upon accepting a new configuration”.

Observation 1: management-based MDT can be overwritten by signaling-based MDT or new management-based MDT configuration, i.e. no override protection is needed for management-based MDT.

Proposal 1: Whether signalling-based MDT is configured at the UE can be implicitly derived based on the indication of “Whether T330 is running”.

Proposal 2: UE sends the flag “Whether T330 is running = {TRUE, FALSE}” only when signaling-based MDT is configured at the UE.

  - Further discussion on logging measurements on non-cellreselection frequencies in logged Measurement Report

In  RAN2#113-emeeting [3], RAN2 had the following agreement:

Agreement:

The network can use a flag in logged MDT configuration to indicate if an early measurement/idle mode configuration has relevance for logged measurement purposes. Upon such an indication, UE can log measurements on non-cellReselection (carrier frequencies not part of SIB4 or SIB5).  AreaConfig and/or InterFreqTargetInfo can be used for filtering of SIB4 and non-SIB4 frequencies. Whether a flag is needed should be FFS.

It is FFS whether a flag is needed to indicate if an early measurement/idle mode configuration has relevance for logged measurement purposes. We believe that idle mode measurements may not always have significance for logged measurement purposes. Therefore, based on the requirements,  a flag can be used to indicate if the early measurement configured frequencies has relevance for logged MDT. To indicate this, we have two options:

- Flag in logged MDT: Network can configure whether the early measurement frequencies have relevance for logged MDT per logged MDT configuration. This remains true for the entire duration of T330.

- Flag in early measurement configuration: Network can configure whether the early measurement frequencies have relevance for logged MDT per early measurement configuration. This provides flexibility to the network to modify the preference during ongoing logged MDT.

Observation 2: To configure whether the early measurement frequencies has relevance for logged MDT, the network can use:

- Flag in logged MDT: Network can configure whether the early measurement frequencies have relevance for logged MDT per logged MDT configuration. This remains true for the entire duration of T330.

- Flag in early measurement configuration: Network can configure whether the early measurement frequencies have relevance for logged MDT per early measurement configuration. This provides flexibility to the network to modify the preference during ongoing logged MDT.

Proposal 3: Use the flag in early measurement configuration instead of logged MDT configuration for higher flexibility.

Further details are being discussed in the post RAN2#115-emeeting email discussion 896, where the Rapporteur outlined the following options:

Option 1 (Explicit flag: EMR configuration and EMR measurement performance on top of extended Logged MDT):

- The network configures the flag to indicate if an early measurement/idle mode configuration has relevance for logged measurement purposes (regardless of extended AreaConfig and/or InterFreqTargetInfo)

- RRCRelease with MeasIdleConfig configures MeasIdleCarrierNR, and MeasIdleCarrierEUTRA (applicable for EMR configuration with (optionally) validity area (see the Annex A) takes the precedence in determining the measurements results in the Logged MDT report for EMR purpose

- The UE performs measurements results according to 5.5a.3 (legacy MDT rules) and 5.7.8.2a (Note: different measurement performance applies than legacy rules for Logged MDT)(see Annex A)

- The Logged MDT report is determined according to:

- Legacy MDT configuration, performing measurements defined in 5.5a.3 (legacy MDT rules with periodical and event-based triggers), and on top of that:

- Detailed configuration parameters of the idle measurement configuration (report quantity, quality threshold, etc) for EMR purposes, and

- Extended AreaConfig and/or InterFreqTargetInfo (if present), following legacy, Logged MDT performance measurements defined in 5.5a.3 (legacy MDT rules)

Option 2 (Explicit flag, but either EMR configuration and EMR measurement performance or extended Logged MDT):

- The network configures the flag to indicate if an early measurement/idle mode configuration has relevance for logged measurement purposes with the dependence of extended AreaConfig and/or InterFreqTargetInfo

  - If extended AreaConfig and/or InterFreqTargetInfo is not present  apply Option1

  - If extended AreaConfig and/or InterFreqTargetInfo is present  ignore the flag

The extended area scope sets the frequencies for non-cellReselection frequencies. I.e. RRCRelease with MeasIdleConfig configuring MeasIdleCarrierNR, and MeasIdleCarrierEUTRA (applicable for idle mode measurement configuration with (optionally) validity area) is not used for determining the measurements results in the Logged MDT report

- The UE performs measurements results according to:

  - Option 1

  - 5.5a.3 (legacy MDT rules) with an extended set of frequencies (report quantity, quality threshold, etc for ERM do not apply)

- The Logged MDT report is determined according to:

  - Option 1

  - Extended AreaConfig and/or InterFreqTargetInfo (if present), following legacy Logged MDT performance measurements defined in 5.5a.3 (legacy MDT rules)

NOTE: Measurement values related to ERM carriers are logged in Logged MDT report without checking qualityThreshold criterion configured in the ERM configuration.

Option 3 (implicit EMR configuration by extended Logged MDT by AreaConfig and/or InterFreqTargetInfo):

- The network does not use an explicit flag, but extended AreaConfig and/or InterFreqTargetInfo in case an early measurement/idle mode configuration frequencies has relevance for logged measurement

  - If extended AreaConfig and/or InterFreqTargetInfo is not present  Logged MDT applies (with no EMR purpose)

  - If extended AreaConfig and/or InterFreqTargetInfo is present it sets the frequencies for non-cellReselection frequencies by reusing EMR frequencies

- The UE performs measurements results according to:

  - 5.5a.3 (legacy MDT rules)

  - 5.5a.3 (legacy MDT rules) with an extended set of frequencies

- The Logged MDT report is determined according to:

  - Legacy MDT configuration,  performing measurements defined in 5.5a.3 (legacy MDT rules with periodical and event-based triggers)

  - Extended AreaConfig and/or InterFreqTargetInfo, following legacy, Logged MDT performance measurements defined in 5.5a.3 (legacy MDT rules)

NOTE: Measurement values related to ERM carriers are logged in Logged MDT report without checking qualityThreshold criterion configured in ERM configuration.

The UE behavior is not clear from option 1, we propose to modify option 1 to include the following UE behavior as described in proposal 4.

Proposal 4: modify option 1 as follows to include the UE behavior:

Option 1 (Explicit flag: EMR configuration and EMR measurement performance on top of extended Logged MDT):

- The network configures the flag to indicate if an early measurement/idle mode configuration has relevance for logged measurement purposes (regardless of extended AreaConfig and/or InterFreqTargetInfo).

  - If extended AreaConfig and/or InterFreqTargetInfo is not present  use MeasIdleConfig to log the early measurement frequencies in logged MDT

  - If extended AreaConfig and/or InterFreqTargetInfo is present  UE should log the measurements only on those EMR frequencies that are included in both MeasIdleConfig and extended AreaConfig and/or InterFreqTargetInfo.

UE should log the measurements on EMR frequencies in logged MDT report only until validityarea (in MeasIdleConfig) remains valid and until T330 is also running.

- The UE performs measurements results according to 5.5a.3 (legacy MDT rules) and 5.7.8.2a (Note: different measurement performance applies than legacy rules for Logged MDT)(see Annex A)

- The Logged MDT report is determined according to:

- Legacy MDT configuration, performing measurements defined in 5.5a.3 (legacy MDT rules with periodical and event-based triggers), and on top of that:

- Detailed configuration parameters of the idle measurement configuration (report quantity, quality threshold, etc) for EMR purposes, and

- Extended AreaConfig and/or InterFreqTargetInfo (if present), following legacy, Logged MDT performance measurements defined in 5.5a.3 (legacy MDT rules)

Proposal 5: Adopt modified option 1 as the solution mechanism for logging measurements on EMR frequencies and cells in logged MDT report.

# Conclusion

Observation 1: management-based MDT can be overwritten by signaling-based MDT or new management-based MDT configuration, i.e. no override protection is needed for management-based MDT.

Proposal 1: Whether signalling-based MDT is configured at the UE can be implicitly derived based on the indication of “Whether T330 is running”.

Proposal 2: UE sends the flag “Whether T330 is running = {TRUE, FALSE}” only when signaling-based MDT is configured at the UE.

Observation 2: To configure whether the early measurement frequencies has relevance for logged MDT, the network can use:

- Flag in logged MDT: Network can configure whether the early measurement frequencies have relevance for logged MDT per logged MDT configuration. This remains true for the entire duration of T330.

- Flag in early measurement configuration: Network can configure whether the early measurement frequencies have relevance for logged MDT per early measurement configuration. This provides flexibility to the network to modify the preference during ongoing logged MDT.

Proposal 3: Use the flag in early measurement configuration instead of logged MDT configuration for higher flexibility.

Proposal 4: modify option 1 as follows to include the UE behavior:

Option 1 (Explicit flag: EMR configuration and EMR measurement performance on top of extended Logged MDT):

- The network configures the flag to indicate if an early measurement/idle mode configuration has relevance for logged measurement purposes (regardless of extended AreaConfig and/or InterFreqTargetInfo).

  - If extended AreaConfig and/or InterFreqTargetInfo is not present  use MeasIdleConfig to log the early measurement frequencies in logged MDT

  - If extended AreaConfig and/or InterFreqTargetInfo is present  UE should log the measurements only on those EMR frequencies that are included in both MeasIdleConfig and extended AreaConfig and/or InterFreqTargetInfo.

UE should log the measurements on EMR frequencies in logged MDT report only until validityarea (in MeasIdleConfig) remains valid and until T330 is also running.

- The UE performs measurements results according to 5.5a.3 (legacy MDT rules) and 5.7.8.2a (Note: different measurement performance applies than legacy rules for Logged MDT)(see Annex A)

- The Logged MDT report is determined according to:

- Legacy MDT configuration, performing measurements defined in 5.5a.3 (legacy MDT rules with periodical and event-based triggers), and on top of that:

- Detailed configuration parameters of the idle measurement configuration (report quantity, quality threshold, etc) for EMR purposes, and

- Extended AreaConfig and/or InterFreqTargetInfo (if present), following legacy, Logged MDT performance measurements defined in 5.5a.3 (legacy MDT rules)

Proposal 5: Adopt modified option 1 as the solution mechanism for logging measurements on EMR frequencies and cells in logged MDT report.

Proposal 6: UE logs the measurements in logged MDT reports for EMR frequencies only if it is part of both MeasIdleConfig (only until validityarea remains valid if configured) and extended AreaConfig and/or InterFreqTargetInfo (if configured).

# Reference

[1] RP-201281, “Revised WID on the enhancement of data collection for SON/MDT in NR and EN-DC”

[2] RAN2#115e meeting report

[3] RAN2#113-emeeting