3GPP TSG RAN WG2 Meeting #116-e                                                        	R2-2110943

e-Meeting, November 1 – November 12, 2021

Agenda item:	8.10.3.2

Source:	Samsung

Title:	Further considerations on idle/inactive behaviours

Document for:	Discussion and Decision

# 1. Introduction

RAN2 made some progress for RRC idle/inactive behaviours in NTN at RAN2#115-e [1]. In addition, the email discussion on 38.304 running CR [2] is under the discussion. In this contribution, we would like RAN2 to discuss further considerations on top of the progress.

# 2. Discussion

At RAN2#115-e, RAN2 made the following agreements for RRC idle/inactive behaviours:

- Broadcast of cell stop time in SIB is only applicable to quasi earth fixed cell (not to moving cell). No further work in this release to address any moving cell specific details on using the cell stop time to assist measurements or cell reselection

- For quasi-earth fixed cell, the reference location of the cell (serving cell or the neighbor cells) is broadcast in system information

- For quasi-earth fixed cell, UE should start measurements on neighbour cells before the serving cell stops covering the current area.

- For quasi-earth fixed cell, the broadcast “timing information on when a cell is going to stop serving the area” refers to the time when a cell stops covering the current area.

- For quasi-earth fixed cell, specify that UE should start measurements on neighbour cells before the broadcast stop time of the serving cell, i.e. the time when the serving cell stops covering the current area, and the exact time to start measurements is up to UE implementation.

- Working assumption: Location assisted cell reselection, with the distance between UE and the reference location of the cell (serving cell and/or neighbor cell) taken into account, is supported for quasi-earth fixed cell, if UE has valid location information, which means location acquisition will not be triggered at UE side only for location assisted cell reselection. FFS on the details.

Location assisted cell reselection:

Location assisted cell reselection was set as working assumption last RAN2 meeting. Considering RSRP/RSRQ is not a good criterion to distinguish cell-center and cell-boundary, it is proposed to confirm the working assumption as an agreement. However, we do not think it is a good idea to only rely to location assisted cell reselection, e.g. the UE performs cell reselection based on the distance even if the target cell has poor RSRP/RSRQ. It is proposed location assisted cell reselection should be applied on top of RSRP/RSRQ criterion.

[Proposal 1]: RAN2 is asked to confirm location assisted cell reselection as an agreement.

[Proposal 2]: Location assisted cell reselection should be applied on top of RSRP/RSRQ criterion.

Measurement rules:

According to RAN2#115-e agreements for quasi-earth fixed cell, the UE should start to perform intra-F or inter-F neighbouring cells measurements before the stopTime of the serving cell. In TN measurement rules, the UE can skip intra-F and inter-F neighbouring cells measurements independently according to SintraSearch and SinterSearch. In NTN quasi-earth fixed cell, two cells (disappearing cell and incoming cell) serve the same geo-location area during the overlapping time. To us, it is not crystal clear whether two cells can be located in the same frequency. It may not be easy to locate two cells serving the same geo-location area at the same time into the same frequency due to heavy interference issue.

[Observation 1]: For quasi-earth fixed cell, it may not be easy to locate two cells (disappearing cell and incoming cell) serving the same geo-location area into the same frequency due to heavy interference issue.

With the observation 1, if the UE always performs all intra-F and inter-F neighbouring cells measurements according to the new measurement rules, it will bring much UE power consumption compared to TN. Furthermore, with the need of GNSS receptions in NTN, the UE power consumption can become worse.

[Observation 2]: For quasi-earth fixed cell, the UE power consumption is concerned if the UE always perform all intra-F and inter-F neighbouring cells measurements according to the new measurement rules.

For quasi-earth fixed cell, the network can know which and when neighbouring cell is incoming. If the serving cell is good enough, the UE only needs to perform measurement on the incoming cell.

[Observation 3]: For quasi-earth fixed cell, the UE only needs to perform measurement on the incoming cell if the serving cell is good enough.

To achieve observation 3, it is proposed that system information includes the new incoming cell with the timing information. Note it is included with FFS in the current 38.304 running CR.

[Proposal 3]: For quasi-earth fixed cell, system information includes the new incoming cell with the timing information. The UE only performs measurement on the incoming cell if the serving cell is good enough.

Once location assisted cell reselection is confirmed as an agreement (see proposal 1 and 2), we think in addition to the RSRP/RSRQ criterion, the location criterion should be also considered in the measurement rules. To us, it sounds natural to consider it since it is applied as one of criteria in cell reselection decision. The UE would decide whether the serving cell is good enough (so the UE can skip neighbouring cell measurements) based on whether (Srxlev > threshold#1, Squal > threshold#2, AND the distance between the UE and the serving cell’s reference < threshold#3) is met if location assisted cell reselection is used. Note legacy criterion (Srxlev > threshold#1 AND Squal > threshold#2) can be used if location assisted cell reselection is not used.

[Proposal 4a]: If location assisted cell reselection is used, the measurement rule is determined based on whether the condition (Srxlev > threshold#1, Squal > threshold#2, AND distance between the UE and serving cell’s reference < threshold#3) is met.

[Proposal 4b]: If location assisted cell reselection is not used, the measurement rule is determined based on whether the condition (Srxlev > threshold#1 AND Squal > threshold#2) is met.

Intra-F or inter-F (with the same cell reselection priority) cell reselection:

Once location assisted cell reselection is confirmed as an agreement (see proposal 1 and 2), we think in addition to the RSRP/RSRQ criterion, the location criterion should be also considered in the ranking procedure. Then it brings the question how to consider two criteria in the ranking procedure. Possible options would be:

- Option1: first ranking based on RSRP/RSRQ criterion then second ranking based on location criterion among the cells ranked higher than the serving cell in the first ranking.

- Option2: first ranking based on location criterion then second ranking based on RSRP/RSRQ criterion among the cells ranked higher than the serving cell in the first ranking.

- Option3: dependent on network configuration, either RSRP/RSRQ or location criterion based ranking.

To our view, legacy RSRP/RSRQ criterion is still baseline criterion, so at least a target cell should be compared to the serving cell in RSRP/RSRQ criterion point of view (Option1).

[Proposal 5]: If location assisted cell reselection is used, RAN2 is asked to discuss options for ranking procedure above.

- Option1: first ranking based on RSRP/RSRQ criterion then second ranking based on location criterion among the cells ranked higher than the serving cell in the first ranking.

- Option2: first ranking based on location criterion then second ranking based on RSRP/RSRQ criterion among the cells ranked higher than the serving cell in the first ranking.

- Option3: dependent on network configuration, either RSRP/RSRQ or location criterion based ranking.

Inter-F (with the different cell reselection priority) / inter-RAT cell reselection:

Once location assisted cell reselection is confirmed as an agreement (see proposal 1 and 2), we think in addition to the RSRP/RSRQ criterion, the location criterion should be also considered in inter-F (with the different cell reselection priority) / inter-RAT cell reselection. In TN, basic inter-F/inter-RAT cell reselection is done with the following principles:

- Inter-F cell reselection to higher cell reselection priority: if a cell’s Srxlev/Squal > threshold#4

- Inter-F cell reselection to lower cell reselection priority: if (the serving cell’s Srxlev/Squal < threshold#5) AND (a cell’s Srxlev/Squal > threshold#6)

Considering the legacy inter-F cell reselection, the possible options would be:

- Option1: Both RSRP/RSRQ AND location criteria are considered. For example:

  - Inter-F cell reselection to higher cell reselection priority: if (a cell’s Srxlev/Squal > threshold#4) AND (distance between the UE and reference < threshold#7)

  - Inter-F cell reselection to lower cell reselection priority: if (the serving cell’s Srxlev/Squal < threshold#5 AND a cell’s Srxlev/Squal > threshold#6) AND (the distance between the UE and the serving cell’s reference > threshold#8 AND the distance between the UE and the cell’s reference < threshold#9)

- Option2: Dependent on network configuration, either RSRP/RSRQ or location criterion is considered.

To our view, legacy RSRP/RSRQ criterion is still baseline criterion, so both criteria should be considered (Option1).

[Proposal 6]: If location assisted cell reselection is used, RAN2 is asked to discuss options for inter-F cell reselection (with the different cell reselection priority) / inter-RAT cell reselection.

- Option1: Both RSRP/RSRQ AND location criteria are considered. For example:

- Option2: Dependent on network configuration, either RSRP/RSRQ or location criterion is considered.

# 3. Conclusion

In this contribution, we have seen further considerations on RRC ide/inactive UE behaviours and asked RAN2 to agree with the following proposals.

[Proposal 1]: RAN2 is asked to confirm location assisted cell reselection as an agreement.

[Proposal 2]: Location assisted cell reselection should be applied on top of RSRP/RSRQ criterion.

[Proposal 3]: For quasi-earth fixed cell, system information includes the new incoming cell with the timing information. The UE only performs measurement on the incoming cell if the serving cell is good enough.

[Proposal 4a]: If location assisted cell reselection is used, the measurement rule is determined based on whether the condition (Srxlev > threshold#1, Squal > threshold#2, AND distance between the UE and serving cell’s reference < threshold#3) is met.

[Proposal 4b]: If location assisted cell reselection is not used, the measurement rule is determined based on whether the condition (Srxlev > threshold#1 AND Squal > threshold#2) is met.

[Proposal 5]: If location assisted cell reselection is used, RAN2 is asked to discuss options for ranking procedure above.

- Option1: first ranking based on RSRP/RSRQ criterion then second ranking based on location criterion among the cells ranked higher than the serving cell in the first ranking.

- Option2: first ranking based on location criterion then second ranking based on RSRP/RSRQ criterion among the cells ranked higher than the serving cell in the first ranking.

- Option3: dependent on network configuration, either RSRP/RSRQ or location criterion based ranking.

[Proposal 6]: If location assisted cell reselection is used, RAN2 is asked to discuss options for inter-F cell reselection (with the different cell reselection priority) / inter-RAT cell reselection.

- Option1: Both RSRP/RSRQ AND location criteria are considered. For example:

- Option2: Dependent on network configuration, either RSRP/RSRQ or location criterion is considered.

# 4. Reference

[1] R2-2108832	Report from break-out session on R17 NTN, REDCAP and CE	ZTE

[2] R2-210xxxx	Stage-3 running 304 CR for NTN	ZTE