3GPP TSG-RAN WG2 #116-e	R2-2110944

1st – 12th November 2021

Online

Source:                    	DENSO CORPORATION

Title:  	RAN2 protocol impacts on preconfigured Measurement Gap

Document for:        	Discussion and decision

Agenda Item:         	8.22	-	NR and MR-DC measurement gap enhancements

- Introduction

RAN2 has received an LS from RAN4 on NR/MR-DC measurement gap enhancements [1]. This paper attempts to investigate the RAN2 protocol impacts based on RAN4 input, in accordance with the WI objective below [2].

- Procedures and signaling for pre-configured MG pattern(s) [RAN2] - Specification of protocol impacts of the mechanisms of activation/deactivation of MG following a DCI or timer based BWP switch, e.g., per BWP MG configuration based on RAN4 input

- Discussion

  - Pre-configuration of measurement gap

With regards to how a measurement gap can be preconfigured, the followings are relevant agreements made by RAN4 so far.

- Pre-configured MG(s) are configured per UE or per FR and cannot be changed after BWP switching. - All existing MG patterns #0~25 in Rel-16 are applicable for the pre-configured MG. - The common configuration parameters of pre-configured MG (e.g. MGRP, MGL, etc) are the same as those of Rel-16 legacy MG. The pre-configured MG can be configured in way similar to the configuration of the Rel-16 legacy MGs. - The RRC parameter(s) used to differentiate pre-configured MG with the legacy MG are needed when pre-configured MG is being configured. - The exact signalling can be designed by RAN2. - NW can control activation/deactivation of pre-configured MG. - RRC-based activation/deactivation method is supported. - Network can indicate active/deactive status per BWP. - Details of signalling are FFS. - Status of pre-configured MG is not fixed after RRC configuration. - FFS: how UE determines the status (active/deactive).

In light of the RAN4 agreement 2) and 3), it is reasonable to infer that a Measurement Gap (MG) is preconfigured via RRC. The existing parameters (e.g. gapOffset, mgl, mgrp, etc.) as shown below can be used to provide a UE with the pre-configuration of MG.

GapConfig ::=                   SEQUENCE {

gapOffset                     INTEGER (0..159),

mgl                           ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6},

mgrp                          ENUMERATED {ms20, ms40, ms80, ms160},

mgta                          ENUMERATED {ms0, ms0dot25, ms0dot5},

...,

[[

refServCellIndicator          ENUMERATED {pCell, pSCell, mcg-FR2} OPTIONAL   -- Cond NEDCorNRDC

]],

[[

refFR2ServCellAsyncCA-r16     ServCellIndex                  OPTIONAL,   -- Cond AsyncCA

mgl-r16                       ENUMERATED {ms10, ms20}        OPTIONAL    -- Cond PRS

]]

}

Likewise, per UE gap or per FR gap can be configured by leveraging the existing signalling as shown below, which is related to the RAN4 agreement 1).

MeasGapConfig ::=              SEQUENCE {

gapFR2                       SetupRelease { GapConfig }    OPTIONAL,   -- Need M

...,

[[

gapFR1                        SetupRelease { GapConfig }   OPTIONAL,   -- Need M

gapUE                         SetupRelease { GapConfig }   OPTIONAL    -- Need M

]]

}

Before plunging into the details of signalling impacts, a baseline needs to be developed such that the pre-configured MG is signalled to the UE via the RRCReconfiguration message and the RRCResume message in which the measurement configuration (i.e. measConfig) can be present. The existing measurement configuration can be extended to support the pre-configured MG. The following is proposed.

Proposal 1:	The preconfigured MG can be provided for the UE as a part of the measurement configuration 			via the RRCReconfiguration message and the RRCResume message.

According to the RAN4 agreement 4), one of the open issues to be discussed by RAN2 is how the preconfigured MG can be distinguished from the legacy MG. Since the configuration of the existing MG is present as a part of the measurement configuration (i.e. MeasConfig), one intuitive approach is to add another measurement configuration specific to the pre-configuration, as shown below (Option 1). In addition, the activation or deactivation of the pre-configured MG can be indicated, which can cover the RAN4 agreement 5) of supporting the RRC-based activation/deactivation. The active/deactive status of the pre-configured MG is applied to the active DL BWP, when it is signalled to the UE, in accordance with the RAN4 agreement 5).

MeasConfig ::=                      SEQUENCE {

measObjectToRemoveList              MeasObjectToRemoveList    OPTIONAL,   -- Need N

measObjectToAddModList              MeasObjectToAddModList    OPTIONAL,   -- Need N

reportConfigToRemoveList            ReportConfigToRemoveList  OPTIONAL,   -- Need N

reportConfigToAddModList            ReportConfigToAddModList  OPTIONAL,   -- Need N

measIdToRemoveList                  MeasIdToRemoveList        OPTIONAL,   -- Need N

measIdToAddModList                  MeasIdToAddModList        OPTIONAL,   -- Need N

s-MeasureConfig                     CHOICE {

ssb-RSRP                            RSRP-Range,

csi-RSRP                            RSRP-Range

}                                                             OPTIONAL,   -- Need M

quantityConfig                      QuantityConfig            OPTIONAL,   -- Need M

measGapConfig                       MeasGapConfig             OPTIONAL,   -- Need M

measGapSharingConfig                MeasGapSharingConfig      OPTIONAL,   -- Need M

...,

[[

interFrequencyConfig-NoGap-r16      ENUMERATED {true}         OPTIONAL    -- Need R

]],

[[

preconfigredMG-r17					MeasGapConfig				OPTIONAL,   -- Need M

preconfiguredGapState-r17	ENUMERATED {activated, deactivated}	OPTIONAL	-- Cond PreConfigMG

]]

}

| Conditional Presence | Explanation |
| --- | --- |
| PreConfigMG | This field is optionally present, Need N, if preconfiguredGap is set to TRUE. Otherwise, it is absent. |

Figure 2.1-1:	Example of ASN.1 impact on Option 1

By doing this, the existing MG and the pre-configured MG can be configured independently. Albeit this approach allows to distinguish between the pre-configured MG and the existing MG, it also allows to configure both simultaneously from signalling point of view. Except for the concurrent MG case [4], it is assumed that the UE is configured with either the pre-configured MG or the existing MG. If so, the network has to ensure such a configuration or the UE behaviour needs to be defined that one of them is discarded when both the pre-configured MG and the existing MG are configured by the network. Hence, this approach is somewhat recalcitrant, albeit the simple extension from signalling viewpoints.

The other approach is to indicate the pre-configuration functionality within the existing measurement gap configuration, as illustrated below (Option 2). The existing GapConfig IE can be extended so as to indicate whether it is the preconfigured gap or the existing gap. This signalling structure allows the UE to be configured with either the pre-configured gap or the existing gap. Therefore, simultaneous configuration as occurred by Option 1 can be avoided. Similar to Option 1, the activation or deactivation of the pre-configured MG can be indicated when the pre-configuration indication is set to TRUE. If the pre-configuration indication is set to FALSE, the measurement gap works as in the legacy (i.e. the gap is always activated).

MeasConfig ::=                      SEQUENCE {

<< skip unrelevant part >>

measGapConfig                       MeasGapConfig             OPTIONAL,   -- Need M

measGapSharingConfig                MeasGapSharingConfig      OPTIONAL,   -- Need M

...,

[[

interFrequencyConfig-NoGap-r16      ENUMERATED {true}         OPTIONAL    -- Need R

]]

}

MeasGapConfig ::=              SEQUENCE {

gapFR2                       SetupRelease { GapConfig }    OPTIONAL,   -- Need M

...,

[[

gapFR1                        SetupRelease { GapConfig }   OPTIONAL,   -- Need M

gapUE                         SetupRelease { GapConfig }   OPTIONAL    -- Need M

]]

}

GapConfig ::=                   SEQUENCE {

gapOffset                     INTEGER (0..159),

mgl                           ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6},

mgrp                          ENUMERATED {ms20, ms40, ms80, ms160},

mgta                          ENUMERATED {ms0, ms0dot25, ms0dot5},

...,

[[

refServCellIndicator          ENUMERATED {pCell, pSCell, mcg-FR2} OPTIONAL   -- Cond NEDCorNRDC

]],

[[

refFR2ServCellAsyncCA-r16     ServCellIndex                  OPTIONAL,   -- Cond AsyncCA

mgl-r16                       ENUMERATED {ms10, ms20}        OPTIONAL    -- Cond PRS

]],

[[

preconfiguredGap-r17		BOOLEAN						 		OPTIONAL,	-- Need M

preconfiguredGapState-r17	ENUMERATED {activated, deactivated}	OPTIONAL	-- Cond PreConfigMG

]]

}

| Conditional Presence | Explanation |
| --- | --- |
| PreConfigMG | This field is optionally present, Need N, if preconfiguredGap is set to TRUE. Otherwise, it is absent. |

Figure 2.1-2:	Example of ASN.1 impact on Option 2

From the above analysis, Option 2 is superior to Option 1, in particular to avoid the simultaneous presence of the pre-configured MG and the existing MG from the signalling structure viewpoint. Therefore, the followings are proposed.

Proposal 2:	The pre-configuration functionality is indicated within the existing GapConfig (to distinguish 				the pre-configure MG from the existing MG).

Proposal 3:	The activation or deactivation of the pre-configured MG can be indicated if the pre-						configuration of MG is indicated in GapConfig.

With regards to the following RAN4 agreement,

• Status of pre-configured MG is not fixed after RRC configuration o FFS: how UE determines the status (active/deactive)

If the active/deactive status of the pre-configured MG is indicated via RRC as in Proposal 3, the network can indicate either the active or deactive status when the pre-configured MG is configured. Although it is FFS in RAN4, it could be discussed and decided by RAN2, since it is relatd to the signalling design. The following is proposed.

Proposal 4:	The network should be able to indicate the active or deactive status when the pre-configured 				MG is configured to the UE via RRC.

The following open issues in the RAN4 LS [1] is to be discussed later, once RAN4 progress and conclude, albeit it is likely to impact on the RAN2 specification.

-	The pre-configured MG activation/deactivation is triggered by the DCI/Timer based BWP switch.

-	FFS if additional conditions for pre-configured MG activation/deactivation shall be considered.

-	Additional explicit rules for pre-configured MG autonomous activation/deactivation shall be defined for the case when signalling is not provided

  - Interaction with measurement gap sharing

When the UE is configured with the pre-configured MG, another potential issue is how to handle the measurement gap sharing, if configured, albeit it is not described in the RAN4 LS [1]. The gap sharing could be valid and applied upon activating the pre-configured MG. If the pre-configured MG is deactivated, the gap sharing could be deactivated, as well. Interaction with the gap sharing mechanism needs to be clarified. Hence, the following is proposed.

Proposal 5:	It has to be clarified how the activation/deactivation of the pre-configured MG interacts with 			the measurement gap sharing.

- Summary and proposal

This paper investigates the RAN2 protocol impacts on NR/MR-DC measurement gap enhancements. In summary, the followings were proposed.

Proposal 1:	The preconfigured MG can be provided for the UE as a part of the measurement configuration 			via the RRCReconfiguration message and the RRCResume message.

Proposal 2:	The pre-configuration functionality is indicated within the existing GapConfig (to distinguish 				the pre-configure MG from the existing MG).

Proposal 3:	The activation or deactivation of the pre-configured MG can be indicated if the pre-						configuration of MG is indicated in GapConfig.

Proposal 4:	The network should be able to indicate the active or deactive status when the pre-configured 				MG is configured to the UE via RRC.

Proposal 5:	It has to be clarified how the activation/deactivation of the pre-configured MG interacts with 			the measurement gap sharing.

- References

[1] R2-2109367, “LS on R17 NR MG enhancements – Pre-configured MG,” RAN4

[2] RP-211591, “Revised WID on NR and MR-DC measurement gap enhancements,” Intel Corporation, MediaTek Inc.

[3] R4-2115398, “Email discussion summary for [100-e][224] NR_MG_Part_2,” Moderator (Intel Corporation)

[4] R2-2109361, “LS on R17 NR MG enhancements – Concurrent MG,” RAN4