3GPP TSG-RAN WG2 Meeting #116-e	R2-2110978

Online, 1 - 12 Nov, 2021

Source: 	Huawei, HiSilicon

Title: 	Left issues on supporting External Credentials and UE onboarding

Agenda Item:	8.16.2

Document for:	Discussion and decision

Introduction

At RAN2#115-e meeting, the RAN2 impact for supporting external credentials and UE onboarding was discussed and the following agreements were achieved [1]:

Wait for SA2 reply LS on the issue whether a common list of GINs used for onboarding and SNPN access using external credentials.

RAN2 has not identified a need for modification of / addition to broadcast of HRNNs.

RAN2 confirms that there is no impact on connected mode mobility when accessing an SNPN through CHs (was already assumed).

maximum number of GINs is specified per cell

new SIB specified to broadcast GINs acc to Option B: Single list of GINs with explicit assignment to SNPNs. Details on the explicit assignment are FFS.

RAN2 didn’t identify a need for modification to access control for SNPN access using external credential (could be discussed in other groups)

RAN2 didn’t identify a need for modification to access control for SNPN access for onboarding (could be discussed in other groups)

Some issues remained open, e.g. the addition of new indicators to SIB1 and the impacts to cell selection. In this paper, we further analyse these remaining issues.

Discussion

- 2.1 Addition of new indicators

At RAN2#115-e, how the three new indicators for external credentials and onboarding (i.e., "access using credentials from a CH is supported ", "registration from UEs that are not explicitly configured to select the SNPN is allowed", "onboarding is supported ") are added to SIB1 was discussed, and the following options were proposed:

- Option A: introduce the new indicators in parallel with npn-IdentityInfoList in CellAccessRelatedInfo

- Option B: introduce the new indicators in npn-IdentityInfo

- Option C: introduce a new list for the new indicators in npn-IdentityInfo

An alternative is to include the new indicators in the nonCriticalExtension field in SIB1 (Option D), as follows:

SIB1 message

-- ASN1START

-- TAG-SIB1-START

SIB1 ::=        SEQUENCE {

cellSelectionInfo                   SEQUENCE {

q-RxLevMin                          Q-RxLevMin,

q-RxLevMinOffset                    INTEGER (1..8)   OPTIONAL,   -- Need S

q-RxLevMinSUL                       Q-RxLevMin       OPTIONAL,   -- Need R

q-QualMin                           Q-QualMin        OPTIONAL,   -- Need S

q-QualMinOffset                     INTEGER (1..8)   OPTIONAL    -- Need S

}                                                                                                   OPTIONAL,   -- Cond Standalone

cellAccessRelatedInfo               CellAccessRelatedInfo,

connEstFailureControl               ConnEstFailureControl  			OPTIONAL,   -- Need R

si-SchedulingInfo                   SI-SchedulingInfo      			OPTIONAL,   -- Need R

servingCellConfigCommon             ServingCellConfigCommonSIB      OPTIONAL,   -- Need R

ims-EmergencySupport                ENUMERATED {true}     			OPTIONAL,   -- Need R

eCallOverIMS-Support                ENUMERATED {true}        		OPTIONAL,   -- Need R

ue-TimersAndConstants               UE-TimersAndConstants    		OPTIONAL,   -- Need R

uac-BarringInfo                     SEQUENCE {

uac-BarringForCommon                UAC-BarringPerCatList  		OPTIONAL,   -- Need S

uac-BarringPerPLMN-List             UAC-BarringPerPLMN-List    	OPTIONAL,   -- Need S

uac-BarringInfoSetList              UAC-BarringInfoSetList,

uac-AccessCategory1-SelectionAssistanceInfo CHOICE {

plmnCommon                           UAC-AccessCategory1-SelectionAssistanceInfo,

individualPLMNList                   SEQUENCE (SIZE (2..maxPLMN)) OF UAC-AccessCategory1-SelectionAssistanceInfo

}    	                       						OPTIONAL    -- Need S

}      	             									OPTIONAL,   -- Need R

useFullResumeID                     ENUMERATED {true}  	OPTIONAL,   -- Need R

lateNonCriticalExtension            OCTET STRING   		OPTIONAL,

nonCriticalExtension                SIB1-v1610-IEs  	OPTIONAL

}

SIB1-v1610-IEs ::=               SEQUENCE {

idleModeMeasurementsEUTRA-r16    ENUMERATED{true}      	OPTIONAL,  -- Need R

idleModeMeasurementsNR-r16       ENUMERATED{true}       OPTIONAL,  -- Need R

posSI-SchedulingInfo-r16         PosSI-SchedulingInfo-r16

nonCriticalExtension             SIB1-v1630-IEs    		OPTIONAL

}

SIB1-v1630-IEs ::=               SEQUENCE {

uac-BarringInfo-v1630            SEQUENCE {

uac-AC1-SelectAssistInfo-r16   SEQUENCE (SIZE (2..maxPLMN)) OF UAC-AC1-SelectAssistInfo-r16

}     											OPTIONAL,  -- Need R

nonCriticalExtension             SIB1-v17xy-IEs    OPTIONAL

}

SIB1-v17xy-IEs ::=               SEQUENCE {

infoForSNPNList-r17 ::=          SEQUENCE (SIZE (1..maxNPN-r16)) OF InfoForSNPN-r17 OPTIONAL,

nonCriticalExtension             SEQUENCE {}    									OPTIONAL

}

UAC-AccessCategory1-SelectionAssistanceInfo ::=    ENUMERATED {a, b, c}

UAC-AC1-SelectAssistInfo-r16 ::=     ENUMERATED {a, b, c, notConfigured}

InfoForSNPN-r17 ::=         	SEQUENCE {

exCredential-Support-r17	ENUMERATED {true}         			OPTIONAL,       	-- Need R

exCredential-OppRegAllowed-r17 	ENUMERATED {true}          			OPTIONAL,       	-- Need R

onboarding-Support-r17 	 		ENUMERATED {true}          			OPTIONAL,       	-- Need R

}

-- TAG-SIB1-STOP

-- ASN1STOP

Some legacy LTE UEs implement the extension marker in SIB1 incorrectly, and the issue was discussed in RAN2 #111-e meeting and resolved in [2]. To avoid similar issues for NR, add the new indicators in the nonCriticalExtension is a more robust solution. Compared with Option A/B/C, Option D is more efficient (a couple of bits can be saved by not using [[]]), but the disadvantage is that Option D is less readable.

If companies confirm that no implementation issue (in terms of decoding the extension marker in SIB1) is foreseen for NR, Option B is better than Option A/C in our understanding. The merit of Option B is that SNPNs with the same configuration can be put into the same npn-IdentityInfo IE, therefore signalling overhead can be reduced.

If no implementation issue (in terms of decoding the extension marker in SIB1) is foreseen for NR, include the new indicators in npn-IdentityInfo, otherwise the new indicators are added in the nonCriticalExtension field in SIB1.

- 2.2 Cell selection

RAN2 has not reached consensus on whether to specify that the broadcast onboarding support indicator is considered during initial cell selection after network selection or leave it to UE implementation. Note that it was agreed that the 1-bit onboarding support indicator is set per cell and can be toggled for access control. If this (i.e., the onboarding support indicator is taken into account for cell suitability check) is not specified and left to UE implementation, the UE may select the cell regardless of this indicator, which conflicts with the intention of toggling the onboarding indicator for access control. Therefore, it is beneficial to modify the definition of suitable cell for onboarding to avoid the undesirable access attempt.

The broadcast onboarding support indicator is taken into account for cell suitability check.

Conclusion

In this paper, we discussed the remaining issues on the addition of new indicators to SIB1 and the impacts to cell selection and proposed the following:

- If no implementation issue (in terms of decoding the extension marker in SIB1) is foreseen for NR, include the new indicators in npn-IdentityInfo, otherwise the new indicators are added in the nonCriticalExtension field in SIB1.

The broadcast onboarding support indicator is taken into account for cell suitability check.

References

- R2-115e Chair Notes EOM rev1.docx

- RP-201939, Technically endorsed RAN2 CRs on LTE SIB extension issue