3GPP TSG-RAN WG2 Meeting #116-e                         R2-2110990

Electronic meeting, 1 Nov – 12 Nov 2021

Source: 			ZTE Corporation, Sanechips

Title: 	Discussion on buffer for NR QoE start and stop

Agenda item:		8.14.2.2

Document for: 	Discussion and Decision

- Introduction

The agreement companies made in RAN2#114e [1] on the NR QoE start and stop section is shown below:

At reception of QoE release, the UE shall discard any unsent QoE reports corresponding to the released QoE configuration.

FFS whether pause resume will affect all configurations or whether pause resume can act selectively per configuration.

On whether to store reports in the AS or the application layer at Pause, Send LS to SA4/SA5/SA3 to inform them about the options and their pros/cons (if possible) and ask them for feedback. RAN2 will continue work on this topic based on the feedback received.

The intention of this contribution is to share our views on the pause/resume QoE reporting based on the agreements and received LS if companies still want to keep developing this function in Rel-17.

- Discussion on buffer handling

A Query LS(R2-2106775) on buffer for paused QoE has been sent after RAN2#114e. RAN2 asks SA3/4/5 to provide their feedback about the options shown below:

- App layer handles the buffer.

- AS layer handles the buffer.

- Drop all reports during paused period.

- Before RAN2#116e, both SA4 and SA5 send the response LS to RAN2. The following content is shown in SA5’s LS(S5-214519):

- ----------------------Content of S5-214519 ------------------------

- SA5 think that QoE reports are useful for the operators and therefore where possible the QoE reports shouldn’t be discarded during a pause, which eliminates option 3. From SA5 perspective either of option 1 and Option 2 are equivalent. Therefore we leave the choice to RAN groups and SA4  to decide while noting some benefits of using option 1  (e.g. larger memory in the application layer).

- In general, SA5 will wait for further RAN2 and SA4 progress and then see if some updates to SA5 specifications are needed or not.

- ----------------------Content of S5-214519 ------------------------

- Based on our understanding, SA5 rejects option3 and believes that option1(app layer buffer) is slightly better than option2(AS layer buffer) in the memory size consideration. RAN groups and SA4 can make the final decision on either option1 or option2.

- Observation 1: It is obvious that app layer can handle larger size buffer than AS layer.

- Besides, SA4 also sends LS(S4-211290) to RAN2 on the QoE buffer issue and shows its concern that the QoE related application may no longer be run due to the poor service quality. And SA4 is able to address this in the future release.

- Observation 2: The application which is relevant to QoE measurement and reporting may not keep running during QoE paused period because of the poor network service.

- In addition, SA4 also sends an LS(S4-211234) to SA5 and CC RAN2.The content is shown below:

- ----------------------Content of S5-214519 ------------------------

TS 28.404 and TS 28.405 seem to roughly represent stage 1/2 and stage 3 documents, respectively, on QoE measurement collection. We are unclear on the apparent misalignment between the requirement in TS 28.404, clause 5.4.6 on UE support for temporary stop and start, and the corresponding specification of UE behavior in TS 28.405, clause 4.2.4. Specifically, TS 28.404 indicates that the UE must continue ongoing recording and storage of QoE data measurements during the entirety of the temporary stop. On the other hand, TS 28.405 indicates that the UE should halt further recording upon the application layer measurements (e.g., QoE measurements) having filled the report container.

- ----------------------Content of S5-214519 ------------------------

- In above green highlighted part, it is clear that different UE behaviors have been explained in the latest TS28.404 and TS28.405. Based on description in TS28.405, UE should halt further recording upon the application layer measurements (e.g., QoE measurements) having filled the report container. SA4 asks SA5 to further clarify the conflict between these two specifications.

- Based on our understanding, it is not appropriate to discuss the QoE buffer issue before SA5 further explain the conflict which is found by SA4.

- Proposal 1: RAN2 shall postpone the discussion on QoE buffer until SA5 clarify the conflict between TS28.404 and TS28.405.

- One procedure for signalling and management based QoE

- In RAN#91e, companies agreed to achieve the pause/resume NR QoE reporting due to RAN overload in Rel-17. And the pause/resume can be implemented to all QoE reporting or reporting per QoE configurations in a UE. Detail procedures should be discussed in RAN2. It is clear that both signalling based QoE and management based QoE are going to be supported in NR. From the content of QoE measurement report perspective, there is no different between signalling based QoE and management based QoE. Hence, both signalling based QoE and management based QoE should support the pause/resume QoE reporting function and share the same procedure.

- Proposal 2: RAN2 shall consider to implement one procedure to pause/resume QoE reporting for both signalling based and management based QoE reporting.

- Partial pause/resume QoE reporting

- In RAN2#114e, companies have different views on whether NG-RAN side can pause partial QoE reporting. Based on our understanding, if the NG-RAN can pause/resume partial QoE reporting, the same mechanism/procedure can be used to pause/resume all QoE reporting in a UE. In other words, pause/resume all QoE reporting is a critical situation of pause/resume partial QoE reporting.

- Observation 3: Pause/resume all QoE reporting is a critical situation of pause/resume partial QoE reporting.

- Table1. Temporary stop and restart of QoE information reporting during RAN overload

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | At RAN overload RAN may stop or delay the QoE information reporting from the UEs that has started it. |  |
| Actors and roles | The RAN node which is the requester of delaying the QoE information reporting. |  |
| Telecom resources | The RAN node and the UE. |  |
| Assumptions | - |  |
| Pre-conditions | Selected UEs have started QoE information collection. |  |
| Begins when | The RAN node detects that it is overloaded. |  |

- As the content in TS 28.404 shown above, the reason why we need the pause/resume QoE reporting function is to relief the RAN overload situation if needed. It is clear that the degree of RAN overload may be various in different cases. Hence, pause/resume all QoE reporting may not be an appropriate selection in all cases. From our point of view, the capability of partial pause/resume QoE reporting can let NG-RAN node have more flexible process to handle different RAN overload situations.

- For example, NG-RAN node can pause all QoE reporting when a severe RAN overload occurs. In addition, if NG-RAN node foresees the RAN overload will happen, it can partially pause some QoE reporting to avoid the potential RAN overload. Or if the RAN overload is not heavy, the RAN node can also pause the management based QoE reporting to treat the overload condition.

- From the procedure prospective, RAN2 does not need to introduce a complex procedure to support pause/resume partial QoE reporting function. Hence, It is proposed that RAN2 shall support pause/resume partial QoE reporting. Details can be FFS.

- Proposal 3: It is proposed that RAN2 shall support pause/resume partial QoE reporting. Details can be FFS.

- Conclusion and proposals

Based on the discussions above, we have the following proposals.

- Observation 1: It is obvious that app layer can handle larger size buffer than AS layer.

- Observation 2: The application which is relevant to QoE measurement and reporting may not keep running during QoE paused period because of the poor network service.

- Proposal 1: RAN2 shall postpone the discussion on QoE buffer until SA5 clarify the conflict between TS28.404 and TS28.405.

- Proposal 2: RAN2 shall consider to implement one procedure to pause/resume QoE reporting for both signalling based and management based QoE reporting.

- Observation 3: Pause/resume all QoE reporting is a critical situation of pause/resume partial QoE reporting.

- Proposal 3: It is proposed that RAN2 shall support pause/resume partial QoE reporting. Details can be FFS.

- References

- Chairman note for RAN2-115-e.

- R2-2106775, LS on QoE report handling at QoE pause.

- S5-214519, Reply LS on QoE report handling at QoE pause.

- S4-211290, LS Reply on QoE report handling at QoE pause.

- S4-211234, LS on TS 28.404/TS 28.405 Clarification.