3GPP TSG-RAN WG2 Meeting #116 electronic	                                          R2-2111054
Online, November 1 – November 12, 2021

Agenda Item:	8.1.3.3

Source: 	CMCC

Title:  	Support of MBS in CA

WID/SID:	         NR_MBS

Document for:	Discussion and Decision

Introduction

In R17 NR MBS, the discussion focuses on the single cell scenario, without touching CA scenario. In this contribution, we aim to provide some analysis on the standard impact from supporting of MBS in CA.

Discussion

Per current running CR TS 38.331 and Parameter List table from RAN1, the MBS configuration, especially for multicast service, are configured in BWP, CFR or serving cell level per UE.

Table 1 Part Parameter for Multicast List table

| Content | Granularity |  |
| --- | --- | --- |
| UE specific common frequency resource configuration for multicast for one dedicated BWP. | Per BWP | UE-Specific |
| G-RNTI is used to scramble the group-common PDCCH and group-common PDSCH. | per serving cell | UE-Specific |
| UE specific group-common PDSCH configuration for multicast for one CFR. | Per CFR, in CFR-Config-Multicast | UE-Specific |
| common search space configured for multicast | Per CFR, in searchSpacesToAddModList-Multicast | UE-Specific |

Therefore, from signaling design perspective, there is no impact on current MBS configuration. This means there is no obvious standard impact on MBS configuration signaling in TS38.331 ASN.1.

Observation 1: to support of MBS in CA, there is no obvious standard impact on MBS configuration signalling in TS38.331 ASN.1.

Proposal 1: Current MBS configuration signalling in TS38.331 ASN.1 can be reused to support of MBS in CA.

However, different UEs may have different capabilities on the simultaneous MBS reception in different carriers, depending on the specific RF and baseband processing capabilities of each UE.

Correspondingly, gNB needs have knowledge of the UE capability of supporting MBS in CA or not to employ appropriate scheduling strategy:

- If a CA UE cannot receive the MBS service in PTM mode from multiple carrier components simultaneously, gNB can switch the MBS delivery mode from PTM to PTP in either one carrier, or just transmit the MBS service via one carrier;

- If a CA UE can receive the MBS service in PTM mode from multiple carrier components simultaneously, gNB can directly schedule MBS service in PTM mode via PRB in multiple carrier components, according to the UE supported band combination for the MBS PTM+MBS PTM simultaneous transmission.

Observation 2: gNB needs have knowledge of the UE capability of supporting MBS in CA or not to employ appropriate scheduling strategy.

Proposal 2: UE capability of simultaneous reception of MBS PTM+MBS PTM simultaneous transmission with supported band combination should be introduced.

Another issue is whether a unified G-RNTI or separate G-RNTI allocated from different carrier components. And now in RAN1 the G-RNTI is defined per serving cell, while in RRC running CR, it is configured per serving cell group. Moreover, in unicast, when CA is configured, the same C-RNTI applies to all serving cells. Although, different TMGI may be mapping to different G-RNTI, we cannot see the necessity of allocating different G-RNTI to different serving cell or serving cell group.

- G-RNTI is used to scramble the group-common PDCCH and group-common PDSCH per serving cell.

MAC-CellGroupConfig ::= SEQUENCE { /**********************text omitted***************************8/ [[ g-RNTI-ConfigToAddModList-r17 SEQUENCE (SIZE (1..maxG-RNTI-r17)) OF G-RNTI-Config-r17 OPTIONAL, -- Need N g-RNTI-ConfigToReleaseList-r17 SEQUENCE (SIZE (1..maxG-RNTI-r17)) OF G-RNTI-ConfigId-r17 OPTIONAL -- Need N ]] } DataInactivityTimer ::= ENUMERATED {s1, s2, s3, s5, s7, s10, s15, s20, s40, s50, s60, s80, s100, s120, s150, s180} G-RNTI-Config-r17 ::= SEQUENCE { g-RNTI-ConfigId-r17 G-RNTI-ConfigId-r17, g-RNTI-r17 RNTI-Value, drx-ConfigPTM-r17 SetupRelease { DRX-ConfigPTM-r17 } OPTIONAL -- Need M } G-RNTI-ConfigId-r17 ::= INTEGER (0..maxG-RNTI-r17)

Observation 3: the granularity of  G-RNTI is defined per serving cell in RAN1, while in RRC running CR, it is configured per serving cell group, which in not align.

Observation 4: in unicast, when CA is configured, the same C-RNTI applies to all serving cells. Although, different TMGI may be mapping to different G-RNTI, we cannot see the necessity of allocating different G-RNTI to different serving cell or serving cell group.

Proposal 3: It is proposed to further discuss the granularity of G-RNTI allocation in case of CA.

Conclusion

Observation 1: to support of MBS in CA, there is no obvious standard impact on MBS configuration signalling in TS38.331 ASN.1.

Observation 2: gNB needs have knowledge of the UE capability of supporting MBS in CA or not to employ appropriate scheduling strategy.

Observation3: the granularity of  G-RNTI is defined per serving cell in RAN1, while in RRC running CR, it is configured per serving cell group, which in not align.

Observation 4: in unicast, when CA is configured, the same C-RNTI applies to all serving cells. Although, different TMGI may be mapping to different G-RNTI, we cannot see the necessity of allocating different G-RNTI to different serving cell or serving cell group.

Proposal 1: Current MBS configuration signalling in TS38.331 ASN.1 can be reused to support of MBS in CA.

Proposal 2: UE capability of simultaneous reception of MBS PTM+MBS PTM simultaneous transmission with supported band combination should be introduced.

Proposal 3: It is proposed to further discuss the granularity of G-RNTI allocation in case of CA.

Reference

[1] Draft_R2-115e_Meeting_Report_v1

[2] R2-xxxx TS running CR 38.331