3GPP TSG RAN WG2#116-e		R2-2111198

e-Meeting, 1st - 12th November, 2021

Source:	vivo

Title:          Discussion on LPP segmentation in LCS message

Agenda Item:	6.3.3

Document for:	Discussion and Decision

- Introduction

In [Post115-e][608][POS] discussion [1], the following proposal regarding the LPP segmentation in LCS message were made:

Proposal 9: LS to SA2 to clarify the potential issue when LPP message (e.g., ProvideLocationInformation) in LCS message (e.g., EventReport) is segmented (12/15). NOTE: The above potential issue in Proposal 9 is independent of RRC state and out of R17 scope. Thus, the LS should be under TEI discussion.

In this contribution, we would discuss the impact of LPP segmentation on LCS message transmission.

- Discussion

In TS 37.355, the LPP message can be segmented if the LPP message size exceeds the maximum message size supported by lower layers. For instance, if the UE is requested to send the LPP ProvideLocationInformation and the message size is larger than 9000 bytes, the LPP message will be segmented.

When receiving the LPP message, the LMF will check the SegmentationInfo to confirm whether there are more messages on the way.

-	If the IE SegmentationInfo is set to moreMessagesOnTheWay, LMF shall store the message for subsequent processing;

-	If the IE SegmentationInfo is set to noMoreMessages, the LMF shall process the new message segment and any stored message segments for the same session and transaction ID.

4.3.5 LPP Message Segmentation An LPP message body may be sent in several shorter LPP messages instead of one long LPP message to deliver a large amount of information (e.g., in case the LPP message size exceeds the maximum message size supported by lower layers). When a sender employs LPP message segmentation, the sender shall include the IE SegmentationInfo in each LPP message segment. The sender shall indicate in all but the final message segment that more messages are on the way. When a receiver receives an LPP message indicating that more messages are on the way, the receiver may store the LPP message. If the receiver receives a subsequent LPP message for the same session and transaction ID, the receiver shall assume that the new LPP message continues the segmentation of the earlier message and may store the new message if the new message indicates that more messages are on the way. If the new message indicates that no more messages are on the way, the receiver shall assume that message segmentation is complete and shall process the new message and any stored message segments for the same session and transaction ID.

In TS 24.080, some LPP messages can be carried in the LCS message. For example, in deferred MT-LR, when the UE needs to send lcs-EventReport message to LMF, the LPP message ProvideLocationInformation which includes any location measurements or location estimate can be included as PositioningProtocolPDU.

Following is excerpt from TS 24.080 about the PositioningProtocolPDU in LCS message.

MultiplePositioningProtocolPDUs ::= SEQUENCE (SIZE (1..maxNumLPPMsg)) OF PositioningProtocolPDU PositioningProtocolPDU ::= OCTET STRING -- PositioningProtocolPDU contains a LPP message defined in 3GPP TS 36.355 [17]. maxNumLPPMsg INTEGER ::= 3

However, if the ProvideLocationInformation carried in the lcs-EventReport message has been segmented due to huge message size, how to process the LPP message segmentation within an LCS message is not explicitly specified from RAN2 perspective. For instance, whether an indication out of the PositioningProtocolPDU is needed.

This is probably in the realm of SA2 but requires close cooperation with RAN2. Besides, if the potential issue is confirmed by SA, some clarification may then also be needed in 37.355 and/or 38.305.

Proposal 1: LS to SA2 to clarify the potential issue when LPP message in LCS message is segmented.

- Conclusion

Based on the discussion, we propose the following:

Proposal 1: LS to SA2 to clarify the potential issue when LPP message in LCS message is segmented.

Note: the draft LS is attached in the Annex.

- Reference

- R2-2109979	Summary of [Post115-e][608][POS] PRS configuration and measurement in RRC_INACTIVE

- Annex

--------------------------------------------------------Draft LS-------------------------------------------------------------

# 1	Overall description

Currently, the LPP message can be segmented if the LPP message size exceeds the maximum message size supported by lower layers. When receiving the LPP message, the LMF will check the indication SegmentationInfo to confirm whether there are more messages on the way. Meanwhile, some LPP messages can be carried in the LCS message. For example, in deferred MT-LR, when the UE needs to send lcs-EventReport message to LMF, the LPP message ProvideLocationInformation which includes any location measurements or location estimate can be included as PositioningProtocolPDU.

However, how to process the LPP message segmentation within an LCS message is not explicitly specified from RAN2 perspective. Therefore, RAN2 would like to ask SA2 the following question:

Q: Does there any potential issue in the above case when the LPP message carried in LCS message is segmented?

# 2	Actions

To SA2 group:

RAN2 respectfully asks SA2 to consider the above question.