3GPP TSG-RAN2 Meeting #116-e	Draft R2-2111383

# Online, 01- 11-November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.822 | CR | 0006 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates based on RAN1 NR positioning features list |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2021-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As mentioned in RAN1 LS in R1-2108427: 1RAN1 agreed to add the value 96 for component 1 in FG13-2b, FG13-3b, and FG13-4b in RAN1 NR UE features list, to align with the TS 37.355. To our understanding, n96 should be added for the component 3 of these FGs since there is no n96 for component 1. 2 For FG 13-7a/9b/10e, it is RAN1 understanding that PRS from a PRS-only TP can be treated as same as PRS from a non-serving cell and no differentiation between the two is needed. It is up to RAN2 to determine if the UE PRS capabilities should explicitly mention PRS-only TP in a note for these FGs in TR38.822. There was no consensus in RAN1 on whether a note is needed or not. So far in TS37.355, it is mentioned clearly that PRS-only TP is not a cell. “PRS-only TP: A TP which only transmits PRS signals or DL-PRS for PRS-based TBS positioning and is not associated with a cell.” The description in TS38.822 is not accurate since only cell is mentioned, it cannot cover PRS-only TP case. 3 Current TR 38.822 FG 13-1 is not aligned with the attachment R1-2108426 in the LS from RAN1 R2-2109313. The description of component 3 of FG 13-1 in R1-2108426 is as follows: Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. - T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms - N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms. The description of component 3 of FG 13-1 in TR 38.822 is as follows: 3. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms. Clearly there was some error when the components is captured in TR 38.822. |
| Summary of change: |  | 1 add n96 for component 3 in FG13-2b, FG13-3b, and FG13-4b; 2 add “Note: A PRS from a PRS-only TP is treated as PRS from a non-serving cell” for FG 13-7a/9b/10e; 3 Bullet a) of componenet 3 from the FG 13-1 in the TR 38.822 is changed to align with the RAN1 UE feature list. Impact analysis Impacted functionality: support of NR positioning; Inter-operability: no inter-operability issues due to this CR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1 n96 is missing for component 3 in FG13-2b, FG13-3b, and FG13-4b; 2 PRS-only TP is missing for FG 13-7a/9b/10e; 3 the description of componenet 3 from the FG 13-1 is not aligned with RAN1 feature lists; |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS37.355 CR 0321 TS38.306 CR 0645 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.1.5	NR positioning

Table 5.1.5-1: Layer-1 feature list for NR positioning

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 | Parent IE in TS 38.331 | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13. NR Positioning | 13-1 | Common DL PRS Processing Capability | 1. Maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} 2. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 3. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 4. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) of no more than 30%. |  | 1 supportedBandwidthPRS-r16 2 dl-PRS-BufferType-r16 3 durationOfPRS-Processing-r16 4 maxNumOfDL-PRS-ResProcessedPerSlot-r16 | PRS-ProcessingCapabilityPerBand-r16 LPP | n/a | n/a | Need for location server to know if the feature is supported. Notes for component 3: a.UE reports one combination of (N, T) values per band, where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth (B) in MHz supported by UE b.UE is not expected to support DL PRS bandwidth that exceeds the reported DL PRS bandwidth value c.UE DL PRS processing capability is defined for a single positioning frequency layer. UE capability for simultaneous DL PRS processing across positioning frequency layers is not supported in Rel.16 (i.e. for a UE supporting multiple positioning frequency layers, a UE is expected to process one frequency layer at a time) d.UE DL PRS processing capability is agnostic to DL PRS comb factor configuration e.The reporting of (N, T) values for maximum BW in MHz is not dependent on SCS Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |
|  | 13-1a | Max number of positioning frequency layers UE supports across all positioning methods across all bands | Max number of positioning frequency layers UE supports across all positioning methods across all bands Values: {1, 2, 3, 4} |  | maxSupportedFreqLayers-r16 | NR-DL-PRS-ProcessingCapability-r16 LPP | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-2 | DL PRS Resources for DL AoD | 1. Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE. Values = {1, 2} 2. Max number of TRPs across all positioning frequency layers per UE. Values = {4, 6, 12, 16, 24, 32, 64, 128, 256} 3. Max number of positioning frequency layers UE supports Values = {1, 2, 3, 4} | 13-1 | 1 maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer-r16 2 maxNrOfTRP-AcrossFreqs-r16 3 maxNrOfPosLayer-r16 | NR-DL-PRS-ResourcesCapability-r16 LPP | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-2a | DL PRS Resources for DL AoD on a band | 1. Max number of DL PRS Resources per DL PRS Resource Set Values = {2, 4, 8, 16, 32, 64} Note: 16, 32, 64 are only applicable to FR2 bands 2. Max number of DL PRS Resources per positioning frequency layer. Values = {6, 24, 32, 64, 96, 128, 256, 512, 1024} Note: 6 is only applicable to FR1 bands | 13-1 | 1 maxNrOfDL-PRS-ResourcesPerResourceSet-r16 2 maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer-r16 | DL-PRS-ResourcesCapabilityPerBand-r16 LPP | n/a | n/a | Need for location server to know if the feature is supported. Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |
|  | 13-2b | DL PRS Resources for DL AoD on a band combination | 1. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only. Values = {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC. 2. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC 3. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation. Values = {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands 4. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands | 13-1 | 1 fr1-Only-r16 2 fr2-Only-r16 3 fr1-r16/ fr1-FR2Mix-r16 4 fr2-r16/ fr1-FR2Mix-r16 | maxNrOfDL-PRS-ResourcesAcrossAllFL-TRP-ResourceSet-r16/ DL-PRS-ResourcesBandCombination-r16 LPP | n/a | n/a | Need for location server to know if the feature is supported. the reported value is the total number across all bands in the corresponding BC Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |
|  | 13-3 | DL PRS Resources for DL-TDOA | 1. Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE. Values = {1, 2} 2. Max number of TRPs across all positioning frequency layers per UE. Values = {4, 6, 12, 16, 24, 32, 64, 128, 256} 3. Max number of positioning frequency layers UE supports Values = {1, 2, 3, 4} | 13-1 | 1 maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer-r16 2 maxNrOfTRP-AcrossFreqs-r16 3 maxNrOfPosLayer-r16 | NR-DL-PRS-ResourcesCapability-r16 LPP | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-3a | DL PRS Resources for DL-TDOA on a band | 1. Max number of DL PRS Resources per DL PRS Resource Set Values = {1, 2, 4, 8, 16, 32, 64} Note: 16, 32, 64 are only applicable to FR2 bands 2. Max number of DL PRS Resources per positioning frequency layer. Values = {6, 24, 32, 64, 96, 128, 256, 512, 1024} Note: 6 is only applicable to FR1 bands | 13-1 | 1 maxNrOfDL-PRS-ResourcesPerResourceSet-r16 2 maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer-r16 | DL-PRS-ResourcesCapabilityPerBand-r16 LPP | n/a | n/a | Need for location server to know if the feature is supported. Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |
|  | 13-3b | DL PRS Resources for DL-TDOA on a band combination | 1. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only. Values = {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC. 2. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC 3. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation. Values = {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands 4. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands | 13-1 | 1 fr1-Only-r16 2 fr2-Only-r16 3 fr1-r16/ fr1-FR2Mix-r16 4 fr2-r16/ fr1-FR2Mix-r16 | maxNrOfDL-PRS-ResourcesAcrossAllFL-TRP-ResourceSet-r16/ DL-PRS-ResourcesBandCombination-r16 LPP | n/a | n/a | Need for location server to know if the feature is supported. the reported value is the total number across all bands in the corresponding BC Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |
|  | 13-4 | DL PRS Resources for Multi-RTT | 1. Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE. Values = {1, 2} 2. Max number of TRPs across all positioning frequency layers per UE. Values = {4, 6, 12, 16, 24, 32, 64, 128, 256} 3. Max number of positioning frequency layers UE supports Values = {1, 2, 3, 4} | 13-1 | 1 maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer-r16 2 maxNrOfTRP-AcrossFreqs-r16 3 maxNrOfPosLayer-r16 | NR-DL-PRS-ResourcesCapability-r16 LPP | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-4a | DL PRS Resources for Multi-RTT on a band | 1. Max number of DL PRS Resources per DL PRS Resource Set Values = {1, 2, 4, 8, 16, 32, 64} Note: 16, 32, 64 are only applicable to FR2 bands 2. Max number of DL PRS Resources per positioning frequency layer. Values = {6, 24, 32, 64, 96, 128, 256, 512, 1024} Note: 6 is only applicable to FR1 bands | 13-1 | 1 maxNrOfDL-PRS-ResourcesPerResourceSet-r16 2 maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer-r16 | DL-PRS-ResourcesCapabilityPerBand-r16 LPP | n/a | n/a | Need for location server to know if the feature is supported. Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |
|  | 13-4b | DL PRS Resources for Multi-RTT on a band combination | 1. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only. Values = {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC. 2. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC 3. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation. Values = {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands 4. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands | 13-1 | 1 fr1-Only-r16 2 fr2-Only-r16 3 fr1-r16/ fr1-FR2Mix-r16 4 fr2-r16/ fr1-FR2Mix-r16 | maxNrOfDL-PRS-ResourcesAcrossAllFL-TRP-ResourceSet-r16/ DL-PRS-ResourcesBandCombination-r16 LPP | n/a | n/a | Need for location server to know if the feature is supported. the reported value is the total number across all bands in the corresponding BC Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |
|  | 13-5 | DL PRS Measurement Report for DL-AoD | 1. Max number of DL PRS RSRP measurements on different PRS resources from the same TRP supported by the UE Values = {1, 2, 3, 4, 5, 6, 7, 8} | 13-2, | maxDL-PRS-RSRP-MeasurementFR1-r16 maxDL-PRS-RSRP-MeasurementFR2-r16 | NR-DL-AoD-MeasurementCapability LPP | No | Yes | Need for location server to know if the feature is supported. the number of RSRP measurement on a particular band is also upper bounded by the number of resources per set supported by UE reported per band | Optional with capability signaling |
|  | 13-6 | DL PRS Measurement Report for DL-TDOA | 1. DL RSTD measurements per pair of TRPs. Values = {1, 2, 3, 4} 2. Support DL PRS-RSRP measurements. Values = {0, 1} | 13-3 | dl-RSTD-MeasurementPerPairOfTRP-FR1-r16 dl-RSTD-MeasurementPerPairOfTRP-FR2-r16 | NR-DL-TDOA-MeasurementCapability-r16 LPP | No | Yes | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-7 | Support of SSB from neighbor cell as QCL source of a DL PRS | 1. Support of SSB from neighbor cell as QCL source of a DL PRS 2. Support of reuse SSB measurement from RRM for receiving PRS Note: Refers to Type-C for FR1 and Type-C & Type-D support for FR2 | 13-1 | ssb-FromNeighCellAsQCL-r16 | DL-PRS-QCL-ProcessingCapabilityPerBand-r16 LPP | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-7a | Support of DL PRS from serving/neighbor cell as QCL source of a DL PRS | 1. Support of DL PRS from serving/neighbor cell as QCL source of a DL PRS Note 1 : Refers to Type-D support for FR2 Note 2: A PRS from a PRS-only TP is treated as PRS from a non-serving cell | 13-1 | prs-FromServNeighCellAsQCL-r16 | DL-PRS-QCL-ProcessingCapabilityPerBand-r16 LPP | n/a | n/a | Need for location server to know if the feature is supported. DL PRSs are in the same band | Optional with capability signaling |
|  | 13-8 | SRS Resources for Positioning | 1. Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 2, 4, 8, 12, 16}. 2. Max number of P/SP/AP SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 3. Max number of P/SP/AP SRS Resources including the SRS resources for positioning per BWP per slot. Values = {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Note: Max number of P/SP/AP SRS Resources in Component 3 include both SRS resources configured by SRS-Resource and SRS resources configured by SRS-PosResource-r16 supported by UE 4. Max number of periodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 5. Max number of periodic SRS Resources for positioning per BWP per slot. Values = {1,2,3,4,5,6,8,10,12,14} OLPC for SRS for positioning based on SSB from serving cell is part of FG13-8 Note: no dedicated capability signaling is intended for this component |  | RRC 1 maxNumberSRS-PosResourceSetPerBWP-r16 2 maxNumberSRS-PosResourcesPerBWP-r16 3 maxNumberSRS-ResourcesPerBWP-PerSlot-r16 4 maxNumberPeriodicSRS-PosResourcesPerBWP-r16 5 maxNumberPeriodicSRS-PosResourcesPerBWP-PerSlot-r16 | RRC SRS-AllPosResources-r16 /SRS-AllPosResources-r16 | n/a | n/a | Note: if the UE does not indicate this capability for a band in a band combination, the UE does not support SRS for Positioning in this band in the band combination. - UE not supporting FG13-8 does not support FG13-8a or FG13-8b in the band in the band combination. - The same approach is applicable to FG13-8c, FG13-8d, and FG13-8e. | Optional with capability signaling |
|  | 13-8a | Support of Aperiodic SRS Resources for positioning | 1. Max number of aperiodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 2. Max number of aperiodic SRS Resources for positioning per BWP per slot. Values = {1,2,3,4,5,6,8,10,12,14} | 13-8 | RRC 1 maxNumberAP-SRS-PosResourcesPerBWP-r16 2 maxNumberAP-SRS-PosResourcesPerBWP-PerSlot-r16 | RRC SRS-PosResourceAP-r16 /SRS-AllPosResources-r16 | n/a | n/a |  | Optional with capability signaling |
|  | 13-8b | Support of Semi-persistent SRS Resources for positioning | 1. Max number of semi-persistent SRS Resources for positioning supported by UE per BWP. Values = {1,2,4,8,16,32,64} 2. Max number of semi-persistent SRS Resources for positioning supported by UE per BWP per slot. Values = {1,2,3,4,5,6,8,10,12,14} | 13-8 | RRC 1 maxNumberSP-SRS-PosResourcesPerBWP-r16 2 maxNumberSP-SRS-PosResourcesPerBWP-PerSlot-r16 | RRC SRS-PosResourceSP-r16 /SRS-AllPosResources-r16 | n/a | n/a |  | Optional with capability signaling |
|  | 13-8c | SRS Resources for Positioning | 1. Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 2, 4, 8, 12, 16}. 2. Max number of P/SP/AP SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 3. Max number of periodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} | 13-8 | LPP 1 maxNumberSRS-PosResourceSetsPerBWP-r16 2 maxNumberSRS-PosResourcesPerBWP-r16 3maxNumberPeriodicSRS-PosResourcesPerBWP-r16 | LPP SRS-PosResourcesPerBand-r16 | n/a | n/a | Need for location server to know if the feature is supported UE only reports the number on bands for the current configured CA band combination. | Optional with capability signaling |
|  | 13-8d | Support of Aperiodic SRS Resources for positioning | 1. Max number of aperiodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} | 13-8a, 13-8c | LPP 1 maxNumberAP-SRS-PosResourcesPerBWP-r16 | LPP SRS-PosResourcesPerBand-r16 | n/a | n/a | Need for location server to know if the feature is supported. UE only reports the number on bands for the current configured CA band combination. | Optional with capability signaling |
|  | 13-8e | Support of Semi-persistent SRS Resources for positioning | 1. Max number of semi-persistent SRS Resources for positioning supported by UE per BWP. Values = {1,2,4,8,16,32,64} | 13-8b,13-8c | LPP 1 maxNumberSP-SRS-PosResourcesPerBWP-r16 | LPP SRS-PosResourcesPerBand-r16 | n/a | n/a | Need for location server to know if the feature is supported. UE only reports the number on bands for the current configured CA band combination. | Optional with capability signaling |
|  | 13-9 | OLPC for SRS for positioning based on PRS from the serving cell | 1. OLPC for SRS for positioning based on PRS from the serving cell in the same band | 13-1 and 13-8 | LPP olpc-SRS-PosBasedOnPRS-Serving-r16 RRC olpc-SRS-PosBasedOnPRS-Serving-r16 | LPP OLPC-SRS-Pos-r16 RRC OLPC-SRS-Pos-r16 | n/a | n/a | RAN1 kindly requests RAN2 to decide on the necessity for location server to know if the feature is supported | Optional with capability signaling |
|  | 13-9a | OLPC for SRS for positioning based on SSB from neighbouring cells | 1. OLPC for SRS for positioning based on SSB from neighbouring cells in the same band | 13-8 | LPP olpc-SRS-PosBasedOnSSB-Neigh-r16 RRC olpc-SRS-PosBasedOnSSB-Neigh-r16 | LPP OLPC-SRS-Pos-r16 RRC OLPC-SRS-Pos-r16 | n/a | n/a | RAN1 kindly requests RAN2 to decide on the necessity for location server to know if the feature is supported | Optional with capability signaling |
|  | 13-9b | OLPC for SRS for positioning based on PRS from the neighbouring cells | 1. OLPC for SRS for positioning based on PRS from the neighbouring cells in the same band Note: A PRS from a PRS-only TP is treated as PRS from a non-serving cell | 13-9 | LPP olpc-SRS-PosBasedOnPRS-Neigh-r16 RRC olpc-SRS-PosBasedOnPRS-Neigh-r16 | LPP OLPC-SRS-Pos-r16 RRC OLPC-SRS-Pos-r16 | n/a | n/a | RAN1 kindly requests RAN2 to decide on the necessity for location server to know if the feature is supported | Optional with capability signaling |
|  | 13-9e | PathLoss estimate maintenance per serving cell | 1. Max number of pathloss estimates that the UE can simultaneously maintain for all the SRS resource sets for positioning per serving cell in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissions Candidate values are {1, 4, 8, 16} Note: SRS in "PUSCH/PUCCH/SRS" refers to SRS configured by SRS-Resource | One of {13-9, 13-9a, 13-9b, 13-9c} | LPP maxNumberPathLossEstimatePerServing-r16 RRC maxNumberPathLossEstimatePerServing-r16 | LPP OLPC-SRS-Pos-r16 RRC OLPC-SRS-Pos-r16 | n/a | n/a | RAN1 kindly requests RAN2 to decide on the necessity for location server to know if the feature is supported SRS and SSB and/or PRS are in the same band Note: if the UE does not indicate this capability for a band, the UE does not support any pathloss estimates in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissions in that band. | Optional with capability signaling |
|  | 13-9f | PathLoss estimate maintenance across all cells | 1. Max number of pathloss estimates that the UE can simultaneously maintain for all the SRS resource sets for positioning across all cells in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissions" Candidate values are {1, 4, 8, 16} Note: SRS in "PUSCH/PUCCH/SRS" refers to SRS configured by SRS-Resource | One of {13-9, 13-9a, 13-9b, 13-9c} | LPP maxNumberSRS-PosPathLossEstimateAllServingCells-r16 RRC maxNumberSRS-PosPathLossEstimateAllServingCells-r16 | LPP NR-UL-SRS-Capability-r16 RRC Phy-ParametersCommon | No | No | RAN1 kindly requests RAN2 to decide on the necessity for location server to know if the feature is supported SRS and SSB and/or PRS are in the same band | Optional with capability signaling |
|  | 13-10 | Spatial relation for SRS for positioning based on SSB from the serving cell | 1. Spatial relation for SRS for positioning based on SSB from the serving cell in the same band | 13-8 | LPP spatialRelation-SRS-PosBasedOnSSB-Serving-r16 RRC spatialRelation-SRS-PosBasedOnSSB-Serving-r16 | LPP SpatialRelationsSRS-Pos-r16 RRC SpatialRelationsSRS-Pos-r16 | n/a | n/a (FR2 only) | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-10a | Spatial relation for SRS for positioning based on CSI-RS from the serving cell | 1. Spatial relation for SRS for positioning based on CSI-RS from the serving cell in the same band | 13-10 | LPP spatialRelation-SRS-PosBasedOnCSI-RS-Serving-r16 RRC spatialRelation-SRS-PosBasedOnCSI-RS-Serving-r16 | LPP SpatialRelationsSRS-Pos-r16 RRC SpatialRelationsSRS-Pos-r16 | n/a | n/a (FR2 only) | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-10b | Spatial relation for SRS for positioning based on PRS from the serving cell | 1. Spatial relation for SRS for positioning based on PRS from the serving cell in the same band | One of {13-2, 13-3, 13-4} and13-8 | LPP spatialRelation-SRS-PosBasedOnPRS-Serving-r16 RRC spatialRelation-SRS-PosBasedOnPRS-Serving-r16 | LPP SpatialRelationsSRS-Pos-r16 RRC SpatialRelationsSRS-Pos-r16 | n/a | n/a (FR2 only) | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-10c | Spatial relation for SRS for positioning based on SRS | 1. Spatial relation for SRS for positioning based on SRS in the same band | 13-8, | LPP spatialRelation-SRS-PosBasedOnSRS-r16 RRC spatialRelation-SRS-PosBasedOnSRS-r16 | LPP SpatialRelationsSRS-Pos-r16 RRC SpatialRelationsSRS-Pos-r16 | n/a | n/a (FR2 only) | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-10d | Spatial relation for SRS for positioning based on SSB from the neighbouring cell | 1. Spatial relation for SRS for positioning based on SSB from the neighbouring cell in the same band | 13-10 | LPP spatialRelation-SRS-PosBasedOnSSB-Neigh-r16 RRC spatialRelation-SRS-PosBasedOnSSB-Neigh-r16 | LPP SpatialRelationsSRS-Pos-r16 RRC SpatialRelationsSRS-Pos-r16 | n/a | n/a (FR2 only) | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-10e | Spatial relation for SRS for positioning based on PRS from the neighbouring cell | 1. Spatial relation for SRS for positioning based on PRS from the neighbouring cell in the same band Note: A PRS from a PRS-only TP is treated as PRS from a non-serving cell | 13-10b | LPP spatialRelation-SRS-PosBasedOnPRS-Neigh-r16 RRC spatialRelation-SRS-PosBasedOnPRS-Neigh-r16 | LPP SpatialRelationsSRS-Pos-r16 RRC SpatialRelationsSRS-Pos-r16 | n/a | n/a (FR2 only) | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-10f | Spatial relation maintenance | 1. Max Number of maintained spatial relations for all the SRS resource sets for positioning across all serving cells in addition to the spatial relations maintained spatial relations per serving cell for the PUSCH/PUCCH/SRS transmissions. Values = {0,1,2,4,8,16} Note: component 1 is for all cells across all bands Note: SRS in "PUSCH/PUCCH/SRS" refers to SRS configured by SRS-Resource | One of {13-10, 13-10a, 13-10b, 13-10d, 13-10e} | LPP maxNumberSRS-PosSpatialRelationsAllServingCells-r16 RRC maxNumberSRS-PosSpatialRelationsAllServingCells-r16 | LPP NR-UL-SRS-Capability-r16 RRC Phy-ParametersFR2 | No | No (FR2 only) | Need for location server to know if the feature is supported. SRS and SSB and/or PRS are in the same band | Optional with capability signaling |
|  | 13-11a | Association between SRS for positioning and DL PRS for Multi-RTT | 1. Support of measurements derived on one or more DL PRS resource/resource sets which may be in different positioning frequency layers for SRS transmitted in a single CC. Note: PRS and SRS may be in a different band | 13-4 and 13-8 | LPP srs-AssocPRS-MultiLayersFR1-r16 srs-AssocPRS-MultiLayersFR2-r16 | LPP NR-Multi-RTT-MeasurementCapability-r16 | No | Yes | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-11 | UE Rx-Tx Measurement Report for Multi-RTT | 1. Max number of UE Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL PRS resource/resource set. Value for component 1: {1,2,3,4} Note: DL PRS resource/sets are on the same frequency layer Note: the number of UE Rx – Tx time difference measurements refers to the measurements for a single TRP 2. Support RSRP measurements. Values = {0, 1} Note: If the UE reports value 1 for component 2, same number of RSRP measurements supported as UE Rx-Tx measurements for component 1 | 13-4 and 13-8 | LPP 1 maxNrOfRx-TX-MeasFR1-r16 maxNrOfRx-TX-MeasFR2-r16 2 supportOfRSRP-MeasFR1-r16 supportOfRSRP-MeasFR2-r16 | LPP NR-Multi-RTT-MeasurementCapability-r16 | No | Yes | Need for location server to know if the feature is supported. FG13-11 covers the case that SRS and DL PRS are on the same band | Optional with capability signaling |
|  | 13-12 | SS-RSRP RRM measurements for NR E-CID Positioning | 1. Support of cell-specific SS-RSRP RRM measurements with LPP report for NR E-CID Positioning 2. Support of beam-specific SS-RSRP RRM measurements with LPP report for NR E-CID Positioning | 1-1 | nr-ECID-MeasSupported-r16 BIT STRING { ssrsrpSup(0), ssrsrqSup(1), csirsrpSup(2), csirsrqSup(3)} (SIZE(1..8)) | NR-ECID-ProvideCapabilities-r16 LPP | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-12a | SS-RSRQ RRM measurements for NR E-CID Positioning | 1. Support of cell-specific SS-RSRQ RRM measurements with LPP report for NR E-CID Positioning 2. Support of beam-specific SS-RSRQ RRM measurements with LPP report for NR E-CID Positioning | 1-1 | nr-ECID-MeasSupported-r16 BIT STRING { ssrsrpSup(0), ssrsrqSup(1), csirsrpSup(2), csirsrqSup(3)} (SIZE(1..8)) | NR-ECID-ProvideCapabilities-r16 LPP | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-12b | CSI-RSRP RRM measurements for NR E-CID Positioning | 1. Support of cell-specific CSI-RSRP RRM measurements with LPP report for NR E-CID Positioning 2. Support of beam-specific CSI-RSRP RRM measurements with LPP report for NR E-CID Positioning | 1-4 | nr-ECID-MeasSupported-r16 BIT STRING { ssrsrpSup(0), ssrsrqSup(1), csirsrpSup(2), csirsrqSup(3)} (SIZE(1..8)) | NR-ECID-ProvideCapabilities-r16 LPP | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-12c | CSI-RSRQ RRM measurements for NR E-CID Positioning | 1. Support of cell-specific CSI-RSRQ RRM measurements with LPP report for NR E-CID Positioning 2. Support of beam-specific CSI-RSRQ RRM measurements with LPP report for NR E-CID Positioning | 1-4 | nr-ECID-MeasSupported-r16 BIT STRING { ssrsrpSup(0), ssrsrqSup(1), csirsrpSup(2), csirsrqSup(3)} (SIZE(1..8)) | NR-ECID-ProvideCapabilities-r16 LPP | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-13 | Simultaneous DL-AoD and DL-TDoA processing | 1. Support of simultaneous processing for DL AoD and DL TDoA measurements If it is not indicated, a UE is not expected to perform simultaneously the processing for deriving DL AoD and DL TDoA measurements | 13-2 and 13-3 | simul-NR-DL-AoD-DL-TDOA-r16 | DL-AoD-MeasCapabilityPerBand LPP | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-14 | Simultaneous DL-AoD and Multi-RTT processing | 1. Support of simultaneous processing for DL AoD and Multi-RTT measurements If it is not indicated, a UE is not expected to perform simultaneously the processing for deriving DL AoD and M-RTT measurements | 13-2, 13-4 and 13-8 | simul-NR-DL-AoD-Multi-RTT-r16 | DL-AoD-MeasCapabilityPerBand LPP | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 13-15 | Simultaneous SRS transmission within a band across multiple CCs | 1. The number of SRS resources for positioning on a symbol within a band Candidate values {2} Note: if the UE does not indicate this capability for a band, the UE does not support the feature in this band | 13-8 | RRC simulSRS-TransWithinBand-r16 | RRC BandNR | n/a | n/a | RAN1 kindly requests RAN2 to decide on the necessity for location server to know if the feature is supported | Optional with capability signaling |
|  | 13-15a | Simultaneous SRS transmission for a given BC | 1. The number of SRS resources for positioning on a symbol for a given BC Candidate values {2} Note: For single-band BCs, it defines the capability for intra-band CA, and for BCs with at least two bands, it defines the capability for inter-band CA. Note: if the UE does not indicate this capability for a band combination, the UE does not support the feature in this band combination | 13-8 | RRC simul-SRS-Trans-BC-r16 | RRC CA-ParametersNR-v1610 | n/a | n/a | RAN1 kindly requests RAN2 to decide on the necessity for location server to know if the feature is supported | Optional with capability signaling |
|  | 13-18 | Support of parallel processing of LTE PRS and NR PRS | 1. Support of parallel processing of LTE PRS and NR PRS |  | simulLTE-NR-PRS-r16 | NR-DL-PRS-ProcessingCapability-r16 LPP | No | No | Need for location server to know if the feature is supported | Optional with capability signaling |
|  | 13-19 | Simultaneous positioning SRS and MIMO SRS transmission within a band across multiple CCs | 1. The number of SRS resources for positioning and SRS resource for MIMO on a symbol within a band Candidate values {2} Note: SRS resource for MIMO refers to SRS resource configured by SRS-Resource. Note: If UE reports 2 for the candidate value, it means both the number of SRS resource for positioning and SRS resource for MIMO equals to 1. Note: if the UE does not indicate this capability for a band, the UE does not support the feature in this band | 13-8 | RRC simulSRS-MIMO-TransWithinBand-r16 | RRC BandNR | n/a | n/a | RAN1 kindly requests RAN2 to decide on the necessity for location server to know if the feature is supported | Optional with capability signaling |
|  | 13-19a | Simultaneous positioning SRS and MIMO SRS transmission for a given BC | 1. The number of SRS resources for positioning and SRS resource for MIMO on a symbol for a given BC Candidate values {2} Note: SRS resource for MIMO refers to SRS resource configured by SRS-Resource. Note: If UE reports 2 for the candidate value, it means both the number of SRS resource for positioning and SRS resource for MIMO equals to 1. Note: For single-band BCs, it defines the capability for intra-band CA, and for BCs with at least two bands, it defines the capability for inter-band CA. Note: if the UE does not indicate this capability for a band combination, the UE does not support the feature in this band combination | 13-8 | RRC simul-SRS-MIMO-Trans-BC-r16 | RRC CA-ParametersNR-v1610 | n/a | n/a | RAN1 kindly requests RAN2 to decide on the necessity for location server to know if the feature is supported | Optional with capability signaling |