3GPP TSG-RAN WG2 Meeting #116-e	R2-2111463

# Electronic, November 1 – November 12, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1827 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR-U RSSI/CO measurement UE capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, xiaomi, vivo, Lenovo, Motorola Mobility, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17, NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2021-10-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | NR-U RSSI/CO measurement UE capability is only carried in UE-CapabilityRAT-ContainerList, which is not required for eNB to decode. However, in TS36.331, NR-U RSSI measurement configuration is enabled. The problem then is eNB cannot make such configuration to UE without knowing UE capability. It was agreed in RAN2 #113 meeting to introduce a new UE capability in TEI17 on this matter. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add the per band inter-RAT NR-U RSSI/CO measurement UE capability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eNB cannot configure UE with NR-U RSSI/CO measurement without knowing the corresponding UE capability. Impact analysis: Impacted 5G architecture options: LTE only, EN-DC Impacted functionality: NR-U Inter-operability: If NW implements this CR and UE does not, UE would not report the NR-U RSSI/CO measurement UE capability to eNB. There is no inter-operability issue. If UE implements this CR and NW does not, NW would not know the UE capability on NR-U RSSI/CO measurement UE capability. There is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 36.331 CR 4729 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.3.6	Measurement parameters

#### <Text omitted>

#### --------------------------------------------------------------------------<Start of change>----------------------------------------------------------------------

4.3.6.XX	nr-RSSI-ChannelOccupancyReporting-r17

#### This field indicates whether the UE supports performing measurements and reporting of RSSI and channel occupancy on each supported NR band.

#### --------------------------------------------------------------------------<End of change>-------------------------------------------------------------------------