3GPP TSG-RAN WG2 Meeting #116-e	R2-21xxxxx

Online, 1-12 November 2021

Source: 	Session chair (MediaTek)

Title:	Report from session on positioning and sidelink relay

# Status of At-Meeting Email Discussions

This subclause is not an Agenda Item. It contains a running summary of the email discussions assigned to take place during the meeting weeks.  This section will be moved to an appendix in the final version of the report.

[AT116-e][600][POS][Relay] Organisational Nathan – Positioning/Relay (MediaTek)

Scope: Organisational discussions and announcements, as needed throughout the meeting weeks

Intended outcome: Well-informed participants

Deadline:  Friday 2021-11-12 1000 UTC

[AT116-e][611][POS] LS to RTCM (ESA)

Scope: Discuss coordination with RTCM, taking into account the way-forward proposals in R2-2109807 and related parts of R2-2110181:

- Conclude on the intention to specify GNSS integrity signalling in Rel-17

- Determine what information we intend to share with RTCM

- Draft an LS reply (TP to be endorsed later)

Intended outcome: Report in R2-2111361 and approvable LS in R2-2111362

Deadline:  Friday 2021-11-05 1000 UTC (comments), Monday 2021-11-08 1100 UTC (output available)

[AT116-e][612][Relay] Non-relay discovery (OPPO)

Scope: Evaluate the spec impact of non-relay discovery specific aspects and determine a way forward for handling this objective.

Intended outcome: Report to CB session, in R2-2111363

Deadline:  Tuesday 2021-11-09 0800 UTC (report available)

[AT116-e][613][POS] BDS B2a and B3I signals (CATT)

Scope: Discuss the CRs in R2-2109485, R2-2109486, R2-2109487, and R2-2109488, collect any comments and produce updates if necessary for endorsement.

Intended outcome: Endorsable CRs

Deadline:  Friday 2021-11-05 1000 UTC (comments), Monday 2021-11-08 1100 UTC (output available)

[AT116-e][614][POS] AI 5.5 CRs (vivo)

Scope: Evaluate and conclude on the CRs in R2-2111126 and R2-2111127.

Intended outcome: Agreed CRs

Deadline:  Thursday 2021-11-11 0200 UTC

[AT116-e][615][POS] PRUs (Qualcomm)

Scope: Discuss the handling of the PRU topic taking the related contributions into account, and determine a way forward.

Intended outcome: Report to positioning session in R2-2111364, and LS out if necessary

Deadline:  Monday 2021-11-08 1000 UTC (report available)

[AT116-e][616][POS] Updates for RAN1 positioning feature list (Intel)

Scope: Review the CRs in R2-2109679, R2-2109680, R2-2109681, R2-2110172, and R2-2110173, and draft a response to RAN1 indicating where we have corrected the implementation of the changes.

Intended outcome: Agreed CRs and approved LS

Deadline:  Tuesday 2021-11-09 0900 UTCs

[AT116-e][617][POS] Correction on BDS B2I clock model (Swift)

Scope: Check and update the CR in R2-2111072.

Intended outcome: Agreeable CR

Deadline:  Tuesday 2021-11-09 0800 UTC

[AT116-e][618][POS] CR to 38.321 on posSRS handling (Huawei)

Scope: Draft a CR to 38.321 capturing the NOTE agreed under agenda item 6.3.4.

Intended outcome: Agreeable CR in R2-2111367

Deadline:  Tuesday 2021-11-09 0800 UTC

[AT116-e][619][POS] Stage 2 Rel-16 positioning CRs (Huawei)

Scope: Check the CRs in R2-2110169 and R2-2110170.

Intended outcome: Agreed CRs

Deadline:  Tuesday 2021-11-09 0800 UTC

# 4	EUTRA corrections Rel-15 and earlier

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.4	Positioning corrections Rel-15 and earlier

Documents in this agenda item will be handled by email.  No web conference is planned for this agenda item.

# 5	Rel-15 WI: New Radio (NR) Access Technology

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

Only essential corrections. Please submit CRs marked “NR_newRAT-Core, TEI16” under one of the below clauses.

## 5.5	Positioning corrections

Corrections to both the stage 2 and stage 3 aspects related to positioning. Stage 2 CRs shall be discussed with the specification rapporteur (Sven Fischer sfischer@qti.qualcomm.com) before submission.  Stage 2 CRs not discussed with the specification rapporteur will not be treated.

Documents in this agenda item will be handled by email.  No web conference is planned for this agenda item.

R2-2111126	Correction on LPP message delivery	vivo	CR	Rel-15	37.355	15.2.0	0324	-	F	NR_pos-Core

R2-2111127	Correction on LPP message delivery	vivo	CR	Rel-16	37.355	16.6.0	0325	-	A	NR_pos-Core

[AT116-e][614][POS] AI 5.5 CRs (vivo)

Scope: Evaluate and conclude on the CRs in R2-2111126 and R2-2111127.

Intended outcome: Agreed CRs

Deadline:  Thursday 2021-11-11 0200 UTC

# 6	Rel-16 NR Work Items

Essential corrections only.

Tdoc Limitation: 18 tdocs in total for all sub agenda items, or the restriction for each sub-AI, whichever is more restrictive.

## 6.3	NR Positioning Support

(NR_pos-Core; leading WG: RAN1; REL-16; started: Mar 19; target; Jun 20; WID: RP-200218).

(NR TEI16 Positioning)

Documents in this agenda item will be handled in a break out session

Tdoc Limitation: See tdoc limitation for Agenda Item 6

### 6.3.1	General and Stage 2 corrections

Including incoming LSs, Including impact to 36.305 and 38.305. Stage 2 corrections shall be discussed with the specification rapporteur (Sven Fischer sfischer@qti.qualcomm.com) before submission. Stage 2 CRs not discussed with the specification rapporteur will not be treated.

This agenda item may use a summary document (decision to be made based on submitted tdocs).

Incoming LS

R2-2109333	Reply LS on E-CID LTE measurement in Rel-15 measurements (R3-212802; contact: Huawei)	RAN3	LS in	Rel-15	NR_pos-Core	To:RAN2

Huawei clarify the issue was resolved last meeting.

- Noted

Feature list

R2-2109313	LS on updated Rel-16 RAN1 UE features lists for NR after RAN1#105-e (R1-2108427; contact: NTT DoCoMo, AT&T)	RAN1	LS in	Rel-16	NR_2step_RACH-Core, NR_unlic-Core, NR_IAB-Core, 5G_V2X_NRSL-Core, NR_L1enh_URLLC-Core, NR_IIOT-Core, NR_eMIMO-Core, NR_UE_pow_sav-Core, NR_pos-Core, NR_Mob_enh-Core, LTE_NR_DC_CA_enh-Core, TEI16, NR_CLI_RIM-Core	To:RAN2	Cc:RAN4

Discussion:

Intel indicate there is a mistake in the LS that changes the wrong component (should be component 3 instead of component 1 of FG13-2b/3b/4b), and this is corrected in the CRs.  Also regarding the secondary change to the note on the PRS-only TP, there are related CRs to this meeting.

Nokia ask if the mistake is only in the LS or if an update is needed within RAN1.  Intel clarify there is only RAN2 impact in the specs, but we may want to send feedback to RAN1 documenting the mistake and the correction in the changes

R2-2109679	Updates based on RAN1 NR positioning features list	Intel Corporation	CR	Rel-16	38.822	16.1.0	0006	-	F	NR_pos-Core

R2-2109680	Updates based on RAN1 NR positioning features list	Intel Corporation	CR	Rel-16	37.355	16.6.0	0321	-	F	NR_pos-Core

R2-2109681	Updates based on RAN1 NR positioning features list	Intel Corporation	CR	Rel-16	38.306	16.6.0	0645	-	F	NR_pos-Core

[AT116-e][616][POS] Updates for RAN1 positioning feature list (Intel)

Scope: Review the CRs in R2-2109679, R2-2109680, R2-2109681, R2-2110172, and R2-2110173, and draft a response to RAN1 indicating where we have corrected the implementation of the changes.

Intended outcome: Agreed CRs and approved LS

Deadline:  Tuesday 2021-11-09 0900 UTCs

Stage 2 CRs checked with rapporteur

R2-2110169	Correction to the alignement between stage2 and stage3	Huawei, HiSilicon	CR	Rel-16	38.305	16.6.0	0081	-	F	NR_pos-Core

Discussion:

vivo are generally fine with the corrections, but want to clarify that the periodical SRS should be treated as released rather than deactivated in section 8.10.2.4.   Huawei understand that the message from the LMF is a deactivation request and the real release is from the gNB.

Nokia are unsure if the activation/deactivation applies to the periodic case and think the alignment might need to change stage 3 to align with stage 2.

Intel wonder how much stage 3 detail should be reflected in stage 2, and think we could just provide general information.

- Check by email

R2-2110170	Correciton to Event Reporting in RRC_IDLE	Huawei, HiSilicon	CR	Rel-16	38.305	16.6.0	0076	-	F	NR_pos-Core	R2-2107333

Discussion:

Huawei think this alignment is needed with respect to the SA2 spec.

Nokia think we concluded last meeting not to pursue this, and they do not see it as an essential correction.

Ericsson agree with Nokia; the description is well captured in SA2 and we don’t need it here as well.

Qualcomm think we captured the equivalent procedure for RRC_CONNECTED and it makes sense to align.

Intel and ZTE agree this does not need to be captured.

vivo wonder if the proposed CR implies that the event report is the only message supported to be transmitted via EDT.  They are concerned that if we capture this procedure in stage 2, we should capture others such as Request Assistance Data.

Huawei think vivo have a fair question, and according to the SA2 spec EDT is only used to transmit LCS messages.  They understand that it might conceivably be used for LPP messages as well but this is not captured in the current spec.

CATT have the same understanding as Qualcomm that it makes sense to capture this; on the point of LCS and LPP messages, they have the same understanding as Huawei.  They see this as an alignment change.

ZTE see no consensus on the detailed procedure for RRC_INACTIVE, and think this case should be the same (the only difference is the transmission mode), so they do not think it needs to be captured.

- Check by email whether to capture anything (content appears to be OK if we want to have a CR)

[AT116-e][619][POS] Stage 2 Rel-16 positioning CRs (Huawei)

Scope: Check the CRs in R2-2110169 and R2-2110170.

Intended outcome: Agreed CRs

Deadline:  Tuesday 2021-11-09 0800 UTC

R2-2110728	Corrections on defintions and scope of information transfer	Ericsson	CR	Rel-16	38.305	16.6.0	0083	-	F	NR_pos-Core

Discussion:

Ericsson consider that the update of definitions is important; there is also a small correction in section 8.9.2.

Nokia think this is a repeat of a discussion from last meeting and they do not see the differentiation in the definitions as essential.  If anything needs to be qualified it can be described as LTE or NR.  The change to section 8.9.2 looks editorial and could be merged.

Ericsson think we made a parallel correction already in stage 3 to introduce the DL-PRS terminology, and it would be good to align.

Intel agree we made this change in stage 3, so they agree with the DL-PRS change, but they are not sure about the change to “UL-SRS” since SRS is always in UL.

Qualcomm agree with Intel, but think we use the term “UL-SRS” consistently in other places.

Nokia acknowledge the change is not harmful.

Apple support the change.

=> Agreed

### 6.3.2	RRC corrections

Including impact to 36.331, 38.331, and 38.306.

This agenda item may use a summary document (decision to be made based on submitted tdocs).

R2-2110172	Correction to posSRS capability associated with PRS-only TP	Huawei, HiSilicon	CR	Rel-16	38.306	16.6.0	0648	-	F	NR_pos-Core

=> Handled in email discussion [AT116-e][616]

### 6.3.3	LPP corrections

This agenda item may use a summary document (decision to be made based on submitted tdocs).

R2-2110173	Correction to posSRS and PRS capability associated with PRS-only TP	Huawei, HiSilicon	CR	Rel-16	37.355	16.6.0	0322	-	F	NR_pos-Core

=> Handled in email discussion [AT116-e][616]

R2-2111072	Correction on BDS B2I clock model	Swift Navigation, Ericsson	CR	Rel-16	37.355	16.6.0	0323	-	F	NR_pos

Discussion:

CATT want to explain three points.  (1) From a requirement perspective, only B1i and B1c were proposed in 3GPP and there is no strong requirement on B2I signal from the BDS ecosystem in China from their understanding.  (2) From metadata perspective, they are concerned that the metadata for this clock model may not be provided to the server.  (3) However, if there is a strong requirement from some market for B2I, they can accept the change.

Nokia are OK with the CR if it matches the ICD, but have some editorial comments (WI code, impact analysis).  Qualcomm agree with Nokia, and they think it makes technical sense to have the change similar to what was captured for other GNSSs.

Swift think LPP should faithfully represent the ICD and we should not make judgements on the priority of the different signals.

Intel point out another editorial issue: -r17 should be -r16.

- Revised in R2-2111366 to address the issues above [CB Tuesday 2021-11-09]

[AT116-e][617][POS] Correction on BDS B2I clock model (Swift)

Scope: Check and update the CR in R2-2111072.

Intended outcome: Agreeable CR

Deadline:  Tuesday 2021-11-09 0800 UTC

R2-2111198	Discussion on LPP segmentation in LCS message	vivo	discussion	Rel-16	NR_pos-Core

Discussion:

Chair wonders if this can be driven by contributions in SA2.  vivo think this would be acceptable but in discussion last meeting, the majority wanted to send an LS to SA2.

Nokia think this should be handled by contributions in SA2.

Qualcomm think this is a Rel-17 issue that does not exist in Rel-16, because EDT is one-shot and segmentation cannot apply.  They think it should be discussed in Rel-17 in the context of SDT and subsequent UL data transmission only.

Intel understand that LPP segmentation has existed for a long time, so this is not a new issue and not specific to EDT, but they agree it could be started in SA2.

Ericsson indicate that the SA2 spec does not allow multiple LCS messages to be sent, so LPP segmentation would require carrying multiple LPP segments in one LCS message; they agree that this should be discussed directly in SA2.

=> Noted (can be originated in SA2)

### 6.3.4	MAC corrections

R2-2110171	Discussion on impacts of TA expiry and SR failure on uplink positoning	Huawei, HiSilicon	discussion	NR_pos-Core

=> Revised in R2-2111272

R2-2111272	Discussion on impacts of TA expiry and SR failure on uplink positoning	Huawei, HiSilicon	discussion	NR_pos-Core

Proposal1: RAN 2 should downselect from the following two options for posSRS at TA expiry or SR failure

	Option1: UE releases posSRS configuration

	Option2: UE keeps the posSRS configuration

Proposal2: If posSRS configuration is kept at TA expiry and SR failure, add to the list of triggers for RACH procedure in the stage2 spec TS 38.300 that random access procedure can be triggered when UL is non-synchronized when the UE is transmitting positioning SRS for uplink positioning. Adopt the TP in Section 6.1

Proposal3: If posSRS configuration is released at TA expiry or SR failure, clarify in the MAC spec that the SRS in the spec includes both mimoSRS and posSRS. Adopt the TP in section 6.2.

Discussion:

vivo think posSRS should be treated the same as normal SRS, i.e. the gNB will update the TA and if the TA timer expires the UE shall consider itself no longer required to send posSRS (option 1).

Qualcomm think this issue is not specific to TA expiry and SR failure; the real question is whether regular SRS rules apply to posSRS.  They understand that the same rules do apply because there is no exception written into the MAC spec; in the past we have asked RAN4 about the applicability of DRX-related conditions, and they indicated that the same rules apply but we did not update our spec.  Where there is a divergence in PHR handling, we documented the difference, and they think this is the only such case.  They also think it is too late to change this in Rel-16.

Ericsson agree with Qualcomm.

Intel agree with the argument from vivo and Qualcomm, and point out that normal SRS can also be used for positioning measurements, which argues for consistency.

Samsung think there is no need to handle posSRS differently.

CATT agree with other companies that we should go with option 1.

Huawei think option 1 is OK, but they have one caveat: SR failure just means the UE cannot send SR to the gNB, which impacts scheduling but has no obvious relation to positioning.  So they understand that SR failure is not a good motivation to release posSRS.  However, they also understand Qualcomm’s point that it is late, and they can accept option 1.  They think a CR may still be needed to add a note for clarification (section 6.2 of the contribution).

Ericsson think we discussed previously if any spec text was needed and there was a majority view to have no impact.  They think it is clear that posSRS follows the behaviour of normal SRS.

Nokia see some value in the clarification, but think it could be more specific about its relation to the MAC procedure.  They think a NOTE is sufficient.

Qualcomm also think a NOTE could be helpful, and would suggest to add “unless explicitly stated otherwise” (as we do for other notes, and this would account for the PHR case).

Agreements:

Capture a NOTE in TS 38.321 indicating that posSRS is treated the same as SRS unless specified otherwise.  CR to be seen in CB session.

[AT116-e][618][POS] CR to 38.321 on posSRS handling (Huawei)

Scope: Draft a CR to 38.321 capturing the NOTE agreed under agenda item 6.3.4.

Intended outcome: Agreeable CR in R2-2111367

Deadline:  Tuesday 2021-11-09 0800 UTC

# 7	Rel-16 EUTRA Work Items

Only essential corrections. No documents should be submitted to 7. Please submit to 7.x

## 7.5	LTE Positioning

(NavIC, LTE TEI16 Positioning)

Documents in this agenda item will be handled by email.  No web conference is planned for this agenda item.

# 8	Rel-17 NR Work Items

## 8.7	NR Sidelink relay

(NR_SL_Relay-Core; leading WG: RAN2; REL-17; WID: RP-212601)

Time budget: 2 TU

Tdoc Limitation: 7 tdocs

Email max expectation: 7 threads

### 8.7.1	Organizational

Incoming LSs, TS updates, rapporteur inputs.  This AI is reserved for rapporteur and organizational inputs.  Documents in this AI do not count towards the tdoc limitation.

Incoming LSs and related documents

R2-2109303	Reply LS on establishment/resume cause value and UAC on L2 SL Relay (C1-214795; contact: OPPO=	CT1	LS in	Rel-17	5G_ProSe, NR_SL_relay-Core	To:RAN2	Cc:SA2, RAN3

R2-2111236	Reply LS on discovery and relay (re)selection (S2-2107972; contact: CATT)	SA2	LS in	Rel-17	5G_ProSe, NR_SL_relay-Core	To:RAN2

R2-2111123	Discussion on LS on discovery and relay (re)selection	OPPO	discussion	Rel-17	NR_SL_relay-Core

R2-2111253	Discussion on LS on discovery and relay (re)selection	CATT	discussion	Late

Organisational documents

R2-2109399	Work planning for R17 SL relay	OPPO, CMCC	Work Plan	Rel-17	NR_SL_relay-Core

R2-2109401	Remaining open issues for R17 SL relay	OPPO	discussion	Rel-17	NR_SL_relay-Core	Late

Running CRs

R2-2109400	Running CR for TS 38.351	OPPO	draft TS	Rel-17	38.351	0.0.0	NR_SL_relay-Core

R2-2109543	Stage 2 Running CR on Introduction of R17 SL Relay	MediaTek Inc.	discussion	Rel-17

R2-2110054	MAC running CR for SL relay	Apple (rapporteur)	draftCR	Rel-17	38.321	16.6.0	B	NR_SL_relay-Core	Late

R2-2110447	Running CR of 38.323 for SL Relay	Samsung	draftCR	Rel-17	38.323	16.5.0	B	NR_SL_relay-Core

R2-2110490	RRC running CR for SL relay	Huawei, HiSilicon	draftCR	Rel-17	38.331	16.6.0	B	NR_SL_relay-Core

R2-2110687	Running CR of 38.304 for SL relay	Ericsson	draftCR	Rel-17	38.304	16.6.0	B	NR_SL_relay-Core

### 8.7.2	L2 relay specific topics

No documents should be submitted to 8.7.2.  Please submit to 8.7.2.x.

#### 8.7.2.1	Control plane procedures

Including connection management, SI delivery, paging, access control for remote UE.  This agenda item will utilise a summary document.

Including outcome of [Post115-e][610][Relay] Control plane procedures (InterDigital)

Email discussion summary

R2-2109928	Summary of [POST115-e][610][Relay] Control Plane Procedures (InterDigital)	InterDigital	discussion	Rel-17	FS_NR_SL_relay

Summary document

The following documents will not be individually treated

R2-2109414	Discussion on Control Plane Aspects for L2 Relay	OPPO	discussion	Rel-17	NR_SL_relay-Core

R2-2109419	Remaining issues on paging and SIB forwarding in L2 U2N relay	Qualcomm Incorporated	discussion	NR_SL_relay-Core

R2-2109427	Remaining issues on RRC connection management of L2 U2N relay	Qualcomm Incorporated	discussion	NR_SL_relay-Core

R2-2109507	Control Plane Procedures of L2 Relay	CATT	discussion	Rel-17	NR_SL_relay-Core

R2-2109508	Discussion on Remote UE's Paging via Dedicated RRC Message	CATT	discussion	Rel-17	NR_SL_relay-Core

R2-2109544	Discussion on SI Modification and PWS Notification	MediaTek Inc.	discussion	Rel-17

R2-2109545	Remaining issue for RLF handling	MediaTek Inc.	discussion	Rel-17

R2-2109556	Discussion on RRC connection management for L2 sidelink relay	Huawei, HiSilicon	discussion	Rel-17	NR_SL_relay-Core

R2-2109557	SI forwarding and paging for L2 sidelink relay	Huawei, HiSilicon	discussion	Rel-17	NR_SL_relay-Core

R2-2109644	Discussion on left issue for paging delivery	SHARP Corporation	discussion	NR_SL_relay-Core

R2-2109696	SI forwarding	NEC Corporation	discussion	Rel-17	NR_SL_relay-Core

R2-2109729	Monitoring Paging by a U2N Relay	Lenovo, Motorola Mobility	discussion	NR_SL_relay-Core

R2-2109763	Discussion on system information delivery open issues	China Telecom	discussion	Rel-17	NR_SL_relay-Core

R2-2109811	SIB handling in sidelink L2 U2N relay	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_SL_relay-Core	R2-2105739

R2-2109859	Consideration on the connection management of SL relay	ZTE, Sanechips	discussion	Rel-17

R2-2109860	Consideration on the system information acquisition and paging in SL relay	ZTE, Sanechips	discussion	Rel-17

R2-2109929	Open Issues on Paging Procedure for L2 UE to NW Relays	InterDigital	discussion	Rel-17	FS_NR_SL_relay

R2-2109930	Open Issues on SI for L2 UE to NW Relays	InterDigital	discussion	Rel-17	FS_NR_SL_relay

R2-2109934	Connection Establishment Procedure for L2 UE to NW Relays	InterDigital	discussion	Rel-17	FS_NR_SL_relay

R2-2109959	Remaining issues of system information forwarding for L2 U2N Remote UE	Intel Corporation	discussion	Rel-17	NR_SL_relay-Core

R2-2109964	Access control support for L2 U2N Relay	Intel Corporation	discussion	Rel-17	NR_SL_relay-Core

R2-2110064	Remaining issues on SIB forwarding	Apple	discussion	Rel-17	NR_SL_relay-Core

R2-2110065	RNA Update via L2 UE-to-NW Relay	Apple	discussion	Rel-17	NR_SL_relay-Core

R2-2110121	Discussion on control plane procedures for L2 U2N relay	Spreadtrum Communications	discussion	Rel-17

R2-2110163	Control plane procedure - SIB delivery, and timer for remote UE	LG Electronics France	discussion	Rel-17	NR_SL_relay

R2-2110165	L2 relay control plane issues 	Kyocera	discussion

R2-2110213	Open issues on L2 Control Plane Procedures	vivo	discussion

R2-2110215	Draft LS on L2 U2N relay issues	vivo	LS out	To:SA2, CT1

R2-2110221	Discussion on SI and short message delivery	Xiaomi	discussion

R2-2110222	Discussion on connection control	Xiaomi	discussion

R2-2110284	Discussion on access control of L2 relay	SHARP Corporation	discussion

R2-2110303	Considerations on control plane issues	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2110350	Area specific SI issue in L2 relay	Sony	discussion	Rel-17	NR_SL_relay-Core

R2-2110363	Discussion on establishment cause of relay UE	Xiaomi, Apple, Lenovo, Motorola Mobility	discussion

R2-2110448	Connection management and PC5/Uu RLC configurations	Samsung	discussion	Rel-17	NR_SL_relay-Core

R2-2110449	Remaining issues for SI message forwarding	Samsung	discussion	Rel-17	NR_SL_relay-Core

R2-2110450	Remaining issues for paging delivery	Samsung	discussion	Rel-17	NR_SL_relay-Core

R2-2110470	Issue with Forwarding SIB9 to remote UE	Nokia, Nokia Shanghai Bell	discussion	NR_SL_relay-Core

R2-2110688	Remaining issues on control plane for L2 sidelink relay	Ericsson	discussion	Rel-17	NR_SL_relay-Core

R2-2111003	Discussion on paging procedure and information for U2N Relay	ASUSTeK	discussion	Rel-17	NR_SL_relay-Core

R2-2111029	Relayed System Information Acquisition	Futurewei	discussion	Rel-17	NR_SL_relay-Core

R2-2111190	SI acquisition, CN Registration and RNAU	Lenovo, Motorola Mobility	discussion	Rel-17	NR_SL_relay-Core

#### 8.7.2.2	Service continuity

Service continuity between Uu and relay paths, limited to intra-gNB cases.  This agenda item will utilise a summary document.

Summary document

R2-2111365	Summary of Agenda item 8.7.2.2: Service continuity	Huawei, HiSilicon	discussion	Rel-17	NR_SL_relay-Core

The following documents will not be individually treated

R2-2109428	Remaining issues on service continuity of L2 U2N relay	Qualcomm Incorporated	discussion	NR_SL_relay-Core

R2-2109509	Service Continuity for L2 U2N Relay	CATT	discussion	Rel-17	NR_SL_relay-Core

R2-2109546	Remaining open issues for Service Continuity	MediaTek Inc.	discussion	Rel-17

R2-2109705	remaining issues on service continuity	NEC Corporation	discussion	Rel-17	NR_SL_relay-Core

R2-2109780	Discussion on remaining issues on service continuity	ZTE Corporation, Sanechips	discussion	Rel-17	NR_SL_relay-Core

R2-2109933	Open Issues on Service Continuity for L2 UE to NW Relays	InterDigital	discussion	Rel-17	FS_NR_SL_relay

R2-2109962	Service continuity left over issues for L2 U2N relaying	Intel Corporation	discussion	Rel-17	NR_SL_relay-Core

R2-2110059	Discussion on U2N Relay UE Identifier	Apple	discussion	Rel-17	NR_SL_relay-Core

R2-2110060	[Draft]LS on U2N relay UE identifier	Apple	LS out	Rel-17	NR_SL_relay-Core	To:SA2

R2-2110066	Discussion on remaining issues of service continuity	Apple	discussion	Rel-17	NR_SL_relay-Core

R2-2110164	Service continuity – depending on relay state	LG Electronics France	discussion	Rel-17	NR_SL_relay

R2-2110214	Remaining issues on service continuity in L2 U2N relay	vivo	discussion

R2-2110220	Discussion on service continuity	Xiaomi	discussion

R2-2110302	Path switching in L2 U2N relay case	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2110351	Service continuity open issues in L2 NR sidelink rela	Sony	discussion	Rel-17	NR_SL_relay-Core

R2-2110371	Discussion on supported relay UE RRC states in direct to indirect path switch	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_SL_relay-Core

R2-2110488	Discussion on service continuity for L2 U2N Relay	Huawei, HiSilicon	discussion	Rel-17	NR_SL_relay-Core

R2-2110499	Discussion on NR sidelink relay service continuity	OPPO	discussion	Rel-17	NR_SL_relay-Core

R2-2110689	Discussion on selecting relay UE in RRC_IDLE or INACTIVE during path switch	Ericsson	discussion	Rel-17	NR_SL_relay-Core

R2-2110690	Remaining Issues on service continuity for L2 Sidelink relay	Ericsson	discussion	Rel-17	NR_SL_relay-Core

R2-2111042	Service continuity for L2 relay	CMCC	discussion	Rel-17	NR_SL_relay-Core

#### 8.7.2.3	Adaptation layer design

Including bearer mapping, remote UE identification, security aspects if any.  This agenda item will utilise a summary document.

Summary document

The following documents will not be individually treated

R2-2109398	Left issues for adaptation layer	OPPO	discussion	Rel-17	NR_SL_relay-Core

R2-2109429	Further discussion on adaptation layer of L2 U2N relay	Qualcomm Incorporated	discussion	NR_SL_relay-Core

R2-2109510	Adaption Layer Design for L2 U2N Relay	CATT	discussion	Rel-17	NR_SL_relay-Core

R2-2109547	Configurations for Bearer Mapping	MediaTek Inc.	discussion	Rel-17

R2-2109558	Adaptation layer functionalities for L2 U2N relay	Huawei, HiSilicon	discussion	Rel-17	NR_SL_relay-Core

R2-2109693	Remaining issues of Adaptation layer	MediaTek Inc.	discussion	Rel-17

R2-2109848	Bearer Mapping Configuration of Adaptation Layer	Futurewei	discussion	Rel-17	NR_SL_relay-Core

R2-2109862	Discussion on adaptation layer design	ZTE, Sanechips	discussion	Rel-17

R2-2109906	UP aspects on Layer 2 SL relay	Ericsson	discussion	Rel-17	NR_SL_relay-Core

R2-2109935	Adaptation Layer Design Remaining Issues	InterDigital	discussion	Rel-17	FS_NR_SL_relay

R2-2109963	L2 U2N relaying Adaptation layer design open aspects	Intel Corporation	discussion	Rel-17	NR_SL_relay-Core

R2-2110216	Adaptation Layer for Uu and PC5	vivo	discussion

R2-2110376	Finalizing design of Adapt layer	Samsung Electronics GmbH	discussion

R2-2110385	On multiplexing of relay UE and remote UE traffic	Samsung Electronics GmbH	discussion

R2-2110987	Discussion on Adaptation Layer for L2 U2N Relay	ETRI	discussion	Rel-17	NR_SL_relay-Core

R2-2111004	Discussion on bearer mapping on PC5 adaptation layer	ASUSTeK	discussion	Rel-17	38.300	NR_SL_relay-Core

R2-2111041	Discussion on adaption layer for L2 U2N relay	CMCC	discussion	Rel-17	NR_SL_relay-Core

#### 8.7.2.4	QoS

Mechanisms for E2E QoS management.  This AI will be treated on a time-available basis.  This agenda item will utilise a summary document.

Including outcome of [Post115-e][604][Relay] Relay QoS (Apple)

Email discussion summary

R2-2110053	Summary of [Post115-e][604][Relay] Relay QoS (Apple)	Apple	discussion	Rel-17	NR_SL_relay-Core

Summary document

R2-2111273	Summary of Agenda item 8.7.2.4: QoS	Qualcomm Incorporated	discussion	NR_SL_relay-Core

The following documents will not be individually treated

R2-2109433	Remaining issues on E2E QoS enforcement in L2 U2N relay	Qualcomm Incorporated	discussion	NR_SL_relay-Core

R2-2109511	QoS Management for L2 Sidelink Relay	CATT	discussion	Rel-17	NR_SL_relay-Core

R2-2109691	Views on QoS for sidelink relay	Continental Automotive GmbH	other	Rel-17

R2-2109822	Considerations on voice and video support for Relays	Philips International B.V., MediaTek, Vivo, FirstNet	discussion	Rel-17	NR_SL_relay-Core

R2-2109853	QoS measurement and reporting for path switch procedure	Nokia, Nokia Shanghai Bell	discussion	NR_SL_relay-Core

R2-2109863	Discussion on QoS of SL relay	ZTE, Sanechips	discussion	Rel-17

R2-2109905	Aspects for QoS management with SL relay	Ericsson	discussion	Rel-17	NR_SL_relay-Core

R2-2109931	Discussion on QoS for L2 UE to NW Relays	InterDigital, Philips	discussion	Rel-17	FS_NR_SL_relay

R2-2110217	Left issues on E2E QoS management	vivo	discussion

R2-2110272	On recommended bit rate	MediaTek Inc.	discussion	Rel-17	NR_SL_relay-Core

R2-2110297	QoS for L2 Sidelink Relay	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-17

R2-2110451	QoS flow control for L2 U2N Relay	Samsung, Philips	discussion	Rel-17	NR_SL_relay-Core	R2-2107712

R2-2110498	Discussion on QoS for layer 2 relay	OPPO	discussion	Rel-17	NR_SL_relay-Core

R2-2110562	Discussion on QoS management of L2 U2N relay	Huawei, HiSilicon	discussion	Rel-17	NR_SL_relay-Core

R2-2110750	QoS priority mapping combinations	Beijing Xiaomi Mobile Softwar	discussion	Rel-17

R2-2111040	Mechanisms for E2E QoS management	CMCC	discussion	Rel-17	NR_SL_relay-Core

### 8.7.3	L2/L3 common topics

For any remaining stage 3 issues related to discovery and (re)selection.  No documents should be submitted to 8.7.3.  Please submit to 8.7.3.x.

#### 8.7.3.1	Discovery

Including 5G ProSe Direct Discovery for the non-relaying case.  Re-using LTE discovery as baseline.  This agenda item may utilise a summary document (decision to be made based on submitted tdocs).

Including outcome of [Post115-e][611][Relay] Discovery shared/dedicated pool issue (Qualcomm)

Email summary

R2-2109430	Summary report of [Post115-e][611][Relay] Discovery shared and dedicated pool issue (Qualcomm)	Qualcomm Incorporated	discussion	NR_SL_relay-Core

Summary document

R2-2111255	Summary of AI 8.7.3.1	CATT	discussion

[AT116-e][612][Relay] Non-relay discovery (OPPO)

Scope: Evaluate the spec impact of non-relay discovery specific aspects and determine a way forward for handling this objective.

Intended outcome: Report to CB session, in R2-2111363

Deadline:  Tuesday 2021-11-09 0800 UTC (report available)

The following documents will not be individually treated

R2-2109431	Remaining issues on discovery	Qualcomm Incorporated	discussion	NR_SL_relay-Core

R2-2109512	Left Issues for Sidelink Discovery	CATT	discussion	Rel-17	NR_SL_relay-Core

R2-2109809	Discussion on SL discovery resource pool configuration	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_SL_relay-Core

R2-2109857	Further discussion on Relay discovery	ZTE, Sanechips	discussion	Rel-17

R2-2109903	Left issues for SL discovery	Ericsson	discussion	Rel-17	NR_SL_relay-Core

R2-2109932	Using Shared and Dedicated Resource Pools for Discovery	InterDigital	discussion	Rel-17	FS_NR_SL_relay

R2-2109960	Leftover aspects of discovery for L2 U2N relaying	Intel Corporation	discussion	Rel-17	NR_SL_relay-Core

R2-2110218	Remaining Issues of Discovery Message Transmission	vivo	discussion

R2-2110271	Remaining issues of Relay Discovery	MediaTek Inc.	discussion	Rel-17	NR_SL_relay-Core

R2-2110304	Relay Discovery for L2 and L3 relay	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2110452	PDCP layer aspects for SL discovery	Samsung	discussion	Rel-17	NR_SL_relay-Core

R2-2110489	Remaining issues on relay discovery	Huawei, HiSilicon	discussion	Rel-17	NR_SL_relay-Core

R2-2110500	Discussion on common issues for relay and non-relay discovery	OPPO	discussion	Rel-17	NR_SL_relay-Core

R2-2110501	Discussion on non-relay discovery	OPPO, Apple, Samsung, Ericsson, Qualcomm	discussion	Rel-17	NR_SL_relay-Core

R2-2110749	Discovery Range for 5G ProSe Direct Discovery	Beijing Xiaomi Mobile Software	discussion	Rel-17

R2-2110751	Discovery with simultaneous Shared and Dedicated Resource Pools	Beijing Xiaomi Mobile Softwar	discussion	Rel-17

#### 8.7.3.2	Relay re/selection

Re-using LTE re/selection as baseline. This agenda item may utilise a summary document (decision to be made based on submitted tdocs).

Summary document

R2-2111223	Summary of AI 8.7.3.2 Relay (re)selection	vivo	discussion	Rel-17	NR_SL_relay-Core

The following documents will not be individually treated

R2-2109432	 Remaining issues on relay (re)selection	Qualcomm Incorporated	discussion	NR_SL_relay-Core

R2-2109513	New Triggers for Relay Reselection	CATT	discussion	Rel-17	NR_SL_relay-Core

R2-2109823	U2N Relay UE operation Threshold Conditions: Impact of UE Mobility	Philips International B.V.	discussion	Rel-17	NR_SL_relay-Core

R2-2109858	Further discussion on Relay selection	ZTE, Sanechips	discussion	Rel-17

R2-2109904	Aspects for  SL relay selection and reselection	Ericsson	discussion	Rel-17	NR_SL_relay-Core

R2-2109961	Open aspects of L2 U2N Relay (re)selection	Intel Corporation	discussion	Rel-17	NR_SL_relay-Core

R2-2110166	Relay reselection upon HO to another gNB 	Kyocera	discussion

R2-2110219	Remaining issues on Relay (re)selection	vivo	discussion

R2-2110285	Discussion on sidelink relay reselection	SHARP Corporation	discussion	R2-2107872

R2-2110305	Relay (re)selection for L2 and L3 relay	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2110370	Uu connection error handling	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_SL_relay-Core

R2-2110502	Discussion on remaining issue of relay reselection	OPPO	discussion	Rel-17	NR_SL_relay-Core

R2-2110617	Discussion on relay reselection aspects	Huawei, HiSilicon	discussion	NR_SL_relay-Core

R2-2110767	Support of idle mode mobility for remote-UE in SL UE-to-Nwk relay	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_SL_relay-Core	R2-2108462

## 8.11	NR positioning enhancements

(NR_pos_enh-Core; leading WG: RAN1; REL-17; WID: RP-210903)

Time budget: 2 TU

Tdoc Limitation: 7 tdocs

Email max expectation: 7 threads

### 8.11.1	Organizational

Rapporteur input. Incoming LS etc. This AI is reserved for rapporteur and organizational inputs; documents in this AI do not count towards the tdoc limitation.

Incoming LSs with RAN2 in Cc:

R2-2109316	Reply LS on determination of location estimates in local co-ordinates (R1-2108509; contact: Ericsson)	RAN1	LS in	Rel-17	5G_eLCS_ph2	To:SA2	Cc:RAN2, RAN3

R2-2109339	Reply LS on determination of location estimates in local co-ordinates (R3-214312; contact: Huawei)	RAN3	LS in	Rel-17	5G_eLCS_ph2	To:SA2	Cc:RAN1, RAN2

R2-2111216	LS on DL PRS reception priority by RRC_INACTIVE UEs (R1-2110644; contact: Intel)	RAN1	LS in	Rel-17	NR_pos_enh-Core	To:RAN4	Cc:RAN2

LS from RTCM

R2-2109392	Liaison Note to 3GPP RAN 2, Reply comments to letter R2-2106596 (RTCM Paper 2021-SC134-0113)	RTCM	LS in	To:RAN2

R2-2109807	Discussion RTCM reply to RAN2 on GNSS integrity coordination	ESA, Intel Corporation	discussion	Rel-17	FS_NR_pos_enh

[AT116-e][611][POS] LS to RTCM (ESA)

Scope: Discuss coordination with RTCM, taking into account the way-forward proposals in R2-2109807 and related parts of R2-2110181:

- Conclude on the intention to specify GNSS integrity signalling in Rel-17

- Determine what information we intend to share with RTCM

- Draft an LS reply (TP to be endorsed later)

Intended outcome: Report in R2-2111361 and approvable LS in R2-2111362

Deadline:  Friday 2021-11-05 1000 UTC (comments), Monday 2021-11-08 1100 UTC (output available)

Other incoming LSs with RAN2 in To:

R2-2109322	LS to RAN2 on SRS for Positioning Transmission by UEs in RRC_INACTIVE State (R1-2108564; contact: Intel)	RAN1	LS in	Rel-17	NR_pos_enh-Core	To:RAN2

R2-2109328	LS on PRS measurement outside the measurement gap (R1-2108639; contact: Huawei)	RAN1	LS in	Rel-17	NR_pos_enh-Core	To:RAN2, RAN3, RAN4

R2-2109329	LS on beam/antenna information for DL AOD in NR positioning (R1-2108646; contact: Ericsson)	RAN1	LS in	Rel-17	NR_pos_enh-Core	To:RAN2, RAN3

R2-2109345	Reply LS on Positioning Reference Units (R3-214457; contact: Ericsson)	RAN3	LS in	Rel-17	NR_pos_enh-Core	To:RAN1, RAN2	Cc:SA2

R2-2111211	LS on support of SP-SRS for positioning by RRC_INACTIVE UEs (R1-2110598; contact: Intel)	RAN1	LS in	Rel-17	NR_pos_enh-Core	To:RAN2

Draft replies

R2-2109480	[Draft] Response LS on the Positioning Reference Units (PRUs) for positioning enhancement	CATT	LS out	Rel-17	To:RAN1,SA2	Cc:RAN3

R2-2110803	Beam/antenna information for DL AOD in NR positioning	Ericsson	discussion	Rel-17

Running CRs and related reports

R2-2109673	Email discussion report on [609][POS] RAT-dependent stage 2 CR (Intel)	Intel Corporation	discussion	Rel-17	NR_pos_enh-Core

R2-2109674	Email discussion [609] Running 38.305 CR for Positioning WI on RAT dependent positioning methods	Intel Corporation	draftCR	Rel-17	38.305	16.6.0	B	NR_pos_enh-Core

R2-2110997	Email discussion report on [614][POS] GNSS Positioning Integrity Stage 2 CR (InterDigital)	InterDigital, Inc.	discussion	Rel-17	NR_pos_enh-Core

R2-2111012	Running CR of 38.305 for GNSS Positioning Integrity	InterDigital, Inc.	draftCR	Rel-17	38.305	16.6.0	B	NR_pos_enh-Core

R2-2111013	Running CR of 36.305 for GNSS Positioning Integrity	InterDigital, Inc.	draftCR	Rel-16	36.305	16.4.0	B	NR_pos_enh-Core

### 8.11.2	Latency enhancements

Enhancements of signalling, and procedures for improving positioning latency of the Rel-16 NR positioning methods, for DL and DL+UL positioning methods.  This agenda item will utilise a summary document.

Including outcome of [Post115-e][605][POS] Pre-configured assistance data (Intel)

Email discussion summary

R2-2109665	Summary of [Post115-e][605][POS] Pre-configured assistance data (Intel)	Intel Corporation	discussion	Rel-17	NR_pos_enh-Core

Summary document

R2-2111252	Summary of agenda 8.11.2: Latency enhancements	Samsung	discussion

The following documents will not be individually treated

R2-2109460	Discussion on positioning latency reduction	ZTE	discussion

R2-2109481	Discussion on Enhancements for Latency Reduction	CATT	discussion	Rel-17	NR_pos_enh-Core

R2-2109663	Leftover issues on Latency reduction	Intel Corporation	discussion	Rel-17	NR_pos_enh-Core

R2-2109824	Positioning Latency Reduction Enhancements	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2109915	Time T and Measurement Gap for Measurement Time Window	Ericsson	discussion	Rel-17

R2-2109978	Discussion on latency enhancement	vivo	discussion	Rel-17	NR_pos_enh-Core

R2-2110103	Further consideration of positioning latency enhancements	OPPO	discussion	Rel-17	NR_pos_enh-Core

R2-2110178	Discussion on latency reduction techniques from other groups	Huawei, HiSilicon	discussion	NR_pos_enh-Core

R2-2110179	Text Proposal for finer granularity of responseTime	Huawei, HiSilicon	discussion	NR_pos_enh-Core

R2-2110180	Discussion on pre-configured PRS	Huawei, HiSilicon	discussion	NR_pos_enh-Core

R2-2110336	Discussion on the response time	Samsung	discussion	Rel-17	NR_pos_enh-Core

R2-2110359	Considerations on positioning latency	Sony	discussion	NR_pos_enh-Core

R2-2110798	PRS Measurements outside measurement Gap	Ericsson	discussion	Rel-17

R2-2110822	Remaining Issues on Scheduling Location in Advance 	Qualcomm Incorporated	discussion

R2-2110928	Discussion on Enhancements for Latency Reduction 	InterDigital, Inc.	discussion	NR_pos_enh

R2-2111075	Discussion on the priority rule for latency reduction	CMCC	discussion	Rel-17	NR_pos_enh-Core

R2-2111081	Simulation study for multiple QoS class handling for latency reduction	Samsung Electronics	discussion	NR_pos_enh-Core

R2-2111083	Handling of multiple QoS for latency reduction	Samsung Electronics	discussion	NR_pos_enh-Core

R2-2111084	Discussion on the Pre-configured Assistance Data	Samsung Electronics	discussion	NR_pos_enh-Core

R2-2111086	Latency reduction via configured grant for positioning	Samsung Electronics	discussion	NR_pos_enh-Core

R2-2111105	Positioning enhancements on latency reduction	Xiaomi	discussion

### 8.11.3	RRC_INACTIVE

Methods, measurements, signalling and procedures to support positioning for UEs in RRC_ INACTIVE state, for UE-based and UE-assisted positioning solutions.  UL and DL+UL NR positioning methods and gNB positioning measurements for UEs in RRC_INACTIVE are treated at lower priority.  This agenda item will utilise a summary document.

Including outcome of [Post115-e][608][POS] PRS configuration and measurement in RRC_INACTIVE (vivo)

Email discussion summary

R2-2109979	Summary of [Post115-e][608][POS] PRS configuration and measurement in RRC_INACTIVE	vivo	discussion	Rel-17	NR_pos_enh-Core

Summary document

R2-2111251	Summary for AI 8.11.3 on positioning in RRC_INACTIVE	OPPO	discussion

The following documents will not be individually treated

R2-2109461	Discussion on positioning in RRC INACTIVE state	ZTE	discussion

R2-2109482	Discussion on UL NR positioning in RRC_INACTIVE	CATT	discussion	Rel-17	NR_pos_enh-Core	Withdrawn

R2-2109758	Supporting positioning in RRC_INACTIVE state	OPPO	discussion	Rel-17	NR_pos_enh-Core

R2-2109759	Discussion on UL Positioning methods in RRC_INACTIVE state	OPPO	discussion	Rel-17	NR_pos_enh-Core

R2-2109825	On Positioning in RRC_INACTIVE state	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2109918	Discussion on RRC Inactive mode Positioning	Ericsson	discussion	Rel-17

R2-2109980	Discussion on UL positioning in RRC_INACTIVE	vivo	discussion	Rel-17	NR_pos_enh-Core

R2-2110021	Support of UL&UL+DL positioning in RRC_INACTIVE	Intel Corporation	discussion	Rel-17	NR_pos_enh-Core

R2-2110174	Way-forward for RRC_INACTIVE positioning	Huawei, CATT, China Unicom, CMCC, Fraunhofer, Futurewei, HiSilicon, Intel Corporation, Spreadtrum Communications, OPPO, VIVO, Xiaomi, ZTE Corporation	discussion	NR_pos_enh-Core

R2-2110249	UE Positioning in RRC_INACTIVE mode	Fraunhofer IIS; Fraunhofer HHI	discussion	Rel-17

R2-2110337	Discussion on the measurement reporting in RRC_INACTIVE	Samsung	discussion	Rel-17	NR_pos_enh-Core

R2-2110360	Considerations on positioning RRC Inactive	Sony	discussion	Rel-17	NR_pos_enh-Core

R2-2110823	Remaining issues for positioning of UEs in RRC_INACTIVE State 	Qualcomm Incorporated	discussion

R2-2110824	[draft] LS on DL-only and RAT-Independent Positioning in RRC_INACTIVE State 	Qualcomm Incorporated	LS out	To:SA2	Cc:RAN3

R2-2110929	Discussion on Positioning in RRC INACTIVE state	InterDigital, Inc.	discussion	NR_pos_enh

R2-2110930	Discussion on reporting of positioning information using SDT 	InterDigital, Inc.	discussion	NR_pos_enh

R2-2111076	Considerations on Positioning in RRC_INACTIVE state	CMCC	discussion	Rel-17	NR_pos_enh-Core

R2-2111106	Discussion on positioning for UEs in RRC Inactive	Xiaomi	discussion

### 8.11.4	On-demand PRS

Specify UE-initiated and LMF-initiated on-demand transmission and reception of DL PRS for DL and DL+UL positioning for UE-based and UE-assisted positioning solutions.  This agenda item will utilise a summary document.

Including outcome of [Post115-e][606][POS] MO-LR for on-demand PRS (CATT)

Email discussion summary

R2-2109483	Report of [Post115-e][606][POS] MO-LR for on-demand PRS (CATT)	CATT	discussion	Rel-17	NR_pos_enh-Core

R2-2110966	[Draft] LS on MO-LR for on-demand PRS	CATT	LS out	Rel-17	NR_pos_enh-Core	To:SA2

Summary document

R2-2111256	Summary of Agenda Item 8.11.4: On-demand PRS	Lenovo, Motorola Mobility	discussion

The following documents will not be individually treated

R2-2109462	Discussion on on-demand PRS	ZTE	discussion

R2-2109484	Discussion on on-demand PRS	CATT	discussion	Rel-17	NR_pos_enh-Core

R2-2109664	Support of On-Demand PRS request	Intel Corporation	discussion	Rel-17	NR_pos_enh-Core

R2-2109757	Discussion on on-demand DL-PRS	OPPO	discussion	Rel-17	NR_pos_enh-Core

R2-2109826	Support of On-Demand DL-PRS	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2109916	On demand PRS	Ericsson	discussion	Rel-17

R2-2109981	Discussion on on-demand PRS	vivo	discussion	Rel-17	NR_pos_enh-Core

R2-2110040	Stage-2 procedure for on-demand PRS	Apple	discussion	NR_pos_enh-Core

R2-2110175	Discussion on on-demand PRS	Huawei, HiSilicon	discussion	NR_pos_enh-Core

R2-2110247	On-demand PRS	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-17

R2-2110361	Considerations on positioning PRS On-demand	Sony	discussion	Rel-17	NR_pos_enh-Core

R2-2110825	Remaining issues for on-demand DL-PRS 	Qualcomm Incorporated	discussion

R2-2110931	Discussion on procedures for On-demand PRS for DL-based positioning	InterDigital, Inc.	discussion	NR_pos_enh

R2-2110932	Discussion on procedure for On-demand PRS for DL+UL based positioning	InterDigital, Inc.	discussion	NR_pos_enh

R2-2110956	Clarifications to on-demand PRS Stage 2	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_pos_enh-Core

R2-2110957	UE-initiated on-demand PRS requests	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_pos_enh-Core

R2-2110958	Pre-configured assistance data for on-demand PRS	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_pos_enh-Core

R2-2111090	[Draft] LS on stage-2 on-demand PRS procedure	CATT	LS out	Rel-17	NR_pos_enh-Core	To:RAN3

R2-2111107	Positioning enhancement to on-demand DL PRS	Xiaomi	discussion

### 8.11.5	GNSS positioning integrity

Signalling, and procedures to support GNSS positioning integrity determination.  This agenda item will utilise a summary document.

Including outcome of [Post115-e][607][POS] Integrity assistance data (Huawei)

Email discussion summary

R2-2110181	Summary of [Post115-e][607][POS] Integrity assistance data	Huawei, HiSilicon	discussion	NR_pos_enh-Core

Summary document

R2-2111263	Summary of Agenda item 8.11.5- GNSS positioning integrity	CATT	discussion	Rel-17	NR_pos_enh-Core

The following documents will not be individually treated

R2-2109463	Discussion on positioning integrity	ZTE	discussion

R2-2109920	On GNSS Integrity	Ericsson	discussion	Rel-17

R2-2109982	Discussion on open issues for GNSS positioning integrity	vivo	discussion	Rel-17	NR_pos_enh-Core

R2-2110102	Discussion on supporting positioing integrity in RAN	OPPO	discussion	Rel-17	NR_pos_enh-Core

R2-2110141	Discussion on GNSS Integrity Assistance Data	Swift Navigation, Mitsubishi Electric Corporation, Intel Corporation, Ericsson	discussion	Rel-17

R2-2110176	Remaining issues on positioning integrity	Huawei, HiSilicon	discussion	NR_pos_enh-Core

R2-2110246	UE-aided detection of threat to GNSS systems and assistance data signaling	Fraunhofer IIS; Fraunhofer HHI; Ericsson; ESA	discussion	R2-2107147

R2-2110445	On GNSS Positioning Integrity	Nokia, Nokia Shanghai Bell	discussion	Rel-17	FS_NR_pos_enh

R2-2110933	Discussion on procedures and signalling for GNSS positioning integrity	InterDigital, Inc.	discussion	NR_pos_enh

R2-2111087	Consideration on the signalling design for Positioning Integrity	Samsung Electronics	discussion	NR_pos_enh-Core

R2-2111108	Discussion on GNSS positioning integrity	Xiaomi	discussion

### 8.11.6	A-GNSS enhancements

Including support of BDS B2a and B3I signals and support of NavIC.

R2-2109485	Introduction of B2a and B3I signal in BDS system in A-GNSS	CATT, CAICT	draftCR	Rel-17	36.305	16.4.0	B	NR_pos_enh-Core	R2-2107138

R2-2109486	Introduction of B2a and B3I signal in BDS system in A-GNSS	CATT, CAICT	draftCR	Rel-17	38.305	16.6.0	B	NR_pos_enh-Core	R2-2107139

R2-2109487	Introduction of B2a signal in BDS system in A-GNSS	CATT, CAICT	draftCR	Rel-17	37.355	16.6.0	B	NR_pos_enh-Core	R2-2107140

R2-2109488	Introduction of B3I signal in BDS system in A-GNSS	CATT, CAICT	draftCR	Rel-17	37.355	16.6.0	B	NR_pos_enh-Core	R2-2107141

[AT116-e][613][POS] BDS B2a and B3I signals (CATT)

Scope: Discuss the CRs in R2-2109485, R2-2109486, R2-2109487, and R2-2109488, collect any comments and produce updates if necessary for endorsement.

Intended outcome: Endorsable CRs

Deadline:  Friday 2021-11-05 1000 UTC (comments), Monday 2021-11-08 1100 UTC (output available)

### 8.11.7	Other

Input on other WI objectives.

PRUs

R2-2109489	Discussion on Positioning Reference Units(PRUs)	CATT, ZTE Coroporation, Intel Coroporation 	discussion	Rel-17	NR_pos_enh-Core

R2-2109827	Support of Positioning Reference Units	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2109919	On the Positioning Reference Units aspects	Ericsson	discussion	Rel-17

R2-2109983	Discussion on support for positioning reference unit	vivo	discussion	Rel-17	NR_pos_enh-Core

R2-2110039	Stage-3 impacts of PRU support	Apple	discussion	NR_pos_enh-Core

R2-2110177	Discussion on PRU	Huawei, HiSilicon	discussion	NR_pos_enh-Core

R2-2110826	Remaining issues for Positioning Reference Units 	Qualcomm Incorporated	discussion

R2-2110827	[draft] Response LS on Positioning Reference Units (PRUs) for enhancing positioning performance 	Qualcomm Incorporated	LS out	To:SA2, RAN1	Cc:RAN3

R2-2110934	Discussion on supporting Positioning Reference Units	InterDigital, Inc.	discussion	NR_pos_enh

R2-2111109	Discussion on how to manage PRU	Xiaomi	discussion

[AT116-e][615][POS] PRUs (Qualcomm)

Scope: Discuss the handling of the PRU topic taking the related contributions into account, and determine a way forward.

Intended outcome: Report to positioning session in R2-2111364, and LS out if necessary

Deadline:  Monday 2021-11-08 1000 UTC (report available)

Other

R2-2109917	On high accuracy aspects	Ericsson	discussion	Rel-17

R2-2111089	Discussion on incoming LSs from RAN1 on positioning	vivo	discussion	Rel-17	NR_pos_enh-Core