3GPP TSG-RAN WG2 Meeting #116-e	R2-211xxxx

Electronic, 1st – 12th Nov, 2021

Agenda item:	8.7.1

Source: 	OPPO

Title: 	Summary of [621]

Document for:	Discussion and decision

# Introduction

This is for the discussion of the following:

| Company | Name | E-mail |
| --- | --- | --- |
| Huawei, HiSilicon | Yulong Shi | shiyulong5@huawei.com |
| CATT | Hao Xu | xuhao@catt.cn |
| Ericsson | Wang Min | min.w.wang@ericsson.com |
| Nokia | Jakob Buthler | Jakob.buthler@nokia.com |
| Samsung | Milos Tesanovic | m.tesanovic@samsung.com |
| Philips | Jesus Gonzalez Tejeria | jesus.gonzalez.tejeria@philips.com |

# Phase-1 Discussion

Based on the submitted Skeleton,

R2-2109400	Running CR for TS 38.351	OPPO	draft TS	Rel-17	38.351	0.0.0	NR_SL_relay-Core

There is one issue being discussed online, i.e., the naming of TS.

Rapp understand the online output includes the following points:

- “Relay” needs to be included in the name

- “L(ayer)” does not needs to be included in the abbreviation

- Some companies claims that “Sidelink” is somehow mis-leading since the adaptation layer includes Uu hop as well

Rapp would like to suggest to do have a Q in this thread on the following two options which address all the concern(s) above

- Option-1: Relay Adaptation Protocol (RAP)

- Option-2: UE Relay adaption protocol (URAP)

Q0: Which name you prefer?

- Option-1: Relay Adaptation Protocol (RAP)

- Option-2: UE Relay adaption protocol (URAP)

- Option-3: Sidelink Relay Adaptation Protocol (SRAP)

- Others

| Company | Option | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | Option-3 | If the common part among options is RAP, we can discuss the difference by using “UE” or “Sidelink” in front. This spec will cover both U2N and U2U in the future. So, “UE relay” cause the confusion that this is only for UE to UE relay. There seems no chance in near release (e.g. R18) to support relay based on other interface other than PC5/sidelink. “Sidelink relay” is just perfect for U2N also even with Uu interface. Please note our SI/WI is called “NR Sidelink relay”, rather than “NR UE relay” or “NR relay”. There seem no reason to argue on the WI code. |
| CATT | Option-3 | Considering there will be different view for the scope of Relay (e.g. IAB is one kind of relay), we prefer Option-3. |
| OPPO | Option-1 or option-2 | We do not see it would be misleading that “This spec will cover both U2N and U2U in the future. So, “UE relay” cause the confusion that this is only for UE to UE relay.”, since it aims at UE-based relay, but not only for UE-2-UE relay.. We tend to see option-2 is a compromise way-out. |
| Ericsson | Option 3 | Option 3 is better, as Huawei commented, this name is generic to cover both U2N and U2U relay. |
| Nokia | Option 3 | We prefer the generic name, and also including the sidelink notation. |
| Samsung | Option-3 | Same reasoning as CATT and Nokia. |
| Philips | Option-3 | Agree with Nokia |

Rapp summary: there seems a majority view on option-3, suggest to follow majority view.

Q1: Besides naming issue in Q0, please share your additional comment on the skeleton in the following table.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | - Whether the data transmission should be split into DL and UL: Considering we may support U2U relay in future release, where there is no UL and DL concept. No sure “5.2 DL Data transfer” and “5.3 UL Data transfer” have enough future compatibility. Is the intention in the future to add “5.4 U2U data transfer”? Another manner is to split as below without explicit UL and DL: Transmitting operation of U2N Relay UE (incl UL and DL); Receiving operation of U2N Relay UE(incl UL and DL); Transmitting operation of U2N Remote UE(incl UL and DL); Receiving operation of U2N Remote UE(incl UL and DL); No strong view from our side, we can trust the judge of spec editor, as long as future addition of U2U relay is considered. [Rapp] thanks for the comment. The main point is there seem some difference between the operation for Tx of relay (or remote) UE w.r.t UL traffic and DL traffic, so anyway we need to split / differentiation. Regarding how to group them, i.e., based on UL / DL (as in the current draft) or relay / remote UE (as in the suggestion), prefer the former one since it would be more comprehensive for the reading to understand the operation (e.g., for DL, the reader can understand the procedure from top to bottom, instead of having to jump between sections (per-entity) to understand how the whole procedure works – where U2U relay comes in later, this may become even more complicated). Typo on 5.5 => 5.4 is corrected. Agree U2U can be covered later using new section. - Add “[SALP] structure” in 4.2, as also defined in PDCP. Or maybe the “4.2.1 General” is for the same purpose? [Rapp] sure structure view and functional view figure/description is needed finally, yet in which clause to capture, different spec uses different clause, rapp understand it is not a critical point. |
| CATT | - Chapter 5.4 is missing. (Typo?) - [Rapp] Corrected. - We wonder the intention of the last part “Agreement” . Will it be retained in the final version? - [Rapp] No (removed from the “Content” clause) |
| Ericsson | Better to have a chapter on channel mapping, same as in the BAP protocol. [Rapp] The similar content will be captured in the clauses of “Egress RLC channel determination” |
| Nokia | We do see a point to Huaweis idea on the (re)structuring, but also trust the spec editor to make the best decision [Rapp] Thanks, response to Huawei comment is provided above. |
| Samsung | - A subsection on Configuration may be needed (e.g. 4.6 Configuration). In the BAP (IAB) spec (which we note the ‘SRAP’ skeleton already follows very closely), we have such a section. This being said, there is a big difference between BAP and ‘SRAP’ – as F1AP is used in addition to RRC to configure a BAP entity at an IAB node. Here we don’t have that functionality, although configuration (using RRC) could still be captured. - [Rapp] thanks, Added. - Regarding sections 5.2 and 5.3, they currently seem to be focused on operation per node and UL/DL, rather than per entity. If we already agreed to have a PC5 and a Uu Adapt entities at the Relay UE, then it seems to us we should describe operation per entity. Each of these entities will then have a Tx and a Rx function which we can centre our descriptions around. We believe this may be what Huawei are proposing as well. - [Rapp] thanks for the comment, the response to Huawei comment is provided above (the main point is to give more comprehensive description to the readers) - 6.3 is missing the agreed ‘D/C’ field (this version of the skeleton was submitted before the relevant agreement was made). - [Rapp] Added (together with R-bit) - In light of our comment 3) immediately above, we should also introduce 6.1.2 Control PDU sub-section (since we agreed the introduction of a ‘D/C’ field), and keep it blank for now. - [Rapp] my understanding is the need of control PDU is still FFS (D/C field was motivated for backwards compatibility anyway), so maybe no need for control PDU section for now (?) |
| Philips | Agree with Ericsson [Rapp] Thanks, response to Ericsson comment is provided above. |

Rapp summary: response provided as above in [Rapp].

# Phase-2 Discussion

Q2: Based on the output of Phase-1 and updated version of the skeleton, please share your additional comment on the skeleton in the following table.

| Company | Comment |
| --- | --- |

# Conclusion

# Reference

- R2-2109400	Running CR for TS 38.351	OPPO	draft TS	Rel-17	38.351	0.0.0	NR_SL_relay-Core