3GPP TSG RAN WG2#116-e	R2-2109321

e-Meeting, 1st - 12th November, 2021

3GPP TSG RAN WG1 #106-e	R1-2108533

e-Meeting, August 16th – 27th, 2021

Title:	Reply LS on on physical layer aspects of small data transmission

Response to:	R2-2106561

Release:	Rel-17

Work Item:	NR_SmallData_INACTIVE-Core

Source:	RAN1

To:	RAN2

Cc:

Attachments:

# Contact Person:

# Name:	                Xiaohang CHEN

E-mail Address: chenxiaohang@vivo.com

# 1. Overall Description:

RAN1 would like to thank RAN2 for the LS on physical layer aspects of small data transmission in R2-2106561.

Regarding RA-SDT, RAN1 discussed the configuration parameters for the PRACH resource configuration when PRACH occasions are shared between SDT and non-SDT and when PRACH occasions are separately configured for SDT and non-SDT. RAN1 made the following agreements/conclusion for RA-SDT.

Agreement: - For RA-SDT, when PRACH occasions are separate between SDT and non-SDT, PRACH resource configurations/parameters for 4-step RACH and/or 2-step RACH should be re-used as much as possible for 4-step RACH and/or 2-step RACH based SDT, respectively. - Note: It is up to RAN2 discussion on the RO configuration for RA-SDT in separate ROs. - For RA-SDT, when PRACH occasions are shared between SDT and non-SDT, at least following parameters can be configured, including 4-step RACH and/or 2-step RACH based SDT operation. - Number of contention-based preambles for SDT per SSB per valid RO - Note: whether starting position of the preambles for SDT per SSB per valid RO needs to be configured for RA-SDT in shared ROs is up to RAN2 discussion. - For RA-SDT, when PRACH occasions are shared between SDT and non-SDT, a PRACH mask can be configured to indicate a subset of ROs for RA-SDT. - For RA-SDT in shared ROs and separate ROs with non-SDT, the power control parameters follow those for non-SDT, - i.e. preambleReceivedTargetPower and power ramping setting follow those for non-SDT. Conclusion: - Further discuss on the case when ROs are shared between SDT and non-SDT, but different RACH types have separate ROs after RAN2’s decision

Regarding the questions for CG-SDT on the working assumption and agreements, RAN1 made the following agreements on BWP for CG-SDT resource. RAN1 kindly requests RAN2 to provide more information on the necessity of the CG-SDT resource configured on a separate SDT BWP. Regarding the search space and L1 feedback for CG-SDT, RAN1 will continue the discussion.

Agreement: - RAN1 confirms the RAN2 agreement that CG-SDT resource can be configured on initial BWP - FFS whether CG-SDT resource can be configured on a separate BWP.

# 2. Actions:

RAN1 respectfully asks RAN2 to take into account the above information.

# 3. Date of Next RAN1 Meetings:

TSG-RAN WG1 Meeting #106b-e	11th Oct – 19th Oct 2021	e-meeting

TSG-RAN WG1 Meeting #107-e	11th Nov – 19th Nov 2021	e-meeting