3GPP TSG RAN WG2#116-e	R2-2109325

e-Meeting, 1st - 12th November, 2021

3GPP TSG-RAN WG1 Meeting #106-e 	R1-2108631

# e-Meeting, 16th – 27th August 2021

Title:	LS on RAN1 agreements on RAN2-led features for RedCap

Response to:

Release:	Rel-17

Work Item:	Support of reduced capability NR devices (NR_redcap-Core)

Source:	RAN1

To:	RAN2

Cc:

Contact person:	Shinya Kumagai, shinya.kumagai (at) docomo-lab.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN1 discussed RAN1 aspects on RAN2-led features for RedCap and agreed to send RAN2-related agreements/working assumptions/conclusions to RAN2 to facilitate their work on RAN2-led features for RedCap. RAN1 respectfully asks RAN2 to take the agreements/working assumptions/conclusions into account in their further work on RAN2-led features for RedCap and provide feedback, if any.

Followings are the agreements/conclusions on RAN1 aspects on RAN2-led features for RedCap made in RAN1#106-e meeting:

Agreement Confirm the following working assumption with the modifications in red: - For 4-step RACH, support the early indication of RedCap UEs at least in Msg1. - The early indication in Msg1 can be configured to be enabled/disabled via SIB - FFS how to support enable/disable the early indication - FFS details e.g.: From RAN1 perspective, the following methods can be used for early indication both for shared initial UL BWP and separate initial UL BWP (if supported) - separate PRACH resource - PRACH preamble partitioning - FFS: whether/how to address RA-RNTI overlapping issue - FFS the possibility of supporting Msg3 for the early indication Whether/how to support early indication of RedCap UEs in Msg3 in Rel-17 is up to RAN2. Conclusion - Whether there is RA-RNTI overlapping issue and how to address RA-RNTI overlapping issue in the early indication of RedCap UEs in Msg1 in Rel-17 is up to RAN2. Conclusion - There is no consensus in RAN1 on whether to have the access barring indication in DCI scheduling SIB1, and RAN1 can come back if triggered by RAN2. Agreement - For the RedCap UE capabilities, current definition of Rel-15/16 L1 UE capabilities mandatory without capability signalling in TR38.822 is reused by default, unless any update is agreed - Note: UE capabilities related to CA, DC and wider max UE bandwidth are not applicable to RedCap UEs - FFS: whether any L1 UE capabilities mandatory/optional with capability signalling are not applicable to RedCap UEs Agreement - A RedCap UE type from RAN1 point of view supports a maximum bandwidth of 20MHz for FR1 and 100MHz for FR2 - Further discuss whether to capture also one or more of the following capabilities to RedCap UE type description - Supports either 1 or 2 Rx branches and corresponding maximum DL MIMO layers - Supports either FD-FDD or Type A HD-FDD operation for FR1 FDD bands - Supports either DL up to 64 QAM or up to 256 QAM for FR1 - Does not support CA/DC

For reference, RAN1 also agreed following RAN2-related agreement in RAN1#106-e meeting:

Agreement - In case a separate initial UL BWP is configured for RedCap Ues, it is supported that the network can enable/disable intra-slot PUCCH frequency hopping within the separate initial UL BWP in the PUCCH resource for HARQ feedback for Msg4/MsgB for RedCap UEs. - Working assumption: The frequency hopping is enabled/disabled at least via SIB.

# 2	Actions

To RAN2:

ACTION: 	RAN1 respectfully asks RAN2 to take the above into account in their further work on RAN2-led features for RedCap and provide feedback, if any.

# 3	Dates of next TSG-RAN WG1 meetings

TSG-RAN WG1 Meeting #106bis-e 	11th – 19th October 2021	Online

TSG-RAN WG1 Meeting #107-e 	11th – 19th November 2021	Online